Pipe Flanges and Flanged Fittings

NPS ½ Through NPS 24 Metric/Inch Standard

### AN  A MERICAN  NA TIONAL  S T AND ARD

Pipe Flanges and

Flanged Fittings

NPS 1⁄2 Through NPS 24 Metric/Inch Standard

AN AMERICAN NATIONAL STANDARD

Date of Issuance: January 29, 2021

The next edition of this Standard is scheduled for publication in 2024.

ASME issues written replies to inquiries concerning interpretations of technical aspects of this Standard. Periodically certain actions of the ASME B16 Committee may be published as Cases. Cases and interpretations are published on the ASME website under the Committee Pages at http://cstools.asme.org/ as they are issued.

Errata to codes and standards may be posted on the ASME website under the Committee Pages to provide corrections to incorrectly published items, or to correct typographical or grammatical errors in codes and standards. Such errata shall be used on the date posted.

The Committee Pages can be found at http://cstools.asme.org/. There is an option available to automatically receive an e-mail notification when errata are posted to a particular code or standard. This option can be found on the appropriate Committee Page after selecting “Errata” in the “Publication Information” section.

ASME is the registered trademark of The American Society of Mechanical Engineers.

This code or standard was developed under procedures accredited as meeting the criteria for American National Standards. The Standards Committee that approved the code or standard was balanced to assure that individuals from competent and concerned interests have had an opportunity to participate. The proposed code or standard was made available for public review and comment that provides an opportunity for additional public input from industry, academia, regulatory agencies, and the public-at-large.

ASME does not “approve,” “rate,” or “endorse” any item, construction, proprietary device, or activity.

ASME does not take any position with respect to the validity of any patent rights asserted in connection with any items mentioned in this document, and does not undertake to insure anyone utilizing a standard against liability for infringement of any applicable letters patent, nor assume any such liability. Users of a code or standard are expressly advised that determination of the validity of any such patent rights, and the risk of infringement of such rights, is entirely their own responsibility.

Participation by federal agency representative(s) or person(s) affiliated with industry is not to be interpreted as government or industry endorsement of this code or standard.

ASME accepts responsibility for only those interpretations of this document issued in accordance with the established ASME procedures and policies, which precludes the issuance of interpretations by individuals.

No part of this document may be reproduced in any form, in an electronic retrieval system or otherwise,

without the prior written permission of the publisher.

The American Society of Mechanical Engineers Two Park Avenue, New York, NY 10016-5990

Copyright © 2021 by

THE AMERICAN SOCIETY OF MECHANICAL ENGINEERS

All rights reserved Printed in U.S.A.

## CONTENTS

Foreword . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	viii

Committee Roster . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	x

Correspondence With the B16 Committee . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	xi

Summary of Changes . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	xiii

List of Changes in Record Number Order . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	xvi

1	General . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	1

2	Pressure–Temperature Ratings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	2

3	Component Size . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	3

4	Marking . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	4

5	Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	4

6	Dimensions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	8

7	Tolerances . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	13

8	Pressure Testing . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .	14

###### Mandatory  Appendices

| I | Threading of Pipe for American National Standard Threaded Flanges . . . . . . . . . . . . . . . . . . | 189 |
| --- | --- | --- |
| II | References . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 191 |
| Nonmandatory Appendices A Method Used for Establishing Pressure–Temperature Ratings . . . . . . . . . . . . . . . . . . . . . . . . |  | 194 |
| B | Limiting Dimensions of Gaskets Other Than Ring Joint Gaskets . . . . . . . . . . . . . . . . . . . . . . . | 198 |
| C | Method for Calculating Bolt Lengths . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 200 |
| D | Quality System Program . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 202 |
| E | Dimensions of Classes 400, 600, 900, 1500, and 2500 Flanged Fittings in U.S. Customary Units | 203 |
| F | Method Used to Establish SI Unit Values . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 231 |
| Figures |  |  |
| 1 | Bevel for Outside Thickness . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 15 |
| 2 | Bevel for Inside Thickness . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 15 |
| 3 | Bevel for Combined Thickness . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 15 |
| 4 | Straight Hub Welding Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 15 |
| 5 | Method of Designating Outlets of Reducing Fittings in Specifications (Flanged Fittings) . . . . . | 16 |
| 6 | End Flange Facings and Their Relationship to Flange Thickness and Center-to-End and End-to-End Dimensions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 17 |
| 7 | Welding Neck Flange Hub and Welding End Profiles for Wall Thicknesses t From 5 mm (0.19 in.) to 22 mm (0.88 in.) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 19 |
| 8 | Welding Neck Flange Hub and Welding End Profiles for Wall Thicknesses t Greater Than 22 mm (0.88 in.) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 20 |

iii

| 9 | Inside Contour for Use With Rectangular Backing Ring . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 21 |
| --- | --- | --- |
| 10 | Inside Contour for Use With Taper Backing Ring . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 21 |
| 11 | Thread Length for Connection Tapping . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 22 |
| 12 | Socket Welding for Connections . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 22 |
| 13 | Bosses for Connections . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 22 |
| 14 | Butt Welding for Connections . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 22 |
| 15 | Method of Designating Location of Auxiliary Connections When Specified (Flanged Fittings) . | 23 |
| C-1 | Specified Machine Bolt Length . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 200 |
| C-2 | Specified Stud Bolt Length . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 200 |
| Tables 1.1-1 | List of Material Specifications . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 5 |
| 1.1-2 | List of Bolting Specifications: Applicable ASTM Specifications . . . . . . . . . . . . . . . . . . . . . . . . | 9 |
| 1.1-3 | Flange Bolting Dimensional Recommendations . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 12 |
| 2-1.1 | Pressure–Temperature Ratings for Group 1.1 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 24 |
| 2-1.1C | Pressure–Temperature Ratings for Group 1.1 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 25 |
| 2-1.2 | Pressure–Temperature Ratings for Group 1.2 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 26 |
| 2-1.2C | Pressure–Temperature Ratings for Group 1.2 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 27 |
| 2-1.3 | Pressure–Temperature Ratings for Group 1.3 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 28 |
| 2-1.3C | Pressure–Temperature Ratings for Group 1.3 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 29 |
| 2-1.4 | Pressure–Temperature Ratings for Group 1.4 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 30 |
| 2-1.4C | Pressure–Temperature Ratings for Group 1.4 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 31 |
| 2-1.5 | Pressure–Temperature Ratings for Group 1.5 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 32 |
| 2-1.5C | Pressure–Temperature Ratings for Group 1.5 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 33 |
| 2-1.7 | Pressure–Temperature Ratings for Group 1.7 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 34 |
| 2-1.7C | Pressure–Temperature Ratings for Group 1.7 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 35 |
| 2-1.9 | Pressure–Temperature Ratings for Group 1.9 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 36 |
| 2-1.9C | Pressure–Temperature Ratings for Group 1.9 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 37 |
| 2-1.10 | Pressure–Temperature Ratings for Group 1.10 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 38 |
| 2-1.10C | Pressure–Temperature Ratings for Group 1.10 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 39 |
| 2-1.11 | Pressure–Temperature Ratings for Group 1.11 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 40 |
| 2-1.11C | Pressure–Temperature Ratings for Group 1.11 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 41 |
| 2-1.13 | Pressure–Temperature Ratings for Group 1.13 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 42 |
| 2-1.13C | Pressure–Temperature Ratings for Group 1.13 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 43 |
| 2-1.14 | Pressure–Temperature Ratings for Group 1.14 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 44 |
| 2-1.14C | Pressure–Temperature Ratings for Group 1.14 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 45 |
| 2-1.15 | Pressure–Temperature Ratings for Group 1.15 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 46 |
| 2-1.15C | Pressure–Temperature Ratings for Group 1.15 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 47 |
| 2-1.17 | Pressure–Temperature Ratings for Group 1.17 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 48 |
| 2-1.17C | Pressure–Temperature Ratings for Group 1.17 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 49 |
| 2-1.18 | Pressure–Temperature Ratings for Group 1.18 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 50 |
| 2-1.18C | Pressure–Temperature Ratings for Group 1.18 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 51 |
| 2-2.1 | Pressure–Temperature Ratings for Group 2.1 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 52 |
| 2-2.1C | Pressure–Temperature Ratings for Group 2.1 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 53 |
| 2-2.2 | Pressure–Temperature Ratings for Group 2.2 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 54 |

iv

| 2-2.2C | Pressure–Temperature Ratings for Group 2.2 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 55 |
| --- | --- | --- | --- |
| 2-2.3 | Pressure–Temperature Ratings for Group 2.3 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 56 |
| 2-2.3C | Pressure–Temperature Ratings for Group 2.3 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 56 |
| 2-2.4 | Pressure–Temperature Ratings for Group 2.4 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 57 |
| 2-2.4C | Pressure–Temperature Ratings for Group 2.4 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 58 |
| 2-2.5 | Pressure–Temperature Ratings for Group 2.5 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 59 |
| 2-2.5C | Pressure–Temperature Ratings for Group 2.5 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 60 |
| 2-2.6 | Pressure–Temperature Ratings for Group 2.6 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 61 |
| 2-2.6C | Pressure–Temperature Ratings for Group 2.6 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 62 |
| 2-2.7 | Pressure–Temperature Ratings for Group 2.7 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 63 |
| 2-2.7C | Pressure–Temperature Ratings for Group 2.7 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 64 |
| 2-2.8 | Pressure–Temperature Ratings for Group 2.8 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 65 |
| 2-2.8C | Pressure–Temperature Ratings for Group 2.8 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 65 |
| 2-2.9 | Pressure–Temperature Ratings for Group 2.9 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 66 |
| 2-2.9C | Pressure–Temperature Ratings for Group 2.9 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 67 |
| 2-2.10 | Pressure–Temperature Ratings for Group 2.10 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 68 |
| 2-2.10C | Pressure–Temperature Ratings for Group 2.10 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 69 |
| 2-2.11 | Pressure–Temperature Ratings for Group 2.11 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 70 |
| 2-2.11C | Pressure–Temperature Ratings for Group 2.11 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 71 |
| 2-2.12 | Pressure–Temperature Ratings for Group 2.12 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 72 |
| 2-2.12C | Pressure–Temperature Ratings for Group 2.12 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 73 |
| 2-3.1 | Pressure–Temperature Ratings for Group 3.1 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 74 |
| 2-3.1C | Pressure–Temperature Ratings for Group 3.1 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 74 |
| 2-3.2 | Pressure–Temperature Ratings for Group 3.2 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 75 |
| 2-3.2C | Pressure–Temperature Ratings for Group 3.2 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 75 |
| 2-3.3 | Pressure–Temperature Ratings for Group 3.3 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 76 |
| 2-3.3C | Pressure–Temperature Ratings for Group 3.3 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 77 |
| 2-3.4 | Pressure–Temperature Ratings for Group 3.4 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 78 |
| 2-3.4C | Pressure–Temperature Ratings for Group 3.4 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 78 |
| 2-3.5 | Pressure–Temperature Ratings for Group 3.5 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 79 |
| 2-3.5C | Pressure–Temperature Ratings for Group 3.5 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 80 |
| 2-3.6 | Pressure–Temperature Ratings for Group 3.6 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 81 |
| 2-3.6C | Pressure–Temperature Ratings for Group 3.6 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 82 |
| 2-3.7 | Pressure–Temperature Ratings for Group 3.7 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 83 |
| 2-3.7C | Pressure–Temperature Ratings for Group 3.7 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 83 |
| 2-3.8 | Pressure–Temperature Ratings for Group 3.8 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 84 |
| 2-3.8C | Pressure–Temperature Ratings for Group 3.8 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 85 |
| 2-3.9 | Pressure–Temperature Ratings for Group 3.9 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 86 |
| 2-3.9C | Pressure–Temperature Ratings for Group 3.9 Materials | . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 87 |
| 2-3.10 | Pressure–Temperature Ratings for Group 3.10 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 88 |
| 2-3.10C | Pressure–Temperature Ratings for Group 3.10 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 88 |
| 2-3.11 | Pressure–Temperature Ratings for Group 3.11 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 89 |
| 2-3.11C | Pressure–Temperature Ratings for Group 3.11 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 89 |
| 2-3.12 | Pressure–Temperature Ratings for Group 3.12 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 90 |
| 2-3.12C | Pressure–Temperature Ratings for Group 3.12 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . |  | 90 |

v

| 2-3.13 | Pressure–Temperature Ratings for Group 3.13 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 91 |
| --- | --- | --- |
| 2-3.13C | Pressure–Temperature Ratings for Group 3.13 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 91 |
| 2-3.14 | Pressure–Temperature Ratings for Group 3.14 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 92 |
| 2-3.14C | Pressure–Temperature Ratings for Group 3.14 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 93 |
| 2-3.15 | Pressure–Temperature Ratings for Group 3.15 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 94 |
| 2-3.15C | Pressure–Temperature Ratings for Group 3.15 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 95 |
| 2-3.16 | Pressure–Temperature Ratings for Group 3.16 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 96 |
| 2-3.16C | Pressure–Temperature Ratings for Group 3.16 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 97 |
| 2-3.17 | Pressure–Temperature Ratings for Group 3.17 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 98 |
| 2-3.17C | Pressure–Temperature Ratings for Group 3.17 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 98 |
| 2-3.19 | Pressure–Temperature Ratings for Group 3.19 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 99 |
| 2-3.19C | Pressure–Temperature Ratings for Group 3.19 Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . | 100 |
| 3 | Permissible Imperfections in Flange Facing Finish for Raised Face and Large Male and Female Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 101 |
| 3C | Permissible Imperfections in Flange Facing Finish for Raised Face and Large Male and Female Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 102 |
| 4 | Dimensions of Facings (Other Than Ring Joints, All Pressure Rating Classes) . . . . . . . . . . . . . | 103 |
| 4C | Dimensions of Facings (Other Than Ring Joints, All Pressure Rating Classes) . . . . . . . . . . . . . | 105 |
| 5 | Dimensions of Ring Joint Facings (All Pressure Rating Classes) . . . . . . . . . . . . . . . . . . . . . . . | 108 |
| 5C | Dimensions of Ring Joint Facings (All Pressure Rating Classes) . . . . . . . . . . . . . . . . . . . . . . . | 116 |
| 6 | Reducing Threaded and Slip-On Pipe Flanges for Classes 150 Through 2500 Pipe Flanges . . . | 123 |
| 6C | Reducing Threaded and Slip-On Pipe Flanges for Classes 150 Through 2500 Pipe Flanges . . . | 124 |
| 7 | Templates for Drilling Class 150 Pipe Flanges and Flanged Fittings . . . . . . . . . . . . . . . . . . . | 125 |
| 7C | Templates for Drilling Class 150 Pipe Flanges and Flanged Fittings . . . . . . . . . . . . . . . . . . . | 126 |
| 8 | Dimensions of Class 150 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 127 |
| 8C | Dimensions of Class 150 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 130 |
| 9 | Dimensions of Class 150 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 134 |
| 9C | Dimensions of Class 150 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 140 |
| 10 | Templates for Drilling Class 300 Pipe Flanges and Flanged Fittings . . . . . . . . . . . . . . . . . . . | 143 |
| 10C | Templates for Drilling Class 300 Pipe Flanges and Flanged Fittings . . . . . . . . . . . . . . . . . . . | 144 |
| 11 | Dimensions of Class 300 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 145 |
| 11C | Dimensions of Class 300 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 148 |
| 12 | Dimensions of Class 300 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 152 |
| 12C | Dimensions of Class 300 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 156 |
| 13 | Templates for Drilling Class 400 Pipe Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 159 |
| 13C | Templates for Drilling Class 400 Pipe Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 160 |
| 14 | Dimensions of Class 400 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 161 |
| 14C | Dimensions of Class 400 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 163 |
| 15 | Templates for Drilling Class 600 Pipe Flanges and Flanged Fittings . . . . . . . . . . . . . . . . . . . | 165 |
| 15C | Templates for Drilling Class 600 Pipe Flanges and Flanged Fittings . . . . . . . . . . . . . . . . . . . | 166 |
| 16 | Dimensions of Class 600 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 167 |
| 16C | Dimensions of Class 600 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 169 |
| 17 | Templates for Drilling Class 900 Pipe Flanges and Flanged Fittings . . . . . . . . . . . . . . . . . . . | 171 |
| 17C | Templates for Drilling Class 900 Pipe Flanges and Flanged Fittings . . . . . . . . . . . . . . . . . . . | 172 |
| 18 | Dimensions of Class 900 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 173 |
| 18C | Dimensions of Class 900 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 175 |

vi

| 19 | Templates for Drilling Class 1500 Pipe Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 177 |
| --- | --- | --- |
| 19C | Templates for Drilling Class 1500 Pipe Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 178 |
| 20 | Dimensions of Class 1500 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 179 |
| 20C | Dimensions of Class 1500 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 181 |
| 21 | Templates for Drilling Class 2500 Pipe Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 183 |
| 21C | Templates for Drilling Class 2500 Pipe Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 184 |
| 22 | Dimensions of Class 2500 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 185 |
| 22C | Dimensions of Class 2500 Flanges . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 187 |
| I-1 | Projection of Threaded Pipe End Through a Ring Gage . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 190 |
| A-1 | Rating Ceiling Pressure — pc, bar . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 196 |
| A-2 | Rating Ceiling Pressure — pc, psi . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 197 |
| B-1 | Gasket Groups and Typical Materials . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 199 |
| C-1 | Thickness for Lapped Joints . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 201 |
| C-2 | F Values . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 201 |
| C-3 | n Values . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 201 |
| E-1 | Dimensions of Class 400 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 204 |
| E-2 | Dimensions of Class 600 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 210 |
| E-3 | Dimensions of Class 900 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 216 |
| E-4 | Dimensions of Class 1500 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 222 |
| E-5 | Dimensions of Class 2500 Flanged Fittings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 228 |

vii

## FOREWORD

In 1920, the American Engineering Standards Committee [later the American Standards Associations (ASA) and currently the American National Standards Institute (ANSI)] organized Sectional Committee B16 to unify and further develop standards for pipe flanges and fittings (and later for valves and gaskets). Cosponsors of the B16 Committee were The American Society of Mechanical Engineers (ASME), the Heating and Piping Contractors National Association [now Mechanical Contractors Association of America (MCAA)], and the Manufacturers Standardization Society of the Valves and Fittings Industry (MSS). Cosponsors were later designated as cosecretariat organizations. The Committee soon recognized the need for standardization of steel pipe flanges. In May 1923, Subcommittee 3 was organized to develop such standards for pressures in the 250-psi to 3,200-psi range and for elevated temperatures. Active work began in October, including on steel flanged fittings. The first proposed standard was submitted to the Committee in April 1926 and approved by letter ballot in December. After favorable review by the three sponsor organizations, the

Standard was approved as American Tentative Standard B16e in June 1927.

Experience in using the Standard showed the need for hub dimensions of companion flanges and for other changes, including rerating of 250-lb and 1,350-lb flanges and development of flanged fittings with integral bases. An investigation was made into the factors determining stiffness of flanges and flange hubs. The revised edition was approved as ASA B16e- 1932.

A revision was initiated in 1936, stimulated by suggestions from Committee members and industrial users. The resulting 1939 edition contained standards for welding neck flanges (completed in March 1937), 1,500-lb flanges in the 14-in. through 24-in. range, 2,500-lb flanges and flanged fittings in the 1∕2-in. through 12-in. range, and dimensions for a full line of ring joint flanges developed by the American Petroleum Institute. Pressure–temperature ratings for alloy steel flanges and fittings, developed by Subcommittee 4, were included for the first time.

In August 1942, the War Production Board requested a review of measures to conserve vital materials in piping components. A special War Committee of B16 was appointed and, operating under War Standard Procedure, developed revised pressure–temperature ratings for all materials and pressure classes. The ratings were published as American War Standard B16e5-1943. In 1945, under normal procedures, Subcommittees 3 and 4 reviewed the 1939 standard and 1943 ratings and recommended adoption of the wartime ratings. Their report was approved as Supplement No. 1 to B16e-1939 and published as ASA B16e6-1949. In addition to ratings, the supplement updated material specification references and added a table of metal wall thickness for welding-end valves.

Subcommittee 3 then began a revision of the entire standard. Technically, the 1949 Supplement was absorbed, new materials were recognized, a general rating method was developed and added as an appendix, and welding end prepara- tions were expanded. Editorially, a new style of presentation was worked out, including tables rearranged for easier use. Approval by Sectional Committee, cosponsors, and ASA resulted in the publication of ASA B16.5-1953 (designation changed from B16e).

Work soon began on further revisions. Class B ratings were deleted, and Class A ratings were clarified as the standard. An appendix defined qualifications for gaskets, other than ring joint, which would merit the ratings. Another appendix defined the method for calculating bolt lengths, including the measurement of stud bolt length between thread ends instead of points. Pressure–temperature ratings for several new materials were added, the table of welding end dimen- sions was expanded, and the temperatures used in determining ratings were redefined. The resulting new edition, after approval, was published as ASA B16.5-1957.

The more modest revision approved as ASA B16.5-1961 changed the text to clarify the intent or to make requirements easier to administer. The next revision began in 1963 with nearly 100 comments and suggestions. No fundamental changes were made, but the text was further clarified, and wall thicknesses less than 1∕4 in. for flanged fittings were recognized in the 1968 edition.

A new joint study of ratings between Subcommittees 3 and 4 was initiated before the next revision. Based on the Subcommittee 4 report, the rating procedure was revised, and a rating basis for Class 150 (150-lb) flanges was developed. New product forms, bar and plate, were added for special applications, including fabricated flanged valves and fittings. Reference to welding-end valves was not included, because a separate standard for them was planned. Bolt length calculations based on worst case tolerances led to a revision of tabulated lengths. Testing of valves subsequently published by Subcommittee 15 closure members was added to the test requirements. Following final approval on October 23, the Standard was published as ANSI B16.5-1973.

viii

Subcommittee N (formerly 15) was assigned responsibility for all valve standards in late 1973. Subcommittee C (formerly 3) continued to have responsibility for flange standards. A revision was accordingly initiated to remove all references to valves. At the same time, comments from users and changes in the ASME Boiler and Pressure Vessel Code (BPVC) led to significant revisions in the Class 150 rating basis and in the ratings of stainless steel and certain alloy steel flanges and flanged fittings in all rating classes. Extensive public review comments led to the addition of considerations for bolting and gaskets for flanged joints and of marking requirements. To avoid having to make frequent and confusing changes to the ratings as a result of further changes to the BPVC-allowable stresses, Subcommittees C and N agreed that ratings would be left alone unless the relevant BPVC stress values were changed by more than 10%. After final approval by the Standards Committee, cosponsors, and ANSI, ANSI B16.5-1977, Steel Pipe Flanges and Flanged Fittings, was published on June 16, 1977.

In 1979, work began on another new edition. Materials coverage was expanded by the addition of nickel and nickel alloys. Bolting rules were revised to cover nickel alloy bolts. Bolt hole and bolting were changed to provide interchange- ability between inch and metric dimensions. Metric dimensional tables were made informational rather than alternative requirements of the Standard. Final approval was granted for ANSI B16.5-1984, Pipe Flanges and Flanged Fittings, on August 14.

In 1982, American National Standards Committee B16 was reorganized as an ASME Committee operating under procedures accredited by ANSI. The 1988 edition of the Standard extended nickel alloy ratings to higher temperatures, clarifying flat face flange requirements, and included other minor revisions. The Committee determined that any metric standard for flanges would stand alone, with metric bolting and gaskets; hence, metric equivalents were deleted. Following approval by the Standards Committee and ASME, approval as an American National Standard was given by ANSI on April 7, 1988, with the new designation ASME/ANSI B16.5-1988.

The 1996 edition allowed flanges marked with more than one material grade or specification, revised flange facing finish requirements, revised pressure–temperature ratings for several material groups, added a nonmandatory quality system annex, and included several other revisions. The 1996 edition was approved by ANSI on October 3, 1996, with the new designation ASME B16.5-1996.

The 2003 edition included metric units as the primary reference units while maintaining U.S. Customary units in either parenthetical or separate forms. New materials were added, some materials were shifted from one group to another, and new material groups were established.

All pressure–temperature ratings were recalculated using data from the latest edition of ASME BPVC, Section II, Part D. Annex F was added to cover pressure–temperature ratings and dimensional data for Classes 150 through 2500 flanges and Classes 150 and 300 flanged fittings in U.S. Customary units. Table and figure numbers in Annex F were prefixed by the letter F and corresponded to table and figure numbers in the main text for the metric version, with the exception of some table and figure numbers that were not used in Annex F. Of note, the flange thickness designations for Classes 150 and 300 were revised with reference to their raised faces. For these classes, the flange thickness dimensional reference planes were altered; however, required flange thickness remained unchanged. The minimum flange thickness designation was changed from C to tf, and it did not include 2.0 mm (0.06 in.) raised face for Classes 150 and 300 raised face flanges and flanged fittings. Because of diminished interest, flanged end fittings conforming to ASME Class 400 and higher were listed only with U.S. Customary units in Annex G. In addition, straight hub welding flanges were incorporated as a new set of flanges in Classes 150 through 2500. Also, there were numerous requirement clarifications and editorial revisions. The 2003 edition was approved by ANSI on July 9, 2003, with the designation ASME B16.5-2003.

The 2009 edition added new materials, updated some pressure–temperature ratings, and designated the annexes as mandatory and nonmandatory appendices. The mandatory appendices were numbered using roman numerals, and the nonmandatory appendices were referenced using capital letters. The 2009 edition was approved by ANSI on February 19, 2009, with the designation ASME B16.5-2009.

The 2013 edition included a revision to para. 5.1 and the addition of perpendicularity tolerances. Additional errata and clarifying revisions were also applied. Following approval of the Standards Committee and ASME, approval for the 2013 edition was granted by ANSI on February 5, 2013.

The 2017 edition included the use of bar stock for blind flanges without hubs; added requirements for forgings, size NPS 22; and updated materials and working pressures.

In the 2020 edition, the U.S. Customary tables in former Mandatory Appendix II have been relocated to the main text and redesignated with a “C” suffix (e.g., Table II-2-1.1 is now Table 2-1.1C) and U.S. Customary figures have been merged with SI figures. Former Mandatory Appendix II has been deleted and the subsequent Mandatory Appendix has been rede- signated. Cross-references have been updated accordingly.

Following approval by the ASME B16 Standards Committee, this revision to the 2017 edition was approved by ANSI as an American National Standard on January 5, 2021, with the new designation ASME B16.5-2020.

ix

ASME B16 COMMITTEE

## Standardization of Valves, Flanges, Fittings, and Gaskets

(The following is the roster of the Committee at the time of approval of this Standard.)

STANDARDS COMMITTEE OFFICERS

C. E. Davila, Chair

R. M. Bojarczuk, Vice Chair

C. Ramcharran, Secretary

STANDARDS COMMITTEE PERSONNEL

A. Appleton, Alloy Stainless Products Co., Inc.

J. E. Barker, DeZURIK, Inc.

R. W. Barnes, ANRIC Enterprises, Inc.

D. C. Bayreuther, Metso Automation, Flow Control Division

W. B. Bedesem, Consultant

R. M. Bojarczuk, ExxonMobil Research and Engineering Co.

A. M. Cheta, Qatar Shell GTL

M. A. Clark, Retired

G. A. Cuccio, Capitol Manufacturing Co.

J. D’Avanzo, Fluoroseal Valves

C. E. Davila, Crane ChemPharma and Energy

K. S. Felder, Valero Energy

D. R. Frikken, Becht Engineering Co., Inc.

J. R. Holstrom, Val-Matic Valve and Manufacturing Corp.

D. Hunt, Jr., Fastenal

G. A. Jolly, Samshin, Ltd.

E. J. Lain, Exelon Nuclear

T. A. McMahon, Emerson Process Management

R. C. Merrick, Fluor Enterprises

W. H. Patrick, The Dow Chemical Co.

D. W. Rahoi, CCM 2000

- Ramcharran, The American Society of Mechanical Engineers

- F. Reid, VSP Technologies

R. A. Schmidt, Canadoil

J. Sekerak, CSA Group

J. P. Tucker, Flowserve

F. Volgstadt, Volgstadt and Associates, Inc.

F. Feng, Delegate, China Productivity Center for Machinery

J. D. Grant, Alternate, DeZURIK APCO HILTON

P. V. Craig, Contributing Member, Jomar Group

B. G. Fabian, Contributing Member, Pennsylvania Machine Works

A. G. Kireta, Jr., Contributing Member, Copper Development Associa- tion, Inc.

SUBCOMMITTEE C — STEEL FLANGES AND FLANGED FITTINGS

A. M. Cheta, Chair, Qatar Shell GTL

A. Carrion, Secretary, The American Society of Mechanical Engineers

A. Appleton, Alloy Stainless Products Co., Inc.

W. B. Bedesem, Consultant

C. E. Davila, Crane ChemPharma and Energy

B. Dennis, Kerkau Manufacturing

J. G. Dominguez, Welding Outlets, Inc.

J. P. Ellenberger, Retired

- R. Frikken, Becht Engineering Co., Inc.

- Gulgun,  International Standard Valve, Inc.

G. B. Hailegiorgis, AFGlobal

M. M. Haynie, Bechtel

J. R. Holstrom, Val-Matic Valve and Manufacturing Corp.

G. A. Jolly, Samshin, Ltd.

R. C. Merrick, Fluor Enterprises

W. H. Patrick, The Dow Chemical Co.

D. W. Rahoi, CCM 2000

T. V. Ramakrishnan, Forged Components, Inc.

R. A. Schmidt, Canadoil

D. E. Tezzo, Emerson Process Management

J. A. Tubb, JCM Industries, Inc.

J. P. Tucker, Flowserve

G. T. Walden, Ferguson

M. M. Zaidi, Worley

x

## CORRESPONDENCE WITH THE B16 COMMITTEE

General. ASME Standards are developed and maintained with the intent to represent the consensus of concerned interests. As such, users of this Standard may interact with the Committee by requesting interpretations, proposing revisions or a case, and attending Committee meetings. Correspondence should be addressed to:

Secretary, B16 Standards Committee

The American Society of Mechanical Engineers Two Park Avenue

New York, NY 10016-5990

http://go.asme.org/Inquiry

Proposing Revisions. Revisions are made periodically to the Standard to incorporate changes that appear necessary or desirable, as demonstrated by the experience gained from the application of the Standard. Approved revisions will be published periodically.

The Committee welcomes proposals for revisions to this Standard. Such proposals should be as specific as possible,

citing the paragraph number(s), the proposed wording, and a detailed description of the reasons for the proposal, including any pertinent documentation.

Proposing a Case. Cases may be issued to provide alternative rules when justified, to permit early implementation of an approved revision when the need is urgent, or to provide rules not covered by existing provisions. Cases are effective immediately upon ASME approval and shall be posted on the ASME Committee web page.

Requests for Cases shall provide a Statement of Need and Background Information. The request should identify the

Standard and the paragraph, figure, or table number(s), and be written as a Question and Reply in the same format as existing Cases. Requests for Cases should also indicate the applicable edition(s) of the Standard to which the proposed Case applies.

Interpretations. Upon request, the B16 Standards Committee will render an interpretation of any requirement of the Standard. Interpretations can only be rendered in response to a written request sent to the Secretary of the B16 Standards Committee.

Requests for interpretation should preferably be submitted through the online Interpretation Submittal Form. The

form is accessible at http://go.asme.org/InterpretationRequest. Upon submittal of the form, the Inquirer will receive an automatic e-mail confirming receipt.

If the Inquirer is unable to use the online form, he/she may e-mail the request to the Secretary of the B16 Standards Committee at SecretaryB16@asme.org, or mail it to the above address. The request for an interpretation should be clear and unambiguous. It is further recommended that the Inquirer submit his/her request in the following format:

Subject:                   Cite the applicable paragraph number(s) and the topic of the inquiry in one or two words. Edition:                Cite the applicable edition of the Standard for which the interpretation is being requested. Question:                           Phrase the question as a request for an interpretation of a specific requirement suitable for

general understanding and use, not as a request for an approval of a proprietary design or

situation. Please provide a condensed and precise question, composed in such a way that a “yes” or “no” reply is acceptable.

Proposed Reply(ies): Provide a proposed reply(ies) in the form of “Yes” or “No,” with explanation as needed. If entering replies to more than one question, please number the questions and replies.

Background Information: Provide the Committee with any background information that will assist the Committee in understanding the inquiry. The Inquirer may also include any plans or drawings that are necessary to explain the question; however, they should not contain proprietary names or information.

xi

Requests that are not in the format described above will be rewritten in the appropriate format by the Committee prior to being answered, which may inadvertently change the intent of the original request.

Moreover, ASME does not act as a consultant for specific engineering problems or for the general application or understanding of the Standard requirements. If, based on the inquiry information submitted, it is the opinion of the Committee that the inquirer should seek assistance, the inquiry will be returned with the recommendation that such assistance be obtained.

ASME procedures provide for reconsideration of any interpretation when or if additional information that might affect an interpretation is available. Further, persons aggrieved by an interpretation may appeal to the cognizant ASME Committee or Subcommittee. ASME does not “approve,” “certify,” “rate,” or “endorse” any item, construction, proprietary device, or activity.

Attending Committee Meetings. The B16 Standards Committee regularly holds meetings and/or telephone confer- ences that are open to the public. Persons wishing to attend any meeting and/or telephone conference should contact the Secretary of the B16 Standards Committee.

xii

## ASME  B16.5-2020 SUMMARY OF CHANGES

Following approval by the ASME B16 Standards Committee and ASME, and after public review, ASME B16.5-2020 was approved by the American National Standards Institute on January 5, 2021.

In ASME B16.5-2020, the U.S. Customary tables formerly in Mandatory Appendix II have been relocated to the main text and redesignated with a "C" suffix, and U.S. Customary figures have been merged with SI figures. Former Mandatory Appendix II has been deleted, and the subsequent appendix redesignated. In addition, this edition includes the following changes identified by a margin note, (20). The Record Numbers listed below are explained in more detail in the "List of Changes in Record Number Order" following this Summary of Changes.

| Page 1 | Location Section 1 | Change Title revised (18-808) |
| --- | --- | --- |
| 1 | 1.1 | Title and subpara. (b)(2) revised (18-808) |
| 1 | 1.6 | Revised (18-2734) |
| 2 | 1.10 | Added (18-808) |
| 3 | 2.8.3 | Revised (11-539) |
| 4 | 4.2.2 | Subparagraph (e) added (19-2978) |
| 4 | 4.2.9 | Added (18-808) |
| 5 | Table 1.1-1 | - Redesignated from Table 1A - Revised A182 F91 to A182 F91 Type 1 (20-887) - Added A182 Gr. N08020 and A240 Gr. N08020 to Material |

Group 3.1, A182 Gr. N08800 and A240 Gr. N08800 to Material

Group 3.6, and A182 Gr. N08810 and A240 Gr. N08810 to Material Group 3.15 (11–540)

| 7 | 5.3.4 | (4) Updated A479 Gr. N08904 to A182 Gr. N08904 (18-2777) Revised (14-2169) |
| --- | --- | --- |
| 10 | 6.4.1 | Revised (19-520) |
| 10 | 6.4.3.2 | Revised (19-520) |
| 11 | 6.7 | Revised in its entirety (19-918) |
| 11 | 6.9.1 | Revised (19-1820) |
| 13 | 7.2 | Subparagraphs (a) through (d) revised (19-520) |
| 13 | 7.3 | Subparagraphs (b), (c), and (e) revised (19-520) |
| 13 | 7.4 | Revised (19-520) |
| 13 | 7.5 | Paragraphs 7.5.1 through 7.5.3 revised (19-520) |
| 14 | 7.6 | Revised (19-520) |
| 14 | 7.7 | Paragraphs 7.7.1 and 7.7.2 revised (19-520) |
| 17 | Figure 6 | Revised (19-520) |
| 19 | Figure 7 | - Title revised (19-918) - Figure revised, General Note (1) deleted, General Note (d) added, and subsequent notes redesignated (14-2172) |
| 20 | Figure 8 | - Title and figure revised (19-918) - General Notes added |
| 22 | Figure 11 | Revised (19-520) |

xiii

| Page 22 | Location Figure 13 | Change Revised (19-520) |
| --- | --- | --- |
| 24 | Table 2-1.1 | Added A350 Gr. LF3, moved A537 CI.1 under C-Mn-Si, and revised Notes 3 and 4 (errata) (19-629) |
| 46 | Table 2-1.15 | - Revised A182 F91 to A182 F91 Type 1 (20-887) - Last three rows revised (20-887) |
| 47 | Table 2-1.15C | - Former Table II-2-1.15, revised A182 F91 to A182 F91 Type 1 (20-887) - Last three rows revised (20-887) |
| 50 | Table 2-1.18 | Note (1) revised |
| 74 | Table 2-3.1 | Revised A182 Gr. N08020 and A240 Gr. N08020 (11-540) |
| 74 | Table 2-3.1C | Former Table II-2-3.1, revised A182 Gr. N08020 and A240 Gr. N08020 (11-540) |
| 81 | Table 2-3.6 | Revised A182 Gr. N08800 and A240 Gr. N08800 (11-540) |
| 82 | Table 2-3.6C | Former Table II-2-3.1, revised A182 Gr. N08800 and A240 Gr. N08800 (11-540) |
| 88 | Table 2-3.10 | Revised (11-540) |
| 88 | Table 2-3.10C | Former Table II-2-3.10, revised (11-540) |
| 89 | Table 2-3.11 | Updated A479 Gr. N08904 to A182 Gr. N08904 (18-2777) |
| 89 | Table 2-3.11C | Former Table II-2-3.11, updated A479 Gr. N08904 to A182 Gr. N08904 (18-2777) |
| 90 | Table 2-3.12 | Revised (11-540) |
| 90 | Table 2-3.12C | Former Table II-2-3.12, revised (11-540) |
| 94 | Table 2-3.15 | Revised A182 Gr. N08810 and A240 Gr. N08810 (11-540) |
| 95 | Table 2-3.15C | Former Table II-2-3.15, revised A182 Gr. N08810 and A240 Gr. N08810 (11-540) |
| 101 | Table 3 | - Third column added (18-578) - Columns 2, 3, and 4 revised (19-520) |
| 102 | Table 3C | - Former Table II-3, third column added (18-578) - Columns 2, 3, and 4 revised (19-520) |
| 103 | Table 4 | Revised (19-520) |
| 108 | Table 5 | (1) Revised metric values (dimensions) to align with U.S. |

Customary values (dimensions) (19-520)

(2) Below Notes, Tolerance for P revised

115	Table 5C	(1) Former Table II-5, for pages 120 and 121, rows rearranged by errata (20-2623)

| 123 | Table 6 | (2) Below table Notes, negative Tolerance for E revised In Example (1), flange dimensions revised (19-520) |
| --- | --- | --- |
| 124 | Table 6C | - In Note (1), “(see Example B)” corrected by errata to “(see Example 2)” (19-520) - In Example (1), flange dimensions revised (19-917) |
| 125 | Table 7 | Revised (19-520) |
| 127 | Table 8 | - Threaded illustration updated (errata) (19-1957) - Table revised (19-520) |
| 130 | Table 8C | - Former Table II-8, threaded illustration updated (errata) (19-1957) - Table revised (19-520) |
| 134 | Table 9 | Revised (19-520) |
| 140 | Table 9C | Former Table II-9, revised (19-1957) |

xiv

| Page 143 | Location Table 10 | Change Revised (19-520) |
| --- | --- | --- |
| 145 | Table 11 | Revised (19-520) |
| 152 | Table 12 | Revised (19-520) |
| 156 | Table 12C | Former Table II-12, revised (19-1957) |
| 159 | Table 13 | Revised (19-520) |
| 161 | Table 14 | Revised (19-520) |
| 165 | Table 15 | Revised (19-520) |
| 167 | Table 16 | Revised (19-520) |
| 171 | Table 17 | Revised (19-520) |
| 173 | Table 18 | Revised (19-520) |
| 177 | Table 19 | Revised (19-520) |
| 179 | Table 20 | Revised (19-520) |
| 183 | Table 21 | Revised (19-520) |
| 185 | Table 22 | Revised (19-520) |
| 191 | Mandatory Appendix II | Former Mandatory Appendix III redesignated and reference list |

updated (20-983)

231	Nonmandatory Appendix F	Added (18-2734)

xv

## LIST OF CHANGES IN RECORD NUMBER ORDER

Record Number	Change

11-539	Revised para. 2.8.3 to clarify “B” dimension.

11-540	Revised Table 1.1-1 (former Table 1A); added materials to Tables 2-3.1, 2-3.6, 2-3.10,

2-3.12, and 2-3.15.

14-2169	Revised reference in para. 5.3.4 to include Nonmandatory Appendix B.

14-2172	Deleted the 1:3 max. slope and the 45-deg max. angle; set a maximum limit for the straight turn; revised values between SI and U.S. Customary units; added a taper (0 deg to 7 deg) for the straight turn; revised General Note (e) for Figures 7 and 8.

18-578	Added a column in Tables 3 and 3C (former Table II-3) to clarify the requirements between maximum depth and maximum radial projection.

18-808	Added paras. 1.10 and 4.2.9 to describe B16 Cases and their uses; revised titles of section 1

and para. 1.1.

18-2734	Added Nonmandatory Appendix F; revised para. 1.6 to reference Nonmandatory Appendix F.

18-2777	Revised Tables 1.1-1 (former Table 1-A), 2-3.11, and 2-3.11C (former Table II-2-3.11), and Mandatory Appendix II (former Mandatory Appendix III).

19-520	Revised paras. 6.4, 6.4.3.2, 7.2, 7.3, 7.4, 7.5, 7.5.2, 7.6, 7.7.1, and 7.7.2; Figures 6, 11, and 13;

and Tables 3 through 22 to align metric dimension with the published U.S. Customary dimension in former Mandatory Appendix II.

19-917	Revised Tables 5C, 6C, 9C, and 12C (former Tables II-5, II-6, II-9, and II-12).

19-918	Revised paras. 6.7, 6.7.1, 6.7.2, and 6.7.3 to clarify text references to Figures 7 and 8; revised titles of Figures 7 and 8.

19-1820	Revised references in para. 6.9.1.

19-2978	Added para. 4.2.2(e) to indicate that when a manufacturer marks a product with the ASTM designation, it is certifying that the material meets all the requirements of the specification marked, including heat treatment.

20-887	Revised Tables 1.1-1 (former Table 1A), 2-1.15, and 2-1.15C (former Table II-2-1.15) to include a new ASTM nomenclature material, A182 F91 Type 1, to replace A182 F91.

20-983	Revised Mandatory Appendix II (former Mandatory Appendix III) to include ASTM Specifications A993, B163, B167, and B407.

xvi

ASME  B16.5-2020

## PIPE FLANGES AND FLANGED FITTINGS

NPS 1⁄2 THROUGH NPS 24 METRIC/INCH STANDARD

ð20Þ ð20Þ

##### GENERAL

  - Scope

    - This Standard covers pressure–temperature ratings, materials, dimensions, tolerances, marking, testing, and methods of designating openings for pipe flanges and flanged fittings. Included are

      - flanges with rating class designations 150, 300,

400, 600, 900, and 1500 in sizes NPS 1∕2 through NPS 24 and flanges with rating class designation 2500 in sizes NPS

1∕2 through NPS 12, with requirements given in both metric

and U.S. Customary units with diameter of bolts and flange

bolt holes expressed in inch units

      - flanged fittings with rating class designation 150 and 300 in sizes NPS 1∕2 through NPS 24, with require- ments given in both metric and U.S. Customary units with diameter of bolts and flange bolt holes expressed in inch units

      - flanged fittings with rating class designation 400, 600, 900, and 1500 in sizes NPS 1∕2 through NPS 24 and flanged fittings with rating class designation 2500 in sizes 1 ∕ 2 through NPS 12 that are acknowledged in Nonmandatory Appendix E in which only U.S. Customary units  are  provided

    - This Standard is limited to

      - flanges and flanged fittings made from cast or forged materials

      - blind flanges and certain reducing flanges made from cast, forged, or plate materials (see Tables 1.1-1 through 1.1-3)

Also included in this Standard are requirements and recommendations regarding flange bolting, gaskets, and joints.

##### References

Codes, standards, and specifications, containing provi- sions to the extent referenced herein, constitute require- ments of this Standard. These reference documents are listed in Mandatory Appendix II.

##### Time of Purchase, Manufacture, or Installation

The pressure–temperature ratings in this Standard are applicable upon its publication to all flanges and flanged fittings within its scope, which otherwise meet its require- ments. For unused flanges or flanged fittings maintained

in inventory, the manufacturer of the flange or flanged fittings may certify conformance to this edition, provided that it can be demonstrated that all requirements of this edition have been met. Where such components were installed in accordance with the pressure–temperature ratings of an earlier edition of this Standard, those ratings are applicable except as may be governed by the applicable code or regulation.

##### User  Accountability

This Standard cites duties and responsibilities that are to be assumed by the flange or flanged fitting user in the areas of, for example, application, installation, system hydrostatic testing, operation, and material selection.

##### Quality Systems

Requirements relating to the product manufacturer’s Quality System Program are described in Nonmandatory Appendix D.

##### Relevant Units                                                     ð20Þ

This Standard states values in both SI (Metric) and U.S. Customary units. As an exception, diameter of bolts and flange bolt holes are expressed in inch units only. These systems of units are to be regarded separately as standard. Within the text, the U.S. Customary units are shown in parentheses or in separate tables following the SI tables. The values stated in each system are not exact equivalents; therefore, it is required that each system of units be used independently of the other. Except for diameter of bolts and flange bolt holes, combining values from the two systems constitutes nonconformance with the Standard. Nonmandatory Appendix F describes how the values expressed in SI units were established.

##### Selection of Materials

Criteria for selection of materials suitable for particular fluid service are not within the scope of this Standard.

##### Convention

For determining conformance with this Standard, the convention for fixing significant digits where limits (maximum and minimum values) are specified shall be as defined in ASTM E29. This requires that an observed

1

ASME  B16.5-2020

ð20Þ

or calculated value be rounded off to the nearest unit in the last right-hand digit used for expressing the limit. Decimal values and tolerances do not imply a particular method of measurement.

##### Denotation

    - Pressure Rating Designation. Class, followed by a dimensionless number, is the designation for pressure– temperature  ratings  as  follows:

Class      150      300      400      600      900      1500      2500

    - Size. NPS, followed by a dimensionless number, is the designation for nominal flange or flange fitting size. NPS is related to the reference nominal diameter, DN, used in international standards. The relationship is, typically, as follows:

NPS	DN

1∕2	15

3∕4	20

1	25

11∕4	32

11∕2	40

2	50

21∕2	65

3	80

4	100

GENERAL NOTE: For NPS ≥ 4, the related DN = 25 multiplied by the NPS number.

##### Cases

ASME issues Cases that are applicable to this Standard by, e.g., adding new materials or alternative construction requirements. The Cases

    - modify the requirements of this Standard.

    - are applicable from their issue dates until the Cases are annulled.

    - may be used only when agreed to by the purchaser

and the manufacturer, and when a Case has been so agreed to, the marking on the flange shall include the Case number.

##### PRESSURE–TEMPERATURE RATINGS

  - General

Pressure–temperature ratings are maximum allowable working gage pressures in bar units at the temperatures in degrees Celsius shown in Tables 2-1.1 through 2-3.19 for the applicable material and class designation. Tables 2-1.1C through 2-3.19C list pressure–temperature ratings using psi units for pressure at the temperature in degrees Fahrenheit. For intermediate temperatures,

linear interpolation is permitted. Interpolation between class designations is not permitted.

##### Flanged Joints

A flanged joint is composed of separate and indepen- dent, although  interrelated, components:  the  flanges, gasket, and bolting, which are assembled by another influ- ence, the assembler. Proper controls must be exercised in the selection and application for all these elements to attain a joint that has acceptable leak tightness. Special techniques, such as controlled bolt tightening, are described in ASME PCC-1.

##### Ratings of Flanged Joints

    - Basis. Pressure–temperature ratings apply to flanged joints that conform to the limitations on bolting in para. 5.3 and on gaskets in para. 5.4, which are made up in accordance with good practice for align- ment and assembly (see para. 2.2). Use of these ratings for flanged joints not conforming to these limitations is the responsibility of the user.

    - Mixed Flanged Joints. If the two flanges in a flanged joint do not have the same pressure–temperature rating, the rating of the joint at any temperature is the lower of the two flange ratings at that temperature.

##### Rating Temperature

The temperature shown for a corresponding pressure rating is the temperature of the pressure–containing shell of the component. In general, this temperature is the same as that of the contained fluid. Use of a pressure rating corresponding to a temperature other than that of the contained fluid is the responsibility of the user, subject to the requirements of applicable codes and regulations. For any temperature below −29°C (−20°F), the rating shall be no greater than the rating shown for −29°C (−20°F) (see also paras. 2.5.3 and 5.1.2).

##### Temperature  Considerations

    - General. Use of flanged joints at either high or low temperatures shall take into consideration the risk of joint leakage due to forces and moments developed in the connected piping or equipment. Provisions in paras. 2.5.2 and 2.5.3 are included as advisory with the aim of lessening these risks.

    - High Temperature. Application at temperatures in the creep range will result in decreasing bolt loads as relaxation of flanges, bolts, and gaskets takes place. Flanged joints subjected to thermal gradients may like- wise be subject to decreasing bolt loads. Decreased bolt loads diminish the capacity of the flanged joint to sustain loads effectively without leakage. At temperatures above 200°C (400°F) for Class 150 and above 400°C (750°F) for other class designations, flanged joints may

2

ASME  B16.5-2020

develop leakage problems unless care is taken to avoid imposing severe external loads, severe thermal gradients, or both.

    - Low Temperature. Some of the materials listed in Tables 1.1-1 and 1.1-2, notably some carbon steels, may undergo a decrease in ductility when used at low tempera- tures to such an extent as to be unable to safely resist shock loading, sudden changes of stress, or high stress concen- tration. Some codes or regulations may require impact testing for applications even where temperatures are higher than −29°C (−20°F). When such requirements apply, it is the responsibility of the user to ensure these requirements are communicated to the manufac- turer prior to the time of purchase.

##### System Hydrostatic Testing

Flanged joints and flanged fittings may be subjected to system hydrostatic tests at a pressure of 1.5 times the 38°C (100°F) rating rounded off to the next higher 1 bar (25 psi) increment. Testing at any higher pressure is the respon- sibility of the user, taking into account the requirements of the applicable code or regulation.

##### Welding Neck Flanges

Ratings for welding neck flanges covered by this Stan- dard are based upon their hubs at the welding end having thickness at least equal to that calculated for pipe having 276 MPa (40,000 psi) specified minimum yield strength.1 In order to ensure adequate flange hub thickness for flange sizes NPS 2 and larger, the bore of a welding neck flange, dimension B in the various dimensional tables, shall not exceed Bmax determined as follows:

applicable code or regulation. See para. 6.7 and Figures 1

through 3 for weld end dimensional requirements.

##### Straight Hub Welding Flanges

    - Hub Dimensions. Straight hub welding flanges have hubs of uniform thickness (see Figure 4). Except as described in paras. 2.8.2 through 2.8.4, the straight hub welding flanges shall have dimensions and tolerances of the welding neck flanges of the same size and class set forth in Tables 8, 11, 14, 16, 18, 20, and 22 (Tables 8C, 11C, 14C, 16C, 18C, 20C, and 22C). In Figure 4 the tolerances described in section 7 are applicable.

    - Length Through Hub. The length through hub shall be 229 mm (9 in.) for NPS 4 and smaller and 305 mm (12 in.) for larger than NPS 4. Other lengths may be furnished by agreement between the end user and manufacturer.

    - Bore. The bore diameter B shall be equal to the B dimension of the welding neck flange. Other bores may be furnished by agreement between the end user and manu- facturer. In no case shall the bore diameter exceed the bore of the same size and class lapped flange.

    - Hub End. The standard flange shall be provided with square cut end. The end user may specify welding end preparation in accordance with para. 6.7.

##### Multiple Material Grades

Material for flanges and flanged fittings may meet the requirements of more than one specification or the re- quirements of more than one grade of a specification listed in Table 1.1-1. In either case, the pressure–tempera-

ture ratings for any of these specifications or grades may

ð20Þ

=	ij

Co pc   yz

be used provided the material is marked in accordance

B	A j1	50,000 z

where

max	hj	z

k	{

with para. 4.2.8.

Ah   = tabulated hub diameter, beginning of chamfer as listed in the dimensional tables

Bmax   =  maximum permissible diameter for the bore of a welding neck flange

Co    =  14.5 when pc   is expressed in bar units or 1.0 when pc   is expressed in psi units

pc   = ceiling pressure value at 38°C (100°F),

Nonmandatory Appendix A, Tables A-1 and A-2

The resultant units for diameter Bmax are the same as those entered for diameter A.

The tabulated ratings for welding neck flanges are inde- pendent of components to which they may be attached, and the pressure rating of the flange shall not be exceeded. Attachment welds should be made in accordance with the

1 For flanges to be attached to high strength pipe with large inside diameters resulting from thin wall sections, see MSS SP-44.

##### COMPONENT SIZE

  - Nominal Pipe Size

As applied in this Standard, the use of the phrase “nominal pipe size” or the designation NPS followed by a dimensionless number is for the purpose of pipe, flange, or flanged fitting end connection size identification. The number is not necessarily the same as the flange or flanged fitting inside diameter.

##### Reducing  Fittings

Reducing fittings shall be designated by the NPS for the openings in the sequence indicated in the sketches of Figure 5.

##### Reducing Flanges

Reducing flanges shall be designated by the NPS for each opening. See examples in Table 6 (Table 6C), Note (4).

3

ASME  B16.5-2020

ð20Þ

##### MARKING

  - General

Except as modified herein, flanges and flanged fittings shall be marked as required in MSS SP-25, except as noted in para. 4.2.

##### Identification Markings

    - Name. The manufacturer’s name or trademark shall be applied.

    - Material. Material shall be identified in the following way:

- Cast flanges and flanged fittings shall be marked with the ASTM specification,2  grade identification

symbol (letters and numbers), and the melt number or melt identification.

- Plate flanges, forged flanges, and flanged fittings

shall be marked with the ASTM specification number and grade identification symbol.2

- A manufacturer may supplement these mandatory material indications with his trade designation for the material grade, but confusion of symbols shall be avoided.

- For flanges and flanged fittings manufactured from

material that meets the requirements of more than one specification or grade of a specification listed in Table 1.1-1, see para. 4.2.8.

- Marking of the ASTM specification number or trade

designation is the manufacturer’s assurance that the material is in full compliance with that specification or specifications when multiple certifications apply.

    - Rating Designation. The flange or flanged fitting shall be marked with the number that corresponds to its pressure rating class designation (i.e., 150, 300, 400, 600,

900, 1500, or 2500).

    - Conformance. The designation B16 or B16.5 shall be applied to the flange or flanged fitting, preferably located adjacent to the class designation, to indicate conformance to this Standard. The use of the prefix ASME is optional.

    - Temperature. Temperature markings are not required on flanges or flanged fittings; however, if marked, the temperature shall be shown with its corre- sponding tabulated pressure rating for the material.

    - Size. The NPS designation shall be marked on flanges and flanged fittings. Reducing flanges and reducing flanged fittings shall be marked with the applicable NPS designations as required by paras. 3.2 and 3.3.

2 An ASME Boiler and Pressure Vessel Code, Section II specification number may be substituted for an ASTM specification number provided the requirements of the ASME specification are identical to or more strin- gent than the ASTM specification for the Grade, Class, or Type of material.

    - Ring Joint Flanges. The edge (periphery) of each ring joint flange shall be marked with the letter R and the corresponding ring groove number.

    - Multiple Material Marking. Material for compo- nents that meet the requirements for more than one spec- ification or grade of a specification listed in Table 1.1-1 may, at the manufacturer’s option, be marked with more than one of the applicable specification or grade symbols. These identification markings shall be placed so as to avoid confusion in identification. The multiple marking shall be in accordance with the guidelines set out in ASME Boiler and Pressure Vessel Code, Section II, Part D, Mandatory Appendix 7.

    - Case Number. When a flange or flanged fitting relies on and meets the requirements of a Case, the Case number shall be included in the markings as “Case ##.”

##### MATERIALS

  - General

    - Materials required for flanges and flanged fittings are listed in Table 1.1-1 with the following restrictions:

      - Plate and flat bar materials may be used only for blind flanges and reducing flanges without hubs.

      - Flanges and flanged fittings shall be manufac-

tured as one piece in accordance with the applicable mate- rial specification. Assembly of multiple pieces into the finished product by welding or other means is not permitted by this Standard.

    - Each forged flange shall be finished from a part that

is brought as nearly as practicable to the finished shape and size by a compressive plastic hot-working operation that consolidates the material to produce an essentially wrought structure, and shall be so processed during the operation as to cause metal flow in the direction most favorable for resisting the stress encountered in service.

    - Recommended bolting materials are  listed  in

Table 1.1-2 (see para. 5.3).

    - Corresponding materials listed in Section II of the ASME Boiler and Pressure Vessel Code may be used provided that the requirements of the ASME specification are identical to or more stringent than the ASTM speci- fication for the Grade, Class, or type of material.

    - Application. Criteria for the selection of materials are not within the scope of this Standard. The possibility of material deterioration in service should be considered by the user. Carbide phase conversion to graphite and exces- sive oxidation of ferritic materials, susceptibility to inter- granular corrosion of austenitic materials, or grain boundary attack of nickel base alloys are among those items requiring attention. A discussion of precautionary considerations can be found in ASME B31.3,

ð20Þ

4

ASME  B16.5-2020

###### Table 1.1-1 List of Material Specifications	ð20Þ

| Material Group No. | Nominal Designation | Pressure– Temperature Rating Table | Applicable ASTM Specifications (1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Forgings | Castings | Plates |

Group 1 Materials

| 1.1 | C–Si C–Mn–Si C–Mn–Si C–Mn–Si–V 31∕2Ni | 2-1.1 2-1.1 2-1.1 2-1.1 2-1.1 | A105 A350 Gr. LF2 … A350 Gr. LF6 CI. 1 A350 Gr. LF3 | A216 Gr. WCB … … … … | A515 Gr. 70 A516 Gr. 70 A537 Cl. 1 … … |
| --- | --- | --- | --- | --- | --- |
| 1.2 | C–Mn–Si C–Mn–Si C–Mn–Si–V 21∕2Ni 31∕2Ni | 2-1.2 2-1.2 2-1.2 2-1.2 2-1.2 | … … A350 Gr. LF6 Cl. 2 … … | A216 Gr. WCC A352 Gr. LCC … A352 Gr. LC2 A352 Gr. LC3 | … … … A203 Gr. B A203 Gr. E |
| 1.3 | C–Si C–Mn–Si 21∕2Ni 31∕2Ni C–1∕2Mo C–1∕2Mo | 2-1.3 2-1.3 2-1.3 2-1.3 2-1.3 2-1.3 | … … … … … … | A352 Gr. LCB … … A217 Gr. WC1 A352 Gr. LC1 | A515 Gr. 65 A516 Gr. 65 A203 Gr. A A203 Gr. D … … |
| 1.4 | C–Si C–Mn–Si | 2-1.4 2-1.4 | … A350 Gr. LF1 Cl. 1 | … … | A515 Gr. 60 A516 Gr. 60 |
| 1.5 | C–1∕2Mo C–1∕2Mo | 2-1.5 2-1.5 | A182 Gr. F1 … | … … | A204 Gr. A A204 Gr. B |
| 1.7 | 1∕2Cr–1∕2Mo Ni–1∕2Cr–1∕2Mo 3∕4Ni–3∕4Cr–1Mo | 2-1.7 2-1.7 2-1.7 | A182 Gr. F2 … … | … A217 Gr. WC4 A217 Gr. WC5 | … … … |
| 1.9 | 11∕4Cr–1∕2Mo 11∕4Cr–1∕2Mo–Si | 2-1.9 2-1.9 | … A182 Gr. F11 CL.2 | A217 Gr. WC6 … | … A387 Gr. 11 Cl. 2 |
| 1.10 | 21∕4Cr–1Mo | 2-1.10 | A182 Gr. F22 Cl. 3 | A 217 Gr. WC9 | A387 Gr. 22 Cl. 2 |
| 1.11 | C–1∕2Mo | 2-1.11 | … | … | A204 Gr. C |
| 1.13 | 5Cr–1∕2Mo | 2-1.13 | A182 Gr. F5a | A217 Gr. C5 | … |
| 1.14 | 9Cr–1Mo | 2-1.14 | A182 Gr. F9 | A217 Gr. C12 | … |
| 1.15 | 9Cr–1Mo–V | 2-1.15 | A182 Gr. F91 Type 1 | A217 Gr. C12A | A387 Gr. 91 Cl. 2 |
| 1.17 | 1Cr–1∕2Mo 5Cr–1∕2Mo | 2-1.17 2-1.17 | A182 Gr. F12 Cl. 2 A182 Gr. F5 | … … | … … |
| 1.18 | 9Cr–2W–V | 2-1.18 | A182 Gr. F92 | … | … |

Group 2 Materials

| 2.1 | 18Cr–8Ni 18Cr–8Ni | 2-2.1 2-2.1 | A182 Gr. F304 A182 Gr. F304H | A351 Gr. CF3 A351 Gr. CF8 | A240 Gr. 304 A240 Gr. 304H |
| --- | --- | --- | --- | --- | --- |
| 2.2 | 16Cr–12Ni–2Mo 16Cr–12Ni–2Mo 18Cr–13Ni–3Mo 19Cr–10Ni–3Mo | 2-2.2 2-2.2 2-2.2 2-2.2 | A182 Gr. F316 A182 Gr. F316H A182 Gr. F317 … | A351 Gr. CF3M A351 Gr. CF8M … A351 Gr. CG8M | A240 Gr. 316 A240 Gr. 316H A240 Gr. 317 … |
| 2.3 | 18Cr–8Ni 16Cr–12Ni–2Mo 18Cr–13Ni–3Mo | 2-2.3 2-2.3 2-2.3 | A182 Gr. F304L A182 Gr. F316L A182 Gr. F317L | … … … | A240 Gr. 304L A240 Gr. 316L … |
| 2.4 | 18Cr–10Ni–Ti 18Cr–10Ni–Ti | 2-2.4 2-2.4 | A182 Gr. F321 A182 Gr. F321H | … … | A240 Gr. 321 A240 Gr. 321H |

5

ASME  B16.5-2020

###### Table 1.1-1 List of Material Specifications (Cont’d)

| Material Group No. | Nominal Designation | Pressure– Temperature Rating Table | Applicable ASTM Specifications (1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Forgings | Castings | Plates |

Group 2 Materials   (Cont’d)

| 2.5 | 18Cr–10Ni–Cb 18Cr–10Ni–Cb 18Cr–10Ni–Cb 18Cr–10Ni–Cb | 2-2.5 2-2.5 2-2.5 2-2.5 | A182 Gr. F347 A182 Gr. F347H A182 Gr. F348 A182 Gr. F348H | … … … … | A240 Gr. 347 A240 Gr. 347H A240 Gr. 348 A240 Gr. 348H |
| --- | --- | --- | --- | --- | --- |
| 2.6 | 23Cr–12Ni | 2-2.6 | … | … | A240 Gr. 309H |
| 2.7 | 25Cr–20Ni | 2-2.7 | A182 Gr. F310 | … | A240 Gr. 310H |
| 2.8 | 20Cr–18Ni–6Mo 22Cr–5Ni–3Mo–N 25Cr–7Ni–4Mo–N 24Cr–10Ni–4Mo–V 25Cr–5Ni–2Mo–3Cu–N 25Cr–7Ni–3.5Mo–W–Cb 25Cr–7.5Ni–3.5Mo–N–Cu–W | 2-2.8 2-2.8 2-2.8 2-2.8 2-2.8 2-2.8 2-2.8 | A182 Gr. F44 A182 Gr. F51 A182 Gr. F53 … … … A182 Gr. F55 | A351 Gr. CK3MCuN … … A995 Gr. CE8MN A995 Gr. CD4MCuN A995 Gr. CD3MWCuN … | A240 Gr. S31254 A240 Gr. S31803 A240 Gr. S32750 … … … A240 Gr. S32760 |
| 2.9 | 23Cr–12Ni 25Cr–12Ni | 2-2.9 2-2.9 | … … | … … | A240 Gr. 309S A240 Gr. 310S |
| 2.10 | 25Cr–12Ni 25Cr–12Ni | 2-2.10 2-2.10 | … … | A351 Gr. CH8 A351 Gr. CH20 | … … |
| 2.11 | 18Cr–10Ni–Cb | 2-2.11 | … | A351 Gr. CF8C | … |
| 2.12 | 25Cr–20Ni | 2-2.12 | … | A351 Gr. CK20 | … |

Group 3 Materials

| 3.1 | 35Ni–35Fe–20Cr–Cb | 2-3.1 | A182 Gr. N08020 | … | A182 Gr. N08020 |
| --- | --- | --- | --- | --- | --- |
| 3.2 | 99.0Ni | 2-3.2 | B564 Gr. N02200 | … | B162 Gr. N02200 |
| 3.3 | 99.0Ni–Low C | 2-3.3 | … | … | B162 Gr. N02201 |
| 3.4 | 67Ni–30Cu | 2-3.4 | B564 Gr. N04400 | … | B127 Gr. N04400 |
| 3.5 | 72Ni–15Cr–8Fe | 2-3.5 | B564 Gr. N06600 | … | B168 Gr. N06600 |
| 3.6 | 33Ni–42Fe–21Cr | 2-3.6 | A182 Gr. N08800 | … | A182 Gr. N08800 |
| 3.7 | 65Ni–28Mo–2Fe 64Ni–29.5Mo–2Cr–2Fe–Mn–W | 2-3.7 2-3.7 | B462 Gr. N10665 B462 Gr. N10675 | … … | B333 Gr. N10665 B333 Gr. N10675 |
| 3.8 | 54Ni–16Mo–15Cr 60Ni–22Cr–9Mo–3.5Cb 62Ni–28Mo–5Fe 70Ni–16Mo–7Cr–5Fe 61Ni–16Mo–16Cr 42Ni–21.5Cr–3Mo–2.3Cu 55Ni–21Cr–13.5Mo 55Ni–23Cr–16Mo–1.6Cu | 2-3.8 2-3.8 2-3.8 2-3.8 2-3.8 2-3.8 2-3.8 2-3.8 | B462 Gr. N10276 B564 Gr. N06625 … … … B564 Gr. N08825 B462 Gr. N06022 B462 Gr. N06200 | … … … … … … … … | B575 Gr. N10276 B443 Gr. N06625 B333 Gr. N10001 B434 Gr. N10003 B575 Gr. N06455 B424 Gr. N08825 B575 Gr. N06022 B575 Gr. N06200 |
| 3.9 | 47Ni–22Cr–9Mo–I8Fe 21Ni–30Fe–22Cr–18Co–3Mo–3W | 2-3.9 2-3.9 | B572 Gr. N06002 B572 Gr. R30556 | … … | B435 Gr. N06002 B435 Gr. R30556 |
| 3.10 | 25Ni–47Fe–21Cr–5Mo | 2-3.10 | … | … | B599 Gr. N08700 |
| 3.11 | 44Fe–25Ni–21Cr–Mo | 2-3.11 | A182 Gr. N08904 | … | A240 Gr. N08904 |
| 3.12 | 26Ni–43Fe–22Cr–5Mo 47Ni–22Cr–20Fe–7Mo 46Fe–24Ni–21Cr–6Mo–Cu–N 46Fe–24Ni–21Cr–6Mo–Cu–N | 2-3.12 2-3.12 2-3.12 2-3.12 | … … A182 Gr. N08367 B462 Gr. N08367 | … … A351 Gr. CN3MN A351 Gr. CN3MN | B620 Gr. N08320 B582 Gr. N06985 B688 Gr. N08367 B688 Gr. N08367 |
| 3.13 | 49Ni–25Cr–18Fe–6Mo Ni–Fe–Cr–Mo–Cu–Low C | 2-3.13 2-3.13 | … B564 Gr. N08031 | … … | B582 Gr. N06975 B625 Gr. N08031 |

6

ASME  B16.5-2020

###### Table 1.1-1 List of Material Specifications (Cont’d)

| Material Group No. | Nominal Designation | Pressure– Temperature Rating Table | Applicable ASTM Specifications (1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Forgings | Castings | Plates |

Group 3 Materials   (Cont’d)

| 3.14 | 47Ni–22Cr–19Fe–6Mo 40Ni–29Cr–15Fe–5Mo 58Ni–33Cr–8Mo | 2-3.14 2-3.14 2-3.14 | … B462 Gr. N06030 B462 Gr. N06035 | … … … | B582 Gr. N06007 B582 Gr. N06030 B575 Gr. N06035 |
| --- | --- | --- | --- | --- | --- |
| 3.15 | 42Ni–42Fe–21Cr 42Ni–42Fe–21Cr | 2-3.15 2-3.15 | A182 Gr. N08810 B564 Gr. N08810 | … … | A240 Gr. N08810 B409 Gr. N08810 |
| 3.16 | 35Ni–19Cr–11∕4Si | 2-3.16 | B511 Gr. N08330 | … | B536 Gr. N08330 |
| 3.17 | 29Ni–20.5Cr–3.5Cu–2.5Mo | 2-3.17 | … | A351 Gr. CN7M | … |
| 3.19 | 57Ni–22Cr–14W–2Mo–La | 2-3.19 | B564 Gr. N06230 | … | B435 Gr. N06230 |

GENERAL NOTES:

- For temperature limitations, see notes in Tables 2-1.1C through 2-3.17C.

- Plate materials are listed only for use as blind flanges and reducing flanges without hubs (see para. 5.1). Additional plate materials listed in ASME B16.34 may also be used with corresponding B16.34, Standard Class ratings.

NOTE: (1) ASME Boiler and Pressure Vessel Code, Section II materials may also be used, provided the requirements of the ASME specification are identical to or more stringent than the corresponding ASTM specification for the Grade, Class, or Type listed.

Nonmandatory Appendix F; Section II, Part D, Nonman- datory Appendix A; and Section III, Division 1, Nonman- datory Appendix W of the ASME Boiler and Pressure Vessel Code.

    - Toughness. Some of the materials listed in Table 1.1-1 undergo a decrease in toughness when used at low temperatures, to the extent that Codes refer- encing the Standard may require impact tests for applica- tion even at temperatures higher than −7°C (+20°F). It is the responsibility of the user to ensure that such testing is performed.

    - Responsibility. When service conditions dictate the implementation of special material require- ments [e.g., using a Group 2 material above 538°C (1,000°F)], it is the user’s responsibility to so specify to the manufacturer in order to ensure compliance with metallurgical requirements  listed  in  the  notes in Tables 2-1.1 through 2-3.19 (Tables 2-1.1C through 2-3.19C).

    - Cast Surfaces. Cast surfaces of component pres- sure boundaries shall be in accordance with MSS SP-55, except that all Type I defects are unacceptable, and defects in excess of Plates “a” and “b” for Type II through Type XII are unacceptable.

##### Mechanical Properties

Mechanical properties shall be obtained from test speci- mens that represent the final heat-treated condition of the material required by the material specification.

##### Bolting

    - General. Bolting listed in Table 1.1-2 is recom- mended for use in flanged joints covered by this Standard. Bolting of other material may be used if permitted by the applicable code or government regulation. Bolting mate- rials are subject to the limitations given in paras. 5.3.2 through 5.3.5.

    - High-Strength Bolting. Bolting materials having allowable stresses not less than those for ASTM A193 Grade B7 are listed as high strength in Table 1.1-2. These and other materials of comparable strength may be used in any flanged joint.

    - Intermediate-Strength Bolting. Bolting mate- rials listed as intermediate strength in Table 1.1-2, and other bolting of comparable strength, may be used in any flanged joint provided the user verifies their ability to seat the selected gasket and maintain a sealed joint under expected operating conditions.

    - Low-Strength Bolting. Bolting materials having no more than 206 MPa (30 ksi) specified minimum yield strength are listed as low strength in Table 1.1-2. These materials and others of comparable strength are to be used only in Class 150 and 300 flanged joints (see para. 5.4.2 and Nonmandatory Appendix B). Flanged assemblies using low-strength carbon steel bolts should not be used above 200°C (400°F) or below

−29°C (−20°F).

    - Bolting to Gray Cast Iron Flanges. The following recommendations are made in recognition of the low ductility of gray cast iron:

ð20Þ

7

ASME  B16.5-2020

- Alignment of flange faces is essential, along with control of assembly bolt torque, so as not to overstress the cast iron flanges. Care must also be exercised to ensure that piping loads transmitted to cast iron

##### 6 DIMENSIONS

6.1 Flanged Fittings Wall Thickness

6.1.1 Minimum Wall Thickness. For inspection

flanges are controlled, taking into account its lack of ducti-

lity and recognizing that cast iron flanges should not be

purposes, the minimum wall thickness, t

m, of flanged

used where suddenly applied loads such as rapid pressure fluctuation may occur.

- Where Class 150 steel flanges are bolted to Class

125 cast iron flanges, the gaskets should be made of Nonmandatory Appendix B, Table B-1, Group No. Ia mate- rials, the steel flanges should have flat faces, and

- low-strength bolting within the limitations of

para. 5.3.4 should be used with ring gaskets extending to the bolt holes or

- bolting of low (para. 5.3.4), intermediate (para.

5.3.3), or high (para. 5.3.2) strength may be used with full face gaskets extending to the outside diameters of the flanges

- Where Class 300 steel flanges are bolted to Class

250 cast iron flanges, the gaskets should be made of Nonmandatory Appendix B, Table B-1, Group No. Ia mate- rials

- low-strength bolting within the limitations of

para. 5.3.4 should be used with gaskets extending to the bolt holes and with the flanges having either raised or flat faces or

- bolting of low (para. 5.3.4), intermediate (para.

5.3.3), or high (para. 5.3.2) strength may be used with full face gaskets extending to the outside diameters of the flanges and with both the Class 300 steel and Class 250 cast iron flanges having flat faces

##### Gaskets

    - General. Ring joint gasket materials shall conform to ASME B16.20. Materials for other gaskets are described in Nonmandatory Appendix B. The user is responsible for selection of gasket materials that will withstand the expected bolt loading without injurious crushing and that are suitable for the service conditions. Particular attention should be given to gasket selection if a system hydrostatic test approaches or exceeds the test pressure specified in para. 2.6.

    - Gaskets for Low-Strength Bolting. If bolting listed as low strength in Table 1.1-2 is used, gaskets shown in Nonmandatory Appendix B, Table B-1, Group No. Ia are recommended.

    - Gaskets for Class 150 Flanged Joints. It is recommended that only Nonmandatory Appendix B, Table B-1, Group No. I gaskets be used for Class 150 flanged joints. When the ring joint or spiral wound gasket is selected, it is recommended that line flanges be of the welding neck or lapped joint type.

fittings at the time of manufacture shall be as shown

in Tables 9 and 12 (Tables 9C and 12C), except as provided in para. 6.1.2. The additional metal thickness needed to withstand installation bolt-up assembly stresses, shapes other than circular, and stress concentrations must be determined by the manufacturer, since these factors vary widely. In particular, 45-deg laterals, true Ys, and crosses may require additional reinforcement to compensate for inherent weaknesses in these shapes.

6.1.2 Fitting Local Areas. Local areas having less than minimum wall thickness are acceptable, provided that all of the following conditions are satisfied:

- The area of subminimum thickness can be enclosed

by a circle whose diameter is no greater than 0.35 dtm, where d is the tabulated fitting inside diameter, and tm is the minimum wall thickness as shown in the tables listed in para. 6.1.1.

- Measured thickness is not less than 0.75tm.

- Enclosure circles are separated from each other by

an edge-to-edge distance of more than 1.75  dtm.

##### Fitting Center-to-Contact Surface and Center- to-End

    - Design. A principle of design in this Standard is to maintain a fixed position for the flange edge with refer- ence to the body of the fitting. In case of raised face flanged fittings, the outside edge of the flange includes the raised face (see para. 6.4).

    - Standard Fittings. Center-to-contact surface, center-to-flange edge, and center-to-end (ring joint) dimensions are shown in Tables 9 and 12 (Tables 9C and 12C).

    - Reducing Fittings. Center-to-contact surface or center-to-flange edge dimensions for all openings shall be the same as those of straight size fittings of the largest opening. The contact surface-to-contact surface or flange edge-to-flange edge dimensions for all combina- tions of reducers and eccentric reducers shall be as listed for the larger opening.

    - Side Outlet Fittings. Side outlet elbows, tees, and crosses shall have all openings on intersecting centerlines, and the center-to-contact surface dimensions of the side outlet shall be the same as for the largest opening. Long radius elbows with one side outlet shall have the side outlet on the radial centerline of the elbow, and the center-to-contact surface dimension of the side outlet shall be the same as for the regular 90-deg elbow of the largest opening.

8

ASME  B16.5-2020

###### Table 1.1-2 List of Bolting Specifications: Applicable ASTM Specifications

| High Strength (2) |  |  | Intermediate Strength |  | (3) | Low Strength (4) |  |  | Nickel and Special Alloy (5) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spec. No. | Grade | Notes | Spec. No. | Grade | Notes | Spec. No. | Grade | Notes | Spec. No. | Grade | Notes |

Bolting Materials (1)

| A193 | B7 | … | A193 | B5 | … | A193 | B8 Cl. 1 | (6) | B164 | … | (7)–(9) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A193 | B16 | … | A193 | B6 | … | A193 | B8C Cl. 1 | (6) | … | … | … |
|  |  | … | A193 | B6X | … | A193 | B8M Cl. 1 | (6) | B166 | … | (7)–(9) |
| A320 | L7 | (10) | A193 | B7M | … | A193 | B8T Cl. 1 | (6) | … | … | … |
| A320 | L7A | (10) |  |  |  |  |  |  |  |  |  |
| A320 | L7B | (10) | A193 | B8 Cl. 2 | (11) | A193 | B8A | (6) | B335 | N10665 | (7) |
|  |  |  |  |  |  |  |  |  |  | N10675 | (7) |
|  |  |  | A193 | B8 Cl. 2B | (11) |  |  |  |  |  |  |
| A320 | L7C | (10) | A193 | B8C Cl. 2 | (11) | A193 | B8CA | … |  |  |  |
| A320 | L43 | (10) | A193 | B8M Cl. 2 | (11) | A193 | B8MA | … | B408 | … | (7)–(9) |
|  |  |  | A193 | B8M Cl. 2B | (11) |  |  |  |  |  |  |
|  |  |  | A193 | B8T Cl. 2 | (11) | A193 | B8TA | (6) |  |  |  |
| A354 | BC | … |  |  |  |  |  |  | B473 | … | (7) |
| A354 | BD | … | A320 | B8 Cl. 2 | (11) | A307 | B | (12) |  |  |  |
|  |  |  | A320 | B8C Cl. 2 | (11) |  |  |  | B574 | … | (7) |
| A540 | B21 | … | A320 | B8F Cl. 2 | (11) | A320 | B8 Cl. 1 | (6) |  |  |  |
| A540 | B22 | … | A320 | B8M Cl. 2 | (11) | A320 | B8C Cl. 1 | (6) |  |  |  |
| A540 | B23 | … | A320 | B8T Cl. 2 | (11) | A320 | B8M Cl. 1 | (6) |  |  |  |
| A540 | B24 | … |  |  |  | A320 | B8T Cl. 1 | (6) |  |  |  |
|  |  |  | A449 |  | (13) |  |  |  |  |  |  |
|  |  |  | A453 | 651 | (14) |  |  |  |  |  |  |
|  |  |  | A453 | 660 | (14) |  |  |  |  |  |  |

GENERAL NOTES:

- Bolting material shall not be used beyond temperature limits specified in the governing code.

- ASME Boiler and Pressure Vessel Code, Section II materials may also be used, provided the requirements of the ASME specification are identical or more stringent than the corresponding ASTM specification for the Grade, Class, or Type listed.

NOTES:

- Repair welding of bolting material is prohibited.

- These bolting materials may be used with all listed materials and gaskets.

- These bolting materials may be used with all listed materials and gaskets, provided it has been verified that a sealed joint can be maintained under rated working pressure and temperature.

- These bolting materials may be used with all listed materials but are limited to Class 150 and Class 300 joints. See para. 5.3.4 for rec- ommended gasket practices.

- These materials may be used as bolting with comparable nickel and special alloy parts.

- This austenitic stainless material has been carbide solution treated but not strain hardened. Use A194 nuts of corresponding material.

- Nuts may be machined from the same material or of a compatible grade of ASTM A194.

- Maximum operating temperature is arbitrarily set at 260°C (500°F), unless the material has been annealed, solution annealed, or hot finished, because hard temper adversely affects design stress in the creep rupture range.

- Forging quality is not permitted unless the producer who heats or works these parts last tests them as required for other permitted conditions in the same specification and certifies their final tensile, yield, and elongation properties to equal or exceed the requirements for one of the other permitted conditions.

- This ferritic material is intended for low temperature service. Use A194 Gr. 4 or Gr. 7 nuts.

- This austenitic stainless material has been carbide solution treated and strain hardened. Use A194 nuts of corresponding material.

- This carbon steel fastener shall not be used above 200°C (400°F) or below −29°C (−20°F) [see also Note (4)]. Bolts with drilled or undersized heads shall not be used.

- Acceptable nuts for use with quenched and tempered bolts are A194 Gr. 2 or Gr. 2H. Mechanical property requirements for studs shall be the same as those for bolts.

- This special alloy is intended for high-temperature service with austenitic stainless steel.

9

ASME  B16.5-2020

ð20Þ

    - Special Degree Elbows. Special degree elbows ranging from 1 deg to 45 deg, inclusive, shall have the same center-to-contact surface dimensions as 45-deg elbows, and those over 45 deg and up to 90 deg, inclusive, shall have the same center-to-contact surface dimensions as 90-deg elbows. The angle designation of an elbow is its deflection from straight line flow and is also the angle between the flange faces.

##### Flat Face Flanges

    - General. This Standard permits flat face flanges in all classes.

    - Conversion. A raised face may be removed from a raised face flange to convert it to a flat face flange, provided that the required dimension, tf, shown in Figure 6 is main- tained.

    - Facing. The flat face flange facing finish shall be in conformance with para. 6.4.5 for the full width of the seating surface for the gasket.

##### Flange Facings

    - General. Figure 6 shows dimensional relation- ships for various flange types and pipe lap facings to be used with lap joints. Table 4 (Table 4C) lists dimensions for facings other than ring joint. Table 5 (Table 5C) lists dimensions for ring joint facings. Classes 150 and 300 pipe flanges and companion flanges of fittings are regularly fur- nished with 1.5 mm (0.06 in.) raised face, which is in addi- tion to the minimum flange thickness, tf. Classes 400, 600, 900, 1500, and 2500 pipe flanges and companion flanges of fittings are regularly furnished with 6.4 mm (0.25 in.) raised face, which is in addition to the minimum flange thickness, tf.

    - Other Than Lapped Joints. For joints other than lapped joints, the requirements of paras. 6.4.2.1 and

6.4.2.2 shall apply.

      - Raised Face and Tongue Face. In the case of flanges having raised face, tongue, or male face, the minimum flange thickness, tf, shall be provided, and then the raised face, tongue, or male face shall be added thereto.

      - Grooves. For flanges that have a ring joint, groove, or female face, the minimum flange thickness shall first be provided and then sufficient thickness added thereto so that the bottom of the ring joint groove, or the contact face of the groove or female face, is in the same plane as the flange edge of a full thick- ness flange.

    - Lapped Joint Flanges. Lapped joint flanges shall be furnished with flat faces as illustrated in Tables 8, 11, 14, 16, 18, 20, and 22 (Tables 8C, 11C, 14C, 16C, 18C, 20C,

and 22C). Lap joint stub ends shall be in accordance with

Figure 6 and paras. 6.4.3.1 through 6.4.3.3.

      - Raised Face. The finished thickness of the lap shall be no less than nominal pipe wall thickness.

      - Large Male and Female. The finished height of the male face shall be the greater of the wall thickness of the pipe used or 6.4 mm (0.25 in.). The thickness of lap that remains after machining the female face shall be no less than the nominal wall thickness of pipe used.

      - Tongue and Groove. The thickness of the lap remaining after machining the tongue or groove face shall be no less than the nominal wall thickness of the pipe used.

      - Ring Joint. The thickness of the lap remaining after machining the ring groove shall be no less than the nominal wall thickness of pipe used.

      - Lap Joint Facing Outside Diameters. The outside diameters of the lap for ring joints are shown in Table 5 (Table 5C), dimension K. The outside diameters of laps for large female, large tongue and groove, and small tongue and groove are shown in Table 4 (Table 4C). Small male and female facings for lapped joints are not covered by this Standard.

    - Blind Flanges. Blind flanges need not be faced in the center if, when this center part is raised, its diameter is at least 25 mm (1 in.) smaller than the inside diameter of fittings of the corresponding pressure class, as given in Tables 9 and 12 (Tables 9C and 12C) or 25 mm (1 in.) smaller than the mating pipe inside diameter. When the center part is depressed, its diameter shall not be greater than the inside diameter of the corresponding pressure class fittings, as given in Tables 9 and 12 (Tables 9C and 12C). Machining of the depressed center is not required.

    - Flange Facing Finish. Flange facing finishes shall be in accordance with paras. 6.4.5.1 through 6.4.5.3, except that other finishes may be furnished by agreement between the user and the manufacturer. The finish of the gasket contact faces shall be judged by visual comparison with Ra standards (see ASME B46.1) and not by instru- ments having stylus tracers and electronic amplification.

      - Tongue and Groove and Small Male and Female. The gasket contact surface finish shall not exceed 3.2 μm (125 μin.) roughness.

      - Ring Joint. The side wall surface finish of the gasket groove shall not exceed 1.6 μm (63 μin.) roughness.

      - Other Flange Facings. Either a serrated concentric or serrated spiral finish having a resultant surface finish from 3.2 μm to 6.3 μm (125 μin. to 250 μin.) average roughness shall be furnished. The cutting tool employed should have an approximate 1.5 mm

(0.06 in.) or larger radius, and there should be from

1.8  grooves/mm  through  2.2  grooves/mm  (45 grooves/in. through 55 grooves/in.).

ð20Þ

10

ASME  B16.5-2020

6.4.6 Flange Facing Finish Imperfections. Imperfec- tions in the flange facing finish shall not exceed the dimen- sions shown in Table 3 (Table 3C). A distance of at least four times the maximum radial projection shall separate adjacent imperfections. A radial projection shall be measured by the difference between an outer radius and inner radius encompassing the imperfection where the radii are struck from the centerline of the bore. Imper- fections less than half the depth of the serrations shall not be considered cause for rejection. Protrusions above the serrations are not permitted.

##### Flange Bolt Holes

Bolt holes are in multiples of four. Bolt holes shall be equally spaced, and pairs of bolt holes shall straddle fitting centerlines.

##### Bolting Bearing Surfaces

Flanges and flanged fittings shall have bearing surfaces for bolting that are parallel to the flange face within 1 deg. Any back facing or spot facing shall not reduce the flange thickness, tf, below the dimensions given in Tables 8, 9, 11, 12, 14, 16, 18, 20, and 22 (Tables 8C, 9C, 11C, 12C, 14C, 16C,

18C, 20C, and 22C). Spot facing or back facing shall be in accordance with MSS SP-9.

6.8.2.2 Welding Neck Flanges. The hub dimensions shall be the same as those of the standard flange of the size to which the reduction is being made.

##### Threaded Flanges

    - Thread Dimensions. Except as provided in Note

(3) of Figure 6 and Note (1) of Table 4 (Table 4C), threaded flanges shall have a taper pipe thread conforming to ASME B1.20.1. The thread shall be concentric with the axis of the flange opening, and variations in alignment (perpendicu- larity with reference to the flange face) shall not exceed 5 mm/m  (0.06  in./ft).

    - Threads for Class 150 Flanges. Class 150 flanges are made without a counterbore. The threads shall be chamfered approximately to the major diameter of the thread at the back of the flange at an angle of approxi- mately 45 deg with the axis of the thread. The chamfer shall be concentric with the thread and shall be included in the measurement of the thread length.

    - Threads for Class 300 and Higher Flanges. Class 300 and higher pressure class flanges shall be made with a counterbore at the back of the flange. The threads shall be chamfered to the diameter of the counterbore at an angle of approximately 45 deg with the axis of the threads. The counterbore and chamfer shall be concentric with the

ð20Þ

ð20Þ

6.7

##### Welding Neck Flanges Hub and Welding End Profiles

thread.

    - Reducing Flange Thread Length. The minimum

    - Welding End Profiles. Welding end profiles are illustrated in Figures 1 through 3 and Figures 7 through

10. Other welding end profiles furnished by agreement between the purchaser and manufacturer shall not inval- idate compliance with this Standard.

    - Hubs. Acceptable profiles of the hub from the welding end to the back of the flange are shown in Figures 1, 3, 7, and 8.

    - Bores. Straight-through bores shown in Figures 7 and 8 are required unless the special conditions illustrated in Figures 2, 3, 9, and 10 are furnished by agreement between the purchaser and manufacturer.

##### Reducing Flanges

    - Drilling, Outside Diameter, Thickness, and Facing Dimensions. Flange drilling, outside diameter, thickness, and facing are the same as those of the standard flange of the size from which the reduction is being made.

###### Hub Dimensions

6.8.2.1 Threaded, Socket Weld, and Slip-On Flanges. The hub dimension shall be at least as large as those of the standard flange of the size to which the reduction is being made. The hub may be larger or omitted as detailed in Table 6 (Table 6C).

length of effective thread in reducing flanges shall be at least equal to dimension T of the corresponding class of threaded flange as shown in Tables 8, 9, 11, 12, 14, 16, 18, 20, and 22 (Tables 8C, 9C, 11C, 12C, 14C, 16C, 18C, 20C, and

22C). Threads do not necessarily extend to the face of the flange. See Table 6 (Table 6C) for reducing threaded flanges.

    - Thread Gaging. Internal threads of flanges shall be checked with gages having a gaging notch. Variations in threading shall be limited to one turn large or one turn small from the gaging notch when using working gages. The reference point for chamfered thread (Class 150) gaging is the starting end of the threaded flange, provided the thread chamfer does not exceed the major diameter of the internal thread. When a chamfer on the internal thread exceeds this limit, the reference point becomes the last thread scratch on this chamfer cone. The reference point for counterbored thread (Classes 300 through 2500) gaging shall be the last thread scratch on the chamfer cone.

    - Assembly Using Power Equipment. For ASME B1.20.1 external pipe threads, Annex A specifies the distance and number of turns that external pipe threads may be made longer than regular for use with the higher pressure flanges to bring the small end of

11

ASME  B16.5-2020

###### Table 1.1-3 Flange Bolting Dimensional Recommendations

| Product | Carbon Steel | Alloy Steel |
| --- | --- | --- |
| Stud bolts | ASME B18.31.2 | ASME B18.31.2 |
| Bolts smaller than 3∕4 in. Bolts equal to or larger than 3∕4 in. | ASME B18.2.1, square or heavy hex head ASME B18.2.1, square or heavy hex head | ASME B18.2.1, heavy hex head ASME B18.2.1, heavy hex head |
| Nuts smaller than 3∕4 in. Nuts equal to or larger than 3∕4 in. | ASME B18.2.2, heavy hex ASME B18.2.2, hex or heavy hex | ASME B18.2.2, heavy hex ASME B18.2.2, heavy hex |
| External threads | ASME B1.1, Cl. 2A coarse series | ASME B1.1, Cl. 2A coarse series up through 1 in.; |

eight thread series for larger bolts

Internal threads	ASME B1.1, Cl. 2B coarse series	ASME B1.1, Cl. 2B coarse series up through 1 in.;

eight thread series for larger bolts

the thread close to the face of the flange when the parts are assembled by power equipment.

##### Flange Bolting Dimensions

    - Dimensional Standards. Stud bolts, threaded at both ends or threaded full length, or bolts may be used in flange joints.  Dimensional recommendations for bolts, stud bolts, and nuts are shown in Table 1.1-3. See para. 5.3 for bolting material recommendations.

    - Bolt Lengths. Stud bolt lengths, including the height of two heavy hexagon nuts, are shown as dimension L in Tables 7, 10, 13, 15, 17, 19, and 21 (Tables 7C, 10C, 13C, 15C, 17C, 19C, and 21C). The tabulated stud bolt length L does not include the height of end points. An end point is defined as an unthreaded length, such as a chamfer, which extends beyond the thread. The method of calculating bolt lengths is explained in Nonmandatory Appendix C. The tabulated bolt lengths are reference dimensions. Users may select other bolting lengths.

    - Bolting Recommendations. For flange joints, stud bolts with a nut at each end are recommended for all applications and especially for high temperature service.

##### Gaskets for Line Flanges

    - Ring Joint. Ring joint gasket dimensions shall conform to ASME B16.20.

    - Contact Width. For flanges having large or small tongue-and-groove faces, all gaskets, except solid flat metal gaskets, shall cover the bottom of the groove with minimum clearance. [See para. 7.3(a) for tolerance applicable to groove.] Solid flat metal gaskets shall have contact width not greater than for Nonmandatory Appendix B, Group III gaskets.

    - Bearing Surface. For flanges with small male- and-female face, care must be taken to ensure that adequate bearing surface is provided for the gaskets. In particular, care is necessary when the joint is made on the end of the pipe as shown in Figure 6.

##### Auxiliary  Connections

    - General. Auxiliary connections or openings for flanged fittings are not required unless specified by the purchaser. Welding to attach auxiliary connections to flanged fittings shall be made by a  qualified welder using a qualified weld procedure in accordance with Section IX of the ASME Boiler and Pressure Vessel Code.

    - Pipe Thread Tapping. Holes may be tapped in the wall of a fitting if the metal is thick enough to allow the effective thread length specified in Figure 11. Where thread length is insufficient or the tapped hole needs re- inforcement, a boss shall be added.

    - Sockets. Sockets for socket welding connections may be provided in the wall of a fitting if the metal is thick enough to afford the depth of socket and retaining wall specified in Figure 12. Where the wall thickness is insuffi- cient, or the size of the connection requires opening re- inforcement, a boss shall be added (see Figure 13).

    - Butt Welding. Connections may be attached by butt welding directly to the wall of the fitting (see Figure 14). Where the size of an opening requires rein- forcement, a boss shall be added.

    - Bosses. Where bosses are required, the diameters shall be no less than those shown in Figure 13, and the height shall provide lengths as specified in Figure 11 or Figure 12.

    - Size. Unless otherwise specified, auxiliary connections shall be of the pipe sizes given below.

Fitting Size	Connection Size (NPS) 2 ≤ NPS ≤ 4		1∕2

5 ≤ NPS ≤ 8	3∕4

NPS ≥ 10	1

    - Designating Locations. The designation of loca- tions for auxiliary connections for flanged fittings is shown in Figure 15. A letter is used to designate each location.

12

ASME  B16.5-2020

##### 7 TOLERANCES

7.1  General

For the purpose of determining conformance with this Standard, the convention for fixing significant digits where limits, maximum or minimum values, are specified shall be rounded as defined in ASTM Practice E29. This requires that an observed or calculated value shall be rounded to the nearest unit in the last right-hand digit used for expres- sing the limit. The listing of decimal tolerances does not imply a particular method of measurement.

Size	Tolerance,  deg

NPS ≤ 5	1

NPS ≥ 6	0.5

##### 7.4  Flange Thickness	ð20Þ

Required tolerances for flange thickness are as follows:

Size	Tolerance, mm (in.)

NPS ≤ 18	+3.0, −0.0 (+0.12, −0.00)

NPS ≥ 20	+5.0, −0.0 (+0.20, −0.00)

ð20Þ

7.2

##### Center-to-Contact Surfaces and Center-to-End Tolerances

The plus tolerance is applicable to bolting bearing

ð20Þ

Required tolerances for various flanges and flanged fitting elements are as follows:

- Center-to-Contact Surfaces Other Than Ring  Joint

Size 	Tolerance, mm (in.)	NPS ≤ 10		±1.0 (±0.04)

NPS ≥ 12	±1.5 (±0.06)

- Center-to-End (Ring Joint)

Size	Tolerance, mm (in.) NPS ≤ 10		±1.0 (±0.04)

NPS ≥ 12	±1.5 (±0.06)

- Contact Surface-to-Contact Surface Other Than Ring Joint

Size	Tolerance, mm (in.) NPS ≤ 10		±2.0 (±0.08)

NPS ≥ 12	±3.0 (±0.12)

- End-to-End (Ring Joint)

Size	Tolerance, mm (in.) NPS ≤ 10		±2.0 (±0.08)

NPS ≥ 12	±3.0 (±0.12)

##### Facings

Tolerances that apply to both flange and flanged fitting facings are as follows:

    - Inside and outside diameter of large and small

tongue and groove and female, ±0.5 mm (±0.02 in.).

    - Outside diameter, 1.5 mm (0.06 in.) raised face, ±1.0 mm (±0.04 in.).

    - Outside diameter, 6.4 mm (0.25 in.) raised face, ±0.5 mm (±0.02 in.).

    - Ring joint groove tolerances are shown in Table 5

(Table 5C).

Tolerances that apply to flanges are as follows:

    - Perpendicularity of the face with the bore

surfaces whether as-forged, as-cast, spot-faced, or

back-faced (see para. 6.6).

##### Welding End Flange Ends and Hubs	ð20Þ

    - Outside Diameter. Required tolerances for the nominal outside diameter dimension A of Figures 7 and 8 of welding ends of welding neck flanges are as follows:

Size	Tolerance, mm (in.)

NPS ≤ 5	+2.0, −1.0 (+0.08, −0.04)

NPS ≥ 6	+4.0, −1.0 (+0.16, −0.04)

    - Inside Diameter. Required tolerances for the nominal inside diameter of welding ends of welding neck flanges and smaller bore of socket welding flanges (dimension B in the referenced figures) are as follows:

- For Figures 4, 7, and 8, the tolerances are

Size	Tolerance, mm (in.) NPS ≤ 10		±1.0 (±0.04)

12 ≤ NPS ≤ 18	±1.5 (±0.06)

NPS ≥ 20	+3.0, −1.5 (+0.12, −0.06)

- For Figure 9, the tolerances are

Size 		Tolerance, mm (in.)	NPS ≤ 10	+0.0, −1.0 (+0.0, −0.04)

NPS ≥ 12	+0.0, −1.5 (+0.0, −0.06)

    - Backing Ring Contact Surface. Required toler- ances for the bore of the backing ring contact surface of welding neck flanges, dimension C of Figures 9 and 10 are as follows:

Size	Tolerance, mm (in.)

2 ≤ NPS ≤ 24	+0.25, −0.0 (+0.01, −0.0)

13

ASME  B16.5-2020

    - Hub Thickness. Despite the tolerances specified for dimensions A and B, the thickness of the hub at the welding end shall not be less than 871 ∕2 % of the nominal thickness of the pipe, having an under-tolerance of 12.5% for the pipe wall thickness to which the flange is to be attached or the minimum wall thickness as specified by the purchaser.

##### ð20Þ  7.6 Length Through Hub on Welding Neck Flanges

The required tolerances for the length through hubs on welding neck flanges are as follows:

Size	Tolerance, mm (in.) NPS ≤ 4		±1.5 (±0.06)

5 ≤ NPS ≤10	+1.5, −3.0 (+0.06, −0.12)

NPS ≥ 12	+3.0, −5.0 (+0.12, −0.20)

7.8.3 Bolt Circle Concentricity. The required toler- ances for concentricity between the flange bolt circle diameter and machine facing diameters are as follows:

Size	Tolerance, mm (in.) NPS ≤ 21∕2		0.8 (0.03)

NPS ≥ 3	1.5 (0.06)

##### PRESSURE TESTING

  - Flange Test

Flanges are not required to be pressure tested.

##### Flanged Fitting Test

8.2.1 Shell Pressure Test. Each flanged fitting shall be given a shell pressure test.

ð20Þ

##### Flange Bore Diameter

    - Lapped and Slip-On Flange Bores. The required tolerances for lapped and slip-on flange bore diameters are as follows:

Size	Tolerance, mm (in.)

NPS ≤ 10	+1.0, −0.0 (+0.04, −0.0)

NPS ≥ 12	+1.5, −0.0 (+0.06, −0.0)

    - Counterbores, Threaded Flanges. The required tolerances for threaded flange counterbores are as follows:

Size 	   Tolerance, mm (in.)	NPS ≤ 10	+1.0, −0.0 (+0.04, −0.0)

NPS ≥ 12	+1.5, −0.0 (+0.06, −0.0)

    - Counterbores, Socket Welding Flanges. The required tolerance for socket end counterbores is as follows:

Size	Tolerance, mm (in.)

1∕2 ≤ NPS ≤3	±0.25 (±0.010)

##### Drilling and Facing

    - Bolt Circle Diameter. The required tolerance for all bolt circle diameters is as follows:

±1.5 mm (±0.06 in.)

    - Bolt Hole to Bolt Hole. The required tolerance for the center-to-center of adjacent bolt holes is as follows:

±0.8 mm (±0.03 in.)

    - Test Conditions. The shell pressure test for flanged fittings shall be at a pressure no less than 1.5 times the 38°C (100°F) pressure rating rounded off to the next higher 1 bar (25 psi) increment.

    - Test Fluid. The pressure test shall be made using water, which may contain a corrosion inhibitor or kero- sene as the test fluid. Other suitable test fluids may be used provided their viscosity is no greater than that of water. The test fluid temperature shall not exceed 50°C (125°F).

    - Test Duration. The test duration shall be as follows:

Fitting Size	Duration, sec NPS ≤ 2		60

21∕2 ≤ NPS ≤ 8	120

NPS ≥ 10	180

    - Acceptance. No visible leakage is permitted through the pressure boundary wall.

14

ASME  B16.5-2020

###### Figure 1 Bevel for Outside Thickness

GENERAL NOTES:

- When the materials joined have equal minimum specified yield strength, there shall be no restriction on the minimum slope.

- Neither t1or t2, nor their sum t1 + t2 , shall exceed 0.5t.

- When the minimum specified yield strengths of the sections to be

###### Figure 3 Bevel for Combined Thickness

GENERAL NOTES:

- When the materials joined have equal minimum specified yield strength, there shall be no restriction on the minimum slope.

joined are unequal, the value of tD  shall at least equal the mating

- Neither t  or t , nor their sum t

+ t  , shall exceed 0.5t.

wall thickness times the ratio of minimum specified yield strength

1	2	1	2

- When the minimum specified yield strengths of the sections to be

of the pipe to minimum specified yield strength of the flange.

- Welding shall be in accordance with the applicable code.

joined are unequal, the value of tD

shall at least equal the mating

###### Figure 2 Bevel for Inside Thickness

GENERAL NOTES:

- When the materials joined have equal minimum specified yield strength, there shall be no restriction on the minimum slope.

- Neither t1or t2, nor their sum t1 + t2 , shall exceed 0.5t.

- When the minimum specified yield strengths of the sections to be joined are unequal, the value of tD shall at least equal the mating wall thickness times the ratio of minimum specified yield strength of the pipe to minimum specified yield strength of the flange.

- Welding shall be in accordance with the applicable code.

wall thickness times the ratio of minimum specified yield strength

of the pipe to minimum specified yield strength of the flange.

(d)  Welding shall be in accordance with the applicable code.

###### Figure 4 Straight Hub Welding Flanges

GENERAL NOTES:

- O = outside diameter of flange (see para. 2.8)

- tf  = minimum thickness of flange (see para. 2.8)

NOTES:

  - X = diameter of hub (see para. 2.8). Tolerance +3.0 mm, −0.0 mm (+0.12 in., −0.00 in.).

  - Hub end (see para. 2.8.4).

  - B = bore (see paras. 2.8.3 and 7.5.2).

  - Y = length through hub (see para. 2.8.2).

15

ASME  B16.5-2020

###### Figure 5 Method of Designating Outlets of Reducing Fittings in Specifications (Flanged Fittings)

GENERAL NOTES:

    - The largest opening establishes the basic size of a reducing fitting. The largest opening is named first, except that for bull head tees, which are reducing on both runs, and for double branch elbows where both branches are reducing, the outlet is the largest opening and named last in both cases.

    - In designating the openings of reducing fittings, they should be read in the order indicated by the sequence of the letters A, B, C, and D. In designating the outlets of side outlet reducing fittings, the side outlet is named last, and in the case of the cross, which is not shown, the side outlet is designated by the letter E.

    - Sketches are illustrative only and do not imply required design (see para. 3.2).

16

ASME  B16.5-2020

###### Figure 6 End Flange Facings and Their Relationship to Flange Thickness and Center-to-End and End-to-End Dimensions

ð20Þ

17

ASME  B16.5-2020

###### Figure 6 End Flange Facings and Their Relationship to Flange Thickness and Center-to-End and End-to-End Dimensions (Cont’d)

NOTES:

      - See paras. 6.2 and 6.4.

      - See Tables 7 through 22 (Tables 7C through 22C).

      - For small male and female joints, care should be taken in the use of these dimensions to ensure that the inside diameter of the pipe fitting is small enough to permit sufficient bearing surface to prevent crushing of the gasket [see Table 4 (Table 4C)]. This applies particularly on lines where the joint is made on the end of the pipe. Threaded companion flanges for small male and female joints are furnished with plain face and threaded with American National Standard Locknut Thread (NPSL).

      - See Table 4 (Table 4C) for dimensions of facings (other than ring joint) and Table 5 (Table 5C) for ring joint facing.

      - Large male and female faces and large tongue and groove are not applicable to Class 150 because of potential dimensional conflicts.

      - See Table 4 (Table 4C).

      - See Table 5 (Table 5C).

      - See para. 6.4.3.

      - See para. 6.4.3.5 and Table 5 (Table 5C).

18

ASME  B16.5-2020

###### Figure 7 Welding Neck Flange Hub and Welding End Profiles for Wall Thicknesses t

From 5 mm (0.19 in.) to 22 mm (0.88 in.)

ð20Þ

Welding Ends (Welding Neck Flanges, No Backing Rings)

A  = nominal outside diameter of pipe

B  = nominal inside diameter of pipe

t  = nominal wall thickness of pipe

x = diameter of hub (see dimensional tables)

GENERAL NOTES:

        - See paras. 6.7, 6.8, and 7.4 for details and tolerances.

        - See Figures 9 and 10 for additional details of welding ends.

        - When the thickness of the hub at the bevel is greater than that of the pipe to which the flange is joined and the additional thickness is provided on the outside diameter, a taper weld having a slope not exceeding 1 to 3 may be used, or, alternatively, the greater outside diameter may be tapered at the same maximum slope or less, from a point on the welding bevel equal to the outside diameter of the mating pipe. Similarly, when the greater thickness is provided on the inside of the flange, it shall be taper-bored from the welding end at a slope not exceeding 1 to 3. When flanges covered by this Standard are intended for services with light wall, higher strength pipe, the thickness of the hub at the bevel may be greater than that of the pipe to which the flange is joined. Under these conditions, a single taper hub may be provided. The additional thickness may be provided on either inside or outside or partially on each side, but the total additional thickness shall not exceed one-half times the nominal wall thickness of intended mating pipe (see Figures 1 through 3).

        - For welding end dimensions, refer to ASME B16.25.

NOTE: (1) For manufacturing purposes, the straight turn from the bevel is permitted to have a taper with the angle not exceeding 7 deg.

19

ASME  B16.5-2020

ð20Þ

###### Figure 8 Welding Neck Flange Hub and Welding End Profiles for Wall Thicknesses t Greater Than 22 mm (0.88 in.)

Welding Ends (Welding Neck Flanges, No Backing Rings)

A  = nominal outside diameter of pipe

B  = nominal inside diameter of pipe

t  = nominal wall thickness of pipe

x = diameter of hub (see dimensional tables)

GENERAL NOTES:

          - See paras. 6.7, 6.8, and 7.4 for details and tolerances.

          - See Figures 9 and 10 for additional details of welding ends.

          - When the thickness of the hub at the bevel is greater than that of the pipe to which the flange is joined and the additional thickness is provided on the outside diameter, a taper weld having a slope not exceeding 1 to 3 may be used, or, alternatively, the greater outside diameter may be tapered at the same maximum slope or less, from a point on the welding bevel equal to the outside diameter of the mating pipe. Similarly, when the greater thickness is provided on the inside of the flange, it shall be taper-bored from the welding end at a slope not exceeding 1 to 3. When flanges covered by this Standard are intended for services with light wall, higher strength pipe, the thickness of the hub at the bevel may be greater than that of the pipe to which the flange is joined. Under these conditions, a single taper hub may be provided. The additional thickness may be provided on either inside or outside or partially on each side, but the total additional thickness shall not exceed one-half times the nominal wall thickness of intended mating pipe (see Figures 1 through 3).

          - For welding end dimensions, refer to ASME B16.25.

NOTE: (1) For manufacturing purposes, the straight turn from the bevel is permitted to have a taper with the angle not exceeding 7 deg.

20

ASME  B16.5-2020

###### Figure 9 Inside Contour for Use With Rectangular Backing Ring

Figure 10 Inside Contour for Use With Taper Backing Ring

Welding Ends (Welding Neck Flanges)

(SI Units)

A  = nominal outside diameter of welding end, mm

B  = nominal inside diameter of pipe (− A − 2t), mm

C  = A − 0.79 − 1.75t − 0.25, mm

t  = nominal wall thickness of pipe, mm

0.79 mm  = minus tolerance on outside diameter of pipe to ASTM A106, etc.

1.75t = 871∕2% of nominal wall (permitted by ASTM A106, etc.) multi- plied by two to convert into terms of diameter

0.25 mm = plus tolerance on diameter C, mm (see para. 7.5.3)

(U.S. Customary Units)

A  = nominal outside diameter of welding end, in.

B  = nominal inside diameter of pipe (− A − 2t), in.

C  = A − 0.031 − 1.75t − 0.010, in.

t  = nominal wall thickness of pipe

0.031 in.  = minus tolerance on outside diameter of pipe to ASTM A106, etc.

1.75t = 871∕2% of nominal wall (permitted by ASTM A106, etc.) multi- plied by two to convert into terms of diameter

0.010 in. = plus tolerance on diameter C, in. (see para. 7.5.3)

GENERAL NOTES:

- See paras. 6.7, 6.8, and 7.5 for details and tolerances.

- See Figures 7 and 8 for welding end details of welding neck flanges.

- For dimensions, see ASME B16.25.

NOTE: (1) 13-mm (0.5-in.) depth based on use of 19-mm (0.75-in.) wide backing ring.

Welding Ends (Welding Neck Flanges)

(SI Units)

A  = nominal outside diameter of welding end, mm

B  = nominal inside diameter of pipe (− A − 2t), mm

C  = A − 0.79 − 1.75t − 0.25, mm

t  = nominal wall thickness of pipe, mm

0.79 mm  = minus tolerance on outside diameter of pipe to ASTM A106, etc.

1.75t = 871∕2% of nominal wall (permitted by ASTM A106, etc.) multi- plied by two to convert into terms of diameter

0.25 mm = plus tolerance on diameter C, mm (see para. 7.5.3)

(U.S. Customary Units)

A  = nominal outside diameter of welding end, in.

B  = nominal inside diameter of pipe (− A − 2t), in.

C  = A − 0.031 − 1.75t − 0.010, in.

t  = nominal wall thickness of pipe

0.031 in.  = minus tolerance on outside diameter of pipe to ASTM A106, etc.

1.75t = 871∕2% of nominal wall (permitted by ASTM A106, etc.) multi- plied by two to convert into terms of diameter

0.010 in. = plus tolerance on diameter C, in. (see para. 7.5.3)

GENERAL NOTES:

- See paras. 6.7, 6.8, and 7.5 for details and tolerances.

- See Figures 7 and 8 for welding end details of welding neck flanges.

- For dimensions, see ASME B16.25.

21

ASME  B16.5-2020

ð20Þ

###### Figure 11 Thread Length for Connection Tapping

Figure 13 Bosses for Connections	ð20Þ

Connection Size, NPS

Thread Length,

T, mm (in.) (1)

3∕8	10.4 (0.41)

1∕2	13.5 (0.53)

Connection Size, NPS

Minimum Boss Diameter, J,  mm (in.)

3∕4	14.0 (0.55)

1	17.3 (0.68)

11∕4	18.0 (0.71)

11∕2	18.3 (0.72)

2	19.3 (0.76)

GENERAL NOTE: See paras. 6.12.2, 6.12.5, and 6.12.6.

NOTE: (1) In no case shall the effective length of thread T be less than that shown in the table above. These lengths are equal to the effective thread length of external pipe threads (ASME B1.20.1).

###### Figure 12 Socket Welding for Connections

3∕8	31.8 (1.25)

1∕2	38.1 (1.50)

3∕4	44.4 (1.75)

1	53.8 (2.12)

11∕4	63.5 (2.50)

11∕2	69.8 (2.75)

2	85.9 (3.38)

GENERAL NOTE: See para. 6.12.5.

###### Figure 14 Butt Welding for Connections

Connection Size, NPS

Minimum Diameter of

Socket,

A, mm

Minimum Depth, B, mm

GENERAL NOTE: See paras. 6.12.4 and 6.12.5.

3∕8	17.5	4.8 (0.19)

1∕2	21.8	4.8 (0.19)

3∕4	26.9	6.4 (0.25)

1	33.8	6.4 (0.25)

11∕4	42.7	6.4 (0.25)

11∕2	48.8	6.4 (0.25)

2	61.2	7.9 (0.31)

GENERAL NOTE: See paras. 6.12.3, 6.12.5, and 6.12.6.

22

ASME  B16.5-2020

###### Figure 15 Method of Designating Location of Auxiliary Connections When Specified (Flanged  Fittings)

GENERAL NOTE: The above sketches show views of the same fitting and represent fittings with symmetrical shapes, with the exception of the side outlet elbow and side outlet tee (straight sizes). Sketches are illustrative only and do not imply required design (see para. 6.12).

23

ASME  B16.5-2020

ð20Þ

###### Table 2-1.1 Pressure–Temperature Ratings for Group 1.1 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–Si | A105 (1) | A216 Gr. WCB (1) | A515 Gr. 70 (1) |
| C–Mn–Si | A350 Gr. LF2 (1) | … | A516 Gr. 70 (1), (2) |
| C–Mn–Si | … | … | A537 Cl. 1 (3) |
| C–Mn–Si–V 31∕2Ni | A350 Gr. LF6 Cl. 1 (4) A350 Gr. LF3 (5) | … … | … … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.6 | 51.1 | 68.1 | 102.1 | 153.2 | 255.3 | 425.5 |
| 50 | 19.2 | 50.1 | 66.8 | 100.2 | 150.4 | 250.6 | 417.7 |
| 100 | 17.7 | 46.6 | 62.1 | 93.2 | 139.8 | 233.0 | 388.3 |
| 150 | 15.8 | 45.1 | 60.1 | 90.2 | 135.2 | 225.4 | 375.6 |
| 200 | 13.8 | 43.8 | 58.4 | 87.6 | 131.4 | 219.0 | 365.0 |
| 250 | 12.1 | 41.9 | 55.9 | 83.9 | 125.8 | 209.7 | 349.5 |
| 300 | 10.2 | 39.8 | 53.1 | 79.6 | 119.5 | 199.1 | 331.8 |
| 325 | 9.3 | 38.7 | 51.6 | 77.4 | 116.1 | 193.6 | 322.6 |
| 350 | 8.4 | 37.6 | 50.1 | 75.1 | 112.7 | 187.8 | 313.0 |
| 375 | 7.4 | 36.4 | 48.5 | 72.7 | 109.1 | 181.8 | 303.1 |
| 400 | 6.5 | 34.7 | 46.3 | 69.4 | 104.2 | 173.6 | 289.3 |
| 425 | 5.5 | 28.8 | 38.4 | 57.5 | 86.3 | 143.8 | 239.7 |
| 450 | 4.6 | 23.0 | 30.7 | 46.0 | 69.0 | 115.0 | 191.7 |
| 475 | 3.7 | 17.4 | 23.2 | 34.9 | 52.3 | 87.2 | 145.3 |
| 500 | 2.8 | 11.8 | 15.7 | 23.5 | 35.3 | 58.8 | 97.9 |
| 538 | 1.4 | 5.9 | 7.9 | 11.8 | 17.7 | 29.5 | 49.2 |

Working Pressures by Classes, bar Class

NOTES:

  - Prolonged use above 425°C is permissible but not recommended. Prolonged exposure above 425°C may cause the carbide phase of steel to convert to graphite.

  - Not to be used over 455°C.

  - Not to be used over 370°C.

  - Not to be used over 260°C.

  - Not to be used over 345°C.

24

ASME  B16.5-2020

###### Table 2-1.1C Pressure–Temperature Ratings for Group 1.1 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–Si | A105 (1) | A216 Gr. WCB (1) | A515 Gr. 70 (1) |
| C–Mn–Si | A350 Gr. LF2 (1) | … | A516 Gr. 70 (1), (2) |
| C–Mn–Si | … | … | A537 Cl. 1 (3) |
| C–Mn–Si–V 31∕2Ni | A350 Gr. LF6 Cl. 1 (4) A350 Gr. LF3 (5) | … … | … … |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 285 | 740 | 985 | 1,480 | 2,220 | 3,705 | 6,170 |
| 200 | 260 | 680 | 905 | 1,360 | 2,035 | 3,395 | 5,655 |
| 300 | 230 | 655 | 870 | 1,310 | 1,965 | 3,270 | 5,450 |
| 400 | 200 | 635 | 845 | 1,265 | 1,900 | 3,170 | 5,280 |
| 500 | 170 | 605 | 805 | 1,205 | 1,810 | 3,015 | 5,025 |
| 600 | 140 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 650 | 125 | 550 | 730 | 1,100 | 1,650 | 2,745 | 4,575 |
| 700 | 110 | 530 | 710 | 1,060 | 1,590 | 2,655 | 4,425 |
| 750 | 95 | 505 | 675 | 1,015 | 1,520 | 2,535 | 4,230 |
| 800 | 80 | 410 | 550 | 825 | 1,235 | 2,055 | 3,430 |
| 850 | 65 | 320 | 425 | 640 | 955 | 1,595 | 2,655 |
| 900 | 50 | 230 | 305 | 460 | 690 | 1,150 | 1,915 |
| 950 | 35 | 135 | 185 | 275 | 410 | 685 | 1,145 |
| 1,000 | 20 | 85 | 115 | 170 | 255 | 430 | 715 |

Working Pressures by Classes, psig Class

NOTES:

- Prolonged use above 800°F is permissible but not recommended. Prolonged exposure above 800°F may cause the carbide phase of steel to convert to graphite.

- Not to be used over 850°F.

- Not to be used over 700°F.

- Not to be used over 500°F.

- Not to be used over 650°F.

25

ASME  B16.5-2020

###### Table 2-1.2 Pressure–Temperature Ratings for Group 1.2 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–Mn–Si | … | A216 Gr. WCC (1) | … |
| C–Mn–Si | … | A352 Gr. LCC (2) | … |
| C–Mn–Si–V | A350 Gr. LF6 Cl.2 (3) | … | … |
| 21∕2Ni 31∕2Ni | … … | A352 Gr. LC2 A352 Gr. LC3 (2) | A203 Gr. B (1) A203 Gr. E (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.8 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.2 | 66.8 | 100.3 | 150.5 | 250.8 | 418.1 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.2 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.0 | 53.4 | 80.0 | 120.1 | 200.1 | 333.5 |
| 375 | 7.4 | 37.8 | 50.4 | 75.7 | 113.5 | 189.2 | 315.3 |
| 400 | 6.5 | 34.7 | 46.3 | 69.4 | 104.2 | 173.6 | 289.3 |
| 425 | 5.5 | 28.8 | 38.4 | 57.5 | 86.3 | 143.8 | 239.7 |
| 450 | 4.6 | 23.0 | 30.7 | 46.0 | 69.0 | 115.0 | 191.7 |
| 475 | 3.7 | 17.1 | 22.8 | 34.2 | 51.3 | 85.4 | 142.4 |
| 500 | 2.8 | 11.6 | 15.4 | 23.2 | 34.7 | 57.9 | 96.5 |
| 538 | 1.4 | 5.9 | 7.9 | 11.8 | 17.7 | 29.5 | 49.2 |

Working Pressures by Classes, bar Class

NOTES:

- Prolonged use above 425°C is permissible but not recommended. Prolonged exposure above 425°C may cause the carbide phase of steel to convert to graphite.

- Not to be used over 340°C.

- Not to be used over 260°C.

26

ASME  B16.5-2020

###### Table 2-1.2C Pressure–Temperature Ratings for Group 1.2 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–Mn–Si | … | A216 Gr. WCC (1) | … |
| C–Mn–Si | … | A352 Gr. LCC (2) | … |
| C–Mn–Si–V | A350 Gr. LF6 Cl. 2 (3) | … | … |
| 21∕2Ni 31∕2Ni | … … | A352 Gr. LC2 A352 Gr. LC3 (2) | A203 Gr. B (1) A203 Gr. E (1) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,405 | 2,110 | 3,520 | 5,865 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 555 | 740 | 1,110 | 1,665 | 2,775 | 4,630 |
| 750 | 95 | 505 | 675 | 1,015 | 1,520 | 2,535 | 4,230 |
| 800 | 80 | 410 | 550 | 825 | 1,235 | 2,055 | 3,430 |
| 850 | 65 | 320 | 425 | 640 | 955 | 1,595 | 2,655 |
| 900 | 50 | 225 | 295 | 445 | 670 | 1,115 | 1,855 |
| 950 | 35 | 135 | 185 | 275 | 410 | 685 | 1,145 |
| 1,000 | 20 | 85 | 115 | 170 | 255 | 430 | 715 |

NOTES:

- Prolonged use above 800°F is permissible but not recommended. Prolonged exposure above 800°F may cause the carbide phase of steel to convert to graphite.

- Not to be used over 650°F

- Not to be used over 500°F.

27

ASME  B16.5-2020

###### Table 2-1.3 Pressure–Temperature Ratings for Group 1.3 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–Si | … | A352 LCB (1) | A515 Gr. 65 (2) |
| C–Mn–Si | … | … | A516 Gr. 65 (2), (3) |
| 21∕2Ni | … | … | A203 Gr. A (2) |
| 31∕2Ni C–1∕2Mo C–1∕2Mo | … … … | … A217 Gr. WC1 (4)–(6) A352 Gr. LC1 (1) | A203 Gr. D (2) … … |

Working Pressures by Classes, bar

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −29 to 38 | 18.4 | 48.0 | 64.0 | 96.0 | 144.1 | 240.1 | 400.1 |
| 50 | 18.2 | 47.5 | 63.3 | 94.9 | 142.4 | 237.3 | 395.6 |
| 100 | 17.4 | 45.3 | 60.5 | 90.7 | 136.0 | 226.7 | 377.8 |
| 150 | 15.8 | 43.9 | 58.6 | 87.9 | 131.8 | 219.7 | 366.1 |
| 200 | 13.8 | 42.5 | 56.7 | 85.1 | 127.6 | 212.7 | 354.4 |
| 250 | 12.1 | 40.8 | 54.4 | 81.6 | 122.3 | 203.9 | 339.8 |
| 300 | 10.2 | 38.7 | 51.6 | 77.4 | 116.1 | 193.4 | 322.4 |
| 325 | 9.3 | 37.6 | 50.1 | 75.2 | 112.7 | 187.9 | 313.1 |
| 350 | 8.4 | 36.4 | 48.5 | 72.8 | 109.2 | 182.0 | 303.3 |
| 375 | 7.4 | 35.0 | 46.6 | 69.9 | 104.9 | 174.9 | 291.4 |
| 400 | 6.5 | 32.6 | 43.5 | 65.2 | 97.9 | 163.1 | 271.9 |
| 425 | 5.5 | 27.3 | 36.4 | 54.6 | 81.9 | 136.5 | 227.5 |
| 450 | 4.6 | 21.6 | 28.8 | 43.2 | 64.8 | 107.9 | 179.9 |
| 475 | 3.7 | 15.7 | 20.9 | 31.3 | 47.0 | 78.3 | 130.6 |
| 500 | 2.8 | 11.1 | 14.8 | 22.1 | 33.2 | 55.4 | 92.3 |
| 538 | 1.4 | 5.9 | 7.9 | 11.8 | 17.7 | 29.5 | 49.2 |

NOTES:

- Not to be used over 340°C.

- Prolonged use above 425°C is permissible but not recommended. Prolonged exposure above 425°C may cause the carbide phase of steel to convert to graphite.

- Not to be used over 455°C.

- Prolonged use above 465°C is permissible but not recommended. Prolonged exposure above 465°C may cause the carbide phase of steel to convert to graphite.

- Use normalized and tempered material only.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

28

ASME  B16.5-2020

###### Table 2-1.3C Pressure–Temperature Ratings for Group 1.3 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–Si | … | A352 Gr. LCB (1) | A515 Gr. 65 (2) |
| C–Mn–Si | … | … | A516 Gr. 65 (2), (3) |
| 21∕2Ni 31∕2Ni C–1∕2Mo C–1∕2Mo | … … … … | … … A217 Gr. WC1 (4)–(6) A352 Gr. LC1 (1) | A203 Gr. A (2) A203 Gr. D (2) … … |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 265 | 695 | 930 | 1,395 | 2,090 | 3,480 | 5,805 |
| 200 | 255 | 660 | 880 | 1,320 | 1,980 | 3,300 | 5,505 |
| 300 | 230 | 640 | 850 | 1,275 | 1,915 | 3,190 | 5,315 |
| 400 | 200 | 615 | 820 | 1,230 | 1,845 | 3,075 | 5,125 |
| 500 | 170 | 585 | 780 | 1,175 | 1,760 | 2,930 | 4,885 |
| 600 | 140 | 550 | 735 | 1,105 | 1,655 | 2,755 | 4,595 |
| 650 | 125 | 535 | 710 | 1,065 | 1,600 | 2,665 | 4,440 |
| 700 | 110 | 510 | 685 | 1,025 | 1,535 | 2,560 | 4,270 |
| 750 | 95 | 475 | 635 | 955 | 1,430 | 2,385 | 3,970 |
| 800 | 80 | 390 | 520 | 780 | 1,175 | 1,955 | 3,255 |
| 850 | 65 | 300 | 400 | 595 | 895 | 1,490 | 2,485 |
| 900 | 50 | 200 | 270 | 405 | 605 | 1,010 | 1,685 |
| 950 | 35 | 135 | 185 | 275 | 410 | 685 | 1,145 |
| 1,000 | 20 | 85 | 115 | 170 | 255 | 430 | 715 |

Working Pressures by Classes, psig Class

NOTES:

- Not to be used over 650°F.

- Prolonged use above 800°F is permissible but not recommended. Prolonged exposure above 800°F may cause the carbide phase of steel to convert to graphite.

- Not to be used over 850°F

- Prolonged use above 875°F is permissible but not recommended. Prolonged exposure above 875°F may cause the carbide phase of steel to convert to graphite.

- Use normalized and tempered material only.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

29

ASME  B16.5-2020

###### Table 2-1.4 Pressure–Temperature Ratings for Group 1.4 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–Si C–Mn–Si | … A350 Gr. LF1, Cl. 1 (1) | … … | A515 Gr. 60 (1) A516 Gr. 60 (1), (2) |

Working Pressures by Classes, bar

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −29 to 38 | 16.3 | 42.6 | 56.7 | 85.1 | 127.7 | 212.8 | 354.6 |
| 50 | 16.0 | 41.8 | 55.7 | 83.5 | 125.3 | 208.9 | 348.1 |
| 100 | 14.9 | 38.8 | 51.8 | 77.7 | 116.5 | 194.2 | 323.6 |
| 150 | 14.4 | 37.6 | 50.1 | 75.1 | 112.7 | 187.8 | 313.0 |
| 200 | 13.8 | 36.4 | 48.5 | 72.8 | 109.2 | 182.1 | 303.4 |
| 250 | 12.1 | 34.9 | 46.6 | 69.8 | 104.7 | 174.6 | 291.0 |
| 300 | 10.2 | 33.2 | 44.2 | 66.4 | 99.5 | 165.9 | 276.5 |
| 325 | 9.3 | 32.2 | 43.0 | 64.5 | 96.7 | 161.2 | 268.6 |
| 350 | 8.4 | 31.2 | 41.7 | 62.5 | 93.7 | 156.2 | 260.4 |
| 375 | 7.4 | 30.4 | 40.5 | 60.7 | 91.1 | 151.8 | 253.0 |
| 400 | 6.5 | 29.3 | 39.1 | 58.7 | 88.0 | 146.7 | 244.5 |
| 425 | 5.5 | 25.8 | 34.4 | 51.5 | 77.3 | 128.8 | 214.7 |
| 450 | 4.6 | 21.4 | 28.5 | 42.7 | 64.1 | 106.8 | 178.0 |
| 475 | 3.7 | 14.1 | 18.8 | 28.2 | 42.3 | 70.5 | 117.4 |
| 500 | 2.8 | 10.3 | 13.7 | 20.6 | 30.9 | 51.5 | 85.9 |
| 538 | 1.4 | 5.9 | 7.9 | 11.8 | 17.7 | 29.5 | 49.2 |

NOTES:

- Prolonged use above 425°C is permissible but not recommended. Prolonged exposure above 425°C may cause the carbide phase of steel to convert to graphite.

- Not to be used over 455°C.

30

ASME  B16.5-2020

###### Table 2-1.4C Pressure–Temperature Ratings for Group 1.4 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–Si C–Mn–Si | … A350 Gr. LF1 Cl. 1 (1) | … … | A515 Gr. 60 (1) A516 Gr. 60 (1), (2) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 |  | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 235 | 615 | 825 |  | 1,235 | 1,850 | 3,085 | 5,145 |
| 200 | 215 | 565 | 755 |  | 1,130 | 1,695 | 2,830 | 4,715 |
| 300 | 210 | 545 | 725 |  | 1,090 | 1,635 | 2,725 | 4,545 |
| 400 | 200 | 525 | 700 |  | 1,055 | 1,580 | 2,635 | 4,390 |
| 500 | 170 | 500 | 670 |  | 1,005 | 1,505 | 2,510 | 4,185 |
| 600 | 140 | 475 | 630 |  | 945 | 1,420 | 2,365 | 3,945 |
| 650 | 125 | 455 | 610 |  | 915 | 1,370 | 2,285 | 3,805 |
| 700 | 110 | 440 | 590 |  | 885 | 1,325 | 2,210 | 3,685 |
| 750 | 95 | 430 | 570 |  | 855 | 1,285 | 2,140 | 3,565 |
| 800 | 80 | 370 | 495 |  | 740 | 1,110 | 1,850 | 3,085 |
| 850 | 65 | 300 | 400 |  | 595 | 895 | 1,490 | 2,485 |
| 900 | 50 | 170 | 230 |  | 345 | 515 | 855 | 1,430 |
| 950 | 35 | 135 | 185 |  | 275 | 410 | 685 | 1,145 |
| 1,000 | 20 | 85 | 115 |  | 170 | 255 | 430 | 715 |

NOTES:

- Prolonged use above 800°F is permissible but not recommended. Prolonged exposure above 800°F may cause the carbide phase of steel to convert to graphite.

- Not to be used over 850°F.

31

ASME  B16.5-2020

###### Table 2-1.5 Pressure–Temperature Ratings for Group 1.5 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–1∕2Mo C–1∕2Mo | A182 Gr. F1 (1) … | … … | A204 Gr. A (1) A204 Gr. B (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 18.4 | 48.0 | 64.0 | 96.0 | 144.1 | 240.1 | 400.1 |
| 50 | 18.4 | 48.0 | 64.0 | 96.0 | 144.1 | 240.1 | 400.1 |
| 100 | 17.7 | 47.9 | 63.9 | 95.9 | 143.8 | 239.7 | 399.5 |
| 150 | 15.8 | 47.3 | 63.1 | 94.7 | 142.0 | 236.7 | 394.5 |
| 200 | 13.8 | 45.8 | 61.1 | 91.6 | 137.4 | 229.0 | 381.7 |
| 250 | 12.1 | 44.5 | 59.3 | 89.0 | 133.5 | 222.5 | 370.9 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 24.1 | 32.1 | 48.1 | 72.2 | 120.3 | 200.5 |
| 538 | 1.4 | 11.3 | 15.1 | 22.7 | 34.0 | 56.7 | 94.6 |

Working Pressures by Classes, bar Class

NOTE:  (1) Upon prolonged exposure to temperatures above 465°C, the carbide phase of carbon-molybdenum steel may be converted to graphite. Permissible but not recommended for prolonged use above 465°C.

32

ASME  B16.5-2020

###### Table 2-1.5C Pressure–Temperature Ratings for Group 1.5 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–1∕2Mo C–1∕2Mo | A182 Gr. F1 (1) … | … … | A204 Gr. A (1) A204 Gr. B (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 265 | 695 | 930 | 1,395 | 2,090 | 3,480 | 5,805 |
| 200 | 260 | 695 | 930 | 1,395 | 2,090 | 3,480 | 5,805 |
| 300 | 230 | 685 | 915 | 1,375 | 2,060 | 3,435 | 5,725 |
| 400 | 200 | 660 | 885 | 1,325 | 1,985 | 3,310 | 5,520 |
| 500 | 170 | 640 | 855 | 1,285 | 1,925 | 3,210 | 5,350 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 280 | 375 | 560 | 845 | 1,405 | 2,345 |
| 1,000 | 20 | 165 | 220 | 330 | 495 | 825 | 1,370 |

Working Pressures by Classes, psig Class

NOTE:  (1) Upon prolonged exposure to temperatures above 875°C, the carbide phase of carbon-molybdenum steel may be converted to graphite. Permissible but not recommended for prolonged use above 875°C.

33

ASME  B16.5-2020

###### Table 2-1.7 Pressure–Temperature Ratings for Group 1.7 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 1∕2Cr–1∕2Mo Ni–1∕2Cr–1∕2Mo | A182 Gr. F2 (1) … | … A217 Gr. WC4 (1)–(3) | … … |
| 3∕4Ni–3∕4Cr–1Mo | … | A217 Gr. WC5 (2), (3) | … |

Working Pressures by Classes, bar

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −29 to 38 | 19.8 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 26.7 | 35.6 | 53.4 | 80.1 | 133.4 | 222.4 |
| 538 | 1.4 | 13.9 | 18.6 | 27.9 | 41.8 | 69.7 | 116.2 |
| 550 | … | 12.6 | 16.8 | 25.2 | 37.8 | 63.0 | 105.0 |
| 575 | … | 7.2 | 9.6 | 14.4 | 21.5 | 35.9 | 59.8 |

NOTES:

- Not to be used over 538°C.

- Use normalized and tempered material only.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

34

ASME  B16.5-2020

###### Table 2-1.7C Pressure–Temperature Ratings for Group 1.7 Materials

| Nominal Designation Forgings |  | Castings | Plates |
| --- | --- | --- | --- |
| 1∕2Cr–1∕2Mo Ni–1∕2Cr–1∕2Mo 3∕4Ni–3∕4Cr–1Mo | A182 Gr. F2 (1) … … | … A217 Gr. WC4 (1)-(3) A217 Gr. WC5 (2), (3) | … … … |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 315 | 420 | 630 | 945 | 1,575 | 2,630 |
| 1,000 | 20 | 200 | 270 | 405 | 605 | 1,010 | 1,685 |
| 1,050 | … | 160 | 210 | 315 | 475 | 790 | 1,315 |

NOTES:

- Not to be used over 1,000°F.

- Use normalized and tempered material only.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

35

ASME  B16.5-2020

###### Table 2-1.9 Pressure–Temperature Ratings for Group 1.9 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 11∕4Cr–1∕2Mo 11∕4Cr–1∕2Mo–Si | … A182 Gr. F11 Cl. 2 (1), (4) | A217 Gr. WC6 (1)–(3) … | … A387 Gr. 11 Cl. 2 (4) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.8 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.6 | 103.0 | 154.4 | 257.4 | 429.0 |
| 150 | 15.8 | 49.7 | 66.3 | 99.5 | 149.2 | 248.7 | 414.5 |
| 200 | 13.8 | 48.0 | 63.9 | 95.9 | 143.9 | 239.8 | 399.6 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 25.7 | 34.3 | 51.5 | 77.2 | 128.6 | 214.4 |
| 538 | 1.4 | 14.9 | 19.9 | 29.8 | 44.7 | 74.5 | 124.1 |
| 550 | … | 12.7 | 16.9 | 25.4 | 38.1 | 63.5 | 105.9 |
| 575 | … | 8.8 | 11.7 | 17.6 | 26.4 | 44.0 | 73.4 |
| 600 | … | 6.1 | 8.1 | 12.2 | 18.3 | 30.5 | 50.9 |
| 625 | … | 4.3 | 5.7 | 8.5 | 12.8 | 21.3 | 35.5 |
| 650 | … | 2.8 | 3.8 | 5.7 | 8.5 | 14.2 | 23.6 |

Working Pressures by Classes, bar Class

NOTES:

- Use normalized and tempered material only.

- Not to be used over 590°C.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

- Prolonged use above 590°C is permissible but not recommended.

36

ASME  B16.5-2020

###### Table 2-1.9C Pressure–Temperature Ratings for Group 1.9 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 11∕4Cr–1∕2Mo 11∕4Cr–1∕2Mo-Si | … A182 Gr. F11 Cl. 2 (1), (4) | A217 Gr. WC6 (1)-(3) … | … A387 Gr. 11 Cl. 2 (4) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 720 | 965 | 1,445 | 2,165 | 3,610 | 6,015 |
| 400 | 200 | 695 | 925 | 1,385 | 2,080 | 3,465 | 5,775 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 320 | 425 | 640 | 955 | 1,595 | 2,655 |
| 1,000 | 20 | 215 | 290 | 430 | 650 | 1,080 | 1,800 |
| 1,050 | … | 145 | 190 | 290 | 430 | 720 | 1,200 |
| 1,100 | … | 95 | 130 | 190 | 290 | 480 | 800 |
| 1,150 | … | 65 | 85 | 130 | 195 | 325 | 545 |
| 1,200 | … | 40 | 55 | 80 | 125 | 205 | 345 |

NOTES:

- Use normalized and tempered material only.

- Not to be used over 1,100°F.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

- Prolonged use above 1,100°F is permissible but not recommended.

37

ASME  B16.5-2020

###### Table 2-1.10 Pressure–Temperature Ratings for Group 1.10 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 21∕4Cr–1Mo | A182 Gr. F22 Cl. 3 (1) | A217 Gr. WC9 (2)–(4) | A387 Gr. 22 Cl. 2 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.8 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 18.4 | 24.6 | 36.9 | 55.3 | 92.2 | 153.7 |
| 550 | … | 15.6 | 20.8 | 31.3 | 46.9 | 78.2 | 130.3 |
| 575 | … | 10.5 | 14.0 | 21.1 | 31.6 | 52.6 | 87.7 |
| 600 | … | 6.9 | 9.2 | 13.8 | 20.7 | 34.4 | 57.4 |
| 625 | … | 4.5 | 6.0 | 8.9 | 13.4 | 22.3 | 37.2 |
| 650 | … | 2.8 | 3.8 | 5.7 | 8.5 | 14.2 | 23.6 |

Working Pressures by Classes, bar Class

NOTES:

- Prolonged use above 590°C is permissible but not recommended.

- Use normalized and tempered material only.

- Not to be used over 590°C.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

38

ASME  B16.5-2020

###### Table 2-1.10C Pressure–Temperature Ratings for Group 1.10 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 21∕4Cr–1Mo | A182 Gr. F22 Cl. 3 (1) | A217 Gr. WC9 (2)–(4) | A387 Gr. 22 Cl. 2 (1) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 265 | 355 | 535 | 800 | 1,335 | 2,230 |
| 1,050 | … | 175 | 235 | 350 | 525 | 875 | 1,455 |
| 1,100 | … | 110 | 145 | 220 | 330 | 550 | 915 |
| 1,150 | … | 70 | 90 | 135 | 205 | 345 | 570 |
| 1,200 | … | 40 | 55 | 80 | 125 | 205 | 345 |

NOTES:

- Prolonged use above 1,100°F is permissible but not recommended.

- Use normalized and tempered material only.

- Not to be used over 1,100°F.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

39

ASME  B16.5-2020

###### Table 2-1.11 Pressure–Temperature Ratings for Group 1.11 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–1∕2Mo | … | … | A204 Gr. C (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 23.6 | 31.4 | 47.1 | 70.7 | 117.8 | 196.3 |
| 538 | 1.4 | 11.3 | 15.3 | 22.7 | 34.4 | 56.7 | 94.6 |
| 550 | … | 11.3 | 15.3 | 22.7 | 34.4 | 56.7 | 94.6 |
| 575 | … | 10.1 | 13.6 | 20.1 | 30.2 | 50.3 | 83.8 |
| 600 | … | 7.1 | 9.5 | 14.2 | 21.3 | 35.6 | 59.3 |
| 625 | … | 5.3 | 7.1 | 10.6 | 15.9 | 26.5 | 44.2 |
| 650 | … | 3.1 | 4.1 | 6.1 | 9.2 | 15.4 | 25.6 |

Working Pressures by Classes, bar Class

NOTE:  (1) Upon prolonged exposure to temperatures above 465°C, the carbide phase of carbon-molybdenum steel may be converted to graphite. Permissible but not recommended for prolonged use above 465°C.

40

ASME  B16.5-2020

###### Table 2-1.11C Pressure–Temperature Ratings for Group 1.11 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| C–1∕2Mo | … | … | A204 Gr. C (1) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,345 | 2,245 | 3,745 |
| 950 | 35 | 280 | 375 | 560 | 845 | 1,405 | 2,345 |
| 1,000 | 20 | 165 | 220 | 330 | 495 | 825 | 1,370 |
| 1,050 | … | 165 | 220 | 330 | 495 | 825 | 1,370 |
| 1,100 | … | 110 | 145 | 220 | 330 | 550 | 915 |
| 1,150 | … | 80 | 110 | 165 | 245 | 410 | 685 |
| 1,200 | … | 45 | 60 | 90 | 135 | 225 | 370 |

NOTE: (1) Upon prolonged exposure to temperatures above 875°F, the carbide phase of carbon-molybdenum steel may be converted to graphite. Permissible but not recommended for prolonged use above 875°F.

41

ASME  B16.5-2020

###### Table 2-1.13 Pressure–Temperature Ratings for Group 1.13 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 5Cr–1∕2Mo | A182 Gr. F5a | A217 Gr. C5 (1), (2) | … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 27.9 | 37.1 | 55.7 | 83.6 | 139.3 | 232.1 |
| 500 | 2.8 | 21.4 | 28.5 | 42.8 | 64.1 | 106.9 | 178.2 |
| 538 | 1.4 | 13.7 | 18.3 | 27.4 | 41.1 | 68.6 | 114.3 |
| 550 | … | 12.0 | 16.1 | 24.1 | 36.1 | 60.2 | 100.4 |
| 575 | … | 8.9 | 11.8 | 17.8 | 26.7 | 44.4 | 74.0 |
| 600 | … | 6.2 | 8.3 | 12.5 | 18.7 | 31.2 | 51.9 |
| 625 | … | 4.0 | 5.3 | 8.0 | 12.0 | 20.0 | 33.3 |
| 650 | … | 2.4 | 3.2 | 4.7 | 7.1 | 11.8 | 19.7 |

Working Pressures by Classes, bar Class

NOTES:

- Use normalized and tempered material only.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

42

ASME  B16.5-2020

###### Table 2-1.13C Pressure–Temperature Ratings for Group 1.13 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 5Cr–1∕2Mo | A182 Gr. F5a | A217 Gr. C5 (1), (2) | … |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 375 | 500 | 745 | 1,120 | 1,870 | 3,115 |
| 950 | 35 | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,000 | 20 | 200 | 265 | 400 | 595 | 995 | 1,655 |
| 1,050 | … | 145 | 190 | 290 | 430 | 720 | 1,200 |
| 1,100 | … | 100 | 135 | 200 | 300 | 495 | 830 |
| 1,150 | … | 60 | 80 | 125 | 185 | 310 | 515 |
| 1,200 | … | 35 | 45 | 70 | 105 | 170 | 285 |

NOTES:

- Use normalized and tempered material only.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

43

ASME  B16.5-2020

###### Table 2-1.14 Pressure–Temperature Ratings for Group 1.14 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 9Cr–1Mo | A182 Gr. F9 | A217 Gr. C12 (1), (2) | … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 17.5 | 23.3 | 35.0 | 52.5 | 87.5 | 145.8 |
| 550 | … | 15.0 | 20.0 | 30.0 | 45.0 | 75.0 | 125.0 |
| 575 | … | 10.5 | 13.9 | 20.9 | 31.4 | 52.3 | 87.1 |
| 600 | … | 7.2 | 9.6 | 14.4 | 21.5 | 35.9 | 59.8 |
| 625 | … | 5.0 | 6.6 | 9.9 | 14.9 | 24.8 | 41.4 |
| 650 | … | 3.5 | 4.7 | 7.1 | 10.6 | 17.7 | 29.5 |

Working Pressures by Classes, bar Class

NOTES:

- Use normalized and tempered material only.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

44

ASME  B16.5-2020

###### Table 2-1.14C Pressure–Temperature Ratings for Group 1.14 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 9Cr–1Mo | A182 Gr. F9 | A217 Gr. C12 (1), (2) | … |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 375 | 505 | 755 | 1,130 | 1,885 | 3,145 |
| 1,000 | 20 | 255 | 340 | 505 | 760 | 1,270 | 2,115 |
| 1,050 | … | 170 | 230 | 345 | 515 | 855 | 1,430 |
| 1,100 | … | 115 | 150 | 225 | 340 | 565 | 945 |
| 1,150 | … | 75 | 100 | 150 | 225 | 375 | 630 |
| 1,200 | … | 50 | 70 | 105 | 155 | 255 | 430 |

NOTES:

- Use normalized and tempered material only.

- The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

45

ASME  B16.5-2020

ð20Þ

###### Table 2-1.15 Pressure–Temperature Ratings for Group 1.15 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 9Cr–1Mo–V | A182 Gr. F91 Type 1 | A217 Gr. C12A (1) | A387 Gr. 91 Cl. 2 |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 18.6 | 24.8 | 37.2 | 55.9 | 93.1 | 155.1 |
| 625 | … | 12.6 | 16.8 | 25.2 | 37.9 | 63.1 | 105.1 |
| 650 | … | 8.2 | 11.0 | 16.5 | 24.7 | 41.1 | 68.6 |

Working Pressures by Classes, bar Class

NOTE: (1) The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

46

ASME  B16.5-2020

###### Table 2-1.15C Pressure–Temperature Ratings for Group 1.15 Materials	ð20Þ

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 9Cr–1Mo–V | A182 Gr. F91 Type 1 | A217 Gr. C12A (1) | A387 Gr. 91 Cl. 2 |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | … | 300 | 400 | 600 | 895 | 1,490 | 2,485 |
| 1,150 | … | 195 | 260 | 390 | 585 | 975 | 1,630 |
| 1,200 | … | 120 | 160 | 240 | 360 | 600 | 1,000 |

NOTE: (1) The deliberate addition of any element not listed in ASTM A217, Table 1 is prohibited, except that calcium (Ca) and manganese (Mn) may be added for deoxidation.

47

ASME  B16.5-2020

###### Table 2-1.17 Pressure–Temperature Ratings for Group 1.17 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 1Cr–1∕2Mo 5Cr–1∕2Mo | A182 Gr. F12 Cl. 2 (1), (2) A182 Gr. F5 | … … | … … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.8 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.5 | 68.7 | 103.0 | 154.5 | 257.5 | 429.2 |
| 100 | 17.7 | 50.4 | 67.3 | 100.9 | 151.3 | 252.2 | 420.4 |
| 150 | 15.8 | 48.2 | 64.2 | 96.4 | 144.5 | 240.9 | 401.5 |
| 200 | 13.8 | 46.3 | 61.7 | 92.5 | 138.8 | 231.3 | 385.6 |
| 250 | 12.1 | 44.8 | 59.8 | 89.6 | 134.5 | 224.1 | 373.5 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 27.9 | 37.1 | 55.7 | 83.6 | 139.3 | 232.1 |
| 500 | 2.8 | 21.4 | 28.5 | 42.8 | 64.1 | 106.9 | 178.2 |
| 538 | 1.4 | 13.7 | 18.3 | 27.4 | 41.1 | 68.6 | 114.3 |
| 550 | … | 12.0 | 16.1 | 24.1 | 36.1 | 60.2 | 100.4 |
| 575 | … | 8.8 | 11.7 | 17.6 | 26.4 | 44.0 | 73.4 |
| 600 | … | 6.1 | 8.1 | 12.1 | 18.2 | 30.3 | 50.4 |
| 625 | … | 4.0 | 5.3 | 8.0 | 12.0 | 20.0 | 33.3 |
| 650 | … | 2.4 | 3.2 | 4.7 | 7.1 | 11.8 | 19.7 |

Working Pressures by Classes, bar Class

NOTES:

- Use normalized and tempered material only.

- Prolonged use above 590°C is permissible but not recommended.

48

ASME  B16.5-2020

###### Table 2-1.17C Pressure–Temperature Ratings for Group 1.17 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 1Cr–1∕2Mo 5Cr–1∕2Mo | A182 Gr. F12 Cl. 2 (1), (2) A182 Gr. F5 | … … | … … |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 735 | 980 | 1,470 | 2,210 | 3,680 | 6,135 |
| 300 | 230 | 700 | 935 | 1,400 | 2,100 | 3,495 | 5,830 |
| 400 | 200 | 670 | 890 | 1,335 | 2,005 | 3,345 | 5,570 |
| 500 | 170 | 645 | 860 | 1,290 | 1,940 | 3,230 | 5,385 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 375 | 500 | 745 | 1,120 | 1,870 | 3,115 |
| 950 | 35 | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,000 | 20 | 200 | 265 | 400 | 595 | 995 | 1,655 |
| 1,050 | … | 145 | 190 | 290 | 430 | 720 | 1,200 |
| 1,100 | … | 95 | 130 | 190 | 290 | 480 | 800 |
| 1,150 | … | 60 | 80 | 125 | 185 | 310 | 515 |
| 1,200 | … | 35 | 45 | 70 | 105 | 170 | 285 |

NOTES:

- Use normalized and tempered material only.

- Prolonged use above 1,100°F is permissible but not recommended.

49

ASME  B16.5-2020

ð20Þ

###### Table 2-1.18 Pressure–Temperature Ratings for Group 1.18 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 9Cr–2W–V | A 182 Gr. F92 (1) | … | … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | 1.4 | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | 1.4 | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | 1.4 | 21.6 | 28.6 | 42.9 | 64.2 | 107.0 | 178.5 |
| 625 | 1.4 | 18.3 | 24.3 | 36.6 | 54.9 | 91.2 | 152.0 |
| 650 | 1.4 | 13.2 | 18.9 | 26.5 | 39.7 | 66.2 | 110.3 |

Working Pressures by Classes, bar Class

NOTE: (1) Applications above 620°C are limited to tubing with maximum outside diameter of 89 mm.

50

ASME  B16.5-2020

###### Table 2-1.18C Pressure–Temperature Ratings for Group 1.18 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 9Cr–2W–V | A182 Gr. F92 (1) | … | … |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | 20 | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | 20 | 325 | 430 | 645 | 965 | 1,610 | 2,685 |
| 1,150 | 20 | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,200 | 20 | 190 | 275 | 385 | 575 | 960 | 1,600 |

NOTE: (1) Application above 1,150°F is limited to tubing with maximum outside diameter of 31∕2 in.

51

ASME  B16.5-2020

###### Table 2-2.1 Pressure–Temperature Ratings for Group 2.1 Materials

Nominal Designation	Forgings	Castings	Plates

18Cr–8Ni	A182 Gr. F304 (1)	A351 Gr. CF3 (2)	A240 Gr. 304 (1)

18Cr–8Ni	A182 Gr. F304H	A351 Gr. CF8 (1)	A240 Gr. 304H

Working Pressures by Classes, bar Class

Temp., °C

150	300	400	600	900	1500	2500

−29 to 38	19.0	49.6	66.2	99.3	148.9	248.2 	413.7

50	18.3	47.8	63.8	95.6	143.5	239.1	398.5

100	15.7	40.9	54.5	81.7	122.6	204.3	340.4

150	14.2	37.0	49.3	74.0	111.0	185.0	308.4

200	13.2	34.5	46.0	69.0	103.4	172.4	287.3

250	12.1	32.5	43.3	65.0	97.5	162.4	270.7

300	10.2	30.9	41.2	61.8	92.7	154.6	257.6

325	9.3	30.2	40.3	60.4	90.7	151.1	251.9

350	8.4	29.6	39.5	59.3	88.9	148.1	246.9

375	7.4	29.0	38.7	58.1	87.1	145.2	241.9

400	6.5	28.4	37.9	56.9	85.3	142.2	237.0

425	5.5	28.0	37.3	56.0	84.0	140.0	233.3

450	4.6	27.4	36.5	54.8	82.2	137.0	228.4

475	3.7	26.9	35.9	53.9	80.8	134.7	224.5

500	2.8	26.5	35.3	53.0	79.5	132.4	220.7

538	1.4	24.4	32.6	48.9	73.3	122.1	203.6

550	…	23.6	31.4	47.1	70.7	117.8	196.3

575	…	20.8	27.8	41.7	62.5	104.2	173.7

600	…	16.9	22.5	33.8	50.6	84.4	140.7

625	…	13.8	18.4	27.6	41.4	68.9	114.9

650	…	11.3	15.0	22.5	33.8	56.3	93.8

675	…	9.3	12.5	18.7	28.0	46.7	77.9

700	…	8.0	10.7	16.1	24.1	40.1	66.9

725	…	6.8	9.0	13.5	20.3	33.8	56.3

NOTES:

750	…	5.8	7.7	11.6	17.3	28.9	48.1

775	…	4.6	6.2	9.0	13.7	22.8	38.0

800	…	3.5	4.8	7.0	10.5	17.4	29.2

816	…	2.8	3.8	5.9	8.6	14.1	23.8

- At temperatures over 538°C, use only when the carbon content is 0.04% or higher.

- Not to be used over 425°C.

52

ASME  B16.5-2020

###### Table 2-2.1C Pressure–Temperature Ratings for Group 2.1 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–8Ni 18Cr–8Ni | A182 Gr. F304 (1) A182 Gr. F304H | A351 Gr. CF3 (2) A351 Gr. CF8 (1) | A240 Gr. 304 (1) A240 Gr. 304H |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 230 | 600 | 800 | 1,200 | 1,800 | 3,000 | 5,000 |
| 300 | 205 | 540 | 715 | 1,075 | 1,615 | 2,690 | 4,480 |
| 400 | 190 | 495 | 660 | 995 | 1,490 | 2,485 | 4,140 |
| 500 | 170 | 465 | 620 | 930 | 1,395 | 2,330 | 3,880 |
| 600 | 140 | 440 | 590 | 885 | 1,325 | 2,210 | 3,680 |
| 650 | 125 | 430 | 575 | 865 | 1,295 | 2,160 | 3,600 |
| 700 | 110 | 420 | 565 | 845 | 1,265 | 2,110 | 3,520 |
| 750 | 95 | 415 | 550 | 825 | 1,240 | 2,065 | 3,440 |
| 800 | 80 | 405 | 540 | 810 | 1,215 | 2,030 | 3,380 |
| 850 | 65 | 395 | 530 | 790 | 1,190 | 1,980 | 3,300 |
| 900 | 50 | 390 | 520 | 780 | 1,165 | 1,945 | 3,240 |
| 950 | 35 | 380 | 510 | 765 | 1,145 | 1,910 | 3,180 |
| 1,000 | 20 | 355 | 470 | 710 | 1,065 | 1,770 | 2,950 |
| 1,050 | … | 325 | 435 | 650 | 975 | 1,630 | 2,715 |
| 1,100 | … | 255 | 345 | 515 | 770 | 1,285 | 2,145 |
| 1,150 | … | 205 | 275 | 410 | 615 | 1,030 | 1,715 |
| 1,200 | … | 165 | 220 | 330 | 495 | 825 | 1,370 |
| 1,250 | … | 135 | 180 | 265 | 400 | 670 | 1,115 |
| 1,300 | … | 115 | 150 | 225 | 340 | 565 | 945 |
| 1,350 | … | 95 | 125 | 185 | 280 | 465 | 770 |
| 1,400 | … | 75 | 100 | 150 | 225 | 380 | 630 |
| 1,450 | … | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,500 | … | 40 | 55 | 85 | 125 | 205 | 345 |

Working Pressures by Classes, psig Class

NOTES:

- At temperatures over 1,000°F, use only when the carbon content is 0.04% or higher.

- Not to be used over 800°F.

53

ASME  B16.5-2020

###### Table 2-2.2 Pressure–Temperature Ratings for Group 2.2 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 16Cr–12Ni–2Mo 16Cr–12Ni–2Mo 18Cr–13Ni–3Mo 19Cr–10Ni–3Mo | A182 Gr. F316 (1) A182 Gr. F316H A182 Gr. F317 (1) … | A351 Gr. CF3M (2) A351 Gr. CF8M (1) … A351 Gr. CG8M (3) | A240 Gr. 316 (1) A240 Gr. 316H A240 Gr. 317 (1) … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.4 | 48.1 | 64.2 | 96.2 | 144.3 | 240.6 | 400.9 |
| 100 | 16.2 | 42.2 | 56.3 | 84.4 | 126.6 | 211.0 | 351.6 |
| 150 | 14.8 | 38.5 | 51.3 | 77.0 | 115.5 | 192.5 | 320.8 |
| 200 | 13.7 | 35.7 | 47.6 | 71.3 | 107.0 | 178.3 | 297.2 |
| 250 | 12.1 | 33.4 | 44.5 | 66.8 | 100.1 | 166.9 | 278.1 |
| 300 | 10.2 | 31.6 | 42.2 | 63.2 | 94.9 | 158.1 | 263.5 |
| 325 | 9.3 | 30.9 | 41.2 | 61.8 | 92.7 | 154.4 | 257.4 |
| 350 | 8.4 | 30.3 | 40.4 | 60.7 | 91.0 | 151.6 | 252.7 |
| 375 | 7.4 | 29.9 | 39.8 | 59.8 | 89.6 | 149.4 | 249.0 |
| 400 | 6.5 | 29.4 | 39.3 | 58.9 | 88.3 | 147.2 | 245.3 |
| 425 | 5.5 | 29.1 | 38.9 | 58.3 | 87.4 | 145.7 | 242.9 |
| 450 | 4.6 | 28.8 | 38.5 | 57.7 | 86.5 | 144.2 | 240.4 |
| 475 | 3.7 | 28.7 | 38.2 | 57.3 | 86.0 | 143.4 | 238.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 19.9 | 26.5 | 39.8 | 59.7 | 99.5 | 165.9 |
| 625 | … | 15.8 | 21.1 | 31.6 | 47.4 | 79.1 | 131.8 |
| 650 | … | 12.7 | 16.9 | 25.3 | 38.0 | 63.3 | 105.5 |
| 675 | … | 10.3 | 13.8 | 20.6 | 31.0 | 51.6 | 86.0 |
| 700 | … | 8.4 | 11.2 | 16.8 | 25.1 | 41.9 | 69.8 |
| 725 | … | 7.0 | 9.3 | 14.0 | 21.0 | 34.9 | 58.2 |
| 750 | … | 5.9 | 7.8 | 11.7 | 17.6 | 29.3 | 48.9 |
| 775 | … | 4.6 | 6.2 | 9.0 | 13.7 | 22.8 | 38.0 |
| 800 | … | 3.5 | 4.8 | 7.0 | 10.5 | 17.4 | 29.2 |
| 816 | … | 2.8 | 3.8 | 5.9 | 8.6 | 14.1 | 23.8 |

Working Pressures by Classes, bar Class

NOTES:

- At temperatures over 538°C, use only when the carbon content is 0.04% or higher.

- Not to be used over 455°C.

- Not to be used over 538°C.

54

ASME  B16.5-2020

###### Table 2-2.2C Pressure–Temperature Ratings for Group 2.2 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 16Cr–12Ni–2Mo 16Cr–12Ni–2Mo 18Cr–13Ni–3Mo 19Cr–10Ni–3Mo | A182 Gr. F316 (1) A182 Gr. F316H A182 Gr. F317 (1) … | A351 Gr. CF3M (2) A351 Gr. CF8M (1) … A351 Gr. CG8M (3) | A240 Gr. 316 (1) A240 Gr. 316H A240 Gr. 317 (1) … |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 235 | 620 | 825 | 1,240 | 1,860 | 3,095 | 5,160 |
| 300 | 215 | 560 | 745 | 1,120 | 1,680 | 2,795 | 4,660 |
| 400 | 195 | 515 | 685 | 1,025 | 1,540 | 2,570 | 4,280 |
| 500 | 170 | 480 | 635 | 955 | 1,435 | 2,390 | 3,980 |
| 600 | 140 | 450 | 600 | 900 | 1,355 | 2,255 | 3,760 |
| 650 | 125 | 440 | 590 | 885 | 1,325 | 2,210 | 3,680 |
| 700 | 110 | 435 | 580 | 870 | 1,305 | 2,170 | 3,620 |
| 750 | 95 | 425 | 570 | 855 | 1,280 | 2,135 | 3,560 |
| 800 | 80 | 420 | 565 | 845 | 1,265 | 2,110 | 3,520 |
| 850 | 65 | 420 | 555 | 835 | 1,255 | 2,090 | 3,480 |
| 900 | 50 | 415 | 555 | 830 | 1,245 | 2,075 | 3,460 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | … | 305 | 405 | 610 | 915 | 1,525 | 2,545 |
| 1,150 | … | 235 | 315 | 475 | 710 | 1,185 | 1,970 |
| 1,200 | … | 185 | 245 | 370 | 555 | 925 | 1,545 |
| 1,250 | … | 145 | 195 | 295 | 440 | 735 | 1,230 |
| 1,300 | … | 115 | 155 | 235 | 350 | 585 | 970 |
| 1,350 | … | 95 | 130 | 190 | 290 | 480 | 800 |
| 1,400 | … | 75 | 100 | 150 | 225 | 380 | 630 |
| 1,450 | … | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,500 | … | 40 | 55 | 85 | 125 | 205 | 345 |

Working Pressures by Classes, psig Class

NOTES:

- At temperatures over 1,000°F, use only when the carbon content is 0.04% or higher.

- Not to be used over 850°F.

- Not to be used over 1,000°F.

55

ASME  B16.5-2020

###### Table 2-2.3 Pressure–Temperature Ratings for Group 2.3 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–8Ni 16Cr–12Ni–2Mo 18Cr–13Ni–3Mo | A182 Gr. F304L (1) A182 Gr. F316L A182 Gr. F317L | … … … | A240 Gr. 304L (1) A240 Gr. 316L … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 15.9 | 41.4 | 55.2 | 82.7 | 124.1 | 206.8 | 344.7 |
| 50 | 15.3 | 40.0 | 53.4 | 80.0 | 120.1 | 200.1 | 333.5 |
| 100 | 13.3 | 34.8 | 46.4 | 69.6 | 104.4 | 173.9 | 289.9 |
| 150 | 12.0 | 31.4 | 41.9 | 62.8 | 94.2 | 157.0 | 261.6 |
| 200 | 11.2 | 29.2 | 38.9 | 58.3 | 87.5 | 145.8 | 243.0 |
| 250 | 10.5 | 27.5 | 36.6 | 54.9 | 82.4 | 137.3 | 228.9 |
| 300 | 10.0 | 26.1 | 34.8 | 52.1 | 78.2 | 130.3 | 217.2 |
| 325 | 9.3 | 25.5 | 34.0 | 51.0 | 76.4 | 127.4 | 212.3 |
| 350 | 8.4 | 25.1 | 33.4 | 50.1 | 75.2 | 125.4 | 208.9 |
| 375 | 7.4 | 24.8 | 33.0 | 49.5 | 74.3 | 123.8 | 206.3 |
| 400 | 6.5 | 24.3 | 32.4 | 48.6 | 72.9 | 121.5 | 202.5 |
| 425 | 5.5 | 23.9 | 31.8 | 47.7 | 71.6 | 119.3 | 198.8 |
| 450 | 4.6 | 23.4 | 31.2 | 46.8 | 70.2 | 117.1 | 195.1 |

Working Pressures by Classes, bar Class

NOTE: (1) Not to be used over 425°C.

###### Table 2-2.3C Pressure–Temperature Ratings for Group 2.3 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–8Ni 16Cr–12Ni–2Mo 18Cr–13Ni–3Mo | A182 Gr. F304L (1) A182 Gr. F316L A182 Gr. F317L | … … … | A240 Gr. 304L (1) A240 Gr. 316L … |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 230 | 600 | 800 | 1,200 | 1,800 | 3,000 | 5,000 |
| 200 | 195 | 510 | 680 | 1,020 | 1,535 | 2,555 | 4,260 |
| 300 | 175 | 455 | 610 | 910 | 1,370 | 2,280 | 3,800 |
| 400 | 160 | 420 | 560 | 840 | 1,260 | 2,100 | 3,500 |
| 500 | 150 | 395 | 525 | 785 | 1,180 | 1,970 | 3,280 |
| 600 | 140 | 370 | 495 | 745 | 1,115 | 1,860 | 3,100 |
| 650 | 125 | 365 | 485 | 730 | 1,095 | 1,825 | 3,040 |
| 700 | 110 | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 750 | 95 | 355 | 470 | 705 | 1,060 | 1,765 | 2,940 |
| 800 | 80 | 345 | 460 | 690 | 1,035 | 1,730 | 2,880 |
| 850 | 65 | 340 | 450 | 675 | 1,015 | 1,690 | 2,820 |

Working Pressures by Classes, psig Class

NOTE: (1) Not to be used over 800°F.

56

ASME  B16.5-2020

###### Table 2-2.4 Pressure–Temperature Ratings for Group 2.4 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–10Ni–Ti 18Cr–10Ni–Ti | A182 Gr. F321 (1) A182 Gr. F321H (2) | … … | A240 Gr. 321 (1) A240 Gr. 321H (2) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.6 | 48.6 | 64.7 | 97.1 | 145.7 | 242.8 | 404.6 |
| 100 | 17.0 | 44.2 | 59.0 | 88.5 | 132.7 | 221.2 | 368.7 |
| 150 | 15.7 | 41.0 | 54.6 | 82.0 | 122.9 | 204.9 | 341.5 |
| 200 | 13.8 | 38.3 | 51.1 | 76.6 | 114.9 | 191.5 | 319.1 |
| 250 | 12.1 | 36.0 | 48.0 | 72.0 | 108.1 | 180.1 | 300.2 |
| 300 | 10.2 | 34.1 | 45.5 | 68.3 | 102.4 | 170.7 | 284.6 |
| 325 | 9.3 | 33.3 | 44.4 | 66.6 | 99.9 | 166.5 | 277.6 |
| 350 | 8.4 | 32.6 | 43.5 | 65.2 | 97.8 | 163.0 | 271.7 |
| 375 | 7.4 | 32.0 | 42.7 | 64.1 | 96.1 | 160.2 | 266.9 |
| 400 | 6.5 | 31.6 | 42.1 | 63.2 | 94.8 | 157.9 | 263.2 |
| 425 | 5.5 | 31.1 | 41.5 | 62.3 | 93.4 | 155.7 | 259.5 |
| 450 | 4.6 | 30.8 | 41.1 | 61.7 | 92.5 | 154.2 | 256.9 |
| 475 | 3.7 | 30.5 | 40.7 | 61.1 | 91.6 | 152.7 | 254.4 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 20.3 | 27.0 | 40.5 | 60.8 | 101.3 | 168.9 |
| 625 | … | 15.8 | 21.1 | 31.6 | 47.4 | 79.1 | 131.8 |
| 650 | … | 12.6 | 16.9 | 25.3 | 37.9 | 63.2 | 105.4 |
| 675 | … | 9.9 | 13.2 | 19.8 | 29.6 | 49.4 | 82.3 |
| 700 | … | 7.9 | 10.5 | 15.8 | 23.7 | 39.5 | 65.9 |
| 725 | … | 6.3 | 8.5 | 12.7 | 19.0 | 31.7 | 52.8 |
| 750 | … | 5.0 | 6.7 | 10.0 | 15.0 | 25.0 | 41.7 |
| 775 | … | 4.0 | 5.3 | 8.0 | 11.9 | 19.9 | 33.2 |
| 800 | … | 3.1 | 4.2 | 6.3 | 9.4 | 15.6 | 26.1 |
| 816 | … | 2.6 | 3.5 | 5.2 | 7.8 | 13.0 | 21.7 |

Working Pressures by Classes, bar Class

NOTES:

- Not to be used over 538°C.

- At temperatures over 538°C, use only if the material is heat treated to a minimum temperature of 1 095°C.

57

ASME  B16.5-2020

###### Table 2-2.4C Pressure–Temperature Ratings for Group 2.4 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–10Ni–Ti 18Cr–10Ni–Ti | A182 Gr. F321 (1) A182 Gr. F321H (2) | … … | A240 Gr. 321 (1) A240 Gr. 321H (2) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 250 | 650 | 865 | 1,295 | 1,945 | 3,240 | 5,400 |
| 300 | 230 | 595 | 795 | 1,190 | 1,785 | 2,975 | 4,960 |
| 400 | 200 | 550 | 735 | 1,105 | 1,655 | 2,760 | 4,600 |
| 500 | 170 | 515 | 690 | 1,030 | 1,550 | 2,580 | 4,300 |
| 600 | 140 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 650 | 125 | 475 | 635 | 950 | 1,425 | 2,375 | 3,960 |
| 700 | 110 | 465 | 620 | 930 | 1,395 | 2,330 | 3,880 |
| 750 | 95 | 460 | 610 | 915 | 1,375 | 2,290 | 3,820 |
| 800 | 80 | 450 | 600 | 900 | 1,355 | 2,255 | 3,760 |
| 850 | 65 | 445 | 595 | 895 | 1,340 | 2,230 | 3,720 |
| 900 | 50 | 440 | 590 | 885 | 1,325 | 2,210 | 3,680 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | … | 310 | 415 | 625 | 935 | 1,560 | 2,600 |
| 1,150 | … | 235 | 315 | 475 | 710 | 1,185 | 1,970 |
| 1,200 | … | 185 | 245 | 370 | 555 | 925 | 1,545 |
| 1,250 | … | 140 | 185 | 280 | 420 | 705 | 1,170 |
| 1,300 | … | 110 | 145 | 220 | 330 | 550 | 915 |
| 1,350 | … | 85 | 115 | 170 | 255 | 430 | 715 |
| 1,400 | … | 65 | 85 | 130 | 195 | 325 | 545 |
| 1,450 | … | 50 | 70 | 105 | 155 | 255 | 430 |
| 1,500 | … | 40 | 50 | 75 | 115 | 190 | 315 |

Working Pressures by Classes, psig Class

NOTES:

- Not to be used over 1,000°F.

- At temperatures over 1,000°F, use only if the material is heat treated to a minimum temperature of 2,000°F.

58

ASME  B16.5-2020

###### Table 2-2.5 Pressure–Temperature Ratings for Group 2.5 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–10Ni–Cb 18Cr–10Ni–Cb 18Cr–10Ni–Cb 18Cr–10Ni–Cb | A182 Gr. F347 (1) A182 Gr. F347H (2) A182 Gr. F348 (1) A182 Gr. F348H (2) | … … … … | A240 Gr. 347 (1) A240 Gr. 347H (2) A240 Gr. 348 (1) A240 Gr. 348H (2) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.7 | 48.8 | 65.0 | 97.5 | 146.3 | 243.8 | 406.4 |
| 100 | 17.4 | 45.3 | 60.4 | 90.6 | 135.9 | 226.5 | 377.4 |
| 150 | 15.8 | 42.5 | 56.6 | 84.9 | 127.4 | 212.4 | 353.9 |
| 200 | 13.8 | 39.9 | 53.3 | 79.9 | 119.8 | 199.7 | 332.8 |
| 250 | 12.1 | 37.8 | 50.4 | 75.6 | 113.4 | 189.1 | 315.1 |
| 300 | 10.2 | 36.1 | 48.1 | 72.2 | 108.3 | 180.4 | 300.7 |
| 325 | 9.3 | 35.4 | 47.1 | 70.7 | 106.1 | 176.8 | 294.6 |
| 350 | 8.4 | 34.8 | 46.3 | 69.5 | 104.3 | 173.8 | 289.6 |
| 375 | 7.4 | 34.2 | 45.6 | 68.4 | 102.6 | 171.0 | 285.1 |
| 400 | 6.5 | 33.9 | 45.2 | 67.8 | 101.7 | 169.5 | 282.6 |
| 425 | 5.5 | 33.6 | 44.8 | 67.2 | 100.8 | 168.1 | 280.1 |
| 450 | 4.6 | 33.5 | 44.6 | 66.9 | 100.4 | 167.3 | 278.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 21.6 | 28.6 | 42.9 | 64.2 | 107.0 | 178.5 |
| 625 | … | 18.3 | 24.3 | 36.6 | 54.9 | 91.2 | 152.0 |
| 650 | … | 14.1 | 18.9 | 28.1 | 42.5 | 70.7 | 117.7 |
| 675 | … | 12.4 | 16.9 | 25.2 | 37.6 | 62.7 | 104.5 |
| 700 | … | 10.1 | 13.4 | 20.0 | 29.8 | 49.7 | 83.0 |
| 725 | … | 7.9 | 10.5 | 15.4 | 23.2 | 38.6 | 64.4 |
| 750 | … | 5.9 | 7.9 | 11.7 | 17.6 | 29.6 | 49.1 |
| 775 | … | 4.6 | 6.2 | 9.0 | 13.7 | 22.8 | 38.0 |
| 800 | … | 3.5 | 4.8 | 7.0 | 10.5 | 17.4 | 29.2 |
| 816 | … | 2.8 | 3.8 | 5.9 | 8.6 | 14.1 | 23.8 |

Working Pressures by Classes, bar Class

NOTES:

- Not to be used over 538°C.

- For temperatures over 538°C, use only if the material is heat treated by heating to a minimum temperature of 1 095°C.

59

ASME  B16.5-2020

###### Table 2-2.5C Pressure–Temperature Ratings for Group 2.5 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–10Ni–Cb 18Cr–10Ni–Cb 18Cr–10Ni–Cb 18Cr–10Ni–Cb | A182 Gr. F347 (1) A182 Gr. F347H (2) A182 Gr. F348 (1) A182 Gr. F348H (2) | … … … … | A240 Gr. 347 (1) A240 Gr. 347H (2) A240 Gr. 348 (1) A240 Gr. 348H (2) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 255 | 660 | 885 | 1,325 | 1,985 | 3,310 | 5,520 |
| 300 | 230 | 615 | 820 | 1,235 | 1,850 | 3,085 | 5,140 |
| 400 | 200 | 575 | 770 | 1,150 | 1,730 | 2,880 | 4,800 |
| 500 | 170 | 540 | 725 | 1,085 | 1,625 | 2,710 | 4,520 |
| 600 | 140 | 515 | 690 | 1,030 | 1,550 | 2,580 | 4,300 |
| 650 | 125 | 505 | 675 | 1,015 | 1,520 | 2,530 | 4,220 |
| 700 | 110 | 495 | 660 | 995 | 1,490 | 2,485 | 4,140 |
| 750 | 95 | 490 | 655 | 985 | 1,475 | 2,460 | 4,100 |
| 800 | 80 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 850 | 65 | 485 | 645 | 970 | 1,455 | 2,425 | 4,040 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | … | 325 | 430 | 645 | 965 | 1,610 | 2,685 |
| 1,150 | … | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,200 | … | 205 | 275 | 410 | 620 | 1,030 | 1,715 |
| 1,250 | … | 180 | 245 | 365 | 545 | 910 | 1,515 |
| 1,300 | … | 140 | 185 | 275 | 410 | 685 | 1,145 |
| 1,350 | … | 105 | 140 | 205 | 310 | 515 | 860 |
| 1,400 | … | 75 | 100 | 150 | 225 | 380 | 630 |
| 1,450 | … | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,500 | … | 40 | 55 | 85 | 125 | 205 | 345 |

Working Pressures by Classes, psig Class

NOTES:

- Not to be used over 1,000°F.

- For temperatures over 1,000°F use only if the material is heat treated by heating to a minimum temperature of 2,000°F.

60

ASME  B16.5-2020

###### Table 2-2.6 Pressure–Temperature Ratings for Group 2.6 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 23Cr–12Ni | … | … | A240 Gr. 309H |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.5 | 48.3 | 64.4 | 96.6 | 144.9 | 241.5 | 402.5 |
| 100 | 16.5 | 43.1 | 57.5 | 86.2 | 129.3 | 215.5 | 359.2 |
| 150 | 15.3 | 40.0 | 53.3 | 80.0 | 120.0 | 200.0 | 333.3 |
| 200 | 13.8 | 37.8 | 50.3 | 75.5 | 113.3 | 188.8 | 314.7 |
| 250 | 12.1 | 36.1 | 48.1 | 72.1 | 108.2 | 180.4 | 300.6 |
| 300 | 10.2 | 34.8 | 46.4 | 69.6 | 104.4 | 173.9 | 289.9 |
| 325 | 9.3 | 34.2 | 45.7 | 68.5 | 102.7 | 171.2 | 285.4 |
| 350 | 8.4 | 33.8 | 45.1 | 67.6 | 101.4 | 169.0 | 281.7 |
| 375 | 7.4 | 33.4 | 44.5 | 66.8 | 100.1 | 166.9 | 278.2 |
| 400 | 6.5 | 33.1 | 44.1 | 66.1 | 99.2 | 165.4 | 275.6 |
| 425 | 5.5 | 32.6 | 43.5 | 65.3 | 97.9 | 163.1 | 271.9 |
| 450 | 4.6 | 32.2 | 42.9 | 64.4 | 96.5 | 160.9 | 268.2 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 22.2 | 29.6 | 44.4 | 66.5 | 110.9 | 184.8 |
| 600 | … | 16.8 | 22.4 | 33.5 | 50.3 | 83.9 | 139.8 |
| 625 | … | 12.5 | 16.7 | 25.0 | 37.5 | 62.5 | 104.2 |
| 650 | … | 9.4 | 12.5 | 18.7 | 28.1 | 46.8 | 78.0 |
| 675 | … | 7.2 | 9.6 | 14.5 | 21.7 | 36.2 | 60.3 |
| 700 | … | 5.5 | 7.3 | 11.0 | 16.5 | 27.5 | 45.9 |
| 725 | … | 4.3 | 5.8 | 8.7 | 13.0 | 21.6 | 36.0 |
| 750 | … | 3.4 | 4.6 | 6.8 | 10.2 | 17.1 | 28.4 |
| 775 | … | 2.7 | 3.6 | 5.4 | 8.1 | 13.5 | 22.4 |
| 800 | … | 2.1 | 2.8 | 4.2 | 6.3 | 10.5 | 17.5 |
| 816 | … | 1.8 | 2.4 | 3.5 | 5.3 | 8.9 | 14.8 |

Working Pressures by Classes, bar Class

61

ASME  B16.5-2020

###### Table 2-2.6C Pressure–Temperature Ratings for Group 2.6 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 23Cr–12Ni | … | … | A240 Gr. 309H |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 240 | 630 | 840 | 1,260 | 1,895 | 3,155 | 5,260 |
| 300 | 225 | 580 | 775 | 1,160 | 1,740 | 2,905 | 4,840 |
| 400 | 200 | 545 | 725 | 1,090 | 1,635 | 2,725 | 4,540 |
| 500 | 170 | 520 | 690 | 1,035 | 1,555 | 2,590 | 4,320 |
| 600 | 140 | 500 | 665 | 1,000 | 1,500 | 2,495 | 4,160 |
| 650 | 125 | 490 | 655 | 985 | 1,475 | 2,460 | 4,100 |
| 700 | 110 | 485 | 645 | 970 | 1,455 | 2,425 | 4,040 |
| 750 | 95 | 480 | 640 | 960 | 1,440 | 2,400 | 4,000 |
| 800 | 80 | 475 | 630 | 945 | 1,420 | 2,365 | 3,940 |
| 850 | 65 | 465 | 620 | 930 | 1,395 | 2,330 | 3,880 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 355 | 470 | 705 | 1,060 | 1,765 | 2,945 |
| 1,100 | … | 260 | 345 | 520 | 780 | 1,305 | 2,170 |
| 1,150 | … | 190 | 250 | 375 | 565 | 945 | 1,570 |
| 1,200 | … | 135 | 185 | 275 | 410 | 685 | 1,145 |
| 1,250 | … | 105 | 135 | 205 | 310 | 515 | 855 |
| 1,300 | … | 75 | 100 | 150 | 225 | 375 | 630 |
| 1,350 | … | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,400 | … | 45 | 60 | 90 | 135 | 225 | 370 |
| 1,450 | … | 35 | 45 | 70 | 105 | 170 | 285 |
| 1,500 | … | 25 | 35 | 50 | 75 | 130 | 215 |

Working Pressures by Classes, psig Class

62

ASME  B16.5-2020

###### Table 2-2.7 Pressure–Temperature Ratings for Group 2.7 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 25Cr–20Ni | A182 Gr. F310 (1), (2) | … | A240 Gr. 310H |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.5 | 48.4 | 64.5 | 96.7 | 145.1 | 241.8 | 403.1 |
| 100 | 16.6 | 43.4 | 57.9 | 86.8 | 130.2 | 217.0 | 361.6 |
| 150 | 15.3 | 40.0 | 53.3 | 80.0 | 120.0 | 200.0 | 333.3 |
| 200 | 13.8 | 37.6 | 50.1 | 75.2 | 112.8 | 188.0 | 313.4 |
| 250 | 12.1 | 35.8 | 47.7 | 71.5 | 107.3 | 178.8 | 298.1 |
| 300 | 10.2 | 34.5 | 45.9 | 68.9 | 103.4 | 172.3 | 287.2 |
| 325 | 9.3 | 33.9 | 45.2 | 67.7 | 101.6 | 169.3 | 282.2 |
| 350 | 8.4 | 33.3 | 44.4 | 66.6 | 99.9 | 166.5 | 277.6 |
| 375 | 7.4 | 32.9 | 43.8 | 65.7 | 98.6 | 164.3 | 273.8 |
| 400 | 6.5 | 32.4 | 43.2 | 64.8 | 97.3 | 162.1 | 270.2 |
| 425 | 5.5 | 32.1 | 42.8 | 64.2 | 96.4 | 160.6 | 267.7 |
| 450 | 4.6 | 31.7 | 42.2 | 63.4 | 95.1 | 158.4 | 264.0 |
| 475 | 3.7 | 31.2 | 41.7 | 62.5 | 93.7 | 156.2 | 260.3 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 22.2 | 29.6 | 44.4 | 66.5 | 110.9 | 184.8 |
| 600 | … | 16.8 | 22.4 | 33.5 | 50.3 | 83.9 | 139.8 |
| 625 | … | 12.5 | 16.7 | 25.0 | 37.5 | 62.5 | 104.2 |
| 650 | … | 9.4 | 12.5 | 18.7 | 28.1 | 46.8 | 78.0 |
| 675 | … | 7.2 | 9.6 | 14.5 | 21.7 | 36.2 | 60.3 |
| 700 | … | 5.5 | 7.3 | 11.0 | 16.5 | 27.5 | 45.9 |
| 725 | … | 4.3 | 5.8 | 8.7 | 13.0 | 21.6 | 36.0 |
| 750 | … | 3.4 | 4.6 | 6.8 | 10.2 | 17.1 | 28.4 |
| 775 | … | 2.7 | 3.5 | 5.3 | 8.0 | 13.3 | 22.1 |
| 800 | … | 2.1 | 2.8 | 4.1 | 6.2 | 10.3 | 17.2 |
| 816 | … | 1.8 | 2.4 | 3.5 | 5.3 | 8.9 | 14.8 |

Working Pressures by Classes, bar Class

NOTES:

- At temperatures over 538°C, use only when the carbon content is 0.04% or higher.

- Service temperatures of 565°C and above should be used only when assurance is provided that grain size is not finer than ASTM No. 6.

63

ASME  B16.5-2020

###### Table 2-2.7C Pressure–Temperature Ratings for Group 2.7 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 25Cr–20Ni | A182 Gr. F310 (1), (2) | … | A240 Gr. 310H |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 245 | 635 | 850 | 1,270 | 1,910 | 3,180 | 5,300 |
| 300 | 225 | 580 | 775 | 1,160 | 1,740 | 2,905 | 4,840 |
| 400 | 200 | 540 | 725 | 1,085 | 1,625 | 2,710 | 4,520 |
| 500 | 170 | 515 | 685 | 1,025 | 1,540 | 2,570 | 4,280 |
| 600 | 140 | 495 | 660 | 990 | 1,485 | 2,470 | 4,120 |
| 650 | 125 | 485 | 645 | 970 | 1,455 | 2,425 | 4,040 |
| 700 | 110 | 480 | 635 | 955 | 1,435 | 2,390 | 3,980 |
| 750 | 95 | 470 | 625 | 940 | 1,410 | 2,350 | 3,920 |
| 800 | 80 | 465 | 620 | 930 | 1,395 | 2,330 | 3,880 |
| 850 | 65 | 460 | 610 | 915 | 1,375 | 2,290 | 3,820 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 355 | 470 | 705 | 1,060 | 1,765 | 2,945 |
| 1,100 | … | 260 | 345 | 520 | 780 | 1,305 | 2,170 |
| 1,150 | … | 190 | 250 | 375 | 565 | 945 | 1,570 |
| 1,200 | … | 135 | 185 | 275 | 410 | 685 | 1,145 |
| 1,250 | … | 105 | 135 | 205 | 310 | 515 | 855 |
| 1,300 | … | 75 | 100 | 150 | 225 | 375 | 630 |
| 1,350 | … | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,400 | … | 45 | 60 | 90 | 135 | 225 | 370 |
| 1,450 | … | 35 | 45 | 65 | 100 | 165 | 275 |
| 1,500 | … | 25 | 35 | 50 | 75 | 130 | 215 |

Working Pressures by Classes, psig Class

NOTES:

- At temperatures over 1,000°F, use only when the carbon content is 0.04% or higher.

- Service temperatures of 1,050°F and above should be used only when assurance is provided that grain size is not finer than ASTM No. 6.

64

ASME  B16.5-2020

###### Table 2-2.8 Pressure–Temperature Ratings for Group 2.8 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 20Cr–18Ni–6Mo | A182 Gr. F44 | A351 Gr. CK3MCuN | A240 Gr. S31254 |
| 22Cr–5Ni–3Mo–N | A182 Gr. F51 (1) | … | A240 Gr. S31803 (1) |
| 25Cr–7Ni–4Mo–N | A182 Gr. F53 (1) | … | A240 Gr. S32750 (1) |
| 24Cr–10Ni–4Mo–V | … | A995 Gr. CE8MN (1) | … |
| 25Cr–5Ni–2Mo–3Cu–N | … | A995 Gr. CD4MCuN (1) | … |
| 25Cr–7Ni–3.5Mo–W–Cb | … | A995 Gr. CD3MWCuN (1) | … |
| 25Cr–7Ni–3.5Mo–N–Cu–W | A182 Gr. F55 (1) | … | A240 Gr. S32760 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 50.7 | 67.5 | 101.3 | 152.0 | 253.3 | 422.2 |
| 150 | 15.8 | 45.9 | 61.2 | 91.9 | 137.8 | 229.6 | 382.7 |
| 200 | 13.8 | 42.7 | 56.9 | 85.3 | 128.0 | 213.3 | 355.4 |
| 250 | 12.1 | 40.5 | 53.9 | 80.9 | 121.4 | 202.3 | 337.2 |
| 300 | 10.2 | 38.9 | 51.8 | 77.7 | 116.6 | 194.3 | 323.8 |
| 325 | 9.3 | 38.2 | 50.9 | 76.3 | 114.5 | 190.8 | 318.0 |
| 350 | 8.4 | 37.6 | 50.2 | 75.3 | 112.9 | 188.2 | 313.7 |
| 375 | 7.4 | 37.4 | 49.8 | 74.7 | 112.1 | 186.8 | 311.3 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |

Working Pressures by Classes, bar Class

NOTE:  (1)  This steel may become brittle after service at moderately elevated temperatures. Not to be used over 315°C.

###### Table 2-2.8C Pressure–Temperature Ratings for Group 2.8 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 20Cr–18Ni–6Mo | A182 Gr. F44 | A351 Gr. CK3MCuN | A240 Gr. S31254 |
| 22Cr–5Ni–3Mo–N | A182 Gr. F51 (1) | … | A240 Gr. S31803 (1) |
| 25Cr–7Ni–4Mo–N | A182 Gr. F53 (1) | … | A240 Gr. S32750 (1) |
| 24Cr–10Ni–4Mo–V | … | A995 Gr. CE8MN (1) | … |
| 25Cr–5Ni–2Mo–3Cu–N | … | A995 Gr. CD4MCuN (1) | … |
| 25Cr–7Ni–3.5Mo–W–Cb | … | A995 Gr. CD3MWCuN (1) | … |
| 25Cr–7Ni–3.5Mo–N–Cu–W | A182 Gr. F55 (1) | … | A240 Gr. S32760 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 745 | 990 | 1,490 | 2,230 | 3,720 | 6,200 |
| 300 | 230 | 665 | 890 | 1,335 | 2,000 | 3,335 | 5,560 |
| 400 | 200 | 615 | 820 | 1,230 | 1,845 | 3,070 | 5,120 |
| 500 | 170 | 580 | 775 | 1,160 | 1,740 | 2,905 | 4,840 |
| 600 | 140 | 555 | 740 | 1,115 | 1,670 | 2,785 | 4,640 |
| 650 | 125 | 545 | 730 | 1,095 | 1,640 | 2,735 | 4,560 |
| 700 | 110 | 540 | 725 | 1,085 | 1,625 | 2,710 | 4,520 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |

Working Pressures by Classes, psig Class

NOTE:  (1)  This steel may become brittle after service at moderately elevated temperatures. Not to be used over 600°F.

65

ASME  B16.5-2020

###### Table 2-2.9 Pressure–Temperature Ratings for Group 2.9 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 23Cr–12Ni 25Cr–20Ni | … … | … … | A240 Gr. 309S (1)–(3) A240 Gr. 310S (1)–(3) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.5 | 48.3 | 64.4 | 96.6 | 144.9 | 241.5 | 402.5 |
| 100 | 16.5 | 43.1 | 57.5 | 86.2 | 129.3 | 215.5 | 359.2 |
| 150 | 15.3 | 40.0 | 53.3 | 80.0 | 120.0 | 200.0 | 333.3 |
| 200 | 13.8 | 37.6 | 50.1 | 75.2 | 112.8 | 188.0 | 313.4 |
| 250 | 12.1 | 35.8 | 47.7 | 71.5 | 107.3 | 178.8 | 298.1 |
| 300 | 10.2 | 34.5 | 45.9 | 68.9 | 103.4 | 172.3 | 287.2 |
| 325 | 9.3 | 33.9 | 45.2 | 67.7 | 101.6 | 169.3 | 282.2 |
| 350 | 8.4 | 33.3 | 44.4 | 66.6 | 99.9 | 166.5 | 277.6 |
| 375 | 7.4 | 32.9 | 43.8 | 65.7 | 98.6 | 164.3 | 273.8 |
| 400 | 6.5 | 32.4 | 43.2 | 64.8 | 97.3 | 162.1 | 270.2 |
| 425 | 5.5 | 32.1 | 42.8 | 64.2 | 96.4 | 160.6 | 267.7 |
| 450 | 4.6 | 31.7 | 42.2 | 63.4 | 95.1 | 158.4 | 264.0 |
| 475 | 3.7 | 31.2 | 41.7 | 62.5 | 93.7 | 156.2 | 260.3 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 23.4 | 31.2 | 46.8 | 70.2 | 117.0 | 195.0 |
| 550 | … | 20.5 | 27.3 | 41.0 | 61.5 | 102.5 | 170.8 |
| 575 | … | 15.1 | 20.1 | 30.2 | 45.3 | 75.5 | 125.8 |
| 600 | … | 11.0 | 14.7 | 22.1 | 33.1 | 55.1 | 91.9 |
| 625 | … | 8.1 | 10.9 | 16.3 | 24.4 | 40.7 | 67.9 |
| 650 | … | 5.8 | 7.8 | 11.6 | 17.4 | 29.1 | 48.5 |
| 675 | … | 3.7 | 4.9 | 7.4 | 11.1 | 18.4 | 30.7 |
| 700 | … | 2.2 | 2.9 | 4.3 | 6.5 | 10.8 | 18.0 |
| 725 | … | 1.4 | 1.8 | 2.7 | 4.1 | 6.8 | 11.4 |
| 750 | … | 1.0 | 1.4 | 2.1 | 3.1 | 5.2 | 8.6 |
| 775 | … | 0.8 | 1.1 | 1.6 | 2.5 | 4.1 | 6.8 |
| 800 | … | 0.6 | 0.8 | 1.2 | 1.8 | 3.0 | 5.0 |
| 816 | … | 0.5 | 0.6 | 0.9 | 1.4 | 2.4 | 3.9 |

Working Pressures by Classes, bar Class

NOTES:

- At temperatures over 538°C, use only when the carbon content is 0.04% or higher.

- At temperatures above 538°C, use only if the material is solution heat treated to the minimum temperature specified in the specification but not lower than 1 035°C, and quenching in water or rapidly cooling by other means.

- This material should be used for service temperatures 565°C and above only when assurance is provided that grain size is not finer than ASTM

No. 6.

66

ASME  B16.5-2020

###### Table 2-2.9C Pressure–Temperature Ratings for Group 2.9 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 23Cr–12Ni 25Cr–20Ni | … … | … … | A240 Gr. 309S (1)–(3) A240 Gr. 310S (1)–(3) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 240 | 630 | 840 | 1,260 | 1,895 | 3,155 | 5,260 |
| 300 | 225 | 580 | 775 | 1,160 | 1,740 | 2,905 | 4,840 |
| 400 | 200 | 540 | 725 | 1,085 | 1,625 | 2,710 | 4,520 |
| 500 | 170 | 515 | 685 | 1,025 | 1,540 | 2,570 | 4,280 |
| 600 | 140 | 495 | 660 | 990 | 1,485 | 2,470 | 4,120 |
| 650 | 125 | 485 | 645 | 970 | 1,455 | 2,425 | 4,040 |
| 700 | 110 | 480 | 635 | 955 | 1,435 | 2,390 | 3,980 |
| 750 | 95 | 470 | 625 | 940 | 1,410 | 2,350 | 3,920 |
| 800 | 80 | 465 | 620 | 930 | 1,395 | 2,330 | 3,880 |
| 850 | 65 | 460 | 610 | 915 | 1,375 | 2,290 | 3,820 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 340 | 455 | 680 | 1,020 | 1,695 | 2,830 |
| 1,050 | … | 245 | 325 | 485 | 730 | 1,215 | 2,030 |
| 1,100 | … | 170 | 230 | 345 | 515 | 855 | 1,430 |
| 1,150 | … | 125 | 165 | 245 | 370 | 615 | 1,030 |
| 1,200 | … | 85 | 115 | 170 | 255 | 430 | 715 |
| 1,250 | … | 50 | 70 | 105 | 155 | 255 | 430 |
| 1,300 | … | 25 | 35 | 55 | 80 | 135 | 230 |
| 1,350 | … | 15 | 25 | 35 | 50 | 85 | 145 |
| 1,400 | … | 15 | 20 | 25 | 40 | 70 | 115 |
| 1,450 | … | 10 | 15 | 20 | 30 | 50 | 85 |
| 1,500 | … | 5 | 10 | 15 | 20 | 35 | 55 |

Working Pressures by Classes, psig Class

NOTES:

- At temperatures over 1,000°F, use only when the carbon content is 0.04% or higher.

- At temperatures above 1,000°F, use only if the material is solution heat treated to the minimum temperature specified in the specification, but not lower than 1,900°F, and quenching in water or rapidly cooling by other means.

- This material should be used for service temperatures 1,050°F and above only when assurance is provided that grain size is not finer than

ASTM No. 6.

67

ASME  B16.5-2020

###### Table 2-2.10 Pressure–Temperature Ratings for Group 2.10 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 25Cr–12Ni 25Cr–12Ni | … … | A351 Gr. CH8 (1) A351 Gr. CH20 (1) | … … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 17.8 | 46.3 | 61.8 | 92.7 | 139.0 | 231.7 | 386.1 |
| 50 | 17.0 | 44.5 | 59.3 | 89.0 | 133.4 | 222.4 | 370.6 |
| 100 | 14.4 | 37.5 | 50.0 | 75.1 | 112.6 | 187.7 | 312.8 |
| 150 | 13.4 | 34.9 | 46.5 | 69.8 | 104.7 | 174.4 | 290.7 |
| 200 | 12.9 | 33.5 | 44.7 | 67.1 | 100.6 | 167.7 | 279.5 |
| 250 | 12.1 | 32.6 | 43.5 | 65.2 | 97.8 | 163.1 | 271.8 |
| 300 | 10.2 | 31.7 | 42.3 | 63.4 | 95.2 | 158.6 | 264.3 |
| 325 | 9.3 | 31.2 | 41.6 | 62.4 | 93.6 | 156.1 | 260.1 |
| 350 | 8.4 | 30.6 | 40.8 | 61.2 | 91.7 | 152.9 | 254.8 |
| 375 | 7.4 | 29.8 | 39.8 | 59.7 | 89.5 | 149.2 | 248.6 |
| 400 | 6.5 | 29.1 | 38.8 | 58.2 | 87.3 | 145.5 | 242.4 |
| 425 | 5.5 | 28.3 | 37.8 | 56.7 | 85.0 | 141.7 | 236.2 |
| 450 | 4.6 | 27.6 | 36.8 | 55.2 | 82.8 | 138.0 | 230.0 |
| 475 | 3.7 | 26.7 | 35.6 | 53.5 | 80.2 | 133.7 | 222.8 |
| 500 | 2.8 | 25.8 | 34.5 | 51.7 | 77.5 | 129.2 | 215.3 |
| 538 | 1.4 | 23.3 | 31.1 | 46.6 | 70.0 | 116.6 | 194.4 |
| 550 | … | 21.9 | 29.2 | 43.8 | 65.7 | 109.5 | 182.5 |
| 575 | … | 18.5 | 24.6 | 37.0 | 55.5 | 92.4 | 154.0 |
| 600 | … | 14.5 | 19.4 | 29.0 | 43.5 | 72.6 | 121.0 |
| 625 | … | 11.4 | 15.2 | 22.8 | 34.3 | 57.1 | 95.2 |
| 650 | … | 8.9 | 11.9 | 17.8 | 26.7 | 44.5 | 74.1 |
| 675 | … | 7.0 | 9.3 | 14.0 | 20.9 | 34.9 | 58.2 |
| 700 | … | 5.7 | 7.6 | 11.3 | 17.0 | 28.3 | 47.2 |
| 725 | … | 4.6 | 6.1 | 9.1 | 13.7 | 22.8 | 38.0 |
| 750 | … | 3.5 | 4.7 | 7.0 | 10.5 | 17.5 | 29.2 |
| 775 | … | 2.6 | 3.4 | 5.1 | 7.7 | 12.8 | 21.4 |
| 800 | … | 2.0 | 2.7 | 4.0 | 6.1 | 10.1 | 16.9 |
| 816 | … | 1.9 | 2.5 | 3.8 | 5.7 | 9.5 | 15.8 |

Working Pressures by Classes, bar Class

NOTE: (1) At temperatures over 538°C, use only when the carbon content is 0.04% or higher.

68

ASME  B16.5-2020

###### Table 2-2.10C Pressure–Temperature Ratings for Group 2.10 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 25Cr–12Ni 25Cr–12Ni | … … | A351 Gr. CH8 (1) A351 Gr. CH20 (1) | … … |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 260 | 670 | 895 | 1,345 | 2,015 | 3,360 | 5,600 |
| 200 | 210 | 550 | 735 | 1,100 | 1,650 | 2,750 | 4,580 |
| 300 | 195 | 505 | 675 | 1,015 | 1,520 | 2,530 | 4,220 |
| 400 | 185 | 485 | 645 | 970 | 1,455 | 2,425 | 4,040 |
| 500 | 170 | 470 | 625 | 940 | 1,410 | 2,350 | 3,920 |
| 600 | 140 | 455 | 610 | 910 | 1,370 | 2,280 | 3,800 |
| 650 | 125 | 445 | 595 | 895 | 1,340 | 2,230 | 3,720 |
| 700 | 110 | 435 | 580 | 870 | 1,305 | 2,170 | 3,620 |
| 750 | 95 | 420 | 565 | 845 | 1,265 | 2,110 | 3,520 |
| 800 | 80 | 410 | 545 | 820 | 1,230 | 2,050 | 3,420 |
| 850 | 65 | 400 | 530 | 795 | 1,195 | 1,990 | 3,320 |
| 900 | 50 | 385 | 510 | 770 | 1,150 | 1,920 | 3,200 |
| 950 | 35 | 370 | 495 | 740 | 1,110 | 1,850 | 3,080 |
| 1,000 | 20 | 340 | 450 | 675 | 1,015 | 1,690 | 2,820 |
| 1,050 | … | 290 | 390 | 585 | 875 | 1,455 | 2,430 |
| 1,100 | … | 225 | 295 | 445 | 670 | 1,115 | 1,855 |
| 1,150 | … | 170 | 230 | 345 | 515 | 855 | 1,430 |
| 1,200 | … | 130 | 175 | 260 | 390 | 650 | 1,085 |
| 1,250 | … | 100 | 135 | 200 | 300 | 495 | 830 |
| 1,300 | … | 80 | 105 | 160 | 235 | 395 | 655 |
| 1,350 | … | 60 | 80 | 125 | 185 | 310 | 515 |
| 1,400 | … | 45 | 60 | 90 | 135 | 225 | 370 |
| 1,450 | … | 30 | 40 | 60 | 95 | 155 | 255 |
| 1,500 | … | 25 | 35 | 55 | 80 | 135 | 230 |

Working Pressures by Classes, psig Class

NOTE: (1) At temperatures over 1,000°F, use only when the carbon content is 0.04% or higher.

69

ASME  B16.5-2020

###### Table 2-2.11 Pressure–Temperature Ratings for Group 2.11 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–10Ni–Cb | … | A351 Gr. CF8C (1) | … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.7 | 48.8 | 65.0 | 97.5 | 146.3 | 243.8 | 406.4 |
| 100 | 17.4 | 45.3 | 60.4 | 90.6 | 135.9 | 226.5 | 377.4 |
| 150 | 15.8 | 42.5 | 56.6 | 84.9 | 127.4 | 212.4 | 353.9 |
| 200 | 13.8 | 39.9 | 53.3 | 79.9 | 119.8 | 199.7 | 332.8 |
| 250 | 12.1 | 37.8 | 50.4 | 75.6 | 113.4 | 189.1 | 315.1 |
| 300 | 10.2 | 36.1 | 48.1 | 72.2 | 108.3 | 180.4 | 300.7 |
| 325 | 9.3 | 35.4 | 47.1 | 70.7 | 106.1 | 176.8 | 294.6 |
| 350 | 8.4 | 34.8 | 46.3 | 69.5 | 104.3 | 173.8 | 289.6 |
| 375 | 7.4 | 34.2 | 45.6 | 68.4 | 102.6 | 171.0 | 285.1 |
| 400 | 6.5 | 33.9 | 45.2 | 67.8 | 101.7 | 169.5 | 282.6 |
| 425 | 5.5 | 33.6 | 44.8 | 67.2 | 100.8 | 168.1 | 280.1 |
| 450 | 4.6 | 33.5 | 44.6 | 66.9 | 100.4 | 167.3 | 278.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 19.8 | 26.4 | 39.6 | 59.4 | 99.0 | 165.1 |
| 625 | … | 13.9 | 18.5 | 27.7 | 41.6 | 69.3 | 115.5 |
| 650 | … | 10.3 | 13.7 | 20.6 | 30.9 | 51.5 | 85.8 |
| 675 | … | 8.0 | 10.6 | 15.9 | 23.9 | 39.8 | 66.3 |
| 700 | … | 5.6 | 7.5 | 11.2 | 16.8 | 28.1 | 46.8 |
| 725 | … | 4.0 | 5.3 | 8.0 | 11.9 | 19.9 | 33.1 |
| 750 | … | 3.1 | 4.1 | 6.2 | 9.3 | 15.5 | 25.8 |
| 775 | … | 2.5 | 3.3 | 4.9 | 7.4 | 12.3 | 20.4 |
| 800 | … | 2.0 | 2.7 | 4.0 | 6.1 | 10.1 | 16.9 |
| 816 | … | 1.9 | 2.5 | 3.8 | 5.7 | 9.5 | 15.8 |

Working Pressures by Classes, bar Class

NOTE: (1) At temperatures over 538°C, use only when the carbon content is 0.04% or higher.

70

ASME  B16.5-2020

###### Table 2-2.11C Pressure–Temperature Ratings for Group 2.11 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 18Cr–10Ni–Cb | … | A351 Gr. CF8C (1) | … |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 255 | 660 | 885 | 1,325 | 1,985 | 3,310 | 5,520 |
| 300 | 230 | 615 | 820 | 1,235 | 1,850 | 3,085 | 5,140 |
| 400 | 200 | 575 | 770 | 1,150 | 1,730 | 2,880 | 4,800 |
| 500 | 170 | 540 | 725 | 1,085 | 1,625 | 2,710 | 4,520 |
| 600 | 140 | 515 | 690 | 1,030 | 1,550 | 2,580 | 4,300 |
| 650 | 125 | 505 | 675 | 1,015 | 1,520 | 2,530 | 4,220 |
| 700 | 110 | 495 | 660 | 995 | 1,490 | 2,485 | 4,140 |
| 750 | 95 | 490 | 655 | 985 | 1,475 | 2,460 | 4,100 |
| 800 | 80 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 850 | 65 | 485 | 645 | 970 | 1,455 | 2,425 | 4,040 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | … | 310 | 415 | 625 | 935 | 1,560 | 2,600 |
| 1,150 | … | 210 | 280 | 420 | 625 | 1,045 | 1,745 |
| 1,200 | … | 150 | 200 | 300 | 455 | 755 | 1,255 |
| 1,250 | … | 115 | 150 | 225 | 340 | 565 | 945 |
| 1,300 | … | 75 | 100 | 150 | 225 | 375 | 630 |
| 1,350 | … | 50 | 70 | 105 | 155 | 255 | 430 |
| 1,400 | … | 40 | 55 | 80 | 125 | 205 | 345 |
| 1,450 | … | 30 | 40 | 60 | 95 | 155 | 255 |
| 1,500 | … | 25 | 35 | 55 | 80 | 135 | 230 |

Working Pressures by Classes, psig Class

NOTE: (1) At temperatures over 1,000°F, use only when the carbon content is 0.04% or higher.

71

ASME  B16.5-2020

###### Table 2-2.12 Pressure–Temperature Ratings for Group 2.12 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 25Cr–20Ni | … | A351 Gr. CK20 (1) | … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 17.8 | 46.3 | 61.8 | 92.7 | 139.0 | 231.7 | 386.1 |
| 50 | 17.0 | 44.5 | 59.3 | 89.0 | 133.4 | 222.4 | 370.6 |
| 100 | 14.4 | 37.5 | 50.0 | 75.1 | 112.6 | 187.7 | 312.8 |
| 150 | 13.4 | 34.9 | 46.5 | 69.8 | 104.7 | 174.4 | 290.7 |
| 200 | 12.9 | 33.5 | 44.7 | 67.1 | 100.6 | 167.7 | 279.5 |
| 250 | 12.1 | 32.6 | 43.5 | 65.2 | 97.8 | 163.1 | 271.8 |
| 300 | 10.2 | 31.7 | 42.3 | 63.4 | 95.2 | 158.6 | 264.3 |
| 325 | 9.3 | 31.2 | 41.6 | 62.4 | 93.6 | 156.1 | 260.1 |
| 350 | 8.4 | 30.6 | 40.8 | 61.2 | 91.7 | 152.9 | 254.8 |
| 375 | 7.4 | 29.8 | 39.8 | 59.7 | 89.5 | 149.2 | 248.6 |
| 400 | 6.5 | 29.1 | 38.8 | 58.2 | 87.3 | 145.5 | 242.4 |
| 425 | 5.5 | 28.3 | 37.8 | 56.7 | 85.0 | 141.7 | 236.2 |
| 450 | 4.6 | 27.6 | 36.8 | 55.2 | 82.8 | 138.0 | 230.0 |
| 475 | 3.7 | 26.7 | 35.6 | 53.5 | 80.2 | 133.7 | 222.8 |
| 500 | 2.8 | 25.8 | 34.5 | 51.7 | 77.5 | 129.2 | 215.3 |
| 538 | 1.4 | 23.3 | 31.1 | 46.6 | 70.0 | 116.6 | 194.4 |
| 550 | … | 22.9 | 30.6 | 45.9 | 68.8 | 114.7 | 191.2 |
| 575 | … | 21.7 | 28.9 | 43.3 | 65.0 | 108.3 | 180.4 |
| 600 | … | 19.4 | 25.9 | 38.8 | 58.2 | 97.1 | 161.8 |
| 625 | … | 16.8 | 22.4 | 33.7 | 50.5 | 84.1 | 140.2 |
| 650 | … | 14.1 | 18.8 | 28.1 | 42.2 | 70.4 | 117.3 |
| 675 | … | 11.5 | 15.4 | 23.0 | 34.6 | 57.6 | 96.0 |
| 700 | … | 8.8 | 11.7 | 17.5 | 26.3 | 43.8 | 73.0 |
| 725 | … | 6.3 | 8.5 | 12.7 | 19.0 | 31.7 | 52.9 |
| 750 | … | 4.5 | 6.0 | 8.9 | 13.4 | 22.3 | 37.2 |
| 775 | … | 3.1 | 4.2 | 6.3 | 9.4 | 15.7 | 26.2 |
| 800 | … | 2.3 | 3.1 | 4.6 | 6.9 | 11.4 | 19.1 |
| 816 | … | 1.9 | 2.5 | 3.8 | 5.7 | 9.5 | 15.8 |

Working Pressures by Classes, bar Class

NOTE: (1) At temperatures over 538°C, use only when the carbon content is 0.04% or higher.

72

ASME  B16.5-2020

###### Table 2-2.12C Pressure–Temperature Ratings for Group 2.12 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 25Cr–20Ni | … | A351 Gr. CK20 (1) | … |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 260 | 670 | 895 | 1,345 | 2,015 | 3,360 | 5,600 |
| 200 | 210 | 550 | 735 | 1,100 | 1,650 | 2,750 | 4,580 |
| 300 | 195 | 505 | 675 | 1,015 | 1,520 | 2,530 | 4,220 |
| 400 | 185 | 485 | 645 | 970 | 1,455 | 2,425 | 4,040 |
| 500 | 170 | 470 | 625 | 940 | 1,410 | 2,350 | 3,920 |
| 600 | 140 | 455 | 610 | 910 | 1,370 | 2,280 | 3,800 |
| 650 | 125 | 445 | 595 | 895 | 1,340 | 2,230 | 3,720 |
| 700 | 110 | 435 | 580 | 870 | 1,305 | 2,170 | 3,620 |
| 750 | 95 | 420 | 565 | 845 | 1,265 | 2,110 | 3,520 |
| 800 | 80 | 410 | 545 | 820 | 1,230 | 2,050 | 3,420 |
| 850 | 65 | 400 | 530 | 795 | 1,195 | 1,990 | 3,320 |
| 900 | 50 | 385 | 510 | 770 | 1,150 | 1,920 | 3,200 |
| 950 | 35 | 370 | 495 | 740 | 1,110 | 1,850 | 3,080 |
| 1,000 | 20 | 340 | 450 | 675 | 1,015 | 1,690 | 2,820 |
| 1,050 | … | 325 | 435 | 650 | 975 | 1,630 | 2,715 |
| 1,100 | … | 290 | 390 | 585 | 875 | 1,455 | 2,430 |
| 1,150 | … | 250 | 335 | 500 | 750 | 1,250 | 2,085 |
| 1,200 | … | 205 | 275 | 410 | 615 | 1,030 | 1,715 |
| 1,250 | … | 165 | 220 | 330 | 495 | 825 | 1,370 |
| 1,300 | … | 120 | 160 | 240 | 360 | 600 | 1,000 |
| 1,350 | … | 80 | 110 | 165 | 245 | 410 | 685 |
| 1,400 | … | 55 | 75 | 110 | 165 | 275 | 455 |
| 1,450 | … | 40 | 50 | 75 | 115 | 190 | 315 |
| 1,500 | … | 25 | 35 | 55 | 80 | 135 | 230 |

Working Pressures by Classes, psig Class

NOTE: (1) At temperatures over 1,000°F, use only when the carbon content is 0.04% or higher.

73

ASME  B16.5-2020

ð20Þ

###### Table 2-3.1 Pressure–Temperature Ratings for Group 3.1 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 35Ni–35Fe–20Cr–Cb 35Ni–35Fe–20Cr–Cb | A182 Gr. N08020 (1) B462 Gr. N08020 (1) | … … | A240 Gr. N08020 (1) B463 Gr. N08020 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 50.9 | 67.8 | 101.7 | 152.6 | 254.4 | 423.9 |
| 150 | 15.8 | 48.9 | 65.3 | 97.9 | 146.8 | 244.7 | 407.8 |
| 200 | 13.8 | 47.2 | 62.9 | 94.3 | 141.5 | 235.8 | 392.9 |
| 250 | 12.1 | 45.5 | 60.7 | 91.0 | 136.5 | 227.5 | 379.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |

Working Pressures by Classes, bar Class

NOTE: (1) Use annealed material only.

ð20Þ

###### Table 2-3.1C Pressure–Temperature Ratings for Group 3.1 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 35Ni–35Fe–20Cr–Cb 35Ni–35Fe–20Cr–Cb | A182 Gr. N08020 (1) B462 Gr. N08020 (1) | … … | A240 Gr. N08020 (1) B463 Gr. N08020 (1) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 740 | 990 | 1,485 | 2,225 | 3,710 | 6,180 |
| 300 | 230 | 710 | 945 | 1,420 | 2,130 | 3,550 | 5,920 |
| 400 | 200 | 680 | 910 | 1,365 | 2,045 | 3,410 | 5,680 |
| 500 | 170 | 655 | 875 | 1,310 | 1,965 | 3,275 | 5,460 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |

NOTE: (1) Use annealed material only.

74

ASME  B16.5-2020

###### Table 2-3.2 Pressure–Temperature Ratings for Group 3.2 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 99.0Ni | B564 Gr. N02200 (1) | … | B162 Gr. N02200 (1) |

Working Pressures by Classes, bar

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −29 to 38 | 12.7 | 33.1 | 44.1 | 66.2 | 99.3 | 165.5 | 275.8 |
| 50 | 12.7 | 33.1 | 44.1 | 66.2 | 99.3 | 165.5 | 275.8 |
| 100 | 12.7 | 33.1 | 44.1 | 66.2 | 99.3 | 165.5 | 275.8 |
| 150 | 12.7 | 33.1 | 44.1 | 66.2 | 99.3 | 165.5 | 275.8 |
| 200 | 12.7 | 33.1 | 44.1 | 66.2 | 99.3 | 165.5 | 275.8 |
| 250 | 12.1 | 31.6 | 42.1 | 63.2 | 94.8 | 158.0 | 263.4 |
| 300 | 10.2 | 29.2 | 39.0 | 58.5 | 87.7 | 146.2 | 243.7 |
| 325 | 7.2 | 18.8 | 25.0 | 37.6 | 56.4 | 93.9 | 156.5 |

NOTE: (1) Use annealed material only.

###### Table 2-3.2C Pressure–Temperature Ratings for Group 3.2 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 99.0Ni | B564 Gr. N02200 (1) | … | B162 Gr. N02200 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 185 | 480 | 640 | 960 | 1,440 | 2,400 | 4,000 |
| 200 | 185 | 480 | 640 | 960 | 1,440 | 2,400 | 4,000 |
| 300 | 185 | 480 | 640 | 960 | 1,440 | 2,400 | 4,000 |
| 400 | 185 | 480 | 640 | 960 | 1,440 | 2,400 | 4,000 |
| 500 | 170 | 455 | 605 | 905 | 1,360 | 2,270 | 3,780 |
| 600 | 140 | 415 | 550 | 825 | 1,240 | 2,065 | 3,440 |

Working Pressures by Classes, psig Class

NOTE: (1) Use annealed material only.

75

ASME  B16.5-2020

###### Table 2-3.3 Pressure–Temperature Ratings for Group 3.3 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 99.0Ni–Low C | … | … | B162 Gr. N02201 (1) |

Working Pressures by Classes, bar

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −29 to 38 | 6.3 | 16.5 | 22.1 | 33.1 | 49.6 | 82.7 | 137.9 |
| 50 | 6.3 | 16.4 | 21.9 | 32.8 | 49.2 | 82.0 | 136.7 |
| 100 | 6.1 | 15.8 | 21.1 | 31.7 | 47.5 | 79.2 | 132.0 |
| 150 | 6.0 | 15.6 | 20.7 | 31.1 | 46.7 | 77.8 | 129.6 |
| 200 | 6.0 | 15.6 | 20.7 | 31.1 | 46.7 | 77.8 | 129.6 |
| 250 | 6.0 | 15.6 | 20.7 | 31.1 | 46.7 | 77.8 | 129.6 |
| 300 | 6.0 | 15.6 | 20.7 | 31.1 | 46.7 | 77.8 | 129.6 |
| 325 | 5.9 | 15.5 | 20.7 | 31.0 | 46.5 | 77.5 | 129.2 |
| 350 | 5.9 | 15.4 | 20.5 | 30.8 | 46.2 | 76.9 | 128.2 |
| 375 | 5.9 | 15.4 | 20.5 | 30.7 | 46.1 | 76.8 | 128.0 |
| 400 | 5.8 | 15.2 | 20.3 | 30.4 | 45.6 | 76.1 | 126.8 |
| 425 | 5.5 | 14.9 | 19.9 | 29.8 | 44.7 | 74.6 | 124.3 |
| 450 | 4.6 | 14.6 | 19.5 | 29.2 | 43.8 | 73.1 | 121.8 |
| 475 | 3.7 | 14.3 | 19.1 | 28.6 | 43.0 | 71.6 | 119.3 |
| 500 | 2.8 | 13.8 | 18.4 | 27.6 | 41.4 | 69.0 | 115.1 |
| 538 | 1.4 | 13.1 | 17.4 | 26.1 | 39.2 | 65.4 | 108.9 |
| 550 | … | 9.8 | 13.1 | 19.6 | 29.5 | 49.1 | 81.8 |
| 575 | … | 5.4 | 7.1 | 10.7 | 16.1 | 26.8 | 44.6 |
| 600 | … | 4.4 | 5.9 | 8.9 | 13.3 | 22.2 | 37.0 |
| 625 | … | 3.4 | 4.6 | 6.9 | 10.3 | 17.2 | 28.7 |
| 650 | … | 2.8 | 3.8 | 5.7 | 8.5 | 14.2 | 23.6 |

NOTE: (1) Use annealed material only.

76

ASME  B16.5-2020

###### Table 2-3.3C Pressure–Temperature Ratings for Group 3.3 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 99.0Ni–Low C | … | … | B162 Gr. N02201 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 90 | 240 | 320 | 480 | 720 | 1,200 | 2,000 |
| 200 | 90 | 230 | 305 | 460 | 690 | 1,150 | 1,920 |
| 300 | 85 | 225 | 300 | 450 | 675 | 1,130 | 1,880 |
| 400 | 85 | 225 | 300 | 450 | 675 | 1,130 | 1,880 |
| 500 | 85 | 225 | 300 | 450 | 675 | 1,130 | 1,880 |
| 600 | 85 | 225 | 300 | 450 | 675 | 1,130 | 1,880 |
| 650 | 85 | 225 | 300 | 445 | 670 | 1,115 | 1,860 |
| 700 | 85 | 225 | 300 | 445 | 670 | 1,115 | 1,860 |
| 750 | 85 | 220 | 295 | 440 | 660 | 1,105 | 1,840 |
| 800 | 80 | 215 | 290 | 430 | 650 | 1,080 | 1,800 |
| 850 | 65 | 210 | 280 | 420 | 635 | 1,055 | 1,760 |
| 900 | 50 | 205 | 275 | 415 | 620 | 1,030 | 1,720 |
| 950 | 35 | 195 | 260 | 395 | 590 | 985 | 1,640 |
| 1,000 | 20 | 190 | 255 | 380 | 570 | 950 | 1,580 |
| 1,050 | … | 80 | 110 | 165 | 245 | 410 | 685 |
| 1,100 | … | 70 | 90 | 135 | 205 | 345 | 570 |
| 1,150 | … | 50 | 70 | 105 | 155 | 255 | 430 |
| 1,200 | … | 40 | 55 | 80 | 125 | 205 | 345 |

Working Pressures by Classes, psig Class

NOTE:

(1)  Use annealed material only.

77

ASME  B16.5-2020

###### Table 2-3.4 Pressure–Temperature Ratings for Group 3.4 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 67Ni–30Cu | B564 Gr. N04400 (1) | … | B127 Gr. N04400 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 15.9 | 41.4 | 55.2 | 82.7 | 124.1 | 206.8 | 344.7 |
| 50 | 15.4 | 40.2 | 53.7 | 80.5 | 120.7 | 201.2 | 335.3 |
| 100 | 13.8 | 35.9 | 47.9 | 71.9 | 107.8 | 179.7 | 299.5 |
| 150 | 12.9 | 33.7 | 45.0 | 67.5 | 101.2 | 168.7 | 281.1 |
| 200 | 12.5 | 32.7 | 43.6 | 65.4 | 98.1 | 163.5 | 272.4 |
| 250 | 12.1 | 32.6 | 43.5 | 65.2 | 97.8 | 163.0 | 271.7 |
| 300 | 10.2 | 32.6 | 43.5 | 65.2 | 97.8 | 163.0 | 271.7 |
| 325 | 9.3 | 32.6 | 43.5 | 65.2 | 97.8 | 163.0 | 271.7 |
| 350 | 8.4 | 32.6 | 43.4 | 65.1 | 97.7 | 162.8 | 271.3 |
| 375 | 7.4 | 32.4 | 43.2 | 64.8 | 97.2 | 161.9 | 269.9 |
| 400 | 6.5 | 32.1 | 42.8 | 64.2 | 96.2 | 160.4 | 267.4 |
| 425 | 5.5 | 31.6 | 42.2 | 63.3 | 94.9 | 158.2 | 263.6 |
| 450 | 4.6 | 26.9 | 35.9 | 53.8 | 80.7 | 134.5 | 224.2 |
| 475 | 3.7 | 20.8 | 27.7 | 41.5 | 62.3 | 103.8 | 173.0 |

Working Pressures by Classes, bar Class

NOTE: (1) Use annealed material only.

###### Table 2-3.4C Pressure–Temperature Ratings for Group 3.4 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 67Ni–30Cu | B564 Gr. N04400 (1) | … | B127 Gr. N04400 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 230 | 600 | 800 | 1,200 | 1,800 | 3,000 | 5,000 |
| 200 | 200 | 525 | 700 | 1,050 | 1,575 | 2,630 | 4,380 |
| 300 | 190 | 490 | 655 | 980 | 1,470 | 2,450 | 4,080 |
| 400 | 180 | 475 | 630 | 945 | 1,420 | 2,365 | 3,940 |
| 500 | 170 | 475 | 630 | 945 | 1,420 | 2,365 | 3,940 |
| 600 | 140 | 475 | 630 | 945 | 1,420 | 2,365 | 3,940 |
| 650 | 125 | 475 | 630 | 945 | 1,420 | 2,365 | 3,940 |
| 700 | 110 | 470 | 625 | 940 | 1,410 | 2,350 | 3,920 |
| 750 | 95 | 465 | 620 | 930 | 1,395 | 2,330 | 3,880 |
| 800 | 80 | 460 | 610 | 915 | 1,375 | 2,290 | 3,820 |
| 850 | 65 | 375 | 505 | 755 | 1,130 | 1,885 | 3,145 |
| 900 | 50 | 275 | 365 | 550 | 825 | 1,370 | 2,285 |

Working Pressures by Classes, psig Class

NOTE: (1) Use annealed material only.

78

ASME  B16.5-2020

###### Table 2-3.5 Pressure–Temperature Ratings for Group 3.5 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 72Ni–15Cr–8Fe | B564 Gr. N06600 (1) | … | B168 Gr. N06600 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 16.5 | 22.1 | 33.1 | 49.6 | 82.7 | 137.9 |
| 550 | … | 13.9 | 18.6 | 27.9 | 41.8 | 69.7 | 116.2 |
| 575 | … | 9.4 | 12.6 | 18.9 | 28.3 | 47.2 | 78.6 |
| 600 | … | 6.6 | 8.9 | 13.3 | 19.9 | 33.2 | 55.3 |
| 625 | … | 5.1 | 6.8 | 10.3 | 15.4 | 25.7 | 42.8 |
| 650 | … | 4.7 | 6.3 | 9.5 | 14.2 | 23.6 | 39.4 |

Working Pressures by Classes, bar Class

NOTE: (1) Use annealed material only.

79

ASME  B16.5-2020

###### Table 2-3.5C Pressure–Temperature Ratings for Group 3.5 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 72Ni–15Cr–8Fe | B564 Gr. N06600 (1) | … | B168 Gr. N06600 (1) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 365 | 485 | 725 | 1,090 | 1,815 | 3,030 |
| 1,000 | 20 | 240 | 320 | 480 | 720 | 1,200 | 2,000 |
| 1,050 | … | 155 | 205 | 310 | 465 | 770 | 1,285 |
| 1,100 | … | 105 | 135 | 205 | 310 | 515 | 855 |
| 1,150 | … | 75 | 100 | 150 | 225 | 375 | 630 |
| 1,200 | … | 70 | 90 | 135 | 205 | 345 | 570 |

NOTE: (1) Use annealed material only.

80

ASME  B16.5-2020

###### Table 2-3.6 Pressure–Temperature Ratings for Group 3.6 Materials	ð20Þ

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 33Ni–42Fe–21Cr 33Ni–42Fe–21Cr | A182 Gr. N08020 (1) B564 Gr. N08800 (1) | … … | A240 Gr. N08020 (1) B409 Gr. N08800 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.7 | 48.8 | 65.1 | 97.6 | 146.4 | 244.0 | 406.7 |
| 100 | 17.5 | 45.6 | 60.8 | 91.2 | 136.9 | 228.1 | 380.1 |
| 150 | 15.8 | 44.0 | 58.7 | 88.0 | 132.0 | 219.9 | 366.6 |
| 200 | 13.8 | 42.8 | 57.1 | 85.6 | 128.4 | 214.0 | 356.7 |
| 250 | 12.1 | 41.7 | 55.7 | 83.5 | 125.2 | 208.7 | 347.9 |
| 300 | 10.2 | 40.8 | 54.4 | 81.6 | 122.5 | 204.1 | 340.2 |
| 325 | 9.3 | 40.3 | 53.8 | 80.6 | 120.9 | 201.6 | 336.0 |
| 350 | 8.4 | 39.8 | 53.0 | 79.5 | 119.3 | 198.8 | 331.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 21.6 | 28.6 | 42.9 | 64.2 | 107.0 | 178.5 |
| 625 | … | 18.3 | 24.3 | 36.6 | 54.9 | 91.2 | 152.0 |
| 650 | … | 14.1 | 18.9 | 28.1 | 42.5 | 70.7 | 117.7 |
| 675 | … | 10.3 | 13.7 | 20.5 | 30.8 | 51.3 | 85.6 |
| 700 | … | 5.6 | 7.4 | 11.1 | 16.7 | 27.8 | 46.3 |
| 725 | … | 4.0 | 5.4 | 8.1 | 12.1 | 20.1 | 33.6 |
| 750 | … | 3.0 | 4.0 | 6.1 | 9.1 | 15.1 | 25.2 |
| 775 | … | 2.5 | 3.3 | 4.9 | 7.4 | 12.4 | 20.6 |
| 800 | … | 2.2 | 2.9 | 4.3 | 6.5 | 10.8 | 18.0 |
| 816 | … | 1.9 | 2.5 | 3.8 | 5.7 | 9.5 | 15.8 |

Working Pressures by Classes, bar Class

NOTE: (1) Use annealed material only.

81

ASME  B16.5-2020

ð20Þ

###### Table 2-3.6C Pressure–Temperature Ratings for Group 3.6 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 33Ni–42Fe–21Cr 33Ni–42Fe–21Cr | A182 Gr. N08810 (1) B564 Gr. N08800 (1) | … … | A240 Gr. N08810 (1) B409 Gr. N08800 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 255 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 300 | 230 | 640 | 850 | 1,275 | 1,915 | 3,190 | 5,320 |
| 400 | 200 | 620 | 825 | 1,240 | 1,860 | 3,095 | 5,160 |
| 500 | 170 | 600 | 805 | 1,205 | 1,805 | 3,010 | 5,020 |
| 600 | 140 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,900 |
| 650 | 125 | 580 | 770 | 1,155 | 1,735 | 2,890 | 4,820 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | … | 325 | 430 | 645 | 965 | 1,610 | 2,685 |
| 1,150 | … | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,200 | … | 205 | 275 | 410 | 620 | 1,030 | 1,715 |
| 1,250 | … | 145 | 190 | 290 | 430 | 720 | 1,200 |
| 1,300 | … | 70 | 90 | 135 | 205 | 345 | 570 |
| 1,350 | … | 55 | 75 | 110 | 165 | 275 | 455 |
| 1,400 | … | 40 | 50 | 75 | 115 | 190 | 315 |
| 1,450 | … | 35 | 45 | 70 | 105 | 170 | 285 |
| 1,500 | … | 25 | 35 | 55 | 80 | 135 | 230 |

Working Pressures by Classes, psig Class

NOTE: (1) Use annealed material only.

82

ASME  B16.5-2020

###### Table 2-3.7 Pressure–Temperature Ratings for Group 3.7 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 65Ni–28Mo–2Fe 64Ni-29.5Mo–2Cr–2Fe–Mn–W | B462 Gr. N10665 (1) B462 Gr. N10675 (1) | … … | B333 Gr. N10665 (1) B333 Gr. N10675 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |

Working Pressures by Classes, bar Class

NOTE:  (1)  Use solution annealed material only.

###### Table 2-3.7C Pressure–Temperature Ratings for Group 3.7 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 65Ni–28Mo–2Fe 64Ni–29.5Mo–2Cr–2Fe–Mn–W | B462 Gr. N10665 (1) B462 Gr. N10675 (1) | … … | B333 Gr. N10665 (1) B333 Gr. N10675 (1) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |

NOTE:  (1)  Use solution annealed material only.

83

ASME  B16.5-2020

###### Table 2-3.8 Pressure–Temperature Ratings for Group 3.8 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 54Ni–16Mo–15Cr | B462 Gr. N10276 (1), (2) | … | B575 Gr. N10276 (1), (2) |
| 60Ni–22Cr–9Mo–3.5Cb | B564 Gr. N06625 (3)–(5) | … | B443 Gr. N06625 (3)–(5) |
| 62Ni–28Mo–5Fe | … | … | B333 Gr. N10001 (1), (6) |
| 70Ni–16Mo–7Cr–5Fe | … | … | B434 Gr. N10003 (3) |
| 61Ni–16Mo–16Cr | … | … | B575 Gr. N06455 (1), (6) |
| 42Ni–21.5Cr–3Mo–2.3Cu | B564 Gr. N08825 (3), (7) | … | B424 Gr. N08825 (3), (7) |
| 55Ni–21Cr–13.5Mo | B462 Gr. N06022 (1), (2), (8) | … | B575 Gr. N06022 (1), (2), (8) |
| 55Ni–23Cr–16Mo–1.6Cu | B462 Gr. N06200 (1), (6) | … | B575 Gr. N06200 (1), (6) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.3 | 64.5 | 96.7 | 145.0 | 241.7 | 402.8 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 21.6 | 28.6 | 42.9 | 64.2 | 107.0 | 178.5 |
| 625 | … | 18.3 | 24.3 | 36.6 | 54.9 | 91.2 | 152.0 |
| 650 | … | 14.1 | 18.8 | 28.1 | 42.2 | 70.4 | 117.3 |
| 675 | … | 11.5 | 15.4 | 23.0 | 34.6 | 57.6 | 96.0 |
| 700 | … | 8.8 | 11.7 | 17.5 | 26.3 | 43.8 | 73.0 |

Working Pressures by Classes, bar Class

NOTES:

- Use solution annealed material only.

- Not to be used over 675°C.

- Use annealed material only.

- Not to be used over 645°C. Alloy N06625 in the annealed condition is subject to severe loss of impact strength at room temperatures after exposure in the range of 538°C to 760°C.

- Grade 1.

- Not to be used over 425°C.

- Not to be used over 538°C.

- Alloy N06022 in the solution annealed condition is subject to severe loss of impact strength at room temperature after exposure to tempera- tures in the range of 538°C to 675°C.

84

ASME  B16.5-2020

###### Table 2-3.8C Pressure–Temperature Ratings for Group 3.8 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 54Ni–16Mo–15Cr | B462 Gr. N10276 (1), (2) | … | B575 Gr. N10276 (1), (2) |
| 60Ni–22Cr–9Mo–3.5Cb | B564 Gr. N06625 (3)–(5) | … | B443 Gr. N06625 (3)–(5) |
| 62Ni–28Mo–5Fe | … | … | B333 Gr. N10001 (1), (6) |
| 70Ni–16Mo–7Cr–5Fe | … | … | B434 Gr. N10003 (3) |
| 61Ni–16Mo–16Cr | … | … | B575 Gr. N06455 (1), (6) |
| 42Ni–21.5Cr–3Mo–2.3Cu | B564 Gr. N08825 (3), (7) | … | B424 Gr. N08825 (3), (7) |
| 55Ni–21Cr–13.5Mo | B462 Gr. N06022 (1), (2), (8) | … | B575 Gr. N06022 (1), (2), (8) |
| 55Ni–23Cr–16Mo–1.6Cu | B462 Gr. N06200 (1), (6) | … | B575 Gr. N06200 (1), (6) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 700 | 930 | 1,395 | 2,095 | 3,490 | 5,820 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | … | 325 | 430 | 645 | 965 | 1,610 | 2,685 |
| 1,150 | … | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,200 | … | 205 | 275 | 410 | 615 | 1,030 | 1,715 |
| 1,250 | … | 165 | 220 | 330 | 495 | 825 | 1,370 |
| 1,300 | … | 120 | 160 | 240 | 360 | 600 | 1,000 |

NOTES:

- Use solution annealed material only.

- Not to be used over 1,250°F.

- Use annealed material only.

- Not to be used over 1,200°F. Alloy N06625 in the annealed condition is subject to severe loss of impact strength at room temperatures after exposure in the range of 1,000°F to 1,400°F.

- Grade 1.

- Not to be used over 800°F.

- Not to be used over 1,000°F.

- Alloy N06022 in the solution annealed condition is subject to severe loss of impact strength at room temperature after exposure to tempera- tures in the range of 1,000°F to 1,250°F.

85

ASME  B16.5-2020

###### Table 2-3.9 Pressure–Temperature Ratings for Group 3.9 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 47Ni–22Cr–9Mo–18Fe 21Ni–30Fe–22Cr–18Co–3Mo–3W | B572 Gr. N06002 (1), (2) B572 Gr. R30556 (1), (2) | … … | B435 Gr. N06002 (1) B435 Gr. R30556 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 47.6 | 63.4 | 95.2 | 142.8 | 237.9 | 396.5 |
| 200 | 13.8 | 44.3 | 59.1 | 88.6 | 132.9 | 221.5 | 369.2 |
| 250 | 12.1 | 41.6 | 55.4 | 83.1 | 124.7 | 207.9 | 346.4 |
| 300 | 10.2 | 39.5 | 52.7 | 79.0 | 118.5 | 197.4 | 329.1 |
| 325 | 9.3 | 38.6 | 51.5 | 77.2 | 115.8 | 193.0 | 321.7 |
| 350 | 8.4 | 37.9 | 50.5 | 75.8 | 113.7 | 189.5 | 315.8 |
| 375 | 7.4 | 37.3 | 49.8 | 74.7 | 112.0 | 186.6 | 311.1 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 21.6 | 28.6 | 42.9 | 64.2 | 107.0 | 178.5 |
| 625 | … | 18.3 | 24.3 | 36.6 | 54.9 | 91.2 | 152.0 |
| 650 | … | 14.1 | 18.9 | 28.1 | 42.5 | 70.7 | 117.7 |
| 675 | … | 12.4 | 16.9 | 25.2 | 37.6 | 62.7 | 104.5 |
| 700 | … | 10.1 | 13.4 | 20.0 | 29.8 | 49.7 | 83.0 |
| 725 | … | 7.9 | 10.5 | 15.4 | 23.2 | 38.6 | 64.4 |
| 750 | … | 5.9 | 7.9 | 11.7 | 17.6 | 29.6 | 49.1 |
| 775 | … | 4.6 | 6.2 | 9.0 | 13.7 | 22.8 | 38.0 |
| 800 | … | 3.5 | 4.8 | 7.0 | 10.5 | 17.4 | 29.2 |
| 816 | … | 2.8 | 3.8 | 5.9 | 8.6 | 14.1 | 23.8 |

Working Pressures by Classes, bar Class

NOTES:

- Use solution annealed material only.

- The chemical composition, mechanical properties, heat treating requirements, and grain size requirements shall conform to the applicable ASTM specification. The manufacturing procedures, tolerances, tests, certification, and markings shall be in accordance with ASTM B564.

86

ASME  B16.5-2020

###### Table 2-3.9C Pressure–Temperature Ratings for Group 3.9 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 47Ni–22Cr–9Mo–18Fe 21Ni–30Fe–22Cr–18Co–3Mo–3W | B572 Gr. N06002 (1) B572 Gr. R30556 (1) | … … | B435 Gr. N06002 (1) B435 Gr. R30556 (1) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 690 | 920 | 1,380 | 2,075 | 3,455 | 5,760 |
| 400 | 200 | 640 | 850 | 1,275 | 1,915 | 3,190 | 5,320 |
| 500 | 170 | 595 | 795 | 1,190 | 1,785 | 2,975 | 4,960 |
| 600 | 140 | 565 | 750 | 1,130 | 1,690 | 2,820 | 4,700 |
| 650 | 125 | 550 | 735 | 1,105 | 1,655 | 2,760 | 4,600 |
| 700 | 110 | 540 | 725 | 1,085 | 1,625 | 2,710 | 4,520 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | … | 325 | 430 | 645 | 965 | 1,610 | 2,685 |
| 1,150 | … | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,200 | … | 205 | 275 | 410 | 620 | 1,030 | 1,715 |
| 1,250 | … | 180 | 245 | 365 | 545 | 910 | 1,515 |
| 1,300 | … | 140 | 185 | 275 | 410 | 685 | 1,145 |
| 1,350 | … | 105 | 140 | 205 | 310 | 515 | 860 |
| 1,400 | … | 75 | 100 | 150 | 225 | 380 | 630 |
| 1,450 | … | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,500 | … | 40 | 55 | 85 | 125 | 205 | 345 |

NOTE:

- Use solution annealed material only.

- The chemical composition, mechanical properties, heat treating requirements, and grain size requirements shall conform to the applicable ASTM specification. The manufacturing procedures, tolerances, tests, certification, and markings shall be in accordance with ASTM B564.

87

ASME  B16.5-2020

ð20Þ

###### Table 2-3.10 Pressure–Temperature Ratings for Group 3.10 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 25Ni–47Fe–21Cr–5Mo 25Ni–47Fe–21Cr–5Mo | … … | … … | A240 Gr. N08700 (1) B599 Gr. N08700 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 47.1 | 62.8 | 94.2 | 141.3 | 235.5 | 392.5 |
| 200 | 13.8 | 44.3 | 59.0 | 88.5 | 132.8 | 221.3 | 368.9 |
| 250 | 12.1 | 42.8 | 57.1 | 85.6 | 128.4 | 214.0 | 356.6 |
| 300 | 10.2 | 41.3 | 55.1 | 82.7 | 124.0 | 206.7 | 344.5 |
| 325 | 9.3 | 40.4 | 53.8 | 80.7 | 121.1 | 201.8 | 336.4 |
| 350 | 8.4 | 38.9 | 51.9 | 77.8 | 116.7 | 194.5 | 324.2 |

Working Pressures by Classes, bar Class

NOTE:  (1)  Use solution annealed material only.

ð20Þ

###### Table 2-3.10C Pressure–Temperature Ratings for Group 3.10 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 25Ni–47Fe–21Cr–5Mo 25Ni–47Fe–21Cr–5Mo | … … | … … | A240 Gr. N08700 (1) B599 Gr. N08700 (1) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 685 | 910 | 1,370 | 2,050 | 3,420 | 5,700 |
| 400 | 200 | 640 | 850 | 1,275 | 1,915 | 3,190 | 5,320 |
| 500 | 170 | 615 | 820 | 1,235 | 1,850 | 3,085 | 5,140 |
| 600 | 140 | 595 | 790 | 1,185 | 1,780 | 2,965 | 4,940 |
| 650 | 125 | 570 | 760 | 1,140 | 1,715 | 2,855 | 4,760 |

NOTE:  (1)  Use solution annealed material only.

88

ASME  B16.5-2020

###### Table 2-3.11 Pressure–Temperature Ratings for Group 3.11 Materials	ð20Þ

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 44Fe–25Ni–21Cr–Mo | A182 Gr. N08904 (1), (2) | … | A240 Gr. N08904 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.7 | 51.3 | 68.4 | 102.6 | 153.9 | 256.5 | 427.5 |
| 50 | 18.8 | 49.1 | 65.5 | 98.3 | 147.4 | 245.7 | 409.6 |
| 100 | 15.7 | 41.1 | 54.7 | 82.1 | 123.2 | 205.3 | 342.1 |
| 150 | 14.4 | 37.5 | 50.0 | 75.0 | 112.5 | 187.5 | 312.5 |
| 200 | 13.3 | 34.7 | 46.2 | 69.3 | 104.0 | 173.4 | 288.9 |
| 250 | 12.1 | 32.0 | 42.6 | 64.0 | 95.9 | 159.9 | 266.5 |
| 300 | 10.2 | 30.0 | 40.0 | 60.0 | 90.0 | 150.1 | 250.1 |
| 325 | 9.3 | 29.2 | 39.0 | 58.5 | 87.7 | 146.1 | 243.6 |
| 350 | 8.4 | 28.7 | 38.2 | 57.3 | 86.0 | 143.4 | 238.9 |
| 375 | 7.4 | 28.2 | 37.7 | 56.5 | 84.7 | 141.2 | 235.4 |

Working Pressures by Classes, bar Class

NOTES:

- Use annealed material only.

- The chemical composition, mechanical properties, heat treating requirements, and grain size requirements shall conform to the applicable ASTM specification. The manufacturing procedures, tolerances, tests, certification, and markings shall be in accordance with ASTM B564.

###### Table 2-3.11C Pressure–Temperature Ratings for Group 3.11 Materials	ð20Þ

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 44Fe–25Ni–21Cr–Mo | A182 Gr. N08904 (1), (2) | … | A240 Gr. N08904 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 285 | 745 | 990 | 1,490 | 2,230 | 3,720 | 6,200 |
| 200 | 230 | 600 | 805 | 1,205 | 1,805 | 3,010 | 5,020 |
| 300 | 210 | 545 | 725 | 1,090 | 1,635 | 2,725 | 4,540 |
| 400 | 190 | 500 | 665 | 1,000 | 1,500 | 2,495 | 4,160 |
| 500 | 170 | 455 | 610 | 910 | 1,370 | 2,280 | 3,800 |
| 600 | 140 | 425 | 570 | 855 | 1,280 | 2,135 | 3,560 |
| 650 | 125 | 420 | 555 | 835 | 1,255 | 2,090 | 3,480 |
| 700 | 110 | 410 | 545 | 820 | 1,230 | 2,050 | 3,420 |

Working Pressures by Classes, psig Class

NOTES:

- Use annealed material only.

- The chemical composition, mechanical properties, heat treating requirements, and grain size requirements shall conform to the applicable ASTM specification. The manufacturing procedures, tolerances, tests, certification, and markings shall be in accordance with ASTM B564.

89

ASME  B16.5-2020

ð20Þ

###### Table 2-3.12 Pressure–Temperature Ratings for Group 3.12 Materials

Nominal Designation	Forgings	Castings	Plates

26Ni–43Fe–22Cr–5Mo	…	…	B620 Gr. N08320 (1)

47Ni–22Cr–20Fe–7Mo	…	…	B582 Gr. N06985 (1)

46Fe–24Ni–21Cr–6Mo–Cu–N	A182 Gr. N08367 (1)	A351 Gr. CN3MN (1)	A240 Gr. N08367 (1)

46Fe–24Ni–21Cr–6Mo–Cu–N	B462 Gr. N08367 (1)	A351 Gr. CN3MN (1)	B688 Gr. N08367 (1)

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 17.8 | 46.3 | 61.8 | 92.7 | 139.0 | 231.7 | 386.1 |
| 50 | 17.5 | 45.6 | 60.8 | 91.1 | 136.7 | 227.8 | 379.7 |
| 100 | 16.3 | 42.5 | 56.7 | 85.1 | 127.6 | 212.7 | 354.5 |
| 150 | 15.4 | 40.1 | 53.5 | 80.3 | 120.4 | 200.7 | 334.6 |
| 200 | 13.8 | 37.3 | 49.8 | 74.6 | 112.0 | 186.6 | 311.0 |
| 250 | 12.1 | 34.9 | 46.5 | 69.8 | 104.7 | 174.5 | 290.8 |
| 300 | 10.2 | 33.1 | 44.1 | 66.2 | 99.3 | 165.5 | 275.9 |
| 325 | 9.3 | 32.3 | 43.1 | 64.6 | 97.0 | 161.6 | 269.3 |
| 350 | 8.4 | 31.6 | 42.1 | 63.2 | 94.8 | 158.1 | 263.4 |
| 375 | 7.4 | 31.0 | 41.4 | 62.0 | 93.0 | 155.1 | 258.5 |
| 400 | 6.5 | 30.4 | 40.6 | 60.8 | 91.3 | 152.1 | 253.5 |
| 425 | 5.5 | 29.8 | 39.8 | 59.7 | 89.5 | 149.1 | 248.5 |

Working Pressures by Classes, bar Class

NOTE:  (1)  Use solution annealed material only.

ð20Þ

###### Table 2-3.12C Pressure–Temperature Ratings for Group 3.12 Materials

Nominal Designation	Forgings	Castings	Plates

| 26Ni–43Fe–22Cr–5Mo | … | … | B620 Gr. N08320 (1) |
| --- | --- | --- | --- |
| 47Ni–22Cr–20Fe–7Mo | … | … | B582 Gr. N06985 (1) |
| 46Fe–24Ni–21Cr–6Mo–Cu–N | A182 Gr. N08367 (1) | A351 Gr. CN3MN (1) | A240 Gr. N08367 (1) |
| 46Fe–24Ni–21Cr–6Mo–Cu–N | B462 Gr. N08367 (1) | A351 Gr. CN3MN (1) | B688 Gr. N08367 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 260 | 670 | 895 | 1,345 | 2,015 | 3,360 | 5,600 |
| 200 | 240 | 620 | 830 | 1,245 | 1,865 | 3,110 | 5,180 |
| 300 | 225 | 585 | 780 | 1,165 | 1,750 | 2,915 | 4,860 |
| 400 | 200 | 540 | 715 | 1,075 | 1,615 | 2,690 | 4,480 |
| 500 | 170 | 500 | 665 | 1,000 | 1,500 | 2,495 | 4,160 |
| 600 | 140 | 475 | 630 | 945 | 1,420 | 2,365 | 3,940 |
| 650 | 125 | 460 | 615 | 920 | 1,380 | 2,305 | 3,840 |
| 700 | 110 | 450 | 600 | 900 | 1,355 | 2,255 | 3,760 |
| 750 | 95 | 440 | 590 | 885 | 1,325 | 2,210 | 3,680 |
| 800 | 80 | 430 | 575 | 865 | 1,295 | 2,160 | 3,600 |

Working Pressures by Classes, psig Class

NOTE:  (1)  Use solution annealed material only.

90

ASME  B16.5-2020

###### Table 2-3.13 Pressure–Temperature Ratings for Group 3.13 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 49Ni–25Cr–18Fe–6Mo Ni–Fe–Cr–Mo–Cu–Low C | … B564 Gr. N08031 (2) | … … | B582 Gr. N06975 (1) B625 Gr. N08031 (2) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 48.2 | 64.2 | 96.3 | 144.5 | 240.8 | 401.4 |
| 150 | 15.8 | 45.8 | 61.0 | 91.6 | 137.4 | 228.9 | 381.6 |
| 200 | 13.8 | 43.6 | 58.1 | 87.1 | 130.7 | 217.8 | 362.9 |
| 250 | 12.1 | 41.5 | 55.3 | 82.9 | 124.4 | 207.3 | 345.5 |
| 300 | 10.2 | 39.4 | 52.5 | 78.7 | 118.1 | 196.8 | 328.1 |
| 325 | 9.3 | 38.4 | 51.3 | 76.9 | 115.3 | 192.2 | 320.3 |
| 350 | 8.4 | 37.7 | 50.3 | 75.5 | 113.2 | 188.7 | 314.5 |
| 375 | 7.4 | 37.2 | 49.5 | 74.3 | 111.5 | 185.8 | 309.7 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |

Working Pressures by Classes, bar Class

NOTES:

- Use solution annealed material only.

- Use annealed material only.

###### Table 2-3.13C Pressure–Temperature Ratings for Group 3.13 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 49Ni–25Cr–18Fe–6Mo Ni–Fe–Cr–Mo–Cu–Low C | … B564 Gr. N08031 (2) | … … | B582 Gr. N06975 (1) B625 Gr. N08031 (2) |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 705 | 940 | 1,405 | 2,110 | 3,515 | 5,860 |
| 300 | 230 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 400 | 200 | 630 | 840 | 1,260 | 1,885 | 3,145 | 5,240 |
| 500 | 170 | 595 | 795 | 1,190 | 1,785 | 2,975 | 4,960 |
| 600 | 140 | 560 | 750 | 1,125 | 1,685 | 2,810 | 4,680 |
| 650 | 125 | 550 | 735 | 1,100 | 1,650 | 2,750 | 4,580 |
| 700 | 110 | 540 | 720 | 1,080 | 1,620 | 2,700 | 4,500 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |

NOTES:

- Use solution annealed material only.

- Use annealed material only.

91

ASME  B16.5-2020

###### Table 2-3.14 Pressure–Temperature Ratings for Group 3.14 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 47Ni–22Cr–19Fe–6Mo 40Ni–29Cr–15Fe–5Mo 58Ni–33Cr–8Mo | … B462 Gr. N06030 (1), (2) B462 Gr. N06035 (1), (2) | … … … | B582 Gr. N06007 (1) B582 Gr. N06030 (1), (2) B575 Gr. N06035 (1), (2) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.6 | 48.6 | 64.7 | 97.1 | 145.7 | 242.8 | 404.6 |
| 100 | 17.0 | 44.3 | 59.0 | 88.6 | 132.8 | 221.4 | 369.0 |
| 150 | 15.8 | 41.3 | 55.1 | 82.6 | 124.0 | 206.6 | 344.3 |
| 200 | 13.8 | 39.1 | 52.1 | 78.2 | 117.3 | 195.4 | 325.7 |
| 250 | 12.1 | 37.4 | 49.9 | 74.8 | 112.2 | 187.0 | 311.6 |
| 300 | 10.2 | 36.1 | 48.2 | 72.2 | 108.3 | 180.6 | 300.9 |
| 325 | 9.3 | 35.6 | 47.4 | 71.1 | 106.7 | 177.9 | 296.4 |
| 350 | 8.4 | 35.2 | 46.9 | 70.3 | 105.5 | 175.8 | 293.1 |
| 375 | 7.4 | 34.9 | 46.5 | 69.7 | 104.6 | 174.3 | 290.6 |
| 400 | 6.5 | 34.6 | 46.1 | 69.2 | 103.7 | 172.9 | 288.1 |
| 425 | 5.5 | 34.4 | 45.9 | 68.9 | 103.3 | 172.1 | 286.9 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |

Working Pressures by Classes, bar Class

NOTES:

- Use solution annealed material only.

- Not to be used over 425°C.

92

ASME  B16.5-2020

###### Table 2-3.14C Pressure–Temperature Ratings for Group 3.14 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 47Ni–22Cr–19Fe–6Mo 40Ni–29Cr–15Fe–5Mo 58Ni–33Cr–8Mo | … B462 Gr. N06030 (1), (2) B462 Gr. N06035 (1), (2) | … … … | B582 Gr. N06007 (1) B582 Gr. N06030 (1), (2) B575 Gr. N06035 (1), (2) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 250 | 650 | 865 | 1,295 | 1,945 | 3,240 | 5,400 |
| 300 | 230 | 600 | 800 | 1,200 | 1,800 | 3,000 | 5,000 |
| 400 | 200 | 565 | 750 | 1,130 | 1,690 | 2,820 | 4,700 |
| 500 | 170 | 540 | 715 | 1,075 | 1,615 | 2,690 | 4,480 |
| 600 | 140 | 520 | 690 | 1,035 | 1,555 | 2,590 | 4,320 |
| 650 | 125 | 510 | 680 | 1,020 | 1,535 | 2,555 | 4,260 |
| 700 | 110 | 505 | 675 | 1,015 | 1,520 | 2,530 | 4,220 |
| 750 | 95 | 500 | 670 | 1,005 | 1,505 | 2,510 | 4,180 |
| 800 | 80 | 500 | 665 | 1,000 | 1,500 | 2,495 | 4,160 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |

Working Pressures by Classes, psig Class

NOTES:

- Use solution annealed material only.

- Not to be used over 800°F.

93

ASME  B16.5-2020

ð20Þ

###### Table 2-3.15 Pressure–Temperature Ratings for Group 3.15 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 33Ni–42Fe–21Cr 33Ni–42Fe–21Cr | A182 Gr. N08810 (1) B564 Gr. N08810 (1) | … … | A240 Gr. N08810 (1) B409 Gr. N08810 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 15.9 | 41.4 | 55.2 | 82.7 | 124.1 | 206.8 | 344.7 |
| 50 | 15.6 | 40.6 | 54.2 | 81.3 | 121.9 | 203.2 | 338.7 |
| 100 | 14.5 | 37.8 | 50.4 | 75.6 | 113.4 | 189.0 | 315.0 |
| 150 | 13.7 | 35.9 | 47.8 | 71.7 | 107.6 | 179.3 | 298.9 |
| 200 | 13.0 | 33.9 | 45.2 | 67.9 | 101.8 | 169.6 | 282.7 |
| 250 | 12.1 | 32.3 | 43.0 | 64.5 | 96.8 | 161.3 | 268.9 |
| 300 | 10.2 | 30.7 | 41.0 | 61.5 | 92.2 | 153.7 | 256.2 |
| 325 | 9.3 | 30.1 | 40.1 | 60.1 | 90.2 | 150.3 | 250.5 |
| 350 | 8.4 | 29.4 | 39.2 | 58.8 | 88.3 | 147.1 | 245.2 |
| 375 | 7.4 | 28.7 | 38.3 | 57.4 | 86.2 | 143.6 | 239.4 |
| 400 | 6.5 | 28.3 | 37.7 | 56.5 | 84.8 | 141.3 | 235.6 |
| 425 | 5.5 | 27.7 | 36.9 | 55.3 | 83.0 | 138.4 | 230.6 |
| 450 | 4.6 | 27.2 | 36.3 | 54.4 | 81.7 | 136.1 | 226.8 |
| 475 | 3.7 | 26.8 | 35.7 | 53.5 | 80.3 | 133.9 | 223.1 |
| 500 | 2.8 | 26.3 | 35.1 | 52.6 | 79.0 | 131.6 | 219.4 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | … | 21.6 | 28.6 | 42.9 | 64.2 | 107.0 | 178.5 |
| 625 | … | 18.3 | 24.3 | 36.6 | 54.9 | 91.2 | 152.0 |
| 650 | … | 14.1 | 18.9 | 28.1 | 42.5 | 70.7 | 117.7 |
| 675 | … | 12.4 | 16.9 | 25.2 | 37.6 | 62.7 | 104.5 |
| 700 | … | 10.1 | 13.4 | 20.0 | 29.8 | 49.7 | 83.0 |
| 725 | … | 7.9 | 10.5 | 15.4 | 23.2 | 38.6 | 64.4 |
| 750 | … | 5.9 | 7.9 | 11.7 | 17.6 | 29.6 | 49.1 |
| 775 | … | 4.6 | 6.2 | 9.0 | 13.7 | 22.8 | 38.0 |
| 800 | … | 3.5 | 4.8 | 7.0 | 10.5 | 17.4 | 29.2 |
| 816 | … | 2.8 | 3.8 | 5.9 | 8.6 | 14.1 | 23.8 |

Working Pressures by Classes, bar Class

NOTE:  (1)  Use solution annealed material only.

94

ASME  B16.5-2020

###### Table 2-3.15C Pressure–Temperature Ratings for Group 3.15 Materials	ð20Þ

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 33Ni–42Fe–21Cr 33Ni–42Fe–21Cr | A182 Gr. N08810 (1) B564 Gr. N08810 (1) | … … | A240 Gr. N08810 (1) B409 Gr. N08810 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 230 | 600 | 800 | 1,200 | 1,800 | 3,000 | 5,000 |
| 200 | 210 | 550 | 735 | 1,105 | 1,655 | 2,760 | 4,600 |
| 300 | 200 | 520 | 695 | 1,040 | 1,560 | 2,605 | 4,340 |
| 400 | 190 | 490 | 655 | 980 | 1,470 | 2,450 | 4,080 |
| 500 | 170 | 465 | 620 | 925 | 1,390 | 2,315 | 3,860 |
| 600 | 140 | 440 | 585 | 880 | 1,320 | 2,195 | 3,660 |
| 650 | 125 | 430 | 575 | 860 | 1,290 | 2,150 | 3,580 |
| 700 | 110 | 420 | 555 | 835 | 1,255 | 2,090 | 3,480 |
| 750 | 95 | 410 | 545 | 820 | 1,230 | 2,050 | 3,420 |
| 800 | 80 | 400 | 535 | 800 | 1,200 | 2,005 | 3,340 |
| 850 | 65 | 395 | 525 | 785 | 1,180 | 1,970 | 3,280 |
| 900 | 50 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 950 | 35 | 380 | 505 | 760 | 1,140 | 1,895 | 3,160 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 350 | 465 | 700 | 1,050 | 1,750 | 2,915 |
| 1,100 | … | 325 | 430 | 645 | 965 | 1,610 | 2,685 |
| 1,150 | … | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,200 | … | 205 | 275 | 410 | 620 | 1,030 | 1,715 |
| 1,250 | … | 180 | 245 | 365 | 545 | 910 | 1,515 |
| 1,300 | … | 140 | 185 | 275 | 410 | 685 | 1,145 |
| 1,350 | … | 105 | 140 | 205 | 310 | 515 | 860 |
| 1,400 | … | 75 | 100 | 150 | 225 | 380 | 630 |
| 1,450 | … | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,500 | … | 40 | 55 | 85 | 125 | 205 | 345 |

Working Pressures by Classes, psig Class

NOTE:  (1)  Use solution annealed material only.

95

ASME  B16.5-2020

###### Table 2-3.16 Pressure–Temperature Ratings for Group 3.16 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 35Ni–19Cr–11∕4Si | B511 Gr. N08330 | … | B536 Gr. N08330 (1) |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 19.0 | 49.6 | 66.2 | 99.3 | 148.9 | 248.2 | 413.7 |
| 50 | 18.5 | 48.4 | 64.5 | 96.7 | 145.1 | 241.8 | 403.1 |
| 100 | 16.7 | 43.5 | 58.0 | 87.0 | 130.5 | 217.5 | 362.4 |
| 150 | 15.6 | 40.8 | 54.4 | 81.6 | 122.5 | 204.1 | 340.2 |
| 200 | 13.8 | 38.6 | 51.5 | 77.2 | 115.8 | 192.9 | 321.6 |
| 250 | 12.1 | 36.8 | 49.0 | 73.5 | 110.3 | 183.8 | 306.3 |
| 300 | 10.2 | 35.2 | 47.0 | 70.4 | 105.6 | 176.1 | 293.4 |
| 325 | 9.3 | 34.5 | 46.0 | 69.0 | 103.6 | 172.6 | 287.7 |
| 350 | 8.4 | 33.9 | 45.2 | 67.8 | 101.7 | 169.4 | 282.4 |
| 375 | 7.4 | 33.2 | 44.2 | 66.3 | 99.5 | 165.8 | 276.4 |
| 400 | 6.5 | 32.6 | 43.4 | 65.1 | 97.7 | 162.9 | 271.4 |
| 425 | 5.5 | 32.0 | 42.6 | 64.0 | 95.9 | 159.9 | 266.5 |
| 450 | 4.6 | 31.4 | 41.8 | 62.8 | 94.1 | 156.9 | 261.5 |
| 475 | 3.7 | 30.8 | 41.0 | 61.6 | 92.4 | 153.9 | 256.5 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | … | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | … | 21.9 | 29.2 | 43.7 | 65.6 | 109.4 | 182.3 |
| 600 | … | 17.4 | 23.2 | 34.8 | 52.3 | 87.1 | 145.1 |
| 625 | … | 13.8 | 18.3 | 27.5 | 41.3 | 68.8 | 114.6 |
| 650 | … | 11.0 | 14.7 | 22.1 | 33.1 | 55.1 | 91.9 |
| 675 | … | 9.1 | 12.1 | 18.2 | 27.3 | 45.6 | 75.9 |
| 700 | … | 7.6 | 10.1 | 15.2 | 22.8 | 38.0 | 63.3 |
| 725 | … | 6.1 | 8.1 | 12.2 | 18.3 | 30.5 | 50.9 |
| 750 | … | 4.8 | 6.4 | 9.5 | 14.3 | 23.8 | 39.7 |
| 775 | … | 3.9 | 5.2 | 7.7 | 11.6 | 19.4 | 32.3 |
| 800 | … | 3.1 | 4.2 | 6.3 | 9.4 | 15.6 | 26.1 |
| 816 | … | 2.6 | 3.5 | 5.2 | 7.8 | 13.0 | 21.7 |

Working Pressures by Classes, bar Class

NOTE:  (1)  Use solution annealed material only.

96

ASME  B16.5-2020

###### Table 2-3.16C Pressure–Temperature Ratings for Group 3.16 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 35Ni–19Cr–11∕4Si | B511 Gr. N08330 | … | B536 Gr. N08330 (1) |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 275 | 720 | 960 | 1,440 | 2,160 | 3,600 | 6,000 |
| 200 | 245 | 635 | 850 | 1,270 | 1,910 | 3,180 | 5,300 |
| 300 | 225 | 595 | 790 | 1,185 | 1,780 | 2,965 | 4,940 |
| 400 | 200 | 555 | 740 | 1,115 | 1,670 | 2,785 | 4,640 |
| 500 | 170 | 530 | 705 | 1,055 | 1,585 | 2,640 | 4,400 |
| 600 | 140 | 505 | 670 | 1,010 | 1,510 | 2,520 | 4,200 |
| 650 | 125 | 495 | 660 | 990 | 1,485 | 2,470 | 4,120 |
| 700 | 110 | 480 | 645 | 965 | 1,445 | 2,410 | 4,020 |
| 750 | 95 | 475 | 630 | 945 | 1,420 | 2,365 | 3,940 |
| 800 | 80 | 465 | 620 | 925 | 1,390 | 2,315 | 3,860 |
| 850 | 65 | 455 | 605 | 905 | 1,360 | 2,270 | 3,780 |
| 900 | 50 | 445 | 590 | 890 | 1,330 | 2,220 | 3,700 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | … | 345 | 455 | 685 | 1,030 | 1,715 | 2,855 |
| 1,100 | … | 265 | 355 | 535 | 800 | 1,335 | 2,230 |
| 1,150 | … | 205 | 275 | 410 | 615 | 1,030 | 1,715 |
| 1,200 | … | 160 | 215 | 320 | 485 | 805 | 1,345 |
| 1,250 | … | 130 | 175 | 260 | 390 | 650 | 1,085 |
| 1,300 | … | 105 | 140 | 215 | 320 | 530 | 885 |
| 1,350 | … | 80 | 110 | 165 | 245 | 410 | 685 |
| 1,400 | … | 60 | 80 | 125 | 185 | 310 | 515 |
| 1,450 | … | 50 | 70 | 105 | 155 | 255 | 430 |
| 1,500 | … | 40 | 50 | 75 | 115 | 190 | 315 |

Working Pressures by Classes, psig Class

NOTE:

- Use solution annealed material only.

97

ASME  B16.5-2020

###### Table 2-3.17 Pressure–Temperature Ratings for Group 3.17 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 29Ni–20.5Cr–3.5Cu–2.5Mo | … | A351 Gr. CN7M (1) | … |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 15.9 | 41.4 | 55.2 | 82.7 | 124.1 | 206.8 | 344.7 |
| 50 | 15.4 | 40.1 | 53.5 | 80.3 | 120.4 | 200.7 | 334.4 |
| 100 | 13.5 | 35.3 | 47.1 | 70.6 | 105.9 | 176.5 | 294.2 |
| 150 | 12.3 | 32.0 | 42.7 | 64.1 | 96.1 | 160.2 | 267.0 |
| 200 | 11.3 | 29.4 | 39.1 | 58.7 | 88.1 | 146.8 | 244.7 |
| 250 | 10.4 | 27.2 | 36.3 | 54.4 | 81.7 | 136.1 | 226.9 |
| 300 | 9.7 | 25.4 | 33.8 | 50.8 | 76.1 | 126.9 | 211.5 |
| 325 | 9.3 | 24.4 | 32.6 | 48.8 | 73.3 | 122.1 | 203.5 |

Working Pressures by Classes, bar Class

NOTE:  (1)  Use solution annealed material only.

###### Table 2-3.17C Pressure–Temperature Ratings for Group 3.17 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 29Ni–20.5Cr–3.5Cu–2.5Mo | … | A351 Gr. CN7M (1) | … |

| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 230 | 600 | 800 | 1,200 | 1,800 | 3,000 | 5,000 |
| 200 | 200 | 520 | 690 | 1,035 | 1,555 | 2,590 | 4,320 |
| 300 | 180 | 465 | 620 | 930 | 1,395 | 2,330 | 3,880 |
| 400 | 160 | 420 | 565 | 845 | 1,265 | 2,110 | 3,520 |
| 500 | 150 | 390 | 520 | 780 | 1,165 | 1,945 | 3,240 |
| 600 | 140 | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |

Working Pressures by Classes, psig Class

NOTE:  (1)  Use solution annealed material only.

98

ASME  B16.5-2020

###### Table 2-3.19 Pressure–Temperature Ratings for Group 3.19 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 57Ni–22Cr–14W–2Mo–La | B564 Gr. N06230 | … | B435 Gr. N06230 |

| Temp., °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | 1.4 | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | 1.4 | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | 1.4 | 21.6 | 28.6 | 42.9 | 64.2 | 107.0 | 178.5 |
| 625 | 1.4 | 18.3 | 24.3 | 36.6 | 54.9 | 91.2 | 152.0 |
| 650 | 1.4 | 14.1 | 18.9 | 28.1 | 42.5 | 70.7 | 117.7 |
| 675 | 1.4 | 12.4 | 16.9 | 25.2 | 37.6 | 62.7 | 104.5 |
| 700 | 1.4 | 10.1 | 13.4 | 20.0 | 29.8 | 49.7 | 83.0 |
| 725 | 1.4 | 7.9 | 10.5 | 15.4 | 23.2 | 38.6 | 64.4 |
| 750 | 1.4 | 5.9 | 7.9 | 11.5 | 17.6 | 29.6 | 49.1 |
| 775 | 1.4 | 4.6 | 6.2 | 9.0 | 13.7 | 22.8 | 38.0 |
| 800 | 1.4 | 3.5 | 4.8 | 7.0 | 10.5 | 17.4 | 29.2 |
| 816 | 1.4 | 2.8 | 3.8 | 5.9 | 8.6 | 14.1 | 23.8 |

Working Pressures by Classes, bar Class

99

ASME  B16.5-2020

###### Table 2-3.19C Pressure–Temperature Ratings for Group 3.19 Materials

| Nominal Designation | Forgings | Castings | Plates |
| --- | --- | --- | --- |
| 57Ni–22Cr–14W–2Mo–La | B564 Gr. N06230 | … | B435 Gr. N06230 |

Working Pressures by Classes, psig

|  |  |  |  | Class |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temp., °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | 20 | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | 20 | 325 | 430 | 645 | 965 | 1,610 | 2,685 |
| 1,150 | 20 | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,200 | 20 | 205 | 275 | 410 | 620 | 1,030 | 1,715 |
| 1,250 | 20 | 180 | 245 | 365 | 545 | 910 | 1,515 |
| 1,300 | 20 | 140 | 185 | 275 | 410 | 685 | 1,145 |
| 1,350 | 20 | 105 | 140 | 205 | 310 | 515 | 860 |
| 1,400 | 20 | 75 | 100 | 150 | 225 | 380 | 630 |
| 1,450 | 20 | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,500 | 20 | 40 | 55 | 85 | 125 | 205 | 345 |

100

ASME  B16.5-2020

###### Table 3 Permissible Imperfections in Flange Facing Finish for Raised Face and Large Male and Female Flanges	ð20Þ

| NPS | Maximum Radial Projection of Imperfections That Are No Deeper Than the Bottom of the Serrations, mm | Maximum Radial Projection of Imperfections That Are Deeper Than the Bottom of the Serrations, mm | Maximum Depth of Imperfections That Are Deeper Than the Bottom of the Serrations, mm |
| --- | --- | --- | --- |
| 1∕2 | 3.0 | 1.5 | 1.5 |
| 3∕4 | 3.0 | 1.5 | 1.5 |
| 1 | 3.0 | 1.5 | 1.5 |
| 11∕4 | 3.0 | 1.5 | 1.5 |
| 11∕2 | 3.0 | 1.5 | 1.5 |
| 2 | 3.0 | 1.5 | 1.5 |
| 21∕2 | 3.0 | 1.5 | 1.5 |
| 3 | 4.6 | 1.5 | 1.5 |
| 31∕2 | 6.4 | 3.0 | 3.0 |
| 4 | 6.4 | 3.0 | 3.0 |
| 5 | 6.4 | 3.0 | 3.0 |
| 6 | 6.4 | 3.0 | 3.0 |
| 8 | 7.9 | 4.6 | 4.6 |
| 10 | 7.9 | 4.6 | 4.6 |
| 12 | 7.9 | 4.6 | 4.6 |
| 14 | 7.9 | 4.6 | 4.6 |
| 16 | 9.6 | 4.6 | 4.6 |
| 18 | 12.7 | 6.4 | 6.4 |
| 20 | 12.7 | 6.4 | 6.4 |
| 22 | 12.7 | 6.4 | 6.4 |
| 24 | 12.7 | 6.4 | 6.4 |

101

ASME  B16.5-2020

ð20Þ

###### Table 3C Permissible Imperfections in Flange Facing Finish for Raised Face and Large Male and Female Flanges

| NPS | Maximum Radial Projection of Imperfections That Are No Deeper Than the Bottom of the Serrations, in. | Maximum Radial Projection of Imperfections That Are Deeper Than the Bottom of the Serrations, in. | Maximum Depth of Imperfections That Are Deeper Than the Bottom of the Serrations, in. |
| --- | --- | --- | --- |
| 1∕2 | 0.12 | 0.06 | 0.06 |
| 3∕4 | 0.12 | 0.06 | 0.06 |
| 1 | 0.12 | 0.06 | 0.06 |
| 11∕4 | 0.12 | 0.06 | 0.06 |
| 11∕2 | 0.12 | 0.06 | 0.06 |
| 2 | 0.12 | 0.06 | 0.06 |
| 21∕2 | 0.12 | 0.06 | 0.06 |
| 3 | 0.18 | 0.06 | 0.06 |
| 31∕2 | 0.25 | 0.12 | 0.12 |
| 4 | 0.25 | 0.12 | 0.12 |
| 5 | 0.25 | 0.12 | 0.12 |
| 6 | 0.25 | 0.12 | 0.12 |
| 8 | 0.31 | 0.18 | 0.18 |
| 10 | 0.31 | 0.18 | 0.18 |
| 12 | 0.31 | 0.18 | 0.18 |
| 14 | 0.31 | 0.18 | 0.18 |
| 16 | 0.38 | 0.18 | 0.18 |
| 18 | 0.50 | 0.25 | 0.25 |
| 20 | 0.50 | 0.25 | 0.25 |
| 22 | 0.50 | 0.25 | 0.25 |
| 24 | 0.50 | 0.25 | 0.25 |

102

###### Table 4 Dimensions of Facings (Other Than Ring Joints, All Pressure Rating Classes)	ð20Þ

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

Minimum Outside Diameter of Raised Portion

Outside Diameter

Outside Diameter

Height

(6), (7)

ASME  B16.5-2020

103

| Nominal Pipe Size | Raised Face Large Male and Large Tongue, R | Small Male, S (1) | Small Tongue, T | Inside Diameter of Large and Small Tongue, U | Inside Diameter of Small Tongue (1) | Large Female and Large Groove, W | Small Female, X (1) | Small Groove, Y | Inside Diameter of Large and Small Groove, Z | Raised Face (2), (3) | Large and Small Male and Tongue (2), (4) | Depth of Groove or Female (2), (5) | Small Female and Groove, K | Large Female and Groove, L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1∕2 | 35.1 | 18.3 | 35.1 | 25.4 | … | 36.6 | 19.8 | 36.6 | 23.9 | … | … | … | 44 | 46 |
| 3∕4 | 42.9 | 23.9 | 42.9 | 33.3 | … | 44.4 | 25.4 | 44.4 | 31.8 | … | … | … | 52 | 54 |
| 1 | 50.8 | 30.2 | 47.8 | 38.1 | … | 52.3 | 31.8 | 49.3 | 36.6 | … | … | … | 57 | 62 |
| 11∕4 | 63.5 | 38.1 | 57.2 | 47.8 | … | 65.1 | 39.6 | 58.7 | 46.0 | … | … | … | 67 | 75 |
| 11∕2 | 73.0 | 44.4 | 63.5 | 53.8 | … | 74.7 | 46.0 | 65.0 | 52.3 | … | … | … | 73 | 84 |
| 2 | 91.9 | 57.2 | 82.6 | 73.2 | … | 93.7 | 58.7 | 84.1 | 71.4 | … | … | … | 92 | 103 |
| 21∕2 | 104.6 | 68.3 | 95.2 | 85.9 | … | 106.4 | 69.8 | 96.8 | 84.1 | … | … | … | 105 | 116 |
| 3 | 127.0 | 84.1 | 117.3 | 108.0 | … | 128.5 | 85.9 | 119.1 | 106.4 | … | … | … | 127 | 138 |
| 31∕2 | 139.7 | 96.8 | 130.0 | 120.6 | … | 141.2 | 98.6 | 131.8 | 119.1 | … | … | … | 140 | 151 |
| 4 | 157.2 | 109.5 | 144.5 | 131.8 | … | 158.8 | 111.3 | 146.0 | 130.0 | … | … | … | 157 | 168 |
| 5 | 185.7 | 136.7 | 173.0 | 160.3 | … | 187.5 | 138.2 | 174.8 | 158.8 | … | … | … | 186 | 197 |
| 6 | 215.9 | 162.1 | 203.2 | 190.5 | … | 217.4 | 163.6 | 204.7 | 189.0 | … | … | … | 216 | 227 |
| 8 | 269.7 | 212.9 | 254.0 | 238.3 | … | 271.5 | 214.4 | 255.5 | 236.5 | … | … | … | 270 | 281 |
| 10 | 323.8 | 266.7 | 304.8 | 285.8 | … | 325.4 | 268.2 | 306.3 | 284.2 | … | … | … | 324 | 335 |
| 12 | 381.0 | 317.5 | 362.0 | 342.9 | … | 382.5 | 319.0 | 363.5 | 341.4 |  |  |  | 381 | 392 |
| 14 | 412.8 | 349.2 | 393.7 | 374.6 | … | 414.3 | 350.8 | 395.2 | 373.1 | … | … | … | 413 | 424 |
| 16 | 469.9 | 400.0 | 447.5 | 425.4 | … | 471.4 | 401.6 | 449.3 | 423.9 | … | … | … | 470 | 481 |
| 18 | 533.4 | 450.8 | 512.0 | 489.0 | … | 534.9 | 452.4 | 512.8 | 487.4 | … | … | … | 533 | 545 |
| 20 | 584.2 | 501.6 | 558.8 | 533.4 | … | 585.7 | 503.2 | 560.3 | 531.9 | … | … | … | 584 | 595 |
| 22 | 641.4 | … | … | … | … | … | … | … | … | … | … | … | … | … |
| 24 | 692.2 | 603.2 | 666.8 | 641.4 | … | 693.7 | 604.8 | 668.3 | 639.8 | … | … | … | 692 | 703 |

GENERAL NOTES:

  - Dimensions are in millimeters.

  - For facing requirements for flanges and flanged fittings, see paras. 6.3 and 6.4 and Figure 6.

  - For facing requirements for lapped joints, see para. 6.4.3 and Figure 6.

  - For facing tolerances, see para. 7.3.

ASME  B16.5-2020

104

###### Table 4 Dimensions of Facings (Other Than Ring Joints, All Pressure Rating Classes) (Cont’d)

NOTES:

    - For small male and female joints, care should be taken in the use of these dimensions to ensure that the inside diameter of the fitting or pipe is small enough to permit sufficient bearing surface to prevent the crushing of the gasket. This applies particularly on lines where the joint is made on the end of the pipe. The inside diameter of the fitting should match the inside diameter of the pipe as specified by the purchaser. Threaded companion flanges for small male and female joints are furnished with plain face and are threaded with American National Standard Locknut Thread (NPSL).

    - See para. 6.4.3 and Figure 6 for thickness and outside diameters of laps.

    - The height of the raised face is either 1.5 mm or 6.4 mm (see para. 6.4.1).

    - The height of the large and small male and tongue is 6.4 mm.

    - The depth of the groove or female is 4.8 mm.

    - The raised portion of the full face may be furnished unless otherwise specified on order.

    - Large male and female faces and large tongue and groove are not applicable to Class 150 because of potential dimensional conflicts.

ASME  B16.5-2020

105

###### Table 4C Dimensions of Facings (Other Than Ring Joints, All Pressure Rating Classes)

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

Minimum Outside Diameter of Raised Portion

| Nominal Pipe Size | Outside Diamet Raised Face Large Male and Large Small Tongue, Male, R S (1) |  | er Small Tongue, T | Inside Diameter of Large and Small Tongue, U | Inside Diameter of Small Male (1) | Outside Diameter Large Female and Large Small Small Groove, Female, Groove, W X (1) Y |  |  | Inside Diameter of Large and Small Groove, Z | Height Large and Small Raised Male and Face Tongue (2), (3) (2), (4) |  | Depth of Groove or Female (2), (5) | (6), Small Female and Groove, K | (7) Large Female and Groove, L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1∕2 | 1.38 | 0.72 | 1.38 | 1.00 | … | 1.44 | 0.78 | 1.44 | 0.94 | … | … | … | 1.75 | 1.81 |
| 3∕4 | 1.69 | 0.94 | 1.69 | 1.31 | … | 1.75 | 1.00 | 1.75 | 1.25 | … | … | … | 2.06 | 2.12 |
| 1 | 2.00 | 1.19 | 1.88 | 1.50 | … | 2.06 | 1.25 | 1.94 | 1.44 | … | … | … | 2.25 | 2.44 |
| 11∕4 | 2.50 | 1.50 | 2.25 | 1.88 | … | 2.56 | 1.56 | 2.31 | 1.81 | … | … | … | 2.62 | 2.94 |
| 11∕2 | 2.88 | 1.75 | 2.50 | 2.12 | … | 2.94 | 1.81 | 2.56 | 2.06 | … | … | … | 2.88 | 3.31 |
| 2 | 3.62 | 2.25 | 3.25 | 2.88 | … | 3.69 | 2.31 | 3.31 | 2.81 | … | … | … | 3.62 | 4.06 |
| 21∕2 | 4.12 | 2.69 | 3.75 | 3.38 | … | 4.19 | 2.75 | 3.81 | 3.31 | … | … | … | 4.12 | 4.56 |
| 3 | 5.00 | 3.31 | 4.62 | 4.25 | … | 5.06 | 3.38 | 4.69 | 4.19 | … | … | … | 5.00 | 5.44 |
| 31∕2 | 5.50 | 3.81 | 5.12 | 4.75 | … | 5.56 | 3.88 | 5.19 | 4.69 | … | … | … | 5.50 | 5.94 |
| 4 | 6.19 | 4.31 | 5.69 | 5.19 | … | 6.25 | 4.38 | 5.75 | 5.12 | … | … | … | 6.19 | 6.62 |
| 5 | 7.31 | 5.38 | 6.81 | 6.31 | … | 7.38 | 5.44 | 6.88 | 6.25 | … | … | … | 7.31 | 7.75 |
| 6 | 8.50 | 6.38 | 8.00 | 7.50 | … | 8.56 | 6.44 | 8.06 | 7.44 | … | … | … | 8.50 | 8.94 |
| 8 | 10.62 | 8.38 | 10.00 | 9.38 | … | 10.69 | 8.44 | 10.06 | 9.31 | … | … | … | 10.62 | 11.06 |
| 10 | 12.75 | 10.50 | 12.00 | 11.25 | … | 12.81 | 10.56 | 12.06 | 11.19 | … | … | … | 12.75 | 13.19 |
| 12 | 15.00 | 12.50 | 14.25 | 13.50 | … | 15.06 | 12.56 | 14.31 | 13.44 | … | … | … | 15.00 | 15.44 |
| 14 | 16.25 | 13.75 | 15.50 | 14.75 | … | 16.31 | 13.81 | 15.56 | 14.69 | … | … | … | 16.25 | 16.69 |
| 16 | 18.50 | 15.75 | 17.62 | 16.75 | … | 18.56 | 15.81 | 17.69 | 16.69 | … | … | … | 18.50 | 18.94 |
| 18 | 21.00 | 17.75 | 20.12 | 19.25 | … | 21.06 | 17.81 | 20.19 | 19.19 | … | … | … | 21.00 | 21.44 |
| 20 | 23.00 | 19.75 | 22.00 | 21.00 | … | 23.06 | 19.81 | 22.06 | 20.94 | … | … | … | 23.00 | 23.44 |
| 22 | 25.25 | … | … | … | … | … | … | … | … | … | … | … | … | … |
| 24 | 27.25 | 23.75 | 26.25 | 25.25 | … | 27.31 | 23.81 | 26.31 | 25.19 | … | … | … | 27.25 | 27.69 |

GENERAL NOTES:

      - Dimensions are in inches.

      - For facing requirements for flanges and flanged fittings, see paras. 6.3 and 6.4 and Figure 6.

      - For facing requirements for lapped Joints, see para. 6.4.3 and Figure 6.

      - For facing tolerances, see para. 7.3.

ASME  B16.5-2020

106

###### Table 4C Dimensions of Facings (Other Than Ring Joints, All Pressure Rating Classes) (Cont’d)

NOTES:

        - For small male and female joints, care should be taken in the use of these dimensions to ensure that the inside diameter of fitting or pipe is small enough to permit sufficient bearing surface to prevent the crushing of the gasket. This applies particularly where the joint is made on the end of the pipe. The inside diameter of the fitting should match the inside diameter of the pipe as specified by the purchaser. Threaded companion flanges for small male and female joints are furnished with plain face and are threaded with American National Standard Locknut Thread (NPSL).

        - See para. 6.4.3 and Figure 6 for thickness and outside diameters of laps.

        - The height of the raised face is either 0.06 in. or 0.25 in. (see para. 6.4.1).

        - The height of the large and small male and tongue is 0.25 in.

        - The depth of the groove or female is 0.19 in.

        - The raised portion of the full face may be furnished unless otherwise specified on order.

        - Large male and female faces and large tongue and groove are not applicable to Class 150 because of potential dimensional conflicts.

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

107

ASME  B16.5-2020

ð20Þ

###### Table 5 Dimensions of Ring Joint Facings (All Pressure Rating Classes)

1	2	3	4	5	6	7	8	9	10	11	12

Nominal Size

Groove Dimensions 	   Radius

Class 150 NPS

Class 300 NPS

Class 400

NPS (2)

Class 600 NPS

Class 900

NPS (3)

Class 1500 NPS

Class 2500 NPS

Groove Number

Pitch Diameter, P

Depth, E

(1)

Width,

F

at Bottom, R

| … | 11 | 34.14 | 5.56 | 7.14 | 0.8 |
| --- | --- | --- | --- | --- | --- |
| … | 12 | 39.67 | 6.35 | 8.74 | 0.8 |

…	1∕2	…	1∕2	…	…

…	…	…	…	…	1∕2

…	3∕4	…	3∕4	…	…	1∕2	13	42.88	6.35	8.74	0.8

| … … … … … 3∕4 … |  |  |  |  |  |  | 14 | 44.45 | 6.35 | 8.74 | 0.8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | … | … | … | … | … | … | 15 | 47.62 | 6.35 | 8.74 | 0.8 |

…	1	…	1	…	1	3∕4	16	50.80	6.35	8.74	0.8

11∕4	…	…	…	…	…	…	17	57.15	6.35	8.74	0.8

…	11∕4	…	11∕4	…	11∕4	1	18	60.33	6.35	8.74	0.8

11∕2	…	…	…	…	…	…	19	65.07	6.35	8.74	0.8

…	11∕2	…	11∕2	…	11∕2	…	20	68.28	6.35	8.74	0.8

…	…	…	…	…	…	11∕4	21	72.24	7.92	11.91	0.8

2	…	…	…	…	…	…	22	82.55	6.35	8.74	0.8

…	2	…	2	…	…	11∕2	23	82.55	7.92	11.91	0.8

…	…	…	…	…	2	…	24	95.25	7.92	11.91	0.8

21∕2	…	…	…	…	…	…	25	101.60	6.35	8.74	0.8

…	21∕2	…	21∕2	…	…	2	26	101.60	7.92	11.91	0.8

…	…	…	…	…	21∕2	…	27	107.95	7.92	11.91	0.8

…	…	…	…	…	…	21∕2	28	111.12	9.522	13.49	1.5

3	…	…	…	…	…	…	29	114.30	6.35	8.74	0.8

…	(4)	…	(4)	…	…	…	30	117.48	7.92	11.91	0.8

…	3 (4)	…	3 (4)	3	…	…	31	123.82	7.92	11.91	0.8

…	…	…	…	…	…	3	32	127.00	9.53	13.49	1.5

31∕2	…	…	…	…	…	…	33	131.78	6.35	8.74	0.8

…	31∕2	…	31∕2	…	…	…	34	131.78	7.92	11.91	0.8

…	…	…	…	…	3	…	35	136.52	7.92	11.91	0.8

4	…	…	…	…	…	…	36	149.22	6.35	8.74	0.8

…	4	4	4	4	…	…	37	149.22	7.92	11.91	0.8

…	…	…	…	…	…	4	38	157.18	11.13	16.66	1.5

…	…	…	…	…	4	…	39	161.92	7.92	11.91	0.8

5	…	…	…	…	…	…	40	171.45	6.35	8.74	0.8

…	5	5	5	5	…	…	41	180.98	7.92	11.91	0.8

108

ASME  B16.5-2020

###### Table 5 Dimensions of Ring Joint Facings (All Pressure Rating Classes)

| Diameter of Raised Portion, |  |  |  | K | Approximate Distance Between Flanges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class |
|  | 300 |  |  |  |  |  |  |  |  |  |  |
| 150 | 400 600 | 900 | 1500 | 2500 | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |

13	14	15	16	17	18	19	20	21	22	23	24

| … … … … 63.5 | 51.0 … 63.5 … … | … … … … … | … 60.5 … 66.5 … | … … 65.0 … … | … … … … 4 | 3 … 4 … … | … … … … … | 3 … 4 … … | … … … … … | … 4 … 4 … | … … 4 … … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | 69.8 | … | 71.4 | 73.2 | … | 4 | … | 4 | … | 4 | 4 |
| 73.2 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 79.2 | … | 81.0 | 82.6 | … | 4 | … | 4 | … | 4 | 4 |
| 82.6 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 90.4 | … | 91.9 | … | … | 4 | … | 4 | … | 4 | … |
| … | … | … | … | 102 | … | … | … | … | … | … | 3 |
| 102 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 108 | … | … | 114 | … | 6 | … | 5 | … | … | 3 |
| … | … | … | 124 | … | … | … | … | … | … | 3 | … |
| 121 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 127 | … | … | 133 | … | 6 | … | 5 | … | … | 3 |
| … | … | … | 137 | … | … | … | … | … | … | 3 | … |
| … | … | … | … | 149 | … | … | … | … | … | … | 3 |
| 133 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | … | … | … | … | … | … | … | … | … | … | … |
| … | 146 | 155 | … | … | … | 6 | … | 5 | 4 | … | … |
| … | … | … | … | 168 | … | … | … | … | … | … | 3 |
| 154 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 159 | … | … | … | … | 6 | … | 5 | … | … | … |
| … | … | … | 168 | … | … | … | … | … | … | 3 | … |
| 171 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 175 | 181 | … | … | … | 6 | 6 | 5 | 4 | … | … |
| … | … | … | … | 203 | … | … | … | … | … | … | 4 |
| … | … | … | 194 | … | … | … | … | … | … | 3 | … |
| 194 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 210 | 216 | … | … | … | 6 | 6 | 5 | 4 | … | … |
|  |  |  |  |  |  | 109 |  |  |  |  |  |

ASME  B16.5-2020

###### Table 5 Dimensions of Ring Joint Facings (All Pressure Rating Classes)  (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12

Nominal Size

Groove Dimensions 	   Radius

| Class 150 NPS | Class 300 NPS | Class 400 NPS (2) | Class 600 NPS | Class 900 NPS (3) | Class 1500 NPS | Class 2500 NPS | Groove Number | Pitch Diameter, P | Depth, E (1) | Width, F | at Bottom, R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | 5 | 42 | 190.50 | 12.70 | 19.84 | 1.5 |
| 6 | … | … | … | … | … | … | 43 | 193.68 | 6.35 | 8.74 | 0.8 |
| … | … | … | … | … | 5 | … | 44 | 193.68 | 7.92 | 11.91 | 0.8 |
| … | 6 | 6 | 6 | 6 | … | … | 45 | 211.12 | 7.92 | 11.91 | 0.8 |
| … | … | … | … | … | 6 | … | 46 | 211.14 | 9.52 | 13.49 | 1.5 |
| … | … | … | … | … | … | 6 | 47 | 228.60 | 12.70 | 19.84 | 1.5 |
| 8 | … | … | … | … | … | … | 48 | 247.65 | 6.35 | 8.74 | 0.8 |
| … | 8 | 8 | 8 | 8 | … | … | 49 | 269.88 | 7.92 | 11.91 | 0.8 |
| … | … | … | … | … | 8 | … | 50 | 269.88 | 11.13 | 16.66 | 1.5 |
| … | … | … | … | … | … | 8 | 51 | 279.40 | 14.27 | 23.01 | 1.5 |
| 10 | … | … | … | … | … | … | 52 | 304.80 | 6.35 | 8.74 | 0.8 |
| … | 10 | 10 | 10 | 10 | … | … | 53 | 323.85 | 7.92 | 11.91 | 0.8 |
| … | … | … | … | … | 10 | … | 54 | 323.85 | 11.13 | 16.66 | 1.5 |
| … | … | … | … | … | … | 10 | 55 | 342.90 | 17.48 | 30.18 | 2.3 |
| 12 | … | … | … | … | … | … | 56 | 381.00 | 6.35 | 8.74 | 0.8 |
| … | 12 | 12 | 12 | 12 | … | … | 57 | 381.00 | 7.92 | 11.91 | 0.8 |
| … | … | … | … | … | 12 | … | 58 | 381.00 | 14.27 | 23.01 | 1.5 |
| 14 | … | … | … | … | … | … | 59 | 396.88 | 6.35 | 8.74 | 0.8 |
| … | … | … | … | … | … | 12 | 60 | 406.40 | 17.48 | 33.32 | 2.3 |
| … | 14 | 14 | 14 | … | … | … | 61 | 419.10 | 7.92 | 11.91 | 0.8 |
| … | … | … | … | 14 | … | … | 62 | 419.10 | 11.13 | 16.66 | 1.5 |
| … | … | … | … | … | 14 | … | 63 | 419.10 | 15.88 | 26.97 | 2.3 |
| 16 | … | … | … | … | … | … | 64 | 454.02 | 6.35 | 8.74 | 0.8 |
| … | 16 | 16 | 16 | … | … | … | 65 | 469.90 | 7.92 | 11.91 | 0.8 |
| … | … | … | … | 16 | … | … | 66 | 469.90 | 11.13 | 16.66 | 1.5 |
| … | … | … | … | … | 16 | … | 67 | 469.90 | 17.48 | 30.18 | 2.3 |
| 18 | … | … | … | … | … | … | 68 | 517.52 | 6.35 | 8.74 | 0.8 |
| … | 18 | 18 | 18 | … | … | … | 69 | 533.40 | 7.92 | 11.91 | 0.8 |
| … | … | … | … | 18 | … | … | 70 | 533.40 | 12.70 | 19.84 | 1.5 |
| … | … | … | … | … | 18 | … | 71 | 533.40 | 17.48 | 30.18 | 2.3 |
| 20 | … | … | … | … | … | … | 72 | 558.80 | 6.35 | 8.74 | 0.8 |
| … | 20 | 20 | 20 | … | … | … | 73 | 584.20 | 9.52 | 13.49 | 1.5 |
|  |  |  |  |  | 110 |  |  |  |  |  |  |

ASME  B16.5-2020

###### Table 5 Dimensions of Ring Joint Facings (All Pressure Rating Classes)  (Cont’d)

| Diameter of Raised Portion, |  |  |  | K | Approximate Distance Between Flanges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class |
|  | 300 |  |  |  |  |  |  |  |  |  |  |
| 150 | 400 600 | 900 | 1500 | 2500 | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |

13	14	15	16	17	18	19	20	21	22	23	24

| … 219 … … | … … … 241 | … … … 241 | … … 229 … | 241 … … … | … 4 … … | … … … 6 | … … … 6 | … … … 5 | … … … 4 | … … 3 … | 4 … … … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | 248 | … | … | … | … | … | … | 3 | … |
| … | … | … | … | 279 | … | … | … | … | … | … | 4 |
| 273 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 302 | 308 | … | … | … | 6 | 6 | 5 | 4 | … | … |
| … | … | … | 318 | … | … | … | … | … | … | 4 | … |
| … | … | … | … | 340 | … | … | … | … | … | … | 5 |
| 330 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 356 | 362 | … | … | … | 6 | 6 | 5 | 4 | … | … |
| … | … | … | 371 | … | … | … | … | … | … | 4 | … |
| … | … | … | … | 425 | … | … | … | … | … | … | 6 |
| 406 | … | … | … | … | 4 | … | … | … | … | … | … |
| … | 413 | 419 | … | … | … | 6 | 6 | 5 | 4 | … | … |
| … | … | … | 438 | … | … | … | … | … | … | 5 | … |
| 425 | … | … | … | … | 3 | … | … | … | … | … | … |
| … | … | … | … | 495 | … | … | … | … | … | … | 8 |
| … | 457 | … | … | … | … | 6 | 6 | 5 | … | … | … |
| … | … | 467 | … | … | … | … | … | … | 4 | … | … |
| … | … | … | 489 | … | … | … | … | … | … | 6 | … |
| 483 | … | … | … | … | 3 | … | … | … | … | … | … |
| … | 508 | … | … | … | … | 6 | 6 | 5 | … | … | … |
| … | … | 524 | … | … | … | … | … | … | 4 | … | … |
| … | … | … | 546 | … | … | … | … | … | … | 8 | … |
| 546 | … | … | … | … | 3 | … | … | … | … | … | … |
| … | 575 | … | … | … | … | 6 | 6 | 5 | … | … | … |
| … | … | 594 | … | … | … | … | … | … | 5 | … | … |
| … | … | … | 613 | … | … | … | … | … | … | 8 | … |
| 597 | … | … | … | … | 3 | … | … | … | … | … | … |
| … | 635 | … | … | … | … | 6 | 6 | 5 | … | … | … |
|  |  |  |  |  |  | 111 |  |  |  |  |  |

ASME  B16.5-2020

###### Table 5 Dimensions of Ring Joint Facings (All Pressure Rating Classes)  (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12

Nominal Size

Groove Dimensions 	   Radius

| 150 NPS | Class 300 NPS | Class 400 NPS (2) | Class 600 NPS | Class 900 NPS (3) | 1500 NPS | 2500 NPS | Groove Diameter, Depth, E Width, Bottom Number P (1) F R |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | 20 | … | … | 74 | 584.20 | 12.70 | 19.84 | 1.5 |
| … | … | … | … | … | 20 | … | 75 | 584.20 | 17.48 | 33.32 | 2.3 |
| 22 | … | … | … | … | … | … | 80 | 615.95 | 6.35 | 8.74 | 0.8 |
| … | 22 | 22 | 22 | … | … | … | 81 | 635.00 | 11.13 | 15.09 | 1.5 |
| 24 | … | … | … | … | … | … | 76 | 673.10 | 6.35 | 8.74 | 0.8 |
| … | 24 | 24 | 24 | … | … | … | 77 | 692.15 | 11.13 | 16.66 | 1.5 |
| … | … | … | … | 24 | … | … | 78 | 692.15 | 15.88 | 26.97 | 2.3 |
| … | … | … | … | … | 24 | … | 79 | 692.15 | 20.62 | 36.53 | 2.3 |

Class

Class

Class

Pitch	at

,

112

ASME  B16.5-2020

###### Table 5 Dimensions of Ring Joint Facings (All Pressure Rating Classes)  (Cont’d)

| Diameter of Raised Portion, |  |  |  | K | Approximate Distance Between Flanges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class |
|  | 300 |  |  |  |  |  |  |  |  |  |  |
| 150 | 400 600 | 900 | 1500 | 2500 | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |

13	14	15	16	17	18	19	20	21	22	23	24

| … … | … … | 648 … | … 673 | … … | … … | … … | … … | … … | 5 … | … 10 | … … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 648 | … | … | … | … | 3 | … | … | … | … | … | … |
| … | 686 | … | … | … | … | 6 | 6 | 6 | … | … | … |
| 711 | … | … | … | … | 3 | … | … | … | … | … | … |
| … | 749 | … | … | … | … | 6 | 6 | 6 | … | … | … |
| … | … | 772 | … | … | … | … | … | … | 6 | … | … |
| … | … | … | 794 | … | … | … | … | … | … | 11 | … |

113

ASME  B16.5-2020

###### Table 5 Dimensions of Ring Joint Facings (All Pressure Rating Classes) (Cont’d)

GENERAL NOTES:

          - Dimensions are in millimeters.

          - For facing requirements for flanges and flanged fitting, see para. 6.4.1 and Figure 6.

          - For facing requirements for lapped joints, see para. 6.4.3 and Figure 6.

          - See para. 4.2.7 for marking requirements.

NOTES:

            - The height of the raised portion is equal to the depth of the groove dimension, E, but is not subjected to the tolerances for E. Former full-face contour may be used.

            - Use Class 600 in sizes NPS 1∕2 to NPS 31∕2 for Class 400.

            - Use Class 1500 in sizes NPS 1∕2 to NPS 21∕2 for Class 900.

            - For ring joints with lapped flanges in Classes 300 and 600, ring and groove number R30 is used instead of R31.

TOLERANCES:

E   = (depth) +0.41, −0.00 mm

F    = (width) ±0.20 mm

P    = (pitch diameter) ±0.13 mm

R    = (radius at bottom)

= R ≤ 1.5 + 0.8, −0.0 mm

= R > 1.5 ± 0.8 mm

23   = deg (angle) ± 1∕2 deg

114

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

115

ASME  B16.5-2020

ð20Þ

###### Table 5C Dimensions of Ring Joint Facings (All Pressure Rating Classes)

1	2	3	4	5	6	7	8	9	10	11	12

Nominal Pipe Size

Groove Dimensions

Class

150	Class 300

Class

400 (2)    Class 600

Class 900 (3)

Class 1500

Class 2500

Groove Number

Pitch Diameter, P

Depth,

E (1)

Width,

F

Radius at Bottom,

R

| … | R11 | 1.344 | 0.219 | 0.281 | 0.03 |
| --- | --- | --- | --- | --- | --- |
| … | 12 | 1.562 | 0.250 | 0.344 | 0.03 |

…	1∕2	…	1∕2	…	…

…	…	…	…	…	1∕2

…	3∕4	…	3∕4	…	…	1∕2	13	1.688	0.250	0.344	0.03

| … … … … … 3∕4 … |  |  |  |  |  |  | 14 | 1.750 | 0.250 | 0.344 | 0.03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | … | … | … | … | … | … | 15 | 1.875 | 0.250 | 0.344 | 0.03 |

…	1	…	1	…	1	3∕4	16	2.000	0.250	0.344	0.03

11∕4	…	…	…	…	…	…	17	2.250	0.250	0.344	0.03

…	11∕4	…	11∕4	…	11∕4	1	18	2.375	0.250	0.344	0.03

11∕2	…	…	…	…	…	…	19	2.562	0.250	0.344	0.03

…	11∕2	…	11∕2	…	11∕2	…	20	2.688	0.250	0.344	0.03

…	…	…	…	…	…	11∕4	21	2.844	0.312	0.469	0.03

2	…	…	…	…	…	…	22	3.250	0.250	0.344	0.03

…	2	…	2	…	…	11∕2	23	3.250	0.312	0.469	0.03

…	…	…	…	…	2	…	24	3.750	0.312	0.469	0.03

21∕2	…	…	…	…	…	…	25	4.000	0.250	0.344	0.03

…	21∕2	…	21∕2	…	…	2	26	4.000	0.312	0.469	0.03

…	…	…	…	…	21∕2	…	27	4.250	0.312	0.469	0.03

…	…	…	…	…	…	21∕2	28	4.375	0.375	0.531	0.06

3	…	…	…	…	…	…	29	4.500	0.250	0.344	0.03

…	(4)	…	(4)	…	…	…	30	4.625	0.312	0.469	0.03

…	3 (4)	…	3 (4)	3	…	…	31	4.875	0.312	0.469	0.03

…	…	…	…	…	…	3	32	5.000	0.375	0.531	0.06

31∕2	…	…	…	…	…	…	33	5.188	0.250	0.344	0.03

…	31∕2	…	31∕2	…	…	…	34	5.188	0.312	0.469	0.03

…	…	…	…	…	3	…	35	5.375	0.312	0.469	0.03

4	…	…	…	…	…	…	36	5.875	0.250	0.344	0.03

…	4	4	4	4	…	…	37	5.875	0.312	0.469	0.03

…	…	…	…	…	…	4	38	6.188	0.438	0.656	0.06

…	…	…	…	…	4	…	39	6.375	0.312	0.469	0.03

5	…	…	…	…	…	…	40	6.750	0.250	0.344	0.03

…	5	5	5	5	…	…	41	7.125	0.312	0.469	0.03

116

ASME  B16.5-2020

###### Table 5C Dimensions of Ring Joint Facings (All Pressure Rating Classes)

| Diameter of Raised Portion, K |  |  |  |  | Approximate Distance Between Flanges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class |
|  | 300 |  |  |  |  |  |  |  |  |  |  |
|  | 400 |  |  |  |  |  |  |  |  |  |  |
| 150 | 600 | 900 | 1500 | 2500 | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |

13	14	15	16	17	18	19	20	21	22	23	24

| … … … … 2.50 | 2.00 … 2.50 … … | … … … … … | … 2.38 … 2.62 … | … … 2.56 … … | … … … … 0.16 | 0.12 … 0.16 … … | … … … … … | 0.12 … 0.16 … … | … … … … … | … 0.16 … 0.16 … | … … 0.16 … … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | 2.75 | … | 2.81 | 2.88 | … | 0.16 | … | 0.16 | … | 0.16 | 0.16 |
| 2.88 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 3.12 | … | 3.19 | 3.25 | … | 0.16 | … | 0.16 | … | 0.16 | 0.16 |
| 3.25 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 3.56 | … | 3.62 | … | … | 0.16 | … | 0.16 | … | 0.16 | … |
| … | … | … | … | 4.00 | … | … | … | … | … | … | 0.12 |
| 4.00 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 4.25 | … | … | 4.50 | … | 0.22 | … | 0.19 | … | … | 0.12 |
| … | … | … | 4.88 | … | … | … | … | … | … | 0.12 | … |
| 4.75 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 5.00 | … | … | 5.25 | … | 0.22 | … | 0.19 | … | … | 0.12 |
| … | … | … | 5.38 | … | … | … | … | … | … | 0.12 | … |
| … | … | … | … | 5.88 | … | … | … | … | … | … | 0.12 |
| 5.25 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | … | … | … | … | … | … | … | … | … | … | … |
| … | 5.75 | 6.12 | … | … | … | 0.22 | … | 0.19 | 0.16 | … | … |
| … | … | … | … | 6.62 | … | … | … | … | … | … | 0.12 |
| 6.06 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 6.25 | … | … | … | … | 0.22 | … | 0.19 | … | … | … |
| … | … | … | 6.62 | … | … | … | … | … | … | 0.12 | … |
| 6.75 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 6.88 | 7.12 | … | … | … | 0.22 | 0.22 | 0.19 | 0.16 | … | … |
| … | … | … | … | 8.00 | … | … | … | … | … | … | 0.16 |
| … | … | … | 7.62 | … | … | … | … | … | … | 0.12 | … |
| 7.62 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 8.25 | 8.50 | … | … | … | 0.22 | 0.22 | 0.19 | 0.16 | … | … |

117

ASME  B16.5-2020

###### Table 5C Dimensions of Ring Joint Facings (All Pressure Rating Classes)  (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12

Nominal Pipe Size

Groove Dimensions

Pitch

Radius at

| Class 150 | Class 300 | Class 400 (2) | Class 600 | Class 900 (3) | Class 1500 | Class 2500 | Groove Number | Diameter, P | Depth, E (1) | Width, F | Bottom, R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | 5 | 42 | 7.500 | 0.500 | 0.781 | 0.06 |
| 6 | … | … | … | … | … | … | 43 | 7.625 | 0.250 | 0.344 | 0.03 |
| … | … | … | … | … | 5 | … | 44 | 7.625 | 0.312 | 0.469 | 0.03 |
| … | 6 | 6 | 6 | 6 | … | … | 45 | 8.312 | 0.312 | 0.469 | 0.03 |
| … | … | … | … | … | 6 | … | 46 | 8.312 | 0.375 | 0.531 | 0.06 |
| … | … | … | … | … | … | 6 | 47 | 9.000 | 0.500 | 0.781 | 0.06 |
| 8 | … | … | … | … | … | … | 48 | 9.750 | 0.250 | 0.344 | 0.03 |
| … | 8 | 8 | 8 | 8 | … | … | 49 | 10.625 | 0.312 | 0.469 | 0.03 |
| … | … | … | … | … | 8 | … | 50 | 10.625 | 0.438 | 0.656 | 0.06 |
| … | … | … | … | … | … | 8 | 51 | 11.000 | 0.562 | 0.906 | 0.06 |
| 10 | … | … | … | … | … | … | 52 | 12.000 | 0.250 | 0.344 | 0.03 |
| … | 10 | 10 | 10 | 10 | … | … | 53 | 12.750 | 0.312 | 0.469 | 0.03 |
| … | … | … | … | … | 10 | … | 54 | 12.750 | 0.438 | 0.656 | 0.06 |
| … | … | … | … | … | … | 10 | 55 | 13.500 | 0.688 | 1.188 | 0.09 |
| 12 | … | … | … | … | … | … | 56 | 15.000 | 0.250 | 0.344 | 0.03 |
| … | 12 | 12 | 12 | 12 | … | … | 57 | 15.000 | 0.312 | 0.469 | 0.03 |
| … | … | … | … | … | 12 | … | 58 | 15.000 | 0.562 | 0.906 | 0.06 |
| 14 | … | … | … | … | … | … | 59 | 15.625 | 0.250 | 0.344 | 0.03 |
| … | … | … | … | … | … | 12 | 60 | 16.000 | 0.688 | 1.312 | 0.09 |
| … | 14 | 14 | 14 | … | … | … | 61 | 16.500 | 0.312 | 0.469 | 0.03 |
| … | … | … | … | 14 | … | … | 62 | 16.500 | 0.438 | 0.656 | 0.06 |
| … | … | … | … | … | 14 | … | 63 | 16.500 | 0.625 | 1.062 | 0.09 |
| 16 | … | … | … | … | … | … | 64 | 17.875 | 0.250 | 0.344 | 0.03 |
| … | 16 | 16 | 16 | … | … | … | 65 | 18.500 | 0.312 | 0.469 | 0.03 |
| … | … | … | … | 16 | … | … | 66 | 18.500 | 0.438 | 0.656 | 0.06 |
| … | … | … | … | … | 16 | … | 67 | 18.500 | 0.688 | 1.188 | 0.09 |
| 18 | … | … | … | … | … | … | 68 | 20.375 | 0.250 | 0.344 | 0.03 |
| … | 18 | 18 | 18 | … | … | … | 69 | 21.000 | 0.312 | 0.469 | 0.03 |
| … | … | … | … | 18 | … | … | 70 | 21.000 | 0.500 | 0.781 | 0.06 |
| … | … | … | … | … | 18 | … | 71 | 21.000 | 0.688 | 1.188 | 0.09 |
| 20 | … | … | … | … | … | … | 72 | 22.000 | 0.250 | 0.344 | 0.03 |
| … | 20 | 20 | 20 | … | … | … | 73 | 23.000 | 0.375 | 0.531 | 0.06 |

118

ASME  B16.5-2020

###### Table 5C Dimensions of Ring Joint Facings (All Pressure Rating Classes)  (Cont’d)

| Diameter of Raised Portion, K |  |  |  |  | Approximate Distance Between Flanges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class |
|  | 300 |  |  |  |  |  |  |  |  |  |  |
|  | 400 |  |  |  |  |  |  |  |  |  |  |
| 150 | 600 | 900 | 1500 | 2500 | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |

13	14	15	16	17	18	19	20	21	22	23	24

| … 8.62 … … | … … … 9.50 | … … … 9.50 | … … 9.00 … | 9.50 … … … | … 0.16 … … | … … … 0.22 | … … … 0.22 | … … … 0.19 | … … … 0.16 | … … 0.12 … | 0.16 … … … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | 9.75 | … | … | … | … | … | … | 0.12 | … |
| … | … | … | … | 11.00 | … | … | … | … | … | … | 0.16 |
| 10.75 | … | … | … | … | 0.16 | … | … | … | … | … |  |
| … | 11.88 | 12.12 | … | … | … | 0.22 | 0.22 | 0.19 | 0.16 | … | … |
| … | … | … | 12.50 | … | … | … | … | … | … | 0.16 | … |
| … | … | … | … | 13.38 | … | … | … | … | … | … | 0.19 |
| 13.00 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 14.00 | 14.25 | … | … | … | 0.22 | 0.22 | 0.19 | 0.16 | … | … |
| … | … | … | 14.62 | … | … | … | … | … | … | 0.16 | … |
| … | … | … | … | 16.75 | … | … | … | … | … | … | 0.25 |
| 16.00 | … | … | … | … | 0.16 | … | … | … | … | … | … |
| … | 16.25 | 16.50 | … | … | … | 0.22 | 0.22 | 0.19 | 0.16 | … | … |
| … | … | … | 17.25 | … | … | … | … | … | … | 0.19 | … |
| 16.75 | … | … | … | … | 0.12 | … | … | … | … | … | … |
| … | … | … | … | 19.50 | … | … | … | … | … | … | 0.31 |
| … | 18.00 | … | … | … | … | 0.22 | 0.22 | 0.19 | … | … | … |
| … | … | 18.38 | … | … | … | … | … | … | 0.16 | … | … |
| … | … | … | 19.25 | … | … | … | … | … | … | 0.22 | … |
| 19.00 | … | … | … | … | 0.12 | … | … | … | … | … | … |
| … | 20.00 | … | … | … | … | 0.22 | 0.22 | 0.19 | … | … | … |
| … | … | 20.62 | … | … | … | … | … | … | 0.16 | … | … |
| … | … | … | 21.50 | … | … | … | … | … | … | 0.31 | … |
| 21.50 | … | … | … | … | 0.12 | … | … | … | … | … | … |
| … | 22.62 | … | … | … | … | 0.22 | 0.22 | 0.19 | … | … | … |
| … | … | 23.38 | … | … | … | … | … | … | 0.19 | … | … |
| … | … | … | 24.12 | … | … | … | … | … | … | 0.31 | … |
| 23.50 | … | … | … | … | 0.12 | … | … | … | … | … | … |
| … | 25.00 | … | … | … | … | 0.22 | 0.22 | 0.19 | … | … | … |

119

ASME  B16.5-2020

###### Table 5C Dimensions of Ring Joint Facings (All Pressure Rating Classes)  (Cont’d)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 11 |  | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Nominal Pipe Size |  |  |  |  |  |  | Groove Dimensions |  |  |
| Class |  | Class |  | Class | Class | Class | Groove | Pitch Diameter, | Depth, Width, |  | Radius at Bottom, |
| 150 | Class 300 | 400 (2) | Class 600 | 900 (3) | 1500 | 2500 | Number | P | E (1) F |  | R |
| … | … | … | … | 20 | … | … | 74 | 23.000 | 0.500 | 0.781 | 0.06 |
| … | … | … | … | … | 20 | … | 75 | 23.000 | 0.688 | 1.312 | 0.09 |
| 22 | … | … | … | … | … | … | 80 | 24.250 | 0.250 | 0.344 | 0.03 |
| … | 22 | 22 | 22 | … | … | … | 81 | 25.000 | 0.438 | 0.594 | 0.06 |
| 24 | … | … | … | … | … | … | 76 | 26.500 | 0.250 | 0.344 | 0.03 |
| … | 24 | 24 | 24 | … | … | … | 77 | 27.250 | 0.438 | 0.656 | 0.06 |
| … | … | … | … | 24 | … | … | 78 | 27.250 | 0.625 | 1.062 | 0.09 |
| … | … | … | … | … | … | … | … | … | … | … | … |
| … | … | … | … | … | 24 | … | 79 | 27.250 | 0.812 | 1.438 | 0.09 |

120

ASME  B16.5-2020

###### Table 5C Dimensions of Ring Joint Facings (All Pressure Rating Classes)  (Cont’d)

| Diameter of Raised Portion, K |  |  |  |  | Approximate Distance Between Flanges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class | Class |
|  | 300 |  |  |  |  |  |  |  |  |  |  |
|  | 400 |  |  |  |  |  |  |  |  |  |  |
| 150 | 600 | 900 | 1500 | 2500 | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |

13	14	15	16	17	18	19	20	21	22	23	24

| … … | … … | 25.50 … | … 26.50 | … … | … … | … … | … … | … … | 0.19 … | … 0.38 | … … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.50 | … | … | … | … | 0.12 | … | … | … | … | … | … |
| … | 27.00 | … | … | … | … | 0.25 | 0.25 | 0.22 | … | … | … |
| … | … | … | … | … | … | … | … | … | … | … | … |
| 28.00 | … | … | … | … | 0.12 | … | … | … | … | … | … |
| … | 29.50 | … | … | … | … | 0.25 | 0.25 | 0.22 | … | … | … |
| … | … | 30.38 | … | … | … | … | … | … | 0.22 | … | … |
| … | … | … | 31.25 | … | … | … | … | … | … | 0.44 | … |

121

ASME  B16.5-2020

###### Table 5C Dimensions of Ring Joint Facings (All Pressure Rating Classes) (Cont’d)

GENERAL NOTES:

- Dimensions are in inches.

- For facing requirements for flanges and flanged fittings, see para. 6.4.1 and Figure 6.

- For facing requirements for lapped joints, see para. 6.4.3 and Figure 6.

- See para 4.2.7 for marking requirements.

NOTES:

  - The height of the raised portion is equal to the depth of groove dimension E, but is not subjected to the tolerances for E. Former full-face contour may be used.

  - Use Class 600 in sizes NPS 1∕2 to NPS 31∕2 for Class 400.

  - Use Class 1500 in sizes NPS 1∕2 to NPS 21∕2 for Class 900.

  - For ring joints with lapped flanges in Classes 300 and 600, ring and groove number R30 are used instead of R31.

TOLERANCES:

E     =  (depth) +0.016, −0.000 in.

F    = (width) ±0.008 in.

P    = (pitch diameter) ±0.005 in.

R    = (radius at bottom)

= R ≤ 0.06 +0.03, −0.00 in.

= R > 0.06 ± 0.03 in.

23 deg   = (angle) ± 1∕2 deg

122

ASME  B16.5-2020

###### Table 6 Reducing Threaded and Slip-On Pipe Flanges for Classes 150 Through 2500 Pipe Flanges	ð20Þ

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| Nominal | Smallest Size of Reducing Outlet Requiring Hub | Nominal | Smallest Size of Reducing Outlet Requiring Hub | Nominal | Smallest Size of Reducing Outlet Requiring Hub |
| Pipe Size(4) | Flanges (1) | Pipe Size (4) | Flanges (1) | Pipe Size (4) | Flanges (1) |
| NPS | NPS | NPS | NPS | NPS | NPS |
| 1 | 1∕2 | 31∕2 | 11∕2 | 12 | 31∕2 |
| 11∕4 | 1∕2 | 4 | 11∕2 | 14 | 31∕2 |
| 11∕2 | 1∕2 | 5 | 11∕2 | 16 | 4 |
| 2 | 1 | 6 | 21∕2 | 18 | 4 |
| 21∕2 | 11∕4 | 8 | 3 | 20 | 4 |
| 3 | 11∕4 | 10 | 31∕2 | 24 | 4 |

GENERAL NOTE: Dimensions are in millimeters.

NOTES:

- The hub dimensions shall be at least as large as those of the standard flanges of the size to which the reduction is being machined, except flanges reducing to a size smaller than those of Columns 2, 4, and 6 may be made from blind flanges (see Example 2).

- Class 150 flanges do not have a counterbore. Class 300 and higher pressure flanges will have depth of counterbore Q of 6.4 mm for NPS 2 and

smaller tapping and 9.7 mm for NPS 21∕2 and larger. The diameter Q of counterbore is the same as that given in the tables of threaded flanges for the corresponding tapping.

- Minimum length of effective threads shall be at least equal dimension T of the corresponding pressure class threaded flange as shown in tables but does not necessarily extend for the face of the flange. For thread of threaded flanges, see para. 6.9.

- For method of designating reducing threaded and reducing slip-on flanges, see para. 3.3 and Examples below.

EXAMPLES:

- The size designation is NPS 6 × 21∕2  — Class 300 reducing threaded flange. This flange has the following dimensions: NPS 21∕2   =  taper pipe thread tapping (ASME B1.20.1)

318 mm = diameter of regular NPS 6 Class 300 threaded flange

35.1 mm = thickness of regular NPS 6 Class 300 threaded flange

178 mm  =  diameter of hub for regular NPS 5 Class 300 threaded flange. Hub diameter may be one size small to reduce machining. In this example, a hub diameter of NPS 21∕2  would be the smallest acceptable.

15.7 mm = height of hub for regular NPS 5 Class 300 threaded flange

- The size designation is NPS 6 × 2 — Class 300 reducing threaded flange. Use regular NPS 6 Class 300 blind flange tapped with NPS 2 taper pipe thread (ASME B1.20.1).

123

ASME  B16.5-2020

ð20Þ

###### Table 6C Reducing Threaded and Slip-On Pipe Flanges for Classes 150 Through 2500 Pipe Flanges

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
|  | Smallest Size of Reducing |  | Smallest Size of Reducing |  | Smallest Size of Reducing |
| Nominal | Outlet Requiring Hub Flanges | Nominal | Outlet Requiring Hub Flanges | Nominal | Outlet Requiring Hub |
| Pipe Size (4) | (1) | Pipe Size (4) | (1) | Pipe Size (4) | Flanges (1) |
| 1 | 1∕2 | 31∕2 | 11∕2 | 12 | 31∕2 |
| 11∕4 | 1∕2 | 4 | 11∕2 | 14 | 31∕2 |
| 11∕2 | 1∕2 | 5 | 11∕2 | 16 | 4 |
| 2 | 1 | 6 | 21∕2 | 18 | 4 |
| 21∕2 | 11∕4 | 8 | 3 | 20 | 4 |
| 3 | 11∕4 | 10 | 31∕2 | 24 | 4 |

GENERAL NOTE: Dimensions are in inches.

NOTES:

  - The hub dimensions shall be at least as large as those of the standard flanges of the size to which the reduction is being made, except flanges reducing to a size smaller than those of columns 2, 4, and 6 may be made from blind flanges (see Example 2).

  - Class 150 flanges do not have a counterbore. Class 300 and higher pressure flanges will have a depth of counterbore of 0.25 in. for NPS 2 and smaller tapping and 0.38 in. for NPS 21∕2 and larger. The diameter Q of counterbore is the same as that given in the tables of the threaded flanges for the corresponding tapping.

  - The minimum length of effective threads shall be at least equal to dimension T of the corresponding pressure class threaded flange as shown in the tables but does not necessarily extend for the face of the flange. For thread of threaded flanges, see para. 6.9.

  - For the method of designating reducing threaded and reducing slip-on flanges, see para. 3.3 and the example below.

EXAMPLES:

- The size designation is NPS 6 × 21∕2  — Class 300 reducing threaded flange. This flange has the following dimensions: NPS 21∕2   =  taper pipe thread tapping (ASME B1.20.1)

12.50 in. = diameter of regular NPS 6 Class 300 threaded flange

1.38 in. = thickness of regular NPS 6 Class 300 threaded flange

7.00 in.  =  diameter of hub for regular NPS 5 Class 300 threaded flange. Hub diameter may be one size smaller to reduce machining. In this example, a hub diameter of NPS 21∕2  would be the smallest acceptable.

0.63 in. = height of hub for regular NPS 5 Class 300 threaded flange

Other dimensions the same as for regular NPS 6 Class 300 threaded flange, Table 12C

- The size designation is NPS 6 × 2 — Class 300 reducing threaded flange. Use regular NPS 6 Class 300 blind flange tapped with NPS 2 taper pipe thread (ASME B1.20.1).

124

ASME  B16.5-2020

###### Table 7 Templates for Drilling Class 150 Pipe Flanges and Flanged Fittings	ð20Þ

| Nominal Pipe Size | Outside Diameter of Flange, O | Drilling (2), (3) |  |  |  | Length of Bolts, L (4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Stud Bolts (1) |  | Machine Bolts |
|  |  | Diameter of Bolt Circle, W | Diameter of Bolt Holes, in. | Number of Bolts | Diameter of Bolts, in. | 1.5-mm Raised Face | Ring Joint | 1.5-mm Raised Face |

1	2	3	4	5	6	7	8	9

| 1∕2 89 60.5 5∕8 4 1∕2 3∕4 99 69.8 5∕8 4 1∕2 1 108 79.2 5∕8 4 1∕2 11∕4 117 88.9 5∕8 4 1∕2 11∕2 127 98.6 5∕8 4 1∕2 2 152 120.6 3∕4 4 5∕8 21∕2 178 139.7 3∕4 4 5∕8 3 190 152.4 3∕4 4 5∕8 31∕2 216 177.8 3∕4 8 5∕8 4 229 190.5 3∕4 8 5∕8 5 254 215.9 7∕8 8 3∕4 6 279 241.3 7∕8 8 3∕4 8 343 298.4 7∕8 8 3∕4 7∕8 7∕8 |  |  |  |  |  | 55 | … | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 65 | … | 50 |
|  |  |  |  |  |  | 65 | 75 | 55 |
|  |  |  |  |  |  | 70 | 85 | 55 |
|  |  |  |  |  |  | 70 | 85 | 65 |
|  |  |  |  |  |  | 85 | 95 | 70 |
|  |  |  |  |  |  | 90 | 100 | 75 |
|  |  |  |  |  |  | 90 | 100 | 75 |
|  |  |  |  |  |  | 90 | 100 | 75 |
|  |  |  |  |  |  | 90 | 100 | 75 |
|  |  |  |  |  |  | 95 | 110 | 85 |
|  |  |  |  |  |  | 100 | 115 | 85 |
|  |  |  |  |  |  | 110 | 120 | 90 |
|  |  |  |  |  |  | 115 | 125 | 100 |
|  |  |  |  |  |  | 120 | 135 | 100 |
| 14 | 533 | 476.2 | 11∕8 | 12 | 1 | 135 | 145 | 115 |
| 16 | 597 | 539.8 | 11∕8 | 16 | 1 | 135 | 145 | 115 |
| 18 | 635 | 577.8 | 11∕4 | 16 | 11∕8 | 145 | 160 | 125 |
| 20 | 693 | 635.0 | 11∕4 | 20 | 11∕8 | 160 | 170 | 140 |
| 22 | 749 | 692.2 | 13∕8 | 20 | 11∕4 | 170 | 185 | 150 |
| 24 | 813 | 749.3 | 13∕8 | 20 | 11∕4 | 170 | 185 | 150 |

| 10 | 406 | 362.0 | 1 | 12 |
| --- | --- | --- | --- | --- |
| 12 | 483 | 431.8 | 1 | 12 |

GENERAL NOTES:

- Dimensions are in millimeters, except for diameters of bolts and bolt holes, which are in inches.

- For other dimensions, see Tables 8 and 9.

NOTES:

  - The length of the stud bolt does not include the height of the points (see para. 6.10.2).

  - For flange bolt holes, see para. 6.5.

  - For spot facing, see para. 6.6.

  - Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

125

ASME  B16.5-2020

###### Table 7C Templates for Drilling Class 150 Pipe Flanges and Flanged Fittings

1	2	3	4	5	6	7	8	9

| Nominal Outside Pipe Diameter Size, of Flange, NPS O | Drilling (2), (3) | Length of Bolts, L (4) |  |
| --- | --- | --- | --- |
|  | Diameter of Bolt Circle, Diameter of Number Diameter W Bolt Holes of Bolts of Bolts | Stud Bolts (1) | Machine Bolts |
|  |  | Raised Face Ring 0.06 in. Joint | Raised Face 0.06 in. |

| 2.25 | … | 2.00 |
| --- | --- | --- |
| 2.50 | … | 2.00 |
| 2.50 | 3.00 | 2.25 |
| 2.75 | 3.25 | 2.25 |
| 2.75 | 3.25 | 2.50 |
| 3.25 | 3.75 | 2.75 |
| 3.50 | 4.00 | 3.00 |
| 3.50 | 4.00 | 3.00 |
| 3.50 | 4.00 | 3.00 |
| 3.50 | 4.00 | 3.00 |
| 3.75 | 4.25 | 3.25 |
| 4.00 | 4.50 | 3.25 |
| 4.25 | 4.75 | 3.50 |
| 4.50 | 5.00 | 4.00 |
| 4.75 | 5.25 | 4.00 |

| 1∕2 3∕4 | 3.50 | 2.38 |
| --- | --- | --- |
|  | 3.88 | 2.75 |
| 1 | 4.25 | 3.12 |
| 11∕4 | 4.62 | 3.50 |
| 11∕2 | 5.00 | 3.88 |
| 2 | 6.00 | 4.75 |
| 21∕2 | 7.00 | 5.50 |
| 3 | 7.50 | 6.00 |
| 31∕2 | 8.50 | 7.00 |
| 4 | 9.00 | 7.50 |

5∕8	4	1∕2

5∕8	4	1∕2

5∕8	4	1∕2

5∕8	4	1∕2

5∕8	4	1∕2

3∕4	4	5∕8

3∕4	4	5∕8

3∕4	4	5∕8

3∕4	8	5∕8

3∕4	8	5∕8

| 5 | 10.00 | 8.50 |
| --- | --- | --- |
| 6 | 11.00 | 9.50 |
| 8 | 13.50 | 11.75 |

7∕8	8	3∕4

7∕8	8	3∕4

| 10 | 16.00 | 14.25 | 1 | 12 |
| --- | --- | --- | --- | --- |
| 12 | 19.00 | 17.00 | 1 | 12 |

7∕8	8	3∕4

7∕8

7∕8

| 14 | 21.00 | 18.75 | 11∕8 | 12 | 1 | 5.25 | 5.75 | 4.50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 23.50 | 21.25 | 11∕8 | 16 | 1 | 5.25 | 5.75 | 4.50 |
| 18 | 25.00 | 22.75 | 11∕4 | 16 | 11∕8 | 5.75 | 6.25 | 5.00 |
| 20 | 27.50 | 25.00 | 11∕4 | 20 | 11∕8 | 6.25 | 6.75 | 5.50 |
| 22 | 29.50 | 27.25 | 13∕8 | 20 | 11∕4 | 6.75 | 7.25 | 6.00 |
| 24 | 32.00 | 29.50 | 13∕8 | 20 | 11∕4 | 6.75 | 7.25 | 6.00 |

GENERAL NOTES:

    - Dimensions are in inches.

    - For other dimensions, see Tables 8C and 9C.

NOTES:

      - The length of the stud bolt does not include the height of the points (see para. 6.10.2).

      - For flange bolt holes, see para. 6.5.

      - For spot facing, see para. 6.6.

      - Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

126

127

###### Table 8 Dimensions of Class 150 Flanges	ð20Þ

ASME  B16.5-2020

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

| Length Through Hub |  |  |  |  |  |  |  |  |  |  | Bore |  | Corner |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Hub |  |  |  |  |  |  |  | Bore |  |
|  |  |  |  |  | Diameter |  |  |  |  |  |  |  | Radius |  |
|  |  | Minimum |  |  | Beginning |  |  |  |  |  |  |  | of |  |
|  | Outside | Thickness |  |  | of | Threaded/ |  |  | Minimum | Minimum |  | Welding | Lapped |  |
| Nominal | Diameter | of | Minimum |  | Chamfer | Slip-on/ |  |  | Thread | Slip-on/ |  | Neck/ | Flange |  |
| Pipe | of | Flange, | Thickness | Diameter | Welding | Socket |  | Welding | Length | Socket | Minimum | Socket | and | Depth of |
| Size, | Flange, | tf | Lap Joint, | of Hub, | Neck, | Welding, | Lapped, | Neck, | Threaded, | Welding, | Lapped, | Welding, B | Pipe, | Socket, |
| NPS | O | (2)–(4) | tf | X | Ah (5) | Y | Y | Y | T (6) | B | B | (7) | r | D |
| 1∕2 | 89 | 9.7 | 11.2 | 30 | 21.3 | 14 | 16 | 46 | 16 | 22.4 | 22.9 | 15.7 | 3 | 10 |
| 3∕4 | 99 | 11.2 | 12.7 | 38 | 26.7 | 14 | 16 | 51 | 16 | 27.7 | 28.2 | 20.8 | 3 | 11 |
| 1 | 108 | 12.7 | 14.2 | 49 | 33.5 | 16 | 18 | 54 | 18 | 34.5 | 35.1 | 26.7 | 3 | 13 |
| 11∕4 | 117 | 14.2 | 15.7 | 59 | 42.2 | 19 | 21 | 56 | 21 | 43.2 | 43.7 | 35.1 | 5 | 14 |

###### Table 8 Dimensions of Class 150 Flanges (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

Minimum

Hub Diameter Beginning

Length Through Hub

Bore	Corner

| Nominal Pipe Size, | Outside Diameter of Flange, | Thickness of Flange, tf | Minimum Thickness Lap Joint, | Diameter of Hub, | of Threaded/ Minimum Minimum Welding Lapped Chamfer Slip-on/ Thread Slip-on/ Neck/ Flange Welding Socket Welding Length Socket Minimum Socket and Depth o Neck, Welding, Lapped, Neck, Threaded, Welding, Lapped, Welding, B Pipe, Socket |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NPS | O | (2)–(4) | tf | X | Ah (5) Y Y Y T (6) B B (7) r D |  |  |  |  |  |  |  |  |  |
| 11∕2 | 127 | 15.9 | 17.5 | 65 | 48.3 | 21 | 22 | 60 | 22 | 49.5 | 50.0 | 40.9 | 6 | 16 |
| 2 | 152 | 17.5 | 19.0 | 78 | 60.5 | 24 | 25 | 62 | 25 | 61.9 | 62.5 | 52.6 | 8 | 18 |
| 21∕2 | 178 | 20.6 | 22.4 | 90 | 73.2 | 27 | 28 | 68 | 28 | 74.7 | 75.4 | 62.7 | 8 | 19 |
| 3 | 190 | 22.4 | 23.9 | 108 | 88.9 | 28 | 30 | 68 | 30 | 90.7 | 91.4 | 78.0 | 10 | 21 |
| 31∕2 | 216 | 22.4 | 23.9 | 122 | 101.6 | 30 | 32 | 70 | 32 | 103.4 | 104.1 | 90.2 | 10 | … |
| 4 | 229 | 22.4 | 23.9 | 135 | 114.3 | 32 | 33 | 75 | 33 | 116.1 | 116.8 | 102.4 | 11 | … |
| 5 | 254 | 22.4 | 23.9 | 164 | 141.2 | 35 | 37 | 87 | 37 | 143.8 | 144.5 | 128.3 | 11 | … |
| 6 | 279 | 23.9 | 25.4 | 192 | 168.4 | 38 | 40 | 87 | 40 | 170.7 | 171.4 | 154.2 | 13 | … |
| 8 | 343 | 26.9 | 28.4 | 246 | 219.2 | 43 | 44 | 100 | 44 | 221.5 | 222.2 | 202.7 | 13 | … |
| 10 | 406 | 28.4 | 30.2 | 305 | 273.0 | 48 | 49 | 100 | 49 | 276.4 | 277.4 | 254.5 | 13 | … |
| 12 | 483 | 30.2 | 31.8 | 365 | 323.8 | 54 | 56 | 113 | 56 | 327.2 | 328.2 | 304.8 | 13 | … |
| 14 | 553 | 33.3 | 35.1 | 400 | 355.6 | 56 | 79 | 125 | 57 | 359.2 | 360.2 | (8) | 13 | … |
| 16 | 597 | 35.1 | 36.6 | 457 | 406.4 | 62 | 87 | 125 | 64 | 410.5 | 411.2 | (8) | 13 | … |
| 18 | 635 | 38.1 | 39.6 | 505 | 457.2 | 67 | 97 | 138 | 68 | 461.8 | 462.3 | (8) | 13 | … |
| 20 | 698 | 41.1 | 42.9 | 559 | 508.0 | 71 | 103 | 143 | 73 | 513.1 | 514.4 | (8) | 13 | … |
| 22 | 749 | 44.4 | 46.0 | 610 | 558.8 | 78 | 108 | 148 | … | 564.4 | 565.2 | (8) | 13 | … |
| 24 | 813 | 46.0 | 47.8 | 663 | 609.6 | 81 | 111 | 151 | 83 | 616.0 | 616.0 | (8) | 13 | … |

Bore Radius of

ASME  B16.5-2020

128

f

,

GENERAL NOTES:

        - Dimensions are in millimeters.

        - For tolerance, see section 7.

        - For facings, see para. 6.4.

        - For flange bolt holes, see para. 6.5 and Table 7.

        - For spot facing, see para. 6.6.

        - For reducing threaded and slip-on flanges, see Table 6.

        - Blind flanges may be made with or without hubs at the manufacturer’s option.

        - For reducing welding neck flanges, see para. 6.8.

NOTES:

          - This dimension is for large end of hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges. This dimension is defined as the diameter at the intersection between the hub taper and back face of the flange.

ASME  B16.5-2020

129

###### Table 8 Dimensions of Class 150 Flanges (Cont’d)

NOTES: (Cont’d)

          - The minimum thickness of these loose flanges, in sizes NPS 31∕2 and smaller, is slightly greater than the thickness of flanges on fittings, Table 9, which are reinforced by being cast integral with the body of the fitting.

          - These flanges may be supplied with a flat face. The flat face may be either the full tf dimension of thickness plus 2 mm or the tf dimension thickness without the raised face height. See para 6.3.2

for additional restrictions.

          - The flange dimensions illustrated are for regularly furnished 1.5-mm raised face (except lapped); for requirements of other facings, see Figure 6.

          - For welding end bevel (see para. 6.7).

          - For thread of threaded flanges, see para. 6.9.

          - Dimensions in Column 13 correspond to the inside diameters of pipe as given in ASME B36.10M for standard wall pipe. Thickness of standard wall is the same as Schedule 40 in sizes NPS 10 and smaller. Tolerances in para. 7.5.2 apply. These bore sizes are furnished unless otherwise specified by the purchaser.

          - To be specified by the purchaser.

ð20Þ

###### Table 8C Dimensions of Class 150 Flanges

ASME  B16.5-2020

130

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

| Nominal | Outside Diameter of Flange, | Minimum Thickness of Flange, tf | Minimum Thickness | Diameter of Hub, | Diameter Beginning of Chamfer Welding Neck, | Length Through Threaded Slip-On Socket Welding, Lapped, |  | Hub Welding Neck, | Minimum Thread Length Threaded, | Minimum Slip-On Socket Welding, | Bore Minimum Lapped, | Welding Neck/ Socket Welding, | Corner Bore Radius of Lapped Flange and Pipe, | Depth of Socket, |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pipe Size | O | (2)–(4) | Lap Joint | X | Ah (5) | Y | Y | Y | T (6) | B | B | B (7) | r | D |
| 1∕2 | 3.50 | 0.38 | 0.44 | 1.19 | 0.84 | 0.56 | 0.62 | 1.81 | 0.62 | 0.88 | 0.90 | 0.62 | 0.12 | 0.38 |
| 3∕4 | 3.88 | 0.44 | 0.50 | 1.50 | 1.05 | 0.56 | 0.62 | 2.00 | 0.62 | 1.09 | 1.11 | 0.82 | 0.12 | 0.44 |
| 1 | 4.25 | 0.50 | 0.56 | 1.94 | 1.32 | 0.62 | 0.69 | 2.12 | 0.69 | 1.36 | 1.38 | 1.05 | 0.12 | 0.50 |
| 11∕4 | 4.62 | 0.56 | 0.62 | 2.31 | 1.66 | 0.75 | 0.81 | 2.19 | 0.81 | 1.70 | 1.72 | 1.38 | 0.19 | 0.56 |
| 11∕2 | 5.00 | 0.62 | 0.69 | 2.56 | 1.90 | 0.81 | 0.88 | 2.38 | 0.88 | 1.95 | 1.97 | 1.61 | 0.25 | 0.62 |
| 2 | 6.00 | 0.69 | 0.75 | 3.06 | 2.38 | 0.94 | 1.00 | 2.44 | 1.00 | 2.44 | 2.46 | 2.07 | 0.31 | 0.69 |

ASME  B16.5-2020

131

###### Table 8C Dimensions of Class 150 Flanges (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

| Nominal | Outside Diameter of Flange, | Minimum Thickness of Flange, tf | Minimum Thickness | Diameter of Hub, | Diameter Beginning of Chamfer Welding Neck, | Length Through Threaded Slip-On Socket Welding, Lapped, |  | Hub Welding Neck, | Minimum Thread Length Threaded, | Minimum Slip-On Socket Welding, | Bore Minimum Lapped, | Welding Neck/ Socket Welding, | Corner Bore Radius of Lapped Flange and Pipe, | Depth of Socket, |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pipe Size | O | (2)–(4) | Lap Joint | X | Ah (5) | Y | Y | Y | T (6) | B | B | B (7) | r | D |
| 21∕2 | 7.00 | 0.81 | 0.88 | 3.56 | 2.88 | 1.06 | 1.12 | 2.69 | 1.12 | 2.94 | 2.97 | 2.47 | 0.31 | 0.75 |
| 3 | 7.50 | 0.88 | 0.94 | 4.25 | 3.50 | 1.12 | 1.19 | 2.69 | 1.19 | 3.57 | 3.60 | 3.07 | 0.38 | 0.81 |
| 31∕2 | 8.50 | 0.88 | 0.94 | 4.81 | 4.00 | 1.19 | 1.25 | 2.75 | 1.25 | 4.07 | 4.10 | 3.55 | 0.38 | … |
| 4 | 9.00 | 0.88 | 0.94 | 5.31 | 4.50 | 1.25 | 1.31 | 2.94 | 1.31 | 4.57 | 4.60 | 4.03 | 0.44 | … |
| 5 | 10.00 | 0.88 | 0.94 | 6.44 | 5.56 | 1.38 | 1.44 | 3.44 | 1.44 | 5.66 | 5.69 | 5.05 | 0.44 | … |
| 6 | 11.00 | 0.94 | 1.00 | 7.56 | 6.63 | 1.50 | 1.56 | 3.44 | 1.56 | 6.72 | 6.75 | 6.07 | 0.50 | … |
| 8 | 13.50 | 1.06 | 1.12 | 9.69 | 8.63 | 1.69 | 1.75 | 3.94 | 1.75 | 8.72 | 8.75 | 7.98 | 0.50 | … |
| 10 | 16.00 | 1.12 | 1.19 | 12.00 | 10.75 | 1.88 | 1.94 | 3.94 | 1.94 | 10.88 | 10.92 | 10.02 | 0.50 | … |
| 12 | 19.00 | 1.19 | 1.25 | 14.38 | 12.75 | 2.12 | 2.19 | 4.44 | 2.19 | 12.88 | 12.92 | 12.00 | 0.50 | … |
| 14 | 21.00 | 1.31 | 1.38 | 15.75 | 14.00 | 2.19 | 3.12 | 4.94 | 2.25 | 14.14 | 14.18 | (8) | 0.50 | … |
| 16 | 23.50 | 1.38 | 1.44 | 18.00 | 16.00 | 2.44 | 3.44 | 4.94 | 2.50 | 16.16 | 16.19 | (8) | 0.50 | … |
| 18 | 25.00 | 1.50 | 1.56 | 19.88 | 18.00 | 2.62 | 3.81 | 5.44 | 2.69 | 18.18 | 18.20 | (8) | 0.50 | … |
| 20 | 27.50 | 1.62 | 1.69 | 22.00 | 20.00 | 2.81 | 4.06 | 5.62 | 2.88 | 20.20 | 20.25 | (8) | 0.50 | … |
| 22 | 29.50 | 1.75 | 1.81 | 24.00 | 22.00 | 3.06 | 4.25 | 5.82 | … | 22.22 | 22.25 | (8) | 0.50 | … |
| 24 | 32.00 | 1.81 | 1.88 | 26.12 | 24.00 | 3.19 | 4.38 | 5.94 | 3.25 | 24.25 | 24.25 | (8) | 0.50 | … |

GENERAL NOTES:

            - Dimensions are in inches.

            - For tolerances, see section 7.

            - For facings, see para. 6.4.

            - For flange bolt holes, see para. 6.5 and Table 7C.

            - For spot facing, see para. 6.6.

            - For reducing threaded and slip-on flanges, see Table 6C.

            - Blind flanges may be made with or without hubs at the manufacturer’s option.

            - For reducing welding neck flanges, see para. 6.8.

NOTES:

              - This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges.

              - The minimum thickness of these loose flanges, in sizes NPS 31∕2 and smaller, is slightly greater than the thickness of flanges on fittings, Table 9C, which are reinforced by being cast integral with the body of the fitting.

              - When these flanges are required with flat face, the flat face may be either the full tf  dimension thickness plus 0.06 in. or the tf dimension thickness without the raised face height. See para. 6.3.2

for additional restrictions.

ASME  B16.5-2020

132

###### Table 8C Dimensions of Class 150 Flanges (Cont’d)

NOTES: (Cont’d)

              - The flange dimensions illustrated are for regularly furnished 0.06-in. raised face (except lapped); for requirements of other facings, see Figure 6.

              - For welding end bevel, see para. 6.7.

              - For thread of threaded flanges, see para. 6.9.

              - Dimensions in column 13 correspond to the inside diameters of pipe as given in ASME B36.10M for standard wall pipe. The thickness of standard wall is the same as Schedule 40 in sizes NPS 10 and smaller. Tolerances in para. 7.5.2 apply. These bore sizes are furnished unless otherwise specked by the purchaser.

              - To be specified by the purchaser.

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

133

ASME  B16.5-2020

ð20Þ

###### Table 9 Dimensions of Class 150 Flanged Fittings

1	2	3	4	5	6	7	8	9	10	11	12

| Minimum Outside Thickness Minimum Inside Nominal Diameter of Wall Diameter Pipe of Flange, Thickness of Size, Flange, tf of Fitting, Fitting, NPS O (1)–(3) tm d | 1.5-mm Raised Face (4) | Ring Joint (4) |
| --- | --- | --- |
|  | Center- to- Short Contact Center- Center- Contact Surface of to- Center- Long to- Surface- Raised Contact to- Center- Contact to- Face Surface of Contact to- Surface of Contact Elbow, Raised Surface of Contact Raised Surface Tee, Face Raised Surface of Face of Cross, Long Face Raised Lateral Raised and Radius 45-deg Face and Face True “Y,” vElbow, Elbow, Lateral, True “Y,” Reducer, AA BB CC EE FF GG (5) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (6) |

1∕2	89	7.9	2.8	13	…	…	…	…	…	…	…

3∕4	99	8.6	3.0	19	…	…	…	…	…	…	…

| 1 | 108 | 9.7 | 4.1 | 25 | 89 | 127 | 44 | 146 | 44 | 114 | 95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11∕4 | 117 | 11.2 | 4.8 | 32 | 95 | 140 | 51 | 159 | 44 | 114 | 102 |
| 11∕2 | 127 | 12.7 | 4.8 | 38 | 102 | 152 | 57 | 178 | 51 | 114 | 108 |
| 2 | 152 | 14.2 | 5.6 | 51 | 114 | 165 | 64 | 203 | 64 | 127 | 121 |
| 21∕2 | 178 | 15.7 | 5.6 | 64 | 127 | 178 | 76 | 241 | 64 | 140 | 133 |
| 3 | 190 | 17.5 | 5.6 | 76 | 140 | 197 | 76 | 254 | 76 | 152 | 146 |
| 31∕2 | 216 | 19.0 | 6.4 | 89 | 152 | 216 | 89 | 292 | 76 | 165 | 159 |
| 4 | 229 | 22.4 | 6.4 | 102 | 165 | 229 | 102 | 305 | 76 | 178 | 171 |
| 5 | 254 | 22.5 | 7.1 | 127 | 190 | 260 | 114 | 343 | 89 | 203 | 197 |
| 6 | 279 | 23.9 | 7.1 | 152 | 203 | 292 | 127 | 368 | 89 | 229 | 210 |
| 8 | 343 | 26.9 | 7.9 | 203 | 229 | 356 | 140 | 444 | 114 | 279 | 235 |
| 10 | 406 | 28.4 | 8.6 | 254 | 279 | 419 | 165 | 521 | 127 | 305 | 286 |
| 12 | 483 | 30.2 | 9.7 | 305 | 305 | 483 | 190 | 622 | 140 | 356 | 311 |
| 14 | 533 | 33.3 | 10.4 | 337 | 356 | 546 | 190 | 686 | 152 | 406 | 362 |
| 16 | 597 | 35.1 | 11.2 | 387 | 381 | 610 | 203 | 762 | 165 | 457 | 387 |

134

ASME  B16.5-2020

###### Table 9 Dimensions of Class 150 Flanged Fittings

13	14	15	16	17	18	19	20	21	22	23	1

| Ring Joint (4) |  |  |  |  | Diameter of Round Base or Center- Width of Thickness to-Base, Square of Thickness R Base, S Base, T of Ribs, U (7)–(9) (7) (7)–(10) (7) | Base Drilling (11) | Nominal Pipe Size, NPS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Center- to-End Long Radius Elbow, JJ (6) | Center- to-End 45-deg Elbow, KK (6) | Long Center- to-End Lateral, LL (6) | Short Center- to-End Lateral and True “Y,” MM (6) | End-to- End Reducer, NN (5), (6) |  | Bolt Circle or Diameter Bolt of Spacing, Drilled W Holes |  |

| … … 133 | … … 51 | … … 152 | … … 51 | … … … | … … … | … … … | … … … | … … … | … … … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 146 | 57 | 165 | 51 | … | … | … | … | … | … |
| 159 | 64 | 184 | 57 | … | … | … | … | … | … |
| 171 | 70 | 210 | 70 | … | 105 | 117 | 13 | 13 | 88.9 |
| 184 | 83 | 248 | 70 | … | 114 | 117 | 13 | 13 | 88.9 |
| 203 | 83 | 260 | 83 | … | 124 | 127 | 14 | 14 | 98.6 |
| 222 | 95 | 298 | 83 | … | 133 | 127 | 14 | 14 | 98.6 |
| 235 | 108 | 311 | 83 | … | 140 | 152 | 16 | 16 | 120.6 |
| 267 | 121 | 349 | 95 | … | 159 | 178 | 18 | 18 | 139.7 |
| 298 | 133 | 375 | 95 | … | 178 | 178 | 18 | 18 | 139.7 |
| 362 | 146 | 451 | 121 | … | 213 | 229 | 24 | 24 | 190.5 |
| 425 | 171 | 527 | 133 | … | 248 | 229 | 24 | 24 | 190.5 |
| 489 | 197 | 629 | 146 | … | 286 | 279 | 25 | 25 | 241.3 |
| 552 | 197 | 692 | 159 | … | 318 | 279 | 25 | 25 | 241.3 |
| 616 | 210 | 768 | 171 | … | 349 | 279 | 25 | 25 | 241.3 |

…	1∕2

…	3∕4

…	1

…	11∕4

…	11∕2

5∕8	2

5∕8	21∕2

5∕8	3

5∕8	31∕2

3∕4	4

3∕4	5

3∕4	6

3∕4	8

3∕4	10

7∕8	12

7∕8	14

7∕8	16

135

| 18 | 635 | 38.1 | 11.9 | 438 | 419 | 673 | 216 | 813 | 178 | 483 | 425 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 698 | 41.1 | 12.7 | 489 | 457 | 737 | 241 | 889 | 203 | 508 | 464 |
| 24 | 813 | 46.0 | 14.5 | 591 | 559 | 864 | 279 | 1029 | 229 | 610 | 565 |

ASME  B16.5-2020

###### Table 9 Dimensions of Class 150 Flanged Fittings  (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12

| Minimum Outside Thickness Minimum Inside Nominal Diameter of Wall Diameter Pipe of Flange, Thickness of Size, Flange, tf of Fitting, Fitting, NPS O (1)–(3) tm d | 1.5-mm Raised Face (4) | Ring Joint (4) |
| --- | --- | --- |
|  | Center- to- Short Contact Center- Center- Contact Surface of to- Center- Long to- Surface- Raised Contact to- Center- Contact to- Face Surface of Contact to- Surface of Contact Elbow, Raised Surface of Contact Raised Surface Tee, Face Raised Surface of Face of Cross, Long Face Raised Lateral Raised and Radius 45-deg Face and Face True “Y,” vElbow, Elbow, Lateral, True “Y,” Reducer, AA BB CC EE FF GG (5) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (6) |

136

ASME  B16.5-2020

###### Table 9 Dimensions of Class 150 Flanged Fittings  (Cont’d)

13	14	15	16	17	18	19	20	21	22	23	1

| Ring Joint (4) |  |  |  |  | Diameter of Round Base or Center- Width of Thickness to-Base, Square of Thickness R Base, S Base, T of Ribs, U (7)–(9) (7) (7)–(10) (7) | Base Drilling (11) | Nominal Pipe Size, NPS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Center- to-End Long Radius Elbow, JJ (6) | Center- to-End 45-deg Elbow, KK (6) | Long Center- to-End Lateral, LL (6) | Short Center- to-End Lateral and True “Y,” MM (6) | End-to- End Reducer, NN (5), (6) |  | Bolt Circle or Diameter Bolt of Spacing, Drilled W Holes |  |

| 679 | 222 | 819 | 184 | … | 381 | 343 | 28 | 28 | 298.4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 743 | 248 | 895 | 210 | … | 406 | 343 | 28 | 28 | 298.4 |
| 870 | 286 | 1035 | 235 | … | 470 | 343 | 28 | 28 | 298.4 |

7∕8	18

7∕8	20

7∕8	24

137

ASME  B16.5-2020

###### Table 9 Dimensions of Class 150 Flanged Fittings (Cont’d)

GENERAL NOTES:

                - Dimensions are in millimeters.

                - For tolerances, see section 7.

                - For facings, see para. 6.4.

                - For flange bolt holes, see para. 6.5 and Table 8.

                - For spot facing, see para. 6.6.

                - For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4.

                - For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

                - For reinforcement of certain fittings, see para. 6.1.

                - For drains, see para. 6.12.

NOTES:

- The thickness of flange minimum dimensions for loose flanges, Table 9 sizes NPS 31∕2 and smaller, are slightly heavier than for flanges on these fittings, which are reinforced by being cast integral with the body of fitting.

- These fittings may be supplied with a flat face flange. The flat face may be either the full tf   dimension thickness plus 1.5 mm or the tf

dimension thickness without the raised face height. See para. 6.3.2 for additional restrictions.

- The thickness of the flange dimension illustrated is for regularly furnished 1.5-mm raised face (except lapped); for thickness requirements of other facings, see Figure 6.

- For center-to-contact surface and center-to-end dimensioins of reducing fittings, see para. 6.2.3.

- For contact surface-to-contact surface and end-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

- These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For center-to-end dimensions of reducing fittings or end-to-end dimensions of reducers, use center-to-contact surface or contact surface-to-contact surface dimensions of 1.5 mm raised face (flange edge) for largest opening, and add the proper height to provide for ring joint groove applying to each flange. See Table 5 for ring joint facing dimensions.

- The base dimensions apply to all straight and reducing sizes.

- For reducing fittings, the size and center-to-face dimension of base are determined by the size of the largest opening of fittings. In the case of reducing base elbows, orders shall specify whether the base shall be opposite the larger or smaller opening.

- Bases shall be plain faced unless otherwise specified, and the center-to-base dimension R shall be the finished dimension.

- Bases may be cast integral or attached as weldments at the option of the manufacturer.

- The bases of these fittings are intended for support in compression and are not to be used for anchors or supports in tension or shear.

138

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

139

ASME  B16.5-2020

ð20Þ

###### Table 9C Dimensions of Class 150 Flanged Fittings

| Minimum Thickness Minimum Inside Nominal Outside of Wall Diameter Pipe Diameter Flange, Thickness of Size, of Flange, tf of Fitting, Fitting, NPS O (1)–(3) tm d | 0.06-in. Raised Face (Flange Edge) (4) | Ring Joint (4) |
| --- | --- | --- |
|  | Center- to- Short Contact Center- Center- Contact Surface of to- Center- Long to- Surface- Raised Contact to- Center- Contact to- Face Surface of Contact to- Surface of Contact Elbow, Raised Surface of Contact Raised Surface Tee, Face Raised Surface of Face of Cross, Long Face Raised Lateral Raised and Radius 45-deg Face and Face True “Y,” Elbow, Elbow, Lateral, True “Y,” Reducer, AA BB CC EE FF GG (5) | Center- to-End Elbow Tee, Cross, and True “Y,” HH (6) |

1	2	3	4	5	6	7	8	9	10	11	12

| 1∕2 3∕4 | 3.50 | 0.31 | 0.11 | 0.50 | … | … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3.88 | 0.34 | 0.12 | 0.75 | … | … | … | … | … | … | … |
| 1 | 4.25 | 0.38 | 0.16 | 1.00 | 3.50 | 5.00 | 1.75 | 5.75 | 1.75 | 4.50 | 3.75 |
| 11∕4 | 4.62 | 0.44 | 0.19 | 1.25 | 3.75 | 5.50 | 2.00 | 6.25 | 1.75 | 4.50 | 4.00 |
| 11∕2 | 5.00 | 0.50 | 0.19 | 1.50 | 4.00 | 6.00 | 2.25 | 7.00 | 2.00 | 4.50 | 4.25 |
| 2 | 6.00 | 0.56 | 0.22 | 2.00 | 4.50 | 6.50 | 2.50 | 8.00 | 2.50 | 5.00 | 4.75 |
| 21∕2 | 7.00 | 0.62 | 0.22 | 2.50 | 5.00 | 7.00 | 3.00 | 9.50 | 2.50 | 5.50 | 5.25 |
| 3 | 7.50 | 0.69 | 0.22 | 3.00 | 5.50 | 7.75 | 3.00 | 10.00 | 3.00 | 6.00 | 5.75 |
| 31∕2 | 8.50 | 0.75 | 0.25 | 3.50 | 6.00 | 8.50 | 3.50 | 11.50 | 3.00 | 6.50 | 6.25 |
| 4 | 9.00 | 0.88 | 0.25 | 4.00 | 6.50 | 9.00 | 4.00 | 12.00 | 3.00 | 7.00 | 6.75 |
| 5 | 10.00 | 0.88 | 0.28 | 5.00 | 7.50 | 10.25 | 4.50 | 13.50 | 3.50 | 8.00 | 7.75 |
| 6 | 11.00 | 0.94 | 0.28 | 6.00 | 8.00 | 11.50 | 5.00 | 14.50 | 3.50 | 9.00 | 8.25 |
| 8 | 13.50 | 1.06 | 0.31 | 8.00 | 9.00 | 14.00 | 5.50 | 17.50 | 4.50 | 11.00 | 9.25 |
| 10 | 16.00 | 1.12 | 0.34 | 10.00 | 11.00 | 16.50 | 6.50 | 20.50 | 5.00 | 12.00 | 11.25 |
| 12 | 19.00 | 1.19 | 0.38 | 12.00 | 12.00 | 19.00 | 7.50 | 24.50 | 5.50 | 14.00 | 12.25 |
| 14 | 21.00 | 1.31 | 0.41 | 13.25 | 14.00 | 21.50 | 7.50 | 27.00 | 6.00 | 16.00 | 14.25 |
| 16 | 23.50 | 1.38 | 0.44 | 15.25 | 15.00 | 24.00 | 8.00 | 30.00 | 6.50 | 18.00 | 15.25 |
| 18 | 25.00 | 1.50 | 0.47 | 17.25 | 16.50 | 26.50 | 8.50 | 32.00 | 7.00 | 19.00 | 16.75 |
| 20 | 27.50 | 1.62 | 0.50 | 19.25 | 18.00 | 29.00 | 9.50 | 35.00 | 8.00 | 20.00 | 18.25 |
| 24 | 32.00 | 1.81 | 0.57 | 23.25 | 22.00 | 34.00 | 11.00 | 40.50 | 9.00 | 24.00 | 22.25 |

140

ASME  B16.5-2020

###### Table 9C Dimensions of Class 150 Flanged Fittings

13	14	15	16	17	18	19	20	21	22	23	1

| Ring Joint (4) |  |  |  |  | Diameter of Round Center- Base or to- Width of Thickness Base, Square of Thickness R Base, S Base, T of Ribs, (7)–(9) (7) (7)–(10) U (7) | Base Drilling (11) |  | Nominal Pipe Size, NPS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center- to-End Long Radius Elbow, JJ (6) | Center- to-End 45-deg Elbow, KK (6) | Long Center- to-End Lateral, LL (6) | Short Center- to-End Lateral and True “Y,” MM (6) | End-to-End Reducer, NN (5), (6) |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

| … | … | … | … | … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | … | … | … | … |
| 5.25 | 2.00 | 6.00 | 2.00 | … | … | … | … | … | … |
| 5.75 | 2.25 | 6.50 | 2.00 | … | … | … | … | … | … |
| 6.25 | 2.50 | 7.25 | 2.25 | … | … | … | … | … | … |
| 6.75 | 2.75 | 8.25 | 2.75 | … | 4.12 | 4.62 | 0.50 | 0.50 | 3.50 |
| 7.25 | 3.25 | 9.75 | 2.75 | … | 4.50 | 4.62 | 0.50 | 0.50 | 3.50 |
| 8.00 | 3.25 | 10.25 | 3.25 | … | 4.88 | 5.00 | 0.56 | 0.56 | 3.88 |
| 8.75 | 3.75 | 11.75 | 3.25 | … | 5.25 | 5.00 | 0.56 | 0.56 | 3.88 |
| 9.25 | 4.25 | 12.25 | 3.25 | … | 5.50 | 6.00 | 0.62 | 0.62 | 4.75 |
| 10.50 | 4.75 | 13.75 | 3.75 | … | 6.25 | 7.00 | 0.69 | 0.65 | 5.50 |
| 11.75 | 5.25 | 14.75 | 3.75 | … | 7.00 | 7.00 | 0.69 | 0.65 | 5.50 |
| 14.25 | 5.75 | 17.75 | 4.75 | … | 8.38 | 9.00 | 0.94 | 0.94 | 7.50 |
| 16.75 | 6.75 | 20.75 | 5.25 | … | 9.75 | 9.00 | 0.94 | 0.94 | 7.50 |
| 19.25 | 7.75 | 24.75 | 5.75 | … | 11.25 | 11.00 | 1.00 | 1.00 | 9.50 |
| 21.75 | 7.75 | 27.25 | 6.25 | … | 12.50 | 11.00 | 1.00 | 1.00 | 9.50 |
| 24.25 | 8.25 | 30.25 | 6.75 | … | 13.75 | 11.00 | 1.00 | 1.00 | 9.50 |
| 26.75 | 8.75 | 32.25 | 7.25 | … | 15.00 | 13.50 | 1.12 | 1.12 | 11.75 |
| 29.25 | 9.75 | 35.25 | 8.25 | … | 16.00 | 13.50 | 1.12 | 1.12 | 11.75 |
| 34.25 | 11.25 | 40.75 | 9.25 | … | 18.50 | 13.50 | 1.12 | 1.12 | 11.75 |

…	1∕2

…	3∕4

…	1

11∕4

11∕2

5∕8	2

5∕8	21∕2

5∕8	3

5∕8	31∕2

3∕4	4

3∕4	5

3∕4	6

3∕4	8

3∕4	10

7∕8	12

7∕8	14

7∕8	16

7∕8	18

7∕8	20

7∕8	24

141

ASME  B16.5-2020

###### Table 9C Dimensions of Class 150 Flanged Fittings (Cont’d)

GENERAL NOTE:

- Dimensions are in inches.

- For tolerances, see section 7.

- For facings, see para. 6.4.

- For flange bolt holes, see para. 6.5 and Table 7C.

- For spot facing, see para. 6.6.

- For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4.

- For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

- For reinforcement of certain fittings, see para. 6.1.

- For drains, see para. 6.12.

NOTES:

- The thickness of flange minimum dimensions for loose flanges, Table 9C, sizes NPS 31∕2 and smaller, are slightly heavier than for flanges on these fittings, which are reinforced by being cast integral with the body of the fitting.

- These fittings may be supplied with a flat face flange. The flat face may be either the full tf  dimension thickness plus 0.06 in. or the tf

dimension thickness without the raised face height. See para. 6.3.2 for additional restrictions.

- The thickness of the flange dimension illustrated is for regularly furnished 0.06-in. raised face (except lapped); for thickness requirements of other facings, see Figure 6.

- For center-to-contact surface and center-to-end dimensions of reducing fittings, see para. 6.2.3.

- For contact surface-to-contact surface and end-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

- These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For center-to-end dimensions of reducing fittings or end-to-end dimensions of reducers, use center-to-contact surface or contact surface-to-contact surface dimensions of 0.06-in. raised face (flange edge) for the largest opening, and add the proper height to provide for ring joint groove applying to each flange. See Table 5C for ring joint facing dimensions.

- The base dimensions apply to all straight and reducing sizes.

- For reducing fittings, the size, and center-to-face dimension of base are determined by the size of the largest opening of fittings. In the case of reducing base elbows, orders shall specify whether the base shall be opposite the larger or smaller opening.

- Bases shall be plain faced, unless otherwise specified, and the center-to-base dimension R shall be the finished dimension.

- Bases may be cast integral or attached as weldments at the option of the manufacturer.

- The bases of these fittings are intended for support in compression and are not to be used for anchors or supports in tension or shear.

142

ASME  B16.5-2020

###### Table 10 Templates for Drilling Class 300 Pipe Flanges and Flanged Fittings	ð20Þ

| Nominal Pipe Size, NPS | Outside Diameter of Flange, O | Drilling (2), (3) |  |  |  | Length of Bolts, L (4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Stud Bolts (1) |  | Machine Bolts |
|  |  | Diameter of Bolt Circle, W | Diameter of Bolt Holes, in. | Number of Bolts | Diameter of Bolts, in. | 1.5-mm Raised Face | Ring Joint | 1.5-mm Raised Face |

1	2	3	4	5	6	7	8	9

| 1∕2 3∕4 1 11∕4 11∕2 2 21∕2 | 95 117 124 133 155 165 190 | 66.5 82.6 88.9 98.6 114.3 127.0 149.4 | 5∕8 3∕4 3∕4 3∕4 7∕8 3∕4 7∕8 | 4 1∕2 65 75 55 4 5∕8 75 90 65 4 5∕8 75 90 65 4 5∕8 85 95 70 4 3∕4 90 100 75 8 5∕8 90 100 75 8 3∕4 100 115 85 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 210 | 168.1 | 7∕8 | 8 | 3∕4 | 110 | 120 | 90 |
| 31∕2 | 229 | 184.2 | 7∕8 | 8 | 3∕4 | 110 | 125 | 95 |
| 4 | 254 | 200.2 | 7∕8 | 8 | 3∕4 | 115 | 125 | 95 |
| 5 | 279 | 235.0 | 7∕8 | 8 | 3∕4 | 120 | 135 | 110 |
| 6 | 318 | 269.7 | 7∕8 | 12 | 3∕4 | 120 | 140 | 110 |
| 8 | 381 | 330.2 | 1 | 12 | 7∕8 | 140 | 150 | 120 |
| 10 | 444 | 387.4 | 11∕8 | 16 | 1 | 160 | 170 | 140 |
| 12 | 521 | 450.8 | 11∕4 | 16 | 11∕8 | 170 | 185 | 145 |
| 14 | 584 | 514.4 | 11∕4 | 20 | 11∕8 | 180 | 190 | 160 |
| 16 | 648 | 571.5 | 13∕8 | 20 | 11∕4 | 190 | 205 | 165 |
| 18 | 711 | 628.6 | 13∕8 | 24 | 11∕4 | 195 | 210 | 170 |
| 20 | 775 | 685.8 | 13∕8 | 24 | 11∕4 | 205 | 220 | 185 |
| 22 | 838 | 743.0 | 15∕8 | 24 | 11∕2 | 230 | 255 | 205 |
| 24 | 914 | 812.8 | 15∕8 | 24 | 11∕2 | 230 | 255 | 205 |

GENERAL NOTES:

- Dimensions are in millimeters, except for diameters of bolts and bolt holes, which are in inches.

- For other dimensions, see Tables 11 and 12.

NOTES:

  - Length of stud bolt does not include the height of the points (see para. 6.10.2).

  - For flange bolt holes, see para. 6.5.

  - For spot facing, see para 6.6.

  - Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

143

ASME  B16.5-2020

###### Table 10C Templates for Drilling Class 300 Pipe Flanges and Flanged Fittings

| Outside Nominal Diameter Pipe of Flange, Size O | Drilling (2), (3) | Length of Bolts, L (4) |  |
| --- | --- | --- | --- |
|  | Diameter of Bolt Circle, Diameter of Number Diameter W Bolt Holes of Bolts of Bolts | Stud Bolts (1) | Machine Bolts |
|  |  | Raised Face Ring 0.06 in. Joint | Raised Face 0.06 in. |

| 1∕2 3∕4 | 3.75 | 2.62 |
| --- | --- | --- |
|  | 4.62 | 3.25 |
| 1 | 4.88 | 3.50 |
| 11∕4 | 5.25 | 3.88 |
| 11∕2 | 6.12 | 4.50 |
| 2 | 6.50 | 5.00 |
| 21∕2 | 7.50 | 5.88 |
| 3 | 8.25 | 6.62 |
| 31∕2 | 9.00 | 7.25 |
| 4 | 10.00 | 7.88 |

1	2	3	4	5	6	7	8	9

|  |  |  | 5∕8 3∕4 3∕4 3∕4 7∕8 3∕4 7∕8 | 4 1∕2 2.50 3.00 2.25 4 5∕8 3.00 3.50 2.50 4 5∕8 3.00 3.50 2.50 4 5∕8 3.25 3.75 2.75 4 3∕4 3.50 4.00 3.00 8 5∕8 3.50 4.00 3.00 8 3∕4 4.00 4.50 3.25 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7∕8 | 8 | 3∕4 | 4.25 | 4.75 | 3.50 |
|  |  |  | 7∕8 | 8 | 3∕4 | 4.25 | 5.00 | 3.75 |
|  |  |  | 7∕8 | 8 | 3∕4 | 4.50 | 5.00 | 3.75 |
| 5 | 11.00 | 9.25 | 7∕8 | 8 | 3∕4 | 4.75 | 5.25 | 4.25 |
| 6 | 12.50 | 10.62 | 7∕8 | 12 | 3∕4 | 4.75 | 5.50 | 4.25 |
| 8 | 15.00 | 13.00 | 1 | 12 | 7∕8 | 5.50 | 6.00 | 4.75 |
| 10 | 17.50 | 15.25 | 11∕8 | 16 | 1 | 6.25 | 6.75 | 5.50 |
| 12 | 20.50 | 17.75 | 11∕4 | 16 | 11∕8 | 6.75 | 7.25 | 5.75 |
| 14 | 23.00 | 20.25 | 11∕4 | 20 | 11∕8 | 7.00 | 7.50 | 6.25 |
| 16 | 25.50 | 22.50 | 13∕8 | 20 | 11∕4 | 7.50 | 8.00 | 6.50 |
| 18 | 28.00 | 24.75 | 13∕8 | 24 | 11∕4 | 7.75 | 8.25 | 6.75 |
| 20 | 30.50 | 27.00 | 13∕8 | 24 | 11∕4 | 8.00 | 8.75 | 7.25 |
| 22 | 33.00 | 29.25 | 15∕8 | 24 | 11∕2 | 9.00 | 10.00 | 8.00 |
| 24 | 36.00 | 32.00 | 15∕8 | 24 | 11∕2 | 9.00 | 10.00 | 8.00 |

GENERAL NOTES:

    - Dimensions are in inches.

    - For other dimensions see, Tables 11C and 12C.

NOTES:

      - The length of the stud bolt does not include the height of the points (see para. 6.10.2).

      - For flange bolt holes, see para. 6.5.

      - For spot facing, see para. 6.6.

      - Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

144

145

###### Table 11 Dimensions of Class 300 Flanges	ð20Þ

ASME  B16.5-2020

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15	16

| Length Through Hub |  |  |  |  |  |  |  |  |  |  | Bore |  | Corner |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Hub |  |  |  |  |  |  |  | Radius |  |  |
|  |  |  |  |  | Diameter |  |  |  |  |  |  |  | of Bore | Minimum |  |
|  | Outside | Minimum |  |  | Beginning | Threaded/ |  |  | Minimum | Minimum |  | Welding | of | Counter- |  |
|  | Diamter | Thickness | Minimum |  | of | Slip-On/ |  |  | Thread | Slip-On/ |  | Neck/ | Lapped | bore | Depth |
| Nominal | of | of Flange, | Thickness | Diameter | Chamfer | Socket |  | Welding | Length | Socket | Minimum | Socket | Flange | Threaded | of |
| Pipe Size | Flange, | tf | Lap Joint, | of Hub, | Welding | Welding, | Lapped, | Neck, | Threaded, | Welding, | Lapped, | Welding, | and | Flange, | Socket, |
| NPS | O | (2)–(4) | tf | X | Neck, Ah (4) | Y | Y | Y | T (5) | B | B | B (6) | Pipe, r | Q | D |
| 1∕2 | 95 | 12.7 | 14.2 | 38 | 21.3 | 21 | 22 | 51 | 16 | 22.4 | 22.9 | 15.7 | 3 | 23.6 | 10 |
| 3∕4 | 117 | 14.2 | 15.7 | 48 | 26.7 | 24 | 25 | 56 | 16 | 27.7 | 28.2 | 20.8 | 3 | 29.0 | 11 |
| 1 | 124 | 15.7 | 17.5 | 54 | 33.5 | 25 | 27 | 60 | 18 | 34.5 | 35.1 | 26.7 | 3 | 35.8 | 13 |
| 11∕4 | 133 | 17.5 | 19.0 | 64 | 42.2 | 25 | 27 | 64 | 21 | 43.2 | 43.7 | 35.1 | 5 | 44.4 | 14 |
| 11∕2 | 155 | 19.0 | 20.6 | 70 | 48.3 | 29 | 30 | 67 | 22 | 49.5 | 50.0 | 40.9 | 6 | 50.3 | 16 |

###### Table 11 Dimensions of Class 300 Flanges (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15	16

Hub Diameter

Length Through Hub

Bore	Corner

Radius of Bore

Minimum

Nominal Pipe Size NPS

Outside Diamter of Flange, O

Minimum Thickness of Flange, tf

(2)–(4)

Minimum Thickness Lap Joint, tf

Diameter of Hub,

X

Beginning of Chamfer Welding

Neck, Ah  (4)

Threaded/ Slip-On/ Socket Welding,

Y

Lapped,

Y

Welding Neck,

Y

Minimum Thread Length Threaded, T (5)

Minimum Slip-On/ Socket Welding, B

Minimum Lapped,

B

Welding Neck/ Socket Welding, B (6)

of Lapped Flange and Pipe, r

Counter- bore Threaded Flange,

Q

Depth of Socket, D

ASME  B16.5-2020

146

| 2 | 165 | 20.6 | 22.4 | 84 | 60.5 | 32 | 33 | 68 | 28 | 62.0 | 62.5 | 52.6 | 8 | 63.5 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21∕2 | 190 | 23.9 | 25.4 | 100 | 73.2 | 37 | 38 | 75 | 32 | 74.7 | 75.4 | 62.7 | 8 | 76.2 | 19 |
| 3 | 210 | 26.9 | 28.4 | 117 | 88.9 | 41 | 43 | 78 | 32 | 90.7 | 91.4 | 78.0 | 10 | 92.2 | 21 |
| 31∕2 | 229 | 28.4 | 30.2 | 133 | 101.6 | 43 | 44 | 80 | 37 | 103.4 | 104.1 | 90.2 | 10 | 104.9 | … |
| 4 | 254 | 30.2 | 31.8 | 146 | 114.3 | 46 | 48 | 84 | 37 | 116.1 | 116.8 | 102.4 | 11 | 117.6 | … |
| 5 | 279 | 33.3 | 35.1 | 178 | 141.2 | 49 | 51 | 97 | 43 | 143.8 | 144.5 | 128.3 | 11 | 144.5 | … |
| 6 | 318 | 35.1 | 36.6 | 206 | 168.4 | 51 | 52 | 97 | 46 | 170.7 | 171.4 | 154.2 | 13 | 171.4 | … |
| 8 | 381 | 39.6 | 41.1 | 260 | 219.2 | 60 | 62 | 110 | 51 | 221.5 | 222.2 | 202.7 | 13 | 222.2 | … |
| 10 | 444 | 46.0 | 47.8 | 321 | 273.0 | 65 | 95 | 116 | 56 | 276.4 | 277.4 | 254.5 | 13 | 276.4 | … |
| 12 | 521 | 49.3 | 50.8 | 375 | 323.8 | 72 | 102 | 129 | 60 | 327.2 | 328.2 | 304.8 | 13 | 328.7 | … |
| 14 | 584 | 52.3 | 53.8 | 425 | 355.6 | 75 | 111 | 141 | 64 | 359.2 | 360.2 | (7) | 13 | 360.4 | … |
| 16 | 648 | 55.6 | 57.2 | 483 | 406.4 | 81 | 121 | 145 | 68 | 410.5 | 411.2 | (7) | 13 | 411.2 | … |
| 18 | 711 | 58.7 | 60.5 | 533 | 457.2 | 87 | 130 | 157 | 70 | 461.8 | 462.3 | (7) | 13 | 462.0 | … |
| 20 | 775 | 62.0 | 63.5 | 587 | 508.0 | 94 | 140 | 161 | 73 | 513.1 | 514.4 | (7) | 13 | 512.8 | … |
| 22 | 838 | 65.0 | 66.5 | 641 | 558.8 | 100 | 145 | 164 | … | 564.4 | 565.2 | (7) | 13 | … | … |
| 24 | 914 | 68.3 | 68.8 | 702 | 609.6 | 105 | 152 | 167 | 83 | 616.0 | 616.0 | (7) | 13 | 614.4 | … |

GENERAL NOTES:

        - Dimensions are in millimeters.

        - For tolerances, see section 7.

        - For facings, see para. 6.4.

        - For flange bolt holes, see para. 6.5 and Table 10.

        - For spot facing, see para. 6.6.

        - For reducing threaded and slip-on flanges, see Table 6.

        - Blind flanges may be made with or without hubs at the manufacturer’s option.

        - For reducing welding neck flanges, see para. 6.8.

NOTES:

          - This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges. This dimension is defined as the diameter at the intersection between the hub taper and back face of the flange.

          - These flanges may be supplied with a flat face. The flat face may be either the full tf dimension thickness plus 1.5 mm or the tf dimension thickness without the raised face height. See para. 6.3.2

for additional restrictions.

          - The flange dimensions illustrated are for regularly furnished 1.5-mm raised face (except lapped); for requirements of other facings, see Figure 6.

ASME  B16.5-2020

147

###### Table 11 Dimensions of Class 300 Flanges (Cont’d)

NOTES: (Cont’d)

          - For welding end bevel, see para. 6.7.

          - For thread of threaded flanges, see para. 6.9.

          - Dimensions in Column 13 correspond to the inside diameters of pipe as given in ASME B36.10M for standard wall pipe. Standard wall dimensions are the same as Schedule 40 in sizes NPS 10 and smaller. Tolerances in para. 7.5.2 apply. These bore sizes are furnished unless otherwise specified by the purchaser.

          - To be specified by the purchaser.

ASME  B16.5-2020

148

###### Table 11C Dimensions of Class 300 Flanges

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15	16

|  |  |  |  |  | Diameter | Length Through Hub Bore Corner |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Minimum |  |  | Beginning |  |  |  |  |  |  |  | Bore | Minimum |  |
|  | Outside | Thickness |  |  | of | Threaded |  |  | Minimum | Minimum |  | Welding | Radius of | Counter- |  |
|  | Diamter | of |  |  | Chamfer | Slip-On |  |  | Thread | Slip-On |  | Neck/ | Lapped | bore | Depth |
|  | of | Flange, | Thickness | Diameter | Welding | Socket |  | Welding | Length | Socket | Minimum | Socket | Flange | Threaded | of |
| Nominal | Flange, | tf | of Lap | of Hub, | Neck, Ah | Welding, | Lapped, | Neck, | Threaded, | Welding, | Lapped, | Welding, B | and | Flange, | Socket, |
| Pipe Size | O | (2)–(4) | Joint, tf | X | (4) | Y | Y | Y | T (5) | B | B | (6) | Pipe, r | Q | D |
| 1∕2 | 3.75 | 0.50 | 0.56 | 1.50 | 0.84 | 0.81 | 0.88 | 2.00 | 0.62 | 0.88 | 0.90 | 0.62 | 0.12 | 0.93 | 0.38 |
| 3∕4 | 4.62 | 0.56 | 0.62 | 1.88 | 1.05 | 0.94 | 1.00 | 2.19 | 0.62 | 1.09 | 1.11 | 0.82 | 0.12 | 1.14 | 0.44 |
| 1 | 4.88 | 0.62 | 0.69 | 2.12 | 1.32 | 1.00 | 1.06 | 2.38 | 0.69 | 1.36 | 1.38 | 1.05 | 0.12 | 1.41 | 0.50 |
| 11∕4 | 5.25 | 0.69 | 0.75 | 2.50 | 1.66 | 1.00 | 1.06 | 2.50 | 0.81 | 1.70 | 1.72 | 1.38 | 0.19 | 1.75 | 0.56 |
| 11∕2 | 6.12 | 0.75 | 0.81 | 2.75 | 1.90 | 1.13 | 1.19 | 2.63 | 0.88 | 1.95 | 1.97 | 1.61 | 0.25 | 1.98 | 0.62 |

ASME  B16.5-2020

149

###### Table 11C Dimensions of Class 300 Flanges (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15	16

| Outside Diamter of |  | Minimum |  |  | Diameter Beginning | Length Through Hub |  |  |  |  | Bore |  | Corner Bore | Minimum |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Thickness |  |  | of | Threaded |  |  | Minimum | Minimum |  | Welding | Radius of | Counter- |  |
|  |  | of |  |  | Chamfer | Slip-On |  |  | Thread | Slip-On |  | Neck/ | Lapped | bore | Depth |
|  |  | Flange, | Thickness | Diameter | Welding | Socket |  | Welding | Length | Socket | Minimum | Socket | Flange | Threaded | of |
| Nominal | Flange, | tf | of Lap | of Hub, | Neck, Ah | Welding, | Lapped, | Neck, | Threaded, | Welding, | Lapped, | Welding, B | and | Flange, | Socket, |
| Pipe Size | O | (2)–(4) | Joint, tf | X | (4) | Y | Y | Y | T (5) | B | B | (6) | Pipe, r | Q | D |
| 2 | 6.50 | 0.81 | 0.88 | 3.31 | 2.38 | 1.25 | 1.31 | 2.69 | 1.12 | 2.44 | 2.46 | 2.07 | 0.31 | 2.50 | 0.69 |
| 21∕2 | 7.50 | 0.94 | 1.00 | 3.94 | 2.88 | 1.44 | 1.50 | 2.94 | 1.25 | 2.94 | 2.97 | 2.47 | 0.31 | 3.00 | 0.75 |
| 3 | 8.25 | 1.06 | 1.12 | 4.62 | 3.50 | 1.63 | 1.69 | 3.06 | 1.25 | 3.57 | 3.60 | 3.07 | 0.38 | 3.63 | 0.81 |
| 31∕2 | 9.00 | 1.12 | 1.19 | 5.25 | 4.00 | 1.69 | 1.75 | 3.13 | 1.44 | 4.07 | 4.10 | 3.55 | 0.38 | 4.13 | … |
| 4 | 10.00 | 1.19 | 1.25 | 5.75 | 4.50 | 1.82 | 1.88 | 3.32 | 1.44 | 4.57 | 4.60 | 4.03 | 0.44 | 4.63 | … |
| 5 | 11.00 | 1.31 | 1.38 | 7.00 | 5.56 | 1.94 | 2.00 | 3.82 | 1.69 | 5.66 | 5.69 | 5.05 | 0.44 | 5.69 | … |
| 6 | 12.50 | 1.38 | 1.44 | 8.12 | 6.63 | 2.00 | 2.06 | 3.82 | 1.81 | 6.72 | 6.75 | 6.07 | 0.50 | 6.75 | … |
| 8 | 15.00 | 1.56 | 1.62 | 10.25 | 8.63 | 2.38 | 2.44 | 4.32 | 2.00 | 8.72 | 8.75 | 7.98 | 0.50 | 8.75 | … |
| 10 | 17.50 | 1.81 | 1.88 | 12.62 | 10.75 | 2.56 | 3.75 | 4.56 | 2.19 | 10.88 | 10.92 | 10.02 | 0.50 | 10.88 | … |
| 12 | 20.50 | 1.94 | 2.00 | 14.75 | 12.75 | 2.82 | 4.00 | 5.06 | 2.38 | 12.88 | 12.92 | 12.00 | 0.50 | 12.94 | … |
| 14 | 23.00 | 2.06 | 2.12 | 16.75 | 14.00 | 2.94 | 4.38 | 5.56 | 2.50 | 14.14 | 14.18 | (7) | 0.50 | 14.19 | … |
| 16 | 25.50 | 2.19 | 2.25 | 19.00 | 16.00 | 3.19 | 4.75 | 5.69 | 2.69 | 16.16 | 16.19 | (7) | 0.50 | 16.19 | … |
| 18 | 28.00 | 2.31 | 2.38 | 21.00 | 18.00 | 3.44 | 5.12 | 6.19 | 2.75 | 18.18 | 18.20 | (7) | 0.50 | 18.19 | … |
| 20 | 30.50 | 2.44 | 2.50 | 23.12 | 20.00 | 3.69 | 5.50 | 6.32 | 2.88 | 20.20 | 20.25 | (7) | 0.50 | 20.19 | … |
| 22 | 33.00 | 2.56 | 2.62 | 25.25 | 22.00 | 3.94 | 5.69 | 6.44 | … | 22.22 | 22.25 | (7) | 0.50 | … | … |
| 24 | 36.00 | 2.69 | 2.75 | 27.62 | 24.00 | 4.13 | 6.00 | 6.56 | 3.25 | 24.25 | 24.25 | (7) | 0.50 | 24.19 | … |

GENERAL NOTES:

            - Dimensions are in inches.

            - For tolerances, see section 7.

            - For facings, see para. 6.4.

            - For flange bolt holes, see para. 6.5 and Table 10C.

            - For spot facing, see para. 6.6.

            - For reducing threaded and slip-on flanges, see Table 6C.

            - Blind flanges may be made with or without hubs at the manufacturer’s option.

            - For reducing welding neck flanges, see para. 6.8.

NOTES:

              - This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges. This dimension is defined as the diameter at the intersection between the hub taper and back face of the flange.

              - These flanges may be supplied with a flat face. The flat face may be either the full tf dimension thickness plus 0.06 in. or the tf dimension thickness without the raised face height. See para. 6.3.2

for additional restrictions.

              - The flange dimensions illustrated are for regularly furnished 0.06-in. raised face (except lapped); for requirements of other facings, see Figure 6.

              - For welding end bevel, see para. 6.7.

ASME  B16.5-2020

150

###### Table 11C Dimensions of Class 300 Flanges (Cont’d)

NOTES: (Cont’d)

              - For thread of threaded flanges, see para. 6.9.

              - Dimensions in Column 13 correspond to the inside diameters of pipe as given in ASME B36.10M for standard wall pipe. Standard wall dimensions are the same as Schedule 40 in sizes NPS 10 and smaller. Tolerances in para. 7.5.2 apply. These bore sizes are furnished unless otherwise specified by the purchaser.

              - To be specified by the purchaser.

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

151

ASME  B16.5-2020

ð20Þ

###### Table 12 Dimensions of Class 300 Flanged Fittings

| Minimum Outside Thickness Minimum Inside Nominal Diameter of Wall Diameter Pipe of Flange, Thickness of Size, Flange, tf of Fitting, Fitting, NPS O (1)–(4) tm d | 1.5-mm Raised Face (Flange Edge) (4) | Ring Joint (4) |
| --- | --- | --- |
|  | Center- to- Short Contact Center- Center- Contact Surface of to- Center- Long to- Surface- Raised Contact to- Center- Contact to- Face Surface of Contact to- Surface of Contact Elbow, Raised Surface of Contact Raised Surface Tee, Face Raised Surface of Face of Cross, Long Face Raised Lateral Raised and Radius 45-deg Face and Face True “Y,” Elbow, Elbow, Lateral, True “Y,” Reducer, AA BB CC EE FF GG (5) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (6) |

1	2	3	4	5	6	7	8	9	10	11	12

| 1 | 124 | 15.7 | 4.8 | 25 | 102 | 127 | 57 | 165 | 51 | 114 | 108 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11∕4 | 133 | 17.5 | 4.8 | 32 | 108 | 140 | 64 | 184 | 57 | 114 | 114 |
| 11∕2 | 155 | 19.0 | 4.8 | 38 | 114 | 152 | 70 | 216 | 64 | 114 | 121 |
| 2 | 165 | 20.6 | 6.4 | 51 | 127 | 165 | 76 | 229 | 64 | 127 | 135 |
| 21∕2 | 190 | 23.9 | 6.4 | 64 | 140 | 178 | 89 | 267 | 64 | 140 | 148 |
| 3 | 210 | 26.9 | 7.1 | 76 | 152 | 197 | 89 | 279 | 76 | 152 | 160 |
| 31∕2 | 229 | 28.4 | 7.4 | 89 | 165 | 216 | 102 | 318 | 76 | 165 | 173 |
| 4 | 254 | 30.2 | 7.9 | 102 | 178 | 229 | 114 | 343 | 76 | 178 | 186 |
| 5 | 279 | 33.3 | 9.7 | 127 | 203 | 260 | 127 | 381 | 89 | 203 | 211 |
| 6 | 318 | 35.1 | 9.7 | 152 | 216 | 292 | 140 | 444 | 102 | 229 | 224 |
| 8 | 381 | 39.6 | 11.2 | 203 | 254 | 356 | 152 | 521 | 127 | 279 | 262 |
| 10 | 444 | 46.0 | 12.7 | 254 | 292 | 419 | 178 | 610 | 140 | 305 | 300 |
| 12 | 521 | 49.3 | 14.2 | 305 | 330 | 483 | 203 | 698 | 152 | 356 | 338 |
| 14 | 584 | 52.3 | 15.7 | 337 | 381 | 546 | 216 | 787 | 165 | 406 | 389 |
| 16 | 648 | 55.6 | 17.5 | 387 | 419 | 610 | 241 | 876 | 190 | 457 | 427 |
| 18 | 711 | 58.7 | 19.0 | 432 | 457 | 673 | 254 | 952 | 203 | 483 | 465 |
| 20 | 775 | 62.0 | 20.6 | 483 | 495 | 737 | 267 | 1029 | 216 | 508 | 505 |
| 24 | 914 | 68.3 | 23.9 | 584 | 572 | 864 | 305 | 1206 | 254 | 610 | 583 |

152

ASME  B16.5-2020

###### Table 12 Dimensions of Class 300 Flanged Fittings

| Ring Joint (5) |  |  |  | Diameter of Round Base or Width of Thickness Center- Square of Base, to-Base, Base, S T R (7)–(9) (7) (7)–(10) | Base Drilling (11) |  | Nominal Pipe Size, NPS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Center- to-End Long Radius Elbow, JJ (7) | Center- to-End 45-deg Elbow, KK (7) | Long Center- to-End Lateral, LL (7) | Short Center- to-End Lateral End-to- and End True “Y,” Reducer, MM (7) NN |  | Thickness of Ribs, U (7) | Bolt Circle or Bolt Diameter Spacing, of Drilled W Holes |  |

13	14	15	16	17	18	19	20	21	22	23	1

| 133 | 64 | 171 | 57 | … | … | … | … | … | . . . | … | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 146 | 70 | 191 | 64 | … | … | … | … | … | . . . | … | 11∕4 |
| 159 | 76 | 222 | 70 | … | … | … | … | … | . . . | … | 11∕2 |
| 173 | 84 | 236 | 71 | … | 114 | 133 | 19 | 13 | 98.6 | 3∕4 | 2 |
| 186 | 97 | 275 | 71 | … | 121 | 133 | 19 | 13 | 98.6 | 3∕4 | 21∕2 |
| 205 | 97 | 287 | 84 | … | 133 | 155 | 21 | 16 | 114.3 | 7∕8 | 3 |
| 224 | 109 | 325 | 84 | … | 143 | 155 | 21 | 16 | 114.3 | 7∕8 | 31∕2 |
| 236 | 124 | 351 | 84 | … | 152 | 165 | 22 | 16 | 127.0 | 3∕4 | 4 |
| 268 | 135 | 389 | 97 | … | 171 | 190 | 25 | 19 | 149.4 | 7∕8 | 5 |
| 300 | 148 | 452 | 109 | … | 190 | 190 | 25 | 19 | 149.4 | 7∕8 | 6 |
| 363 | 160 | 529 | 135 | … | 229 | 254 | 32 | 22 | 200.2 | 7∕8 | 8 |
| 427 | 186 | 617 | 148 | … | 267 | 254 | 32 | 22 | 200.2 | 7∕8 | 10 |
| 490 | 211 | 706 | 160 | … | 305 | 318 | 37 | 25 | 269.7 | 7∕8 | 12 |
| 554 | 224 | 795 | 173 | … | 343 | 318 | 37 | 25 | 269.7 | 7∕8 | 14 |
| 617 | 249 | 884 | 198 | … | 375 | 318 | 37 | 28 | 269.7 | 7∕8 | 16 |
| 681 | 262 | 960 | 211 | … | 413 | 381 | 41 | 28 | 330.2 | 1 | 18 |
| 746 | 276 | 1038 | 226 | … | 454 | 381 | 41 | 32 | 330.2 | 1 | 20 |
| 875 | 316 | 1218 | 265 | … | 527 | 444 | 48 | 32 | 387.4 | 11∕8 | 24 |

153

ASME  B16.5-2020

###### Table 12 Dimensions of Class 300 Flanged Fittings (Cont’d)

GENERAL NOTES:

                - Dimensions are in millimeters.

                - For tolerances, see section 7.

                - For facings, see para. 6.4.

                - For flange bolt holes, see para. 6.5 and Table 8.

                - For spot facing, see para. 6.6

                - For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4.

                - For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

                - For reinforcement of certain fittings, see para. 6.1.1.

                - For drains, see para. 6.12.

NOTES:

- These fittings may be supplied with a flat face flange. The flat face may be either the full tf   dimension thickness plus 1.5 mm or the tf

dimension thickness without the raised face height. See para. 6.3.2 for additional restrictions.

- The thickness of the flange dimension illustrated is for regularly furnished 1.5-mm raised face (except lapped); for thickness requirements of other facings, see Figure 6.

- The thickness of flange minimum dimensions for loose flanges, Table 9, size NPS 31∕2 and smaller, are slightly heavier than for flanges on

these fittings that are reinforced by being cast integral with the body of fitting.

- For center-to-contact surface and center-to-end dimensions of reducing fittings, see para. 6.2.3.

- For contact surface-to-contact surface and end-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

- These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For center-to-end dimensions of reducing fittings or end-to-end dimensions of reducers, use center-to-contact surface or contact surface-to-contact surface dimensions of 1.5-mm raised face (flange edge) for the largest opening, and add the proper height to provide for the ring joint groove applying to each flange. See Table 5 for ring joint facing dimensions.

- The base dimensions apply to all straight and reducing sizes.

- For reducing fittings, the size and center-to-face dimension of base are determined by the size of the largest opening of fittings. In the case of reducing base elbows, orders shall specify whether the base shall be opposite the larger or smaller opening.

- Bases shall be plain faced unless otherwise specified, and the center-to-base dimension R shall be the finished dimension.

- Bases may be cast integral or attached as weldments at the option of the manufacturer.

- The bases of these fittings are intended for support in compression and are not to be used for anchors or supports in tension or shear.

154

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

155

ASME  B16.5-2020

ð20Þ

###### Table 12C Dimensions of Class 300 Flanged Fittings

| Minimum Thickness Minimum Inside Nominal Outside of Wall Diameter Pipe Diameter Flange, Thickness of Size, of Flange, tf of Fitting, Fitting, NPS O (1)–(4) tm d | 0.06-in. Raised Face (Flange Edge) (4) | Ring Joint (4) |
| --- | --- | --- |
|  | Center- to- Short Contact Center- Center- Contact Surface of to- Center- Long to- Surface- Raised Contact to- Center- Contact to- Face Surface of Contact to- Surface of Contact Elbow, Raised Surface of Contact Raised Surface Tee, Face Raised Surface of Face of Cross, Long Face Raised Lateral Raised and Radius 45-deg Face and Face True “Y,” Elbow, Elbow, Lateral, True “Y,” Reducer, AA BB CC EE FF GG (5) | Center- to-End Elbow Tee, Cross, and True “Y,” HH (6) |

1	2	3	4	5	6	7	8	9	10	11	12

| 1 | 4.88 | 0.62 | 0.19 | 1.00 | 4.00 | 5.00 | 2.25 | 6.50 | 2.00 | 4.50 | 4.25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11∕4 | 5.25 | 0.69 | 0.19 | 1.25 | 4.25 | 5.50 | 2.50 | 7.25 | 2.25 | 4.50 | 4.50 |
| 11∕2 | 6.12 | 0.75 | 0.19 | 1.50 | 4.50 | 6.00 | 2.75 | 8.50 | 2.50 | 4.50 | 4.75 |
| 2 | 6.50 | 0.81 | 0.25 | 2.00 | 5.00 | 6.50 | 3.00 | 9.00 | 2.50 | 5.00 | 5.31 |
| 21∕2 | 7.50 | 0.94 | 0.25 | 2.50 | 5.50 | 7.00 | 3.50 | 10.50 | 2.50 | 5.50 | 5.81 |
| 3 | 8.25 | 1.06 | 0.28 | 3.00 | 6.00 | 7.75 | 3.50 | 11.00 | 3.00 | 6.00 | 6.31 |
| 31∕2 | 9.00 | 1.12 | 0.29 | 3.50 | 6.50 | 8.50 | 4.00 | 12.50 | 3.00 | 6.50 | 6.81 |
| 4 | 10.00 | 1.19 | 0.31 | 4.00 | 7.00 | 9.00 | 4.50 | 13.50 | 3.00 | 7.00 | 7.31 |
| 5 | 11.00 | 1.31 | 0.38 | 5.00 | 8.00 | 10.25 | 5.00 | 15.00 | 3.50 | 8.00 | 8.31 |
| 6 | 12.50 | 1.38 | 0.38 | 6.00 | 8.50 | 11.50 | 5.50 | 17.50 | 4.00 | 9.00 | 8.81 |
| 8 | 15.00 | 1.56 | 0.44 | 8.00 | 10.00 | 14.00 | 6.00 | 20.50 | 5.00 | 11.00 | 10.31 |
| 10 | 17.50 | 1.81 | 0.50 | 10.00 | 11.50 | 16.50 | 7.00 | 24.00 | 5.50 | 12.00 | 11.81 |
| 12 | 20.50 | 1.94 | 0.56 | 12.00 | 13.00 | 19.00 | 8.00 | 27.50 | 6.00 | 14.00 | 13.31 |
| 14 | 23.00 | 2.06 | 0.62 | 13.25 | 15.00 | 21.50 | 8.50 | 31.00 | 6.50 | 16.00 | 15.31 |
| 16 | 25.50 | 2.19 | 0.69 | 15.25 | 16.50 | 24.00 | 9.50 | 34.50 | 7.50 | 18.00 | 16.81 |
| 18 | 28.00 | 2.31 | 0.75 | 17.00 | 18.00 | 26.50 | 10.00 | 37.50 | 8.00 | 19.00 | 18.31 |
| 20 | 30.50 | 2.44 | 0.81 | 19.00 | 19.50 | 29.00 | 10.50 | 40.50 | 8.50 | 20.00 | 19.89 |
| 24 | 36.00 | 2.69 | 0.94 | 23.00 | 22.50 | 34.00 | 12.00 | 47.50 | 10.00 | 24.00 | 22.94 |

156

ASME  B16.5-2020

###### Table 12C Dimensions of Class 300 Flanged Fittings

| Ring Joint (5) |  |  |  | Diameter of Round Base Center- or Width Thickness to- of of Base, Square Base, Thickness R Base, T of Ribs, (7)–(9) S (7) (7)-(10) U (7) | Base Drilling (11) |  | Nominal Pipe Size, NPS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Center- to-End Long Radius Elbow, JJ (7) | Center- to-End 45-deg Elbow, KK (7) | Long Center- to-End Lateral, LL (7) | Short Center- to-End Lateral End-to- and End True “Y,” Reducer, MM (7) NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

13	14	15	16	17	18	19	20	21	22	23	1

| 5.25 | 2.50 | 6.75 | 2.25 | … | … | … | … | … | … | … | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.75 | 2.75 | 7.50 | 2.50 | … | … | … | … | … | … | … | 11∕4 |
| 6.25 | 3.00 | 8.75 | 2.75 | … | … | … | … | … | … | … | 11∕2 |
| 6.81 | 3.31 | 9.31 | 2.81 | … | 4.50 | 5.25 | 0.75 | 0.50 | 3.88 | 3∕4 | 2 |
| 7.31 | 3.81 | 10.81 | 2.81 | … | 4.75 | 5.25 | 0.75 | 0.50 | 3.88 | 3∕4 | 21∕2 |
| 8.06 | 3.81 | 11.31 | 3.31 | … | 5.25 | 6.12 | 0.81 | 0.62 | 4.50 | 7∕8 | 3 |
| 8.81 | 4.31 | 12.81 | 3.31 | … | 5.62 | 6.12 | 0.81 | 0.62 | 4.50 | 7∕8 | 31∕2 |
| 9.31 | 4.88 | 13.81 | 3.31 | … | 6.00 | 6.50 | 0.88 | 0.62 | 5.00 | 3∕4 | 4 |
| 10.56 | 5.31 | 15.31 | 3.81 | … | 6.75 | 7.50 | 1.00 | 0.75 | 5.88 | 7∕8 | 5 |
| 11.81 | 5.81 | 17.81 | 4.31 | … | 7.50 | 7.50 | 1.00 | 0.75 | 5.88 | 7∕8 | 6 |
| 14.31 | 6.31 | 20.81 | 5.31 | … | 9.00 | 10.00 | 1.25 | 0.88 | 7.88 | 7∕8 | 8 |
| 16.81 | 7.31 | 24.31 | 5.81 | … | 10.50 | 10.00 | 1.25 | 0.88 | 7.88 | 7∕8 | 10 |
| 19.31 | 8.31 | 27.81 | 6.31 | … | 12.00 | 12.50 | 1.44 | 1.00 | 10.62 | 7∕8 | 12 |
| 21.81 | 8.81 | 31.31 | 6.81 | … | 13.50 | 12.50 | 1.44 | 1.00 | 10.62 | 7∕8 | 14 |
| 24.31 | 9.81 | 34.81 | 7.81 | … | 14.75 | 12.50 | 1.44 | 1.12 | 10.62 | 7∕8 | 16 |
| 26.81 | 10.31 | 37.81 | 8.31 | … | 16.25 | 15.00 | 1.62 | 1.12 | 13.00 | 1 | 18 |
| 29.38 | 10.88 | 40.88 | 8.88 | … | 17.88 | 15.00 | 1.62 | 1.25 | 13.00 | 1 | 20 |
| 34.44 | 12.44 | 47.94 | 10.44 | … | 20.75 | 17.50 | 1.88 | 1.25 | 15.25 | 11∕8 | 24 |

157

ASME  B16.5-2020

###### Table 12C Dimensions of Class 300 Flanged Fittings (Cont’d)

GENERAL NOTES:

- Dimensions are in inches.

- For tolerances, see section 7.

- For facings, see para. 6.4.

- For flange bolt holes, see para. 6.5 and Table 10C.

- For spot facing, see para. 6.6.

- For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4.

- For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

- For reinforcement of certain fittings, see para. 6.1.1.

- For drains, see para. 6.12.

NOTES:

- These fittings may be supplied with a flat face flange. The flat face may be either the full tf  dimension thickness plus 0.06 in. or the tf

dimension thickness without the raised face height. See para. 6.3.2 for additional restrictions.

- The thickness of the flange dimension illustrated is for regularly furnished 0.06-in. raised face (except lapped); for thickness requirements of other facings, see Figure 6.

- The thickness of flange minimum dimensions for loose flanges, Table 9, size NPS 31∕2 and smaller are slightly heavier than for flanges on these

fittings that are reinforced by being cast integral with the body of fitting.

- For center-to-contact surface and center-to-end dimensioins of reducing fittings, see para. 6.2.3.

- For contact surface-to-contact surface and end-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

- These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For center-to-end dimensions of reducing fittings or end-to-end dimensions of reducers, use center-to-contact surface or contact surface-to-contact surface dimensions of 0.06-in. raised face (flange edge) for the largest opening, and add the proper height to provide for the ring joint groove applying to each flange. See Table 5C for ring joint facing dimensions.

- The base dimensions apply to all straight and reducing sizes.

- The reducing fittings, the size, and center-to-face dimension of base are determined by the size of the largest opening of fittings. In the case of reducing base elbows, orders shall specify whether the base shall be opposite the larger or smaller opening.

- Bases shall be plain faced unless otherwise specified, and the center-to-base dimension R shall be the finished dimension.

- Bases may be cast integral or attached as weldments at the option of the manufacturer.

- The bolt hole template for round base is the same as for Class 300 flanges (Table 11) of corresponding outside diameter, except using only four holes in all cases so placed as to straddle centerlines. The bases of these fittings are intended for support in compression and are not for anchors or supports in tension or shear.

158

ASME  B16.5-2020

###### Table 13 Templates for Drilling Class 400 Pipe Flanges	ð20Þ

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Drilling (2), (3) Length of Bolts, L (1), (4) |  |  |  |  |  |  |  |  |
| Nominal | Outside Diameter | Diameter of |  |  |  | Raised | Male and Female/ |  |
| Pipe | of Flange, | Bolt Circle, | Diameter of | Number | Diameter | Face | Tongue | Ring |
| Size, NPS | O | W | Bolt Holes | of Bolts | of Bolts | 0.25 in. | and Groove | Joint |
| 1∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 3∕4 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 1 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 11∕4 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 11∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 21∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 3 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 31∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 4 | 254 | 200.2 | 1 | 8 | 7∕8 | 140 | 135 | 140 |
| 5 | 279 | 235.0 | 1 | 8 | 7∕8 | 145 | 135 | 145 |
| 6 | 318 | 269.7 | 1 | 12 | 7∕8 | 150 | 145 | 150 |

| 8 | 381 | 330.2 | 11∕8 | 12 | 1 | 170 | 165 | 170 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 444 | 387.4 | 11∕4 | 16 | 11∕8 | 190 | 185 | 190 |
| 12 | 521 | 450.8 | 13∕8 | 16 | 11∕4 | 205 | 195 | 205 |
| 14 | 584 | 514.4 | 13∕8 | 20 | 11∕4 | 210 | 205 | 210 |
| 16 | 648 | 571.5 | 11∕2 | 20 | 13∕8 | 220 | 215 | 220 |
| 18 | 711 | 628.6 | 11∕2 | 24 | 13∕8 | 230 | 220 | 230 |
| 20 | 775 | 685.8 | 15∕8 | 24 | 11∕2 | 240 | 235 | 250 |
| 22 | 838 | 743.0 | 13∕4 | 24 | 15∕8 | 255 | 250 | 260 |
| 24 | 914 | 812.8 | 17∕8 | 24 | 13∕4 | 265 | 260 | 280 |

GENERAL NOTES:

- Dimensions are in millimeters, except for diameters of bolts and bolt holes, which are in inches.

- For other dimensions, see Table 14.

NOTES:

  - The length of the stud bolt does not include the height of the points. See para. 6.10.2.

  - For flange bolt holes, see para. 6.5.

  - For spot facing, see para. 6.6.

  - Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

  - Use Class 600 dimensions in this size.

159

ASME  B16.5-2020

###### Table 13C Templates for Drilling Class 400 Pipe Flanges

1	2	3	4	5	6	7	8	9

Drilling (2), (3)	Length of Bolts, L (1), (4)

| Nominal Pipe Size | Outside Diameter of Flange, O | Diameter of Bolt Circle, W | Diameter of Bolt Holes | Number of Bolts | Diameter of Bolts | Raised Face 0.25 in. | Male and Female/ Tongue and Groove | Ring Joint |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 3∕4 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 1 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 11∕4 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 11∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 21∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 3 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 31∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 4 | 10.00 | 7.88 | 1 | 8 | 7∕8 | 5.50 | 5.25 | 5.50 |
| 5 | 11.00 | 9.25 | 1 | 8 | 7∕8 | 5.75 | 5.25 | 5.75 |
| 6 | 12.50 | 10.62 | 1 | 12 | 7∕8 | 6.00 | 5.75 | 6.00 |

| 8 | 15.00 | 13.00 | 11∕8 | 12 | 1 | 6.75 | 6.50 | 6.75 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 17.50 | 15.25 | 11∕4 | 16 | 11∕8 | 7.50 | 7.25 | 7.50 |
| 12 | 20.50 | 17.75 | 13∕8 | 16 | 11∕4 | 8.00 | 7.75 | 8.00 |
| 14 | 23.00 | 20.25 | 13∕8 | 20 | 11∕4 | 8.25 | 8.00 | 8.25 |
| 16 | 25.50 | 22.50 | 11∕2 | 20 | 13∕8 | 8.75 | 8.50 | 8.75 |
| 18 | 28.00 | 24.75 | 11∕2 | 24 | 13∕8 | 9.00 | 8.75 | 9.00 |
| 20 | 30.50 | 27.00 | 15∕8 | 24 | 11∕2 | 9.50 | 9.25 | 9.75 |
| 22 | 33.00 | 29.25 | 13∕4 | 24 | 15∕8 | 10.00 | 9.75 | 10.25 |
| 24 | 36.00 | 32.00 | 17∕8 | 24 | 13∕4 | 10.50 | 10.25 | 11.00 |

GENERAL NOTES:

    - Dimensions are in inches.

    - For other dimensions, see Table 14C.

NOTES:

      - The length of the stud bolt does not include the height of the points (see para. 6.10.2).

      - For flange bolt holes, see para. 6.5.

      - For spot facing, see para. 6.6.

      - Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

      - Use Class 600 dimensions in this size.

160

ASME  B16.5-2020

161

###### Table 14 Dimensions of Class 400 Flanges	ð20Þ

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outside | Min. |  | Hub Diam. Beginning | Length | Through | Hub | Minimum Thread |  | Bore |  | Corner Bore | Minimum |
| Nom. | Diam. | Thickness | Diam. | of Chamfer |  |  |  | Length | Min. |  |  | Radius of | Counterbore |
| Pipe | of | of | of | Welding | Threaded/ |  | Welding | Threaded | Slip- | Min. | Welding | Lapped | Threaded |
| Size, | Flange, | Flange, | Hub, | Neck, | Slip-On, | Lapped, | Neck, | Flange, | On, | Lapped, | Neck, | Flange and | Flange, |
| NPS | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | Pipe, r | Q |
| 1∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 3∕4 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 1 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 11∕4 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 11∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 21∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |

ASME  B16.5-2020

162

###### Table 14 Dimensions of Class 400 Flanges (Cont’d)

| 1 | 2 | 3 | 4 | 5 | 6 7 8 |  |  | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outside | Min. |  | Hub Diam. Beginning | Length Through Hub |  |  | Minimum Thread |  | Bore |  | Corner Bore | Minimum |
| Nom. | Diam. | Thickness | Diam. | of Chamfer |  |  |  | Length | Min. |  |  | Radius of | Counterbore |
| Pipe | of | of | of | Welding | Threaded/ Welding |  |  | Threaded | Slip- | Min. | Welding | Lapped | Threaded |
| Size, | Flange, | Flange, | Hub, | Neck, | Slip-On, Lapped, Neck, |  |  | Flange, | On, | Lapped, | Neck, | Flange and | Flange, |
| NPS | O | tf | X | Ah (2) | Y Y Y |  |  | T (3) | B | B | B | Pipe, r | Q |
| 3 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 31∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 4 | 254 | 35.1 | 146 | 114.3 | 51 | 51 | 89 | 37 | 116.1 | 116.8 | (5) | 11 | 117.6 |
| 5 | 279 | 38.1 | 178 | 141.2 | 54 | 54 | 102 | 43 | 143.8 | 144.5 | (5) | 11 | 144.5 |
| 6 | 318 | 41.1 | 206 | 168.4 | 57 | 57 | 103 | 46 | 170.7 | 171.4 | (5) | 13 | 171.4 |
| 8 | 381 | 47.8 | 260 | 219.2 | 68 | 68 | 117 | 51 | 221.5 | 222.2 | (5) | 13 | 222.2 |
| 10 | 444 | 53.8 | 321 | 273.0 | 73 | 102 | 124 | 56 | 276.4 | 277.4 | (5) | 13 | 276.4 |
| 12 | 521 | 57.2 | 375 | 323.8 | 79 | 108 | 137 | 60 | 327.2 | 328.2 | (5) | 13 | 328.7 |
| 14 | 584 | 60.5 | 425 | 355.6 | 84 | 117 | 149 | 64 | 359.2 | 360.2 | (5) | 13 | 360.4 |
| 16 | 648 | 63.5 | 483 | 406.4 | 94 | 127 | 152 | 68 | 410.5 | 411.2 | (5) | 13 | 411.2 |
| 18 | 711 | 66.5 | 533 | 457.2 | 99 | 137 | 165 | 70 | 461.8 | 462.3 | (5) | 13 | 462.0 |
| 20 | 775 | 69.8 | 587 | 508.0 | 102 | 146 | 168 | 73 | 513.1 | 514.4 | (5) | 13 | 512.8 |
| 22 | 838 | 73.2 | 641 | 558.8 | 108 | 152 | 171 | … | 564.4 | 565.2 | (5) | 13 | … |
| 24 | 914 | 76.2 | 702 | 609.6 | 114 | 159 | 175 | 83 | 616.0 | 616.0 | (5) | 13 | 614.4 |

GENERAL NOTES:

        - Dimensions are in millimeters, except for the diameter of bolts and bolt holes, which are in inches.

        - For tolerances, see section 7.

        - For facings, see para. 6.4.

        - For flange bolt holes, see para. 6.5 and Table 13.

        - For spot facing, see para 6.6.

        - For reducing threaded and slip-on flanges, see Table 6.

        - Blind flanges may be made with or without hubs at the manufacturer’s option.

        - For reducing welding neck flanges, see para 6.8.

NOTES:

          - This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges. This dimension is defined as the diameter at the intersection between the hub taper and back face of the flange.

          - For welding end bevel, see para. 6.7.

          - For thread of threaded flanges, see para. 6.9.

          - Use Class 600 dimensions in this size.

          - To be specified by the purchaser.

ASME  B16.5-2020

163

###### Table 14C Dimensions of Class 400 Flanges

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Hub Diam. | Length | Through | Hub | Minimum |  | Bore |  | Corner Bore |  |
|  | Outside | Min. |  | Beginning |  |  |  | Thread |  |  |  | Radius of | Minimum |
|  | Diam. | Thickness | Diam. | of Chamfer |  |  |  | Length |  |  |  | Lapped | Counterbore |
| Nom. | of | of | of | Welding | Threaded |  | Welding | Threaded | Min. | Min. | Welding | Flange | Threaded |
| Pipe | Flange, | Flange, | Hub, | Neck, | Slip-On, | Lapped, | Neck, | Flange, | Slip-On, | Lapped, | Neck, | and Pipe, | Flange, |
| Size | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q |
| 1∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 3∕4 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 1 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 11∕4 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 11∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |

ASME  B16.5-2020

164

###### Table 14C Dimensions of Class 400 Flanges (Cont’d)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Hub Diam. | Length | Through | Hub | Minimum |  | Bore |  | Corner Bore |  |
|  | Outside | Min. |  | Beginning |  |  |  | Thread |  |  |  | Radius of | Minimum |
|  | Diam. | Thickness | Diam. | of Chamfer |  |  |  | Length |  |  |  | Lapped | Counterbore |
| Nom. | of | of | of | Welding | Threaded |  | Welding | Threaded | Min. | Min. | Welding | Flange | Threaded |
| Pipe | Flange, | Flange, | Hub, | Neck, | Slip-On, | Lapped, | Neck, | Flange, | Slip-On, | Lapped, | Neck, | and Pipe, | Flange, |
| Size | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q |
| 21∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 3 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 31∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 4 | 10.00 | 1.38 | 5.75 | 4.50 | 2.00 | 2.00 | 3.50 | 1.44 | 4.57 | 4.60 | (5) | 0.44 | 4.63 |
| 5 | 11.00 | 1.50 | 7.00 | 5.56 | 2.12 | 2.12 | 4.00 | 1.69 | 5.66 | 5.69 | (5) | 0.44 | 5.69 |
| 6 | 12.50 | 1.62 | 8.12 | 6.63 | 2.25 | 2.25 | 4.06 | 1.81 | 6.72 | 6.75 | (5) | 0.50 | 6.75 |
| 8 | 15.00 | 1.88 | 10.25 | 8.63 | 2.69 | 2.69 | 4.62 | 2.00 | 8.72 | 8.75 | (5) | 0.50 | 8.75 |
| 10 | 17.50 | 2.12 | 12.62 | 10.75 | 2.88 | 4.00 | 4.88 | 2.19 | 10.88 | 10.92 | (5) | 0.50 | 10.88 |
| 12 | 20.50 | 2.25 | 14.75 | 12.75 | 3.12 | 4.25 | 5.38 | 2.38 | 12.88 | 12.92 | (5) | 0.50 | 12.94 |
| 14 | 23.00 | 2.38 | 16.75 | 14.00 | 3.31 | 4.62 | 5.88 | 2.50 | 14.14 | 14.18 | (5) | 0.50 | 14.19 |
| 16 | 25.50 | 2.50 | 19.00 | 16.00 | 3.69 | 5.00 | 6.00 | 2.69 | 16.16 | 16.19 | (5) | 0.50 | 16.19 |
| 18 | 28.00 | 2.62 | 21.00 | 18.00 | 3.88 | 5.38 | 6.50 | 2.75 | 18.18 | 18.20 | (5) | 0.50 | 18.19 |
| 20 | 30.50 | 2.75 | 23.12 | 20.00 | 4.00 | 5.75 | 6.62 | 2.88 | 20.20 | 20.25 | (5) | 0.50 | 20.19 |
| 22 | 33.00 | 2.88 | 25.25 | 22.00 | 4.25 | 6.00 | 6.75 | … | 22.22 | 22.25 | (5) | 0.50 | … |
| 24 | 36.00 | 3.00 | 27.62 | 24.00 | 4.50 | 6.25 | 6.88 | 3.25 | 24.25 | 24.25 | (5) | 0.50 | 24.19 |

GENERAL NOTES:

            - Dimensions are in inches.

            - For tolerances, see section 7.

            - For facings, see para. 6.4.

            - For flange bolt holes, see para. 6.5 and Table 13C.

            - For spot facing, see para. 6.6.

            - For reducing threaded and slip-on flanges, see Table 6C.

            - Blind flanges may be made with or without hubs at the manufacturer’s option.

            - For reducing welding neck flanges, see para. 6.8.

NOTES:

              - This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, and lapped flanges.

              - For welding end bevel, see para. 6.7.

              - For thread in threaded flanges, see para. 6.9.

              - Use Class 600 dimensions in this size.

              - To be specified by the purchaser.

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Drilling (2), (3) Length of Bolts, L (1), (4) |  |  |  |  |  |  |  |  |
| Nominal Pipe | Outside Diameter | Diameter of Bolt | Diameter of Bolt | Number | Diameter | 6.4-mm | Male and Female/ |  |
| Size, | of Flange, | Circle, | Holes, | of | of Bolts, | Raised | Tongue | Ring |
| NPS | O | W | in. | Bolts | in. | Face | and Groove | Joint |

| 31∕2 | 229 | 184.2 | 1 | 8 |
| --- | --- | --- | --- | --- |
| 4 | 273 | 215.9 | 1 | 8 |

| 95 | 66.5 |
| --- | --- |
| 117 | 82.6 |
| 124 | 88.9 |
| 133 | 98.6 |
| 155 | 114.3 |

ASME  B16.5-2020

###### Table 15 Templates for Drilling Class 600 Pipe Flanges and Flanged Fittings	ð20Þ

| 1∕2 5∕8 4 1∕2 3∕4 3∕4 4 5∕8 1 3∕4 4 5∕8 11∕4 3∕4 4 5∕8 11∕2 7∕8 4 3∕4 2 165 127.0 3∕4 8 5∕8 21∕2 190 149.4 7∕8 8 3∕4 3 210 168.1 7∕8 8 3∕4 7∕8 7∕8 |  |  |  |  |  | 75 | 70 | 75 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 90 | 85 | 90 |
|  |  |  |  |  |  | 90 | 85 | 90 |
|  |  |  |  |  |  | 95 | 90 | 95 |
|  |  |  |  |  |  | 110 | 100 | 110 |
|  |  |  |  |  |  | 110 | 100 | 110 |
|  |  |  |  |  |  | 120 | 115 | 120 |
|  |  |  |  |  |  | 125 | 120 | 125 |
|  |  |  |  |  |  | 140 | 135 | 140 |
|  |  |  |  |  |  | 145 | 140 | 145 |
| 5 | 330 | 266.7 | 11∕8 | 8 | 1 | 165 | 160 | 165 |
| 6 | 356 | 292.1 | 11∕8 | 12 | 1 | 170 | 165 | 170 |
| 8 | 419 | 349.2 | 11∕4 | 12 | 11∕8 | 190 | 185 | 195 |
| 10 | 508 | 431.8 | 13∕8 | 16 | 11∕4 | 215 | 210 | 215 |
| 12 | 559 | 489.0 | 13∕8 | 20 | 11∕4 | 220 | 215 | 220 |
| 14 | 603 | 527.0 | 11∕2 | 20 | 13∕8 | 235 | 230 | 235 |
| 16 | 686 | 603.2 | 15∕8 | 20 | 11∕2 | 255 | 250 | 255 |
| 18 | 743 | 654.0 | 13∕4 | 20 | 15∕8 | 275 | 265 | 275 |
| 20 | 813 | 723.9 | 13∕4 | 24 | 15∕8 | 285 | 280 | 290 |
| 22 | 870 | 777.7 | 17∕8 | 24 | 13∕4 | 305 | … | 310 |
| 24 | 940 | 838.2 | 2 | 24 | 17∕8 | 330 | 325 | 335 |

GENERAL NOTES:

                - Dimensions are in millimeters, except for the diameters of the bolts and bolt holes, which are expressed in inches.

                - For other dimensions, see Table 16.

NOTES:

(1) The length of the stud bolt does not include the height of the points (see para 6.10.2).

(2)  For flange bolt holes, see para. 6.5.

(3)  For spot facing, see para 6.6.

(4) Bolt lengths not shown in the table may be in accordance with Nonmandatory Appendix C (see para. 6.10.2).

165

| 31∕2 | 9.00 | 7.25 | 1 | 8 |
| --- | --- | --- | --- | --- |
| 4 | 10.75 | 8.50 | 1 | 8 |

ASME  B16.5-2020

###### Table 15C Templates for Drilling Class 600 Pipe Flanges and Flanged Fittings

|  | Outside | Drilling (2), (3) Length of Bolts, L (1), (4) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Diameter | Diameter of |  |  |  | Raised |  |  |
| Nominal | of Flange, | Bolt Circle, | Diameter of | Number of | Diameter of | Face | Male and Female/ | Ring |
| Pipe Size | O | W | Bolt Holes | Bolts | Bolts | 0.25 in. | Tongue and Groove | Joint |

1	2	3	4	5	6	7	8	9

| 1∕2 3.75 2.62 5∕8 4 1∕2 3∕4 4.62 3.25 3∕4 4 5∕8 1 4.88 3.50 3∕4 4 5∕8 11∕4 5.25 3.88 3∕4 4 5∕8 11∕2 6.12 4.50 7∕8 4 3∕4 2 6.50 5.00 3∕4 8 5∕8 21∕2 7.50 5.88 7∕8 8 3∕4 3 8.25 6.62 7∕8 8 3∕4 7∕8 7∕8 |  |  |  |  |  | 3.00 | 2.75 | 3.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 3.50 | 3.25 | 3.50 |
|  |  |  |  |  |  | 3.50 | 3.25 | 3.50 |
|  |  |  |  |  |  | 3.75 | 3.50 | 3.75 |
|  |  |  |  |  |  | 4.25 | 4.00 | 4.25 |
|  |  |  |  |  |  | 4.25 | 4.00 | 4.25 |
|  |  |  |  |  |  | 4.75 | 4.50 | 4.75 |
|  |  |  |  |  |  | 5.00 | 4.75 | 5.00 |
|  |  |  |  |  |  | 5.50 | 5.25 | 5.50 |
|  |  |  |  |  |  | 5.75 | 5.50 | 5.75 |
| 5 | 13.00 | 10.50 | 11∕8 | 8 | 1 | 6.50 | 6.25 | 6.50 |
| 6 | 14.00 | 11.50 | 11∕8 | 12 | 1 | 6.75 | 6.50 | 6.75 |
| 8 | 16.50 | 13.75 | 11∕4 | 12 | 11∕8 | 7.50 | 7.25 | 7.75 |
| 10 | 20.00 | 17.00 | 13∕8 | 16 | 11∕4 | 8.50 | 8.25 | 8.50 |
| 12 | 22.00 | 19.25 | 13∕8 | 20 | 11∕4 | 8.75 | 8.50 | 8.75 |
| 14 | 23.75 | 20.75 | 11∕2 | 20 | 13∕8 | 9.25 | 9.00 | 9.25 |
| 16 | 27.00 | 23.75 | 15∕8 | 20 | 11∕2 | 10.00 | 9.75 | 10.00 |
| 18 | 29.25 | 25.75 | 13∕4 | 20 | 15∕8 | 10.75 | 10.50 | 10.75 |
| 20 | 32.00 | 28.50 | 13∕4 | 24 | 15∕8 | 11.25 | 11.00 | 11.50 |
| 22 | 34.25 | 30.62 | 17∕8 | 24 | 13∕4 | 12.00 | … | 12.25 |
| 24 | 37.00 | 33.00 | 2 | 24 | 17∕8 | 13.00 | 12.75 | 13.25 |

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For other dimensions, see Table 16C.

NOTES:

(1) The length of the stud bolt does not include the height of the points (see para. 6.10.2).

(2)  For flange bolt holes, see para. 6.5.

(3)  For spot facing, see para. 6.6.

(4) Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

166

167

###### Table 16 Dimensions of Class 600 Flanges	ð20Þ

ASME  B16.5-2020

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

|  |  |  |  | Hub | Length Through Hub Bore Corner |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Diameter |  |  |  | Minimum |  |  |  | Bore |  |  |
|  | Outside |  |  | Beginning of | Threaded/ |  |  | Thread | Minimum |  | Welding | Radius of | Minimum |  |
| Nominal | Diameter | Minimum |  | Chamfer | Slip-On/ |  |  | Length | Slip-On/ |  | Neck/ | Lapped | Counterbore |  |
| Pipe | of | Thickness | Diameter | Welding | Socket |  | Welding | Threaded | Socket | Minimum | Socket | Flange and | Threaded | Depth of |
| Size, | Flange, | of Flange, | of Hub, | Neck, | Welding, | Lapped, | Neck, | Flange, | Welding, | Lapped, | Welding, | Pipe, | Flange, | Socket, |
| NPS | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q | D |
| 1∕2 | 95 | 14.2 | 38 | 21.3 | 22 | 22 | 52 | 16 | 22.4 | 22.9 | (4) | 3 | 23.6 | 10 |
| 3∕4 | 117 | 15.7 | 48 | 26.7 | 25 | 25 | 57 | 16 | 27.7 | 28.2 | (4) | 3 | 29.0 | 11 |
| 1 | 124 | 17.5 | 54 | 33.5 | 27 | 27 | 62 | 18 | 34.5 | 35.1 | (4) | 3 | 35.8 | 13 |
| 11∕4 | 133 | 20.6 | 64 | 42.2 | 28 | 28 | 67 | 21 | 43.2 | 43.7 | (4) | 5 | 44.4 | 14 |
| 11∕2 | 155 | 22.4 | 70 | 48.3 | 32 | 32 | 70 | 22 | 49.5 | 50.0 | (4) | 6 | 50.5 | 16 |

ASME  B16.5-2020

168

| Table 16 Dimensions of Class 600 Flanges (Cont’d) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Hub Length Through Hub Bore Corner Diameter Minimum Bore |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Outside |  |  | Beginning of | Threaded/ |  |  | Thread | Minimum |  | Welding | Radius of | Minimum |  |
| Nominal | Diameter | Minimum |  | Chamfer | Slip-On/ |  |  | Length | Slip-On/ |  | Neck/ | Lapped | Counterbore |  |
| Pipe | of | Thickness | Diameter | Welding | Socket |  | Welding | Threaded | Socket | Minimum | Socket | Flange and | Threaded | Depth of |
| Size, | Flange, | of Flange, | of Hub, | Neck, | Welding, | Lapped, | Neck, | Flange, | Welding, | Lapped, | Welding, | Pipe, | Flange, | Socket, |
| NPS | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q | D |
| 2 | 165 | 25.4 | 84 | 60.5 | 37 | 37 | 73 | 28 | 62.0 | 62.5 | (4) | 8 | 63.5 | 18 |
| 21∕2 | 190 | 28.4 | 100 | 73.2 | 41 | 41 | 79 | 32 | 74.7 | 75.4 | (4) | 8 | 76.2 | 19 |
| 3 | 210 | 31.8 | 117 | 88.9 | 46 | 46 | 83 | 35 | 90.7 | 91.4 | (4) | 10 | 92.2 | 21 |
| 31∕2 | 229 | 35.1 | 133 | 101.6 | 49 | 49 | 86 | 40 | 103.4 | 104.1 | (4) | 10 | 104.9 | … |
| 4 | 273 | 38.1 | 152 | 114.3 | 54 | 54 | 102 | 41 | 116.1 | 116.8 | (4) | 11 | 117.6 | … |
| 5 | 330 | 44.4 | 189 | 141.2 | 60 | 60 | 114 | 48 | 143.8 | 144.5 | (4) | 11 | 144.5 | … |
| 6 | 356 | 47.8 | 222 | 168.4 | 67 | 67 | 117 | 51 | 170.7 | 171.4 | (4) | 13 | 171.4 | … |
| 8 | 419 | 55.6 | 273 | 219.2 | 76 | 76 | 133 | 57 | 221.5 | 222.2 | (4) | 13 | 222.2 | … |
| 10 | 508 | 63.5 | 343 | 273.0 | 86 | 111 | 152 | 65 | 276.2 | 277.4 | (4) | 13 | 276.4 | … |
| 12 | 559 | 66.5 | 400 | 323.8 | 92 | 117 | 155 | 70 | 327.0 | 328.2 | (4) | 13 | 328.7 | … |
| 14 | 603 | 69.8 | 432 | 355.6 | 94 | 127 | 165 | 73 | 359.2 | 360.2 | (4) | 13 | 360.4 | … |
| 16 | 686 | 76.2 | 495 | 406.4 | 106 | 140 | 178 | 78 | 410.5 | 411.2 | (4) | 13 | 411.2 | … |
| 18 | 743 | 82.6 | 546 | 457.2 | 117 | 152 | 184 | 79 | 461.8 | 462.3 | (4) | 13 | 462.0 | … |
| 20 | 813 | 88.9 | 610 | 508.0 | 127 | 165 | 190 | 83 | 513.1 | 514.4 | (4) | 13 | 512.8 | … |
| 22 | 870 | 95.2 | 663 | 558.8 | 133 | 175 | 197 | … | 564.4 | 565.2 | (4) | 13 | … | … |
| 24 | 940 | 101.6 | 718 | 609.6 | 140 | 184 | 203 | 92 | 616.0 | 616.0 | (4) | 13 | 614.4 | … |

GENERAL NOTES:

(a)  Dimensions are in millimeters.

(b)  For tolerance, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 15C.

(e)  For spot facing, see para. 6.6.

(f)  For reducing threaded and slip-on flanges, see Table 6.

(g)  Blind flanges may be made with or without hubs at the manufacturer’s option.

(h) For reducing welding neck flanges, see para. 6.8.

NOTES:

(1)  This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges. This dimension is defined as the diameter at the intersection between the hub taper and back face of the flange.

(2) For welding end bevel, see para. 6.7.

(3)  For thread of threaded flanges, see para. 6.9.

(4)  To be specified by the purchaser.

ASME  B16.5-2020

169

###### Table 16C Dimensions of Class 600 Flanges

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

|  |  |  |  | Hub | Length Through Hub Bore Corner |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Diameter |  |  |  | Minimum |  |  |  | Bore | Minimum |  |
|  | Outside |  |  | Beginning of | Threaded |  |  | Thread | Minimum |  | Welding | Radius of | Counter- |  |
|  | Diameter | Minimum |  | Chamfer | Slip-On |  |  | Length | Slip-On/ |  | Neck/ | Lapped | bore |  |
| Nominal | of | Thickness | Diameter | Welding | Socket |  | Welding | Threaded | Socket | Minimum | Socket | Flange and | Threaded | Depth of |
| Pipe | Flange, | of Flange, | of Hub, | Neck, | Welding, | Lapped, | Neck, | Flange, | Welding, | Lapped, | Welding, | Pipe, | Flange, | Socket, |
| Size | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q | D |
| 1∕2 | 3.75 | 0.56 | 1.50 | 0.84 | 0.88 | 0.88 | 2.06 | 0.62 | 0.88 | 0.90 | (4) | 0.12 | 0.93 | 0.38 |
| 3∕4 | 4.62 | 0.62 | 1.88 | 1.05 | 1.00 | 1.00 | 2.25 | 0.62 | 1.09 | 1.11 | (4) | 0.12 | 1.14 | 0.44 |
| 1 | 4.88 | 0.69 | 2.12 | 1.32 | 1.06 | 1.06 | 2.44 | 0.69 | 1.36 | 1.38 | (4) | 0.12 | 1.41 | 0.50 |
| 11∕4 | 5.25 | 0.81 | 2.50 | 1.66 | 1.12 | 1.12 | 2.62 | 0.81 | 1.70 | 1.72 | (4) | 0.19 | 1.75 | 0.56 |
| 11∕2 | 6.12 | 0.88 | 2.75 | 1.90 | 1.25 | 1.25 | 2.75 | 0.88 | 1.95 | 1.97 | (4) | 0.25 | 1.99 | 0.62 |

ASME  B16.5-2020

170

###### Table 16C Dimensions of Class 600 Flanges (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

|  |  |  |  | Hub Diameter | Length Through Hub |  |  |  | Minimum |  | Bore |  | Corner Bore | Minimum |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outside |  |  | Beginning of | Threaded |  |  |  | Thread | Minimum |  | Welding | Radius of | Counter- |  |
|  | Diameter | Minimum |  | Chamfer | Slip-On |  |  |  | Length | Slip-On/ |  | Neck/ | Lapped | bore |  |
| Nominal | of | Thickness | Diameter | Welding | Socket |  | Welding |  | Threaded | Socket | Minimum | Socket | Flange and | Threaded | Depth of |
| Pipe | Flange, | of Flange, | of Hub, | Neck, | Welding, | Lapped, | Neck, |  | Flange, | Welding, | Lapped, | Welding, | Pipe, | Flange, | Socket, |
| Size | O | tf | X | Ah (2) | Y | Y | Y |  | T (3) | B | B | B | r | Q | D |
| 2 | 6.50 | 1.00 | 3.31 | 2.38 | 1.44 | 1.44 | 2.88 |  | 1.12 | 2.44 | 2.46 | (4) | 0.31 | 2.50 | 0.69 |
| 21∕2 | 7.50 | 1.12 | 3.94 | 2.88 | 1.62 | 1.62 | 3.12 |  | 1.25 | 2.94 | 2.97 | (4) | 0.31 | 3.00 | 0.75 |
| 3 | 8.25 | 1.25 | 4.62 | 3.50 | 1.81 | 1.81 | 3.25 |  | 1.38 | 3.57 | 3.60 | (4) | 0.38 | 3.63 | 0.81 |
| 31∕2 | 9.00 | 1.38 | 5.25 | 4.00 | 1.94 | 1.94 | 3.38 |  | 1.56 | 4.07 | 4.10 | (4) | 0.38 | 4.13 | … |
| 4 | 10.75 | 1.50 | 6.00 | 4.50 | 2.12 | 2.12 | 4.00 |  | 1.62 | 4.57 | 4.60 | (4) | 0.44 | 4.63 | … |
| 5 | 13.00 | 1.75 | 7.44 | 5.56 | 2.38 | 2.38 | 4.50 |  | 1.88 | 5.66 | 5.69 | (4) | 0.44 | 5.69 | … |
| 6 | 14.00 | 1.88 | 8.75 | 6.63 | 2.62 | 2.62 | 4.62 |  | 2.00 | 6.72 | 6.75 | (4) | 0.50 | 6.75 | … |
| 8 | 16.50 | 2.19 | 10.75 | 8.63 | 3.00 | 3.00 | 5.25 |  | 2.25 | 8.72 | 8.75 | (4) | 0.50 | 8.75 | … |
| 10 | 20.00 | 2.50 | 13.50 | 10.75 | 3.38 | 4.38 | 6.00 |  | 2.56 | 10.88 | 10.92 | (4) | 0.50 | 10.88 | … |
| 12 | 22.00 | 2.62 | 15.75 | 12.75 | 3.62 | 4.62 | 6.12 |  | 2.75 | 12.88 | 12.92 | (4) | 0.50 | 12.94 | … |
| 14 | 23.75 | 2.75 | 17.00 | 14.00 | 3.69 | 5.00 | 6.50 |  | 2.88 | 14.14 | 14.18 | (4) | 0.50 | 14.19 | … |
| 16 | 27.00 | 3.00 | 19.50 | 16.00 | 4.19 | 5.50 | 7.00 |  | 3.06 | 16.16 | 16.19 | (4) | 0.50 | 16.19 | … |
| 18 | 29.25 | 3.25 | 21.50 | 18.00 | 4.62 | 6.00 | 7.25 |  | 3.12 | 18.18 | 18.20 | (4) | 0.50 | 18.19 | … |
| 20 | 32.00 | 3.50 | 24.00 | 20.00 | 5.00 | 6.50 | 7.50 |  | 3.25 | 20.20 | 20.25 | (4) | 0.50 | 20.19 | … |
| 22 | 34.25 | 3.75 | 26.25 | 22.00 | 5.25 | 6.88 | 7.75 |  | … | 22.22 | 22.25 | (4) | 0.50 | … | … |
| 24 | 37.00 | 4.00 | 28.25 | 24.00 | 5.50 | 7.25 | 8.00 |  | 3.62 | 24.25 | 24.25 | (4) | 0.50 | 24.19 | … |

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For tolerances, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 15C.

(e)  For spot facing, see para. 6.6.

(f)  For reducing threaded and slip-on flanges, see Table 6C.

(g)  Blind flanges may be made with or without hubs at the manufacturer’s option.

(h) For reducing welding neck flanges, see para. 6.8.

NOTES:

(1)  This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges.

(2) For welding end bevel, see para. 6.7.

(3) For threads in threaded flanges, see para. 6.9.

(4)  To be specified by the purchaser.

ASME  B16.5-2020

###### Table 17 Templates for Drilling Class 900 Pipe Flanges and Flanged Fittings	ð20Þ

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Drilling (2), (3) Length of Bolts, L (1), (4) |  |  |  |  |  |  |  |  |
| Nominal Pipe | Outside Diameter | Diameter of | Diameter of |  |  | 6.4-mm | Male and Female/ |  |
| Size, | of Flange, | Bolt Circle, | Bolt Holes, | Number of | Diameter of | Raised | Tongue and | Ring |
| NPS | O | W | in. | Bolts | Bolts, in. | Face | Groove | Joint |
| 1∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 3∕4 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 1 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |

| 11∕4 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 21∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |

3	241	190.5	1	8	7∕8	145	140	145

4	292	235.0	11∕4	8	11∕8	170	165	170

5	349	279.4	13∕8	8	11∕4	190	185	190

6	381	317.5	11∕4	12	11∕8	190	185	195

8	470	393.7	11∕2	12	13∕8	220	215	220

10	546	469.9	11∕2	16	13∕8	235	230	235

12	610	533.4	11∕2	20	13∕8	255	250	255

14	641	558.8	15∕8	20	11∕2	275	265	280

16	705	616.0	13∕4	20	15∕8	285	280	290

18	787	685.8	2	20	17∕8	325	320	335

20	857	749.3	21∕8	20	2	350	345	360

24	1,041	901.7	25∕8	20	21∕2	440	430	455

GENERAL NOTES:

(a) Dimensions are in millimeters, except for diameters of bolts and bolt holes, which are in inches.

(b)  For other dimensions, see Tables 18 and 19.

NOTES:

(1) The length of the stud bolt does not include the height of the points (see para 6.10.2).

(2)  For flange bolt holes, see para. 6.5.

(3)  For spot facing, see para. 6.6.

(4) Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

(5)  Use Class 1500 dimensions in this size.

171

ASME  B16.5-2020

###### Table 17C Templates for Drilling Class 900 Pipe Flanges and Flanged Fittings

1	2	3	4	5	6	7	8	9

|  | Outside | Drilling (2), (3) Length of Bolts, L (1), (4) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Diameter of | Diameter of |  |  |  | Raised | Male and Female/ |  |
| Nominal | Flange, | Bolt Circle, | Diameter of | Number of | Diameter of | Face | Tongue and | Ring |
| Pipe Size | O | W | Bolt Holes | Bolts | Bolts | 0.25 in. | Groove | Joint |
| 1∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 3∕4 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 1 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |

| 11∕4 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |
| 21∕2 | (5) | (5) | (5) | (5) | (5) | (5) | (5) | (5) |

3	9.50	7.50	1	8	7∕8	5.75	5.50	5.75

4	11.50	9.25	11∕4	8	11∕8	6.75	6.50	6.75

5	13.75	11.00	13∕8	8	11∕4	7.50	7.25	7.50

6	15.00	12.50	11∕4	12	11∕8	7.50	7.25	7.75

8	18.50	15.50	11∕2	12	13∕8	8.75	8.50	8.75

10	21.50	18.50	11∕2	16	13∕8	9.25	9.00	9.25

12	24.00	21.00	11∕2	20	13∕8	10.00	9.75	10.00

14	25.25	22.00	15∕8	20	11∕2	10.75	10.50	11.00

16	27.75	24.25	13∕4	20	15∕8	11.25	11.00	11.50

18	31.00	27.00	2	20	17∕8	12.75	12.50	13.25

20	33.75	29.50	21∕8	20	2	13.75	13.50	14.25

24	41.00	35.50	25∕8	20	21∕2	17.25	17.00	18.00

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For other dimensions, see Table 18C and Table 19C.

NOTES:

(1) The length of the stud bolt does not include the height of the points (see para. 6.10.2).

(2)  For flange bolt holes, see para. 6.5.

(3)  For spot facing, see para. 6.6.

(4) Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

(5)  Use Class 1500 dimensions in this size.

172

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length Through Hub Bore Corner Min. Hub Diam. Minimum Bore Minimum |  |  |  |  |  |  |  |  |  |  |  |  |  |

ASME  B16.5-2020

###### Table 18 Dimensions of Class 900 Flanges	ð20Þ

| Nom. Pipe Size, NPS | Outside Diam. of Flange, O | Thick- ness of Flange, tf | Diam. of Hub, X | Beginning of Chamfer Welding Neck, Ah (2) | Threaded/ Slip-On, Y | Lapped, Y | Welding Neck, Y | Thread Length Threaded Flange, T (3) | Min. Slip-On, B | Min. Lapped, B | Welding Neck, B | Radius of Lapped Flange and Pipe, r | Counter- bore Threaded Flange, Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 3∕4 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 1 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 11∕4 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 11∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 21∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 3 | 241 | 38.1 | 127 | 88.9 | 54 | 54 | 102 | 41 | 90.7 | 91.4 | (5) | 10 | 92.2 |
| 4 | 292 | 44.5 | 159 | 114.3 | 70 | 70 | 114 | 48 | 116.1 | 116.8 | (5) | 11 | 117.6 |
| 5 | 349 | 50.8 | 190 | 141.2 | 79 | 79 | 127 | 54 | 143.8 | 144.5 | (5) | 11 | 144.5 |
| 6 | 381 | 55.6 | 235 | 168.4 | 86 | 86 | 140 | 57 | 170.7 | 171.4 | (5) | 13 | 171.4 |
| 8 | 470 | 63.5 | 298 | 219.2 | 102 | 114 | 162 | 64 | 221.5 | 222.2 | (5) | 13 | 222.2 |
| 10 | 546 | 69.8 | 368 | 273.0 | 108 | 127 | 184 | 71 | 276.4 | 277.4 | (5) | 13 | 276.4 |
| 12 | 610 | 79.2 | 419 | 323.8 | 117 | 143 | 200 | 76 | 327.2 | 328.2 | (5) | 13 | 328.7 |
| 14 | 641 | 85.9 | 451 | 355.6 | 130 | 155 | 213 | 83 | 359.2 | 360.2 | (5) | 13 | 360.4 |

173

ASME  B16.5-2020

###### Table 18 Dimensions of Class 900 Flanges (Cont’d)

| Hub Diam. |  |  |  | Minimum |  |  |  | Bore | Minimum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beginning |  |  |  | Thread |  |  |  | Radius of | Counter- |
| of Chamfer |  |  |  | Length |  |  |  | Lapped | bore |
| Welding | Threaded/ |  | Welding | Threaded | Min. | Min. | Welding | Flange | Threaded |
| Neck, | Slip-On, | Lapped, | Neck, | Flange, T | Slip-On, | Lapped, | Neck, | and Pipe, | Flange, |
| Ah (2) | Y | Y | Y | (3) | B | B | B | r | Q |
| 406.4 | 133 | 165 | 216 | 86 | 410.5 | 411.2 | (5) | 13 | 411.2 |
| 457.2 | 152 | 190 | 229 | 89 | 461.8 | 462.3 | (5) | 13 | 462.0 |
| 508.0 | 159 | 210 | 248 | 92 | 513.1 | 514.4 | (5) | 13 | 512.8 |
| 609.6 | 203 | 267 | 292 | 102 | 616.0 | 616.0 | (5) | 13 | 614.4 |

1	2	3	4	5	6	7	8	9	10	11	12	13	14

Nom. Pipe Size, NPS

Outside Diam. of Flange, O

Min. Thick- ness of

Flange,

tf

Diam. of Hub, X

Length Through Hub

Bore	Corner

16	705	88.9	508

18	787	101.6	565

20	857	108.0	622

24	1,041	139.7	749

GENERAL NOTES:

(a)  Dimensions are in millimeters.

(b)  For tolerances, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 17.

(e)  For spot facing, see para 6.6.

(f)  For reducing threaded and slip-on flanges, see Table 6.

(g)  Blind flanges may be made with or without hubs at the manufacture’s option.

(h) For reducing welding neck flanges, see para. 6.8.

NOTES:

(1) This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket- welding, and lapped flanges. This dimension is defined as the diameter at the intersection between the hub taper and back face of the flange.

(2) For welding end bevel, see para. 6.7.

(3)  For thread of threaded flanges, see para. 6.9.

(4)  Use Class 1500 dimensions in this size.

(5)  To be specified by the purchaser.

174

ASME  B16.5-2020

###### Table 18C Dimensions of Class 900 Flanges

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| Nom. Pipe Size | Outside Diamet- er of Flange, O | Minim- um Thick- ness of Flange, tf | Diam. of Hub, X | Hub Diameter Beginning of Chamfer Welding Neck, Ah (2) | Length Through Threaded Slip-On, Lapped, Y Y |  | Hub Welding Neck, Y | Minimum Thread Length Threaded Flange, T (3) | Min. Slip- On, B | Bore Min. Lapped, B | Welding Neck, B | Corner Bore Radius of Lapped Flange and Pipe, r | Minimum Counter- bore Threaded Flange, Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 3∕4 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 1 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 11∕4 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 11∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 21∕2 | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) | (4) |
| 3 | 9.50 | 1.50 | 5.00 | 3.50 | 2.12 | 2.12 | 4.00 | 1.62 | 3.57 | 3.60 | (5) | 0.38 | 3.63 |
| 4 | 11.50 | 1.75 | 6.25 | 4.50 | 2.75 | 2.75 | 4.50 | 1.88 | 4.57 | 4.60 | (5) | 0.44 | 4.63 |
| 5 | 13.75 | 2.00 | 7.50 | 5.56 | 3.12 | 3.12 | 5.00 | 2.12 | 5.66 | 5.69 | (5) | 0.44 | 5.69 |
| 6 | 15.00 | 2.19 | 9.25 | 6.63 | 3.38 | 3.38 | 5.50 | 2.25 | 6.72 | 6.75 | (5) | 0.50 | 6.75 |
| 8 | 18.50 | 2.50 | 11.75 | 8.63 | 4.00 | 4.50 | 6.38 | 2.50 | 8.72 | 8.75 | (5) | 0.50 | 8.75 |
| 10 | 21.50 | 2.75 | 14.50 | 10.75 | 4.25 | 5.00 | 7.25 | 2.81 | 10.88 | 10.92 | (5) | 0.50 | 10.88 |
| 12 | 24.00 | 3.12 | 16.50 | 12.75 | 4.62 | 5.62 | 7.88 | 3.00 | 12.88 | 12.92 | (5) | 0.50 | 12.94 |
| 14 | 25.25 | 3.38 | 17.75 | 14.00 | 5.12 | 6.12 | 8.38 | 3.25 | 14.14 | 14.18 | (5) | 0.50 | 14.19 |

175

ASME  B16.5-2020

###### Table 18C Dimensions of Class 900 Flanges (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14

|  |  |  |  | Hub | Length Through Hub Bore |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Minim- |  | Diameter |  |  |  |  |  |  |  | Corner |  |
|  | Outside | um |  | Beginning |  |  |  | Minimum |  |  |  | Bore | Minimum |
|  | Diamet- | Thick- |  | of |  |  |  | Thread |  |  |  | Radius of | Counter- |
|  | er | ness | Diam. | Chamfer |  |  |  | Length | Min. |  |  | Lapped | bore |
| Nom. | of | of | of | Welding | Threaded |  | Welding | Threaded | Slip- | Min. | Welding | Flange and | Threaded |
| Pipe | Flange, | Flange, | Hub, | Neck, | Slip-On, | Lapped, | Neck, | Flange, | On, | Lapped, | Neck, | Pipe, | Flange, |
| Size | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q |
| 16 | 27.75 | 3.50 | 20.00 | 16.00 | 5.25 | 6.50 | 8.50 | 3.38 | 16.16 | 16.19 | (5) | 0.50 | 16.19 |
| 18 | 31.00 | 4.00 | 22.25 | 18.00 | 6.00 | 7.50 | 9.00 | 3.50 | 18.18 | 18.20 | (5) | 0.50 | 18.19 |
| 20 | 33.75 | 4.25 | 24.50 | 20.00 | 6.25 | 8.25 | 9.75 | 3.62 | 20.20 | 20.25 | (5) | 0.50 | 20.19 |
| 24 | 41.00 | 5.50 | 29.50 | 24.00 | 8.00 | 10.50 | 11.50 | 4.00 | 24.25 | 24.25 | (5) | 0.50 | 24.19 |

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For tolerances, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 17C.

(e)  For spot facing, see para. 6.6.

(f)  For reducing threaded and slip-on flanges, see Table 6C.

(g)  Blind flanges may be made with or without hubs at the manufacturer’s option.

(h) For reducing welding neck flanges, see para. 6.8.

NOTES:

(1) This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket- welding, and lapped flanges.

(2) For welding end bevel, see para. 6.7.

(3) For threads in threaded flanges, see para. 6.9.

(4)  Use Class 1500 dimensions in this size.

(5)  To be specified by the purchaser.

176

ASME  B16.5-2020

###### Table 19 Templates for Drilling Class 1500 Pipe Flanges	ð20Þ

1	2	3	4	5	6	7	8	9

Outside

Drilling (2), (3)	Length of Bolts, L (1), (4)

| 121 | 82.6 |
| --- | --- |
| 130 | 88.9 |

Nominal Pipe Size, NPS

Diameter of Flange, O

Diameter of Bolt Circle, W

Diameter of Bolt Holes, in.

Number of Bolts

Diameter of Bolts,

in.

6.4-mm Raised Face

Male and Female/

Tongue and Groove    Ring Joint

1∕2

| 1 | 149 | 101.6 | 1 | 4 |
| --- | --- | --- | --- | --- |
| 11∕4 | 159 | 111.1 | 1 | 4 |

3∕4

7∕8	4	3∕4

| 110 | 100 | 110 |
| --- | --- | --- |
| 115 | 110 | 115 |
| 125 | 120 | 125 |
| 125 | 120 | 125 |

7∕8	4	3∕4

7∕8

7∕8

| 11∕2 | 178 | 124.0 | 11∕8 | 4 | 1 | 140 | 135 | 140 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 216 | 165.1 | 1 | 8 | 7∕8 | 145 | 140 | 145 |
| 21∕2 | 244 | 190.5 | 11∕8 | 8 | 1 | 160 | 150 | 160 |
| 3 | 267 | 203.2 | 11∕4 | 8 | 11∕8 | 180 | 170 | 180 |
| 4 | 311 | 241.3 | 13∕8 | 8 | 11∕4 | 195 | 190 | 195 |
| 5 | 375 | 292.1 | 15∕8 | 8 | 11∕2 | 250 | 240 | 250 |
| 6 | 394 | 317.5 | 11∕2 | 12 | 13∕8 | 260 | 255 | 265 |
| 8 | 483 | 393.7 | 13∕4 | 12 | 15∕8 | 290 | 285 | 300 |
| 10 | 584 | 482.6 | 2 | 12 | 17∕8 | 335 | 330 | 345 |
| 12 | 673 | 571.5 | 21∕8 | 16 | 2 | 375 | 370 | 385 |
| 14 | 749 | 635.0 | 23∕8 | 16 | 21∕4 | 405 | 400 | 425 |
| 16 | 826 | 704.8 | 25∕8 | 16 | 21∕2 | 445 | 440 | 470 |
| 18 | 914 | 774.7 | 27∕8 | 16 | 23∕4 | 495 | 490 | 525 |
| 20 | 984 | 831.8 | 31∕8 | 16 | 3 | 540 | 535 | 565 |
| 24 | 1 168 | 990.6 | 35∕8 | 16 | 31∕2 | 615 | 610 | 650 |

GENERAL NOTES:

(a) Dimensions are in millimeters, except for the diameters of the bolts and bolt holes, which are in inches.

(b)  For other dimensions, see Table 20.

NOTES:

(1) The length of the stud bolt does not include the height of the points (see para. 6.10.2).

(2)  For flange bolt holes, see para. 6.5.

(3)  For spot facing, see para. 6.6.

(4) Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

177

ASME  B16.5-2020

###### Table 19C Templates for Drilling Class 1500 Pipe Flanges

1	2	3	4	5	6	7	8	9

|  | Outside | Drilling (2), (3) Length of Bolts, L (1), (4) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Diameter of | Diameter of |  |  |  | Raised | Male and Female/ |  |
| Nominal | Flange, | Bolt Circle, | Diameter of | Number of | Diameter of | Face | Tongue and | Ring |
| Pipe Size | O | W | Bolt Holes | Bolts | Bolts | 0.25 in. | Groove | Joint |

| 4.75 | 3.25 |
| --- | --- |
| 5.12 | 3.50 |

1∕2

| 1 | 5.88 | 4.00 | 1 | 4 |
| --- | --- | --- | --- | --- |
| 11∕4 | 6.25 | 4.38 | 1 | 4 |

3∕4

7∕8	4	3∕4

| 4.25 | 4.00 | 4.25 |
| --- | --- | --- |
| 4.50 | 4.25 | 4.50 |
| 5.00 | 4.75 | 5.00 |
| 5.00 | 4.75 | 5.00 |

7∕8	4	3∕4

7∕8

7∕8

| 11∕2 | 7.00 | 4.88 | 11∕8 | 4 | 1 | 5.50 | 5.25 | 5.50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 8.50 | 6.50 | 1 | 8 | 7∕8 | 5.75 | 5.50 | 5.75 |
| 21∕2 | 9.62 | 7.50 | 11∕8 | 8 | 1 | 6.25 | 6.00 | 6.25 |
| 3 | 10.50 | 8.00 | 11∕4 | 8 | 11∕8 | 7.00 | 6.75 | 7.00 |
| 4 | 12.25 | 9.50 | 13∕8 | 8 | 11∕4 | 7.75 | 7.50 | 7.75 |
| 5 | 14.75 | 11.50 | 15∕8 | 8 | 11∕2 | 9.75 | 9.50 | 9.75 |
| 6 | 15.50 | 12.50 | 11∕2 | 12 | 13∕8 | 10.25 | 10.00 | 10.50 |
| 8 | 19.00 | 15.50 | 13∕4 | 12 | 15∕8 | 11.50 | 11.25 | 11.75 |
| 10 | 23.00 | 19.00 | 2 | 12 | 17∕8 | 13.25 | 13.00 | 13.50 |
| 12 | 26.50 | 22.50 | 21∕8 | 16 | 2 | 14.75 | 14.50 | 15.25 |
| 14 | 29.50 | 25.00 | 23∕8 | 16 | 21∕4 | 16.00 | 15.75 | 16.75 |
| 16 | 32.50 | 27.75 | 25∕8 | 16 | 21∕2 | 17.50 | 17.25 | 18.50 |
| 18 | 36.00 | 30.50 | 27∕8 | 16 | 23∕4 | 19.50 | 19.25 | 20.75 |
| 20 | 38.75 | 32.75 | 31∕8 | 16 | 3 | 21.25 | 21.00 | 22.25 |
| 24 | 46.00 | 39.00 | 35∕8 | 16 | 31∕2 | 24.25 | 24.00 | 25.50 |

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For other dimensions, see Table 20C.

NOTES:

(1) The length of the stud bolt does not include the height of the points (see para. 6.10.2).

(2)  For flange bolt holes, see para. 6.5.

(3)  For spot facing, see para. 6.6.

(4) Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

178

179

###### Table 20 Dimensions of Class 1500 Flanges	ð20Þ

ASME  B16.5-2020

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

|  |  |  |  | Hub | Length Through Hub Bore |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Diameter |  |  |  | Minimum |  |  |  | Corner Bore |  |  |
|  | Outside |  |  | Beginning of | Threaded/ |  |  | Thread | Minimum |  | Welding | Radius of | Minimum |  |
|  | Diameter | Minimum |  | Chamfer | Slip-On/ |  |  | Length | Slip-On/ |  | Neck/ | Lapped | Counterbore | Depth |
| Nominal | of | Thickness | Diameter | Welding | Socket |  | Welding | Threaded | Socket | Minimum | Socket | Flange | Threaded | of |
| Pipe Size, | Flange, | of Flange, | of Hub, | Neck, | Welding, | Lapped, | Neck, | Flange, | Welding, | Lapped, | Welding, | and Pipe, | Flange, | Socket, |
| NPS | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q | D |
| 1∕2 | 121 | 22.4 | 38 | 21.3 | 32 | 32 | 60 | 22 | 22.4 | 22.9 | (4) | 3 | 23.6 | 10 |
| 3∕4 | 130 | 25.4 | 44 | 26.7 | 35 | 35 | 70 | 25 | 27.7 | 28.2 | (4) | 3 | 29.0 | 11 |
| 1 | 149 | 28.4 | 52 | 33.5 | 41 | 41 | 73 | 28 | 34.5 | 35.4 | (4) | 3 | 35.8 | 13 |
| 11∕4 | 159 | 28.4 | 64 | 42.2 | 41 | 41 | 73 | 30 | 43.2 | 43.7 | (4) | 5 | 44.4 | 14 |
| 11∕2 | 178 | 31.8 | 70 | 48.3 | 44 | 44 | 83 | 32 | 49.5 | 50.0 | (4) | 6 | 50.5 | 16 |

ASME  B16.5-2020

180

| Table 20 Dimensions of Class 1500 Flanges (Cont’d) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Hub Length Through Hub Bore Diameter Minimum Corner Bore |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Outside |  |  | Beginning of | Threaded/ |  |  | Thread | Minimum |  | Welding | Radius of | Minimum |  |
|  | Diameter | Minimum |  | Chamfer | Slip-On/ |  |  | Length | Slip-On/ |  | Neck/ | Lapped | Counterbore | Depth |
| Nominal | of | Thickness | Diameter | Welding | Socket |  | Welding | Threaded | Socket | Minimum | Socket | Flange | Threaded | of |
| Pipe Size, | Flange, | of Flange, | of Hub, | Neck, | Welding, | Lapped, | Neck, | Flange, | Welding, | Lapped, | Welding, | and Pipe, | Flange, | Socket, |
| NPS | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q | D |
| 2 | 216 | 38.1 | 105 | 60.5 | 57 | 57 | 102 | 38 | 62.0 | 62.5 | (4) | 8 | 63.5 | 18 |
| 21∕2 | 244 | 41.1 | 124 | 73.2 | 64 | 64 | 105 | 48 | 74.7 | 75.4 | (4) | 8 | 76.2 | 19 |
| 3 | 267 | 47.8 | 133 | 88.9 | … | 73 | 117 | … | … | 91.4 | (4) | 10 | … | … |
| 4 | 311 | 53.8 | 162 | 114.3 | … | 90 | 124 | … | … | 116.8 | (4) | 11 | … | … |
| 5 | 375 | 73.2 | 197 | 141.2 | … | 105 | 155 | … | … | 144.5 | (4) | 11 | … | … |
| 6 | 394 | 82.6 | 229 | 168.4 | … | 119 | 171 | … | … | 171.4 | (4) | 13 | … | … |
| 8 | 483 | 91.9 | 292 | 219.2 | … | 143 | 213 | … | … | 222.2 | (4) | 13 | … | … |
| 10 | 584 | 108.0 | 368 | 273.0 | … | 178 | 254 | … | … | 277.4 | (4) | 13 | … | … |
| 12 | 673 | 124.0 | 451 | 323.8 | … | 219 | 282 | … | … | 328.2 | (4) | 13 | … | … |
| 14 | 749 | 133.4 | 495 | 355.6 | … | 241 | 298 | … | … | 360.2 | (4) | 13 | … | … |
| 16 | 826 | 146.0 | 552 | 406.4 | … | 260 | 311 | … | … | 411.2 | (4) | 13 | … | … |
| 18 | 914 | 162.1 | 597 | 457.2 | … | 276 | 327 | … | … | 462.3 | (4) | 13 | … | … |
| 20 | 984 | 177.8 | 641 | 508.0 | … | 292 | 356 | … | … | 514.4 | (4) | 13 | … |  |
| 24 | 1 168 | 203.2 | 762 | 609.6 | … | 330 | 406 | … | … | 616.0 | (4) | 13 | … | … |

GENERAL NOTES:

(a)  Dimensions are in millimeters.

(b)  For tolerances, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 19.

(e)  For spot facing, see para 6.6.

(f)  For reducing threaded and slip-on flanges, see Table 6.

(g)  Blind flanges may be made with or without hubs at the manufacturer’s option.

(h) For reducing welding neck flanges, see para 6.8.

NOTES:

(1)  This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges. This dimension is defined as the diameter at the intersection between the hub taper and back face of the flange.

(2) For welding end bevel, see para. 6.7.

(3)  For thread of threaded flanges, see para. 6.9.

(4)  To be specified by the purchaser.

ASME  B16.5-2020

181

###### Table 20C Dimensions of Class 1500 Flanges

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

|  |  |  |  | Hub Diameter | Length Through |  | Hub | Minimum |  | Bore |  | Corner Bore |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outside |  |  | Beginning of | Threaded |  |  | Thread | Minimum |  | Welding | Radius of | Minimum |  |
|  | Diameter | Minimum |  | Chamfer | Slip-On |  |  | Length | Slip-On |  | Neck/ | Lapped | Counterbore | Depth |
|  | of | Thickness | Diameter | Welding | Socket |  | Welding | Threaded | Socket | Minimum | Socket | Flange and | Threaded | of |
| Nominal | Flange, | of Flange, | of Hub, | Neck, | Welding, | Lapped | Neck, | Flange, | Welding, | Lapped, | Welding, | Pipe, | Flange, | Socket, |
| Pipe Size | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | B | r | Q | D |
| 1∕2 | 4.75 | 0.88 | 1.50 | 0.84 | 1.25 | 1.25 | 2.38 | 0.88 | 0.88 | 0.90 | (4) | 0.12 | 0.93 | 0.38 |
| 3∕4 | 5.12 | 1.00 | 1.75 | 1.05 | 1.38 | 1.38 | 2.75 | 1.00 | 1.09 | 1.11 | (4) | 0.12 | 1.14 | 0.44 |
| 1 | 5.88 | 1.12 | 2.06 | 1.32 | 1.62 | 1.62 | 2.88 | 1.12 | 1.36 | 1.38 | (4) | 0.12 | 1.41 | 0.50 |
| 11∕4 | 6.25 | 1.12 | 2.50 | 1.66 | 1.62 | 1.62 | 2.88 | 1.19 | 1.70 | 1.72 | (4) | 0.19 | 1.75 | 0.56 |
| 11∕2 | 7.00 | 1.25 | 2.75 | 1.90 | 1.75 | 1.75 | 3.25 | 1.25 | 1.95 | 1.97 | (4) | 0.25 | 1.99 | 0.62 |
| 2 | 8.5O | 1.50 | 4.12 | 2.38 | 2.25 | 2.25 | 4.00 | 1.50 | 2.44 | 2.46 | (4) | 0.31 | 2.50 | 0.69 |

ASME  B16.5-2020

182

###### Table 20C Dimensions of Class 1500 Flanges (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

| Nominal Pipe Size | Outside Diameter of Flange, O | Minimum Thickness of Flange, tf | Diameter of Hub, X | Hub Diameter Beginning of Chamfer Welding Neck, Ah (2) | Length Through Threaded Slip-On Socket Welding, Lapped Y Y |  | Hub Welding Neck, Y | Minimum Thread Length Threaded Flange, T (3) | Minimum Slip-On Socket Welding, B | Bore Minimum Lapped, B | Welding Neck/ Socket Welding, B | Corner Bore Radius of Lapped Flange and Pipe, r | Minimum Counterbore Threaded Flange, Q | Depth of Socket, D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21∕2 | 9.62 | 1.62 | 4.88 | 2.88 | 2.50 | 2.50 | 4.12 | 1.88 | 2.94 | 2.97 | (4) | 0.31 | 3.00 | 0.75 |
| 3 | 10.50 | 1.88 | 5.25 | 3.50 | … | 2.88 | 4.62 | … | … | 3.60 | (4) | 0.38 | … | … |
| 4 | 12.25 | 2.12 | 6.38 | 4.50 | … | 3.56 | 4.88 | … | … | 4.60 | (4) | 0.44 | … | … |
| 5 | 14.75 | 2.88 | 7.75 | 5.56 | … | 4.12 | 6.12 | … | … | 5.69 | (4) | 0.44 | … | … |
| 6 | 15.50 | 3.25 | 9.00 | 6.63 | … | 4.69 | 6.75 | … | … | 6.75 | (4) | 0.50 | … | … |
| 8 | 19.00 | 3.62 | 11.50 | 8.63 | … | 5.62 | 8.38 | … | … | 8.75 | (4) | 0.50 | … | … |
| 10 | 23.00 | 4.25 | 14.50 | 10.75 | … | 7.00 | 10.00 | … | … | 10.92 | (4) | 0.50 | … | … |
| 12 | 26.50 | 4.88 | 17.75 | 12.75 | … | 8.62 | 11.12 | … | … | 12.92 | (4) | 0.50 | … | … |
| 14 | 29.50 | 5.25 | 19.50 | 14.00 | … | 9.50 | 11.75 | … | … | 14.18 | (4) | 0.50 | … | … |
| 16 | 32.50 | 5.75 | 21.75 | 16.00 | … | 10.25 | 12.25 | … | … | 16.19 | (4) | 0.50 | … | … |
| 18 | 36.00 | 6.38 | 23.50 | 18.00 | … | 10.88 | 12.88 | … | … | 18.20 | (4) | 0.50 | … | … |
| 20 | 38.75 | 7.00 | 25.25 | 20.00 | … | 11.50 | 14.00 | … | … | 20.25 | (4) | 0.50 | … | … |
| 24 | 46.00 | 8.00 | 30.00 | 24.00 | … | 13.00 | 16.00 | … | … | 24.25 | (4) | 0.50 | … | … |

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For tolerances, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 19C.

(e)  For spot facing, see para. 6.6.

(f)  For reducing threaded and slip-on flanges, see Table 6C.

(g)  Blind flanges may be made with or without hubs at the manufacturer’s option.

(h) For reducing welding neck flanges, see para. 6.8.

NOTES:

(1)  This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded, slip-on, socket-welding, and lapped flanges.

(2) For welding end bevel, see para. 6.7.

(3) For threads in threaded flanges, see para. 6.9.

(4)  To be specified by the purchaser.

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Drilling (2), (3) Length of Bolts, L (1), (4) |  |  |  |  |  |  |  |  |
| Nominal | Outside Diameter | Diameter of | Diameter of |  | Diameter | 6.4-mm | Male and Female/ |  |
| Pipe Size, | of Flange, | Bolt Circle, | Bolt Holes, | Number of | of Bolts, | Raised | Tongue and | Ring |
| NPS | O | W | in. | Bolts | in. | Face | Groove | Joint |

ASME  B16.5-2020

###### Table 21 Templates for Drilling Class 2500 Pipe Flanges	ð20Þ

| 1∕2 | 133 | 88.9 | 7∕8 | 4 3∕4 120 115 120 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3∕4 | 140 | 95.2 | 7∕8 | 4 | 3∕4 | 125 | 120 | 125 |
| 1 | 159 | 108.0 | 1 | 4 | 7∕8 | 140 | 135 | 140 |
| 11∕4 | 184 | 130.0 | 11∕8 | 4 | 1 | 150 | 145 | 150 |
| 11∕2 | 203 | 146.0 | 11∕4 | 4 | 11∕8 | 170 | 165 | 170 |
| 2 | 235 | 171.4 | 11∕8 | 8 | 1 | 180 | 170 | 180 |
| 21∕2 | 267 | 196.8 | 11∕4 | 8 | 11∕8 | 195 | 190 | 205 |
| 3 | 305 | 228.6 | 13∕8 | 8 | 11∕4 | 220 | 215 | 230 |
| 4 | 356 | 273.0 | 15∕8 | 8 | 11∕2 | 255 | 250 | 260 |
| 5 | 419 | 323.8 | 17∕8 | 8 | 13∕4 | 300 | 290 | 310 |
| 6 | 483 | 368.3 | 21∕8 | 8 | 2 | 345 | 335 | 355 |
| 8 | 552 | 438.2 | 21∕8 | 12 | 2 | 380 | 375 | 395 |
| 10 | 673 | 539.8 | 25∕8 | 12 | 21∕2 | 490 | 485 | 510 |
| 12 | 762 | 619.3 | 27∕8 | 12 | 23∕4 | 540 | 535 | 560 |

GENERAL NOTES:

(a) Dimensions are in millimeters, except for diameters of bolts and bolt holes, which are in inches.

(b)  For other dimensions, see Table 22.

NOTES:

(1) The length of the stud bolt does not include the height of the points (see para. 6.10.2).

(2)  For flange bolt holes, see para. 6.5.

(3)  For spot facing, see para. 6.6.

(4) Bolt lengths not shown in the table may be determined with Nonmandatory Appendix C (see para. 6.10.2).

183

ASME  B16.5-2020

###### Table 21C Templates for Drilling Class 2500 Pipe Flanges

|  | Outside | Drilling (2), (3) Length of Bolts, L (1), (4) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Diameter of | Diameter of |  |  |  | Raised | Male and Female/ |  |
| Nominal | Flange, | Bolt Circle, | Diameter of | Number of | Diameter of | Face | Tongue and | Ring |
| Pipe Size | O | W | Bolt Holes | Bolts | Bolts | 0.25 in. | Groove | Joint |

1	2	3	4	5	6	7	8	9

| 1∕2 | 5.25 | 3.50 | 7∕8 | 4 | 3∕4 | 4.75 | 4.50 | 4.75 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3∕4 | 5.50 | 3.75 | 7∕8 | 4 | 3∕4 | 5.00 | 4.75 | 5.00 |
| 1 | 6.25 | 4.25 | 1 | 4 | 7∕8 | 5.50 | 5.25 | 5.50 |
| 11∕4 | 7.25 | 5.12 | 11∕8 | 4 | 1 | 6.00 | 5.75 | 6.00 |
| 11∕2 | 8.00 | 5.75 | 11∕4 | 4 | 11∕8 | 6.75 | 6.50 | 6.75 |
| 2 | 9.25 | 6.75 | 11∕8 | 8 | 1 | 7.00 | 6.75 | 7.00 |
| 21∕2 | 10.50 | 7.75 | 11∕4 | 8 | 11∕8 | 7.75 | 7.50 | 8.00 |
| 3 | 12.00 | 9.00 | 13∕8 | 8 | 11∕4 | 8.75 | 8.50 | 9.00 |
| 4 | 14.00 | 10.75 | 15∕8 | 8 | 11∕2 | 10.00 | 9.75 | 10.25 |
| 5 | 16.50 | 12.75 | 17∕8 | 8 | 13∕4 | 11.75 | 11.50 | 12.25 |
| 6 | 19.00 | 14.50 | 21∕8 | 8 | 2 | 13.50 | 13.25 | 14.00 |
| 8 | 21.75 | 17.25 | 21∕8 | 12 | 2 | 15.00 | 14.75 | 15.50 |
| 10 | 26.50 | 21.25 | 25∕8 | 12 | 21∕2 | 19.25 | 19.00 | 20.00 |
| 12 | 30.00 | 24.38 | 27∕8 | 12 | 23∕4 | 21.25 | 21.00 | 22.00 |

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For other dimensions, see Table 22C.

NOTES:

(1) The length of the stud bolt does not include the height of the points (see para. 6.10.2).

(2)  For flange bolt holes, see para. 6.5.

(3)  For spot facing, see para. 6.6.

(4) Bolt lengths not shown in the table may be determined in accordance with Nonmandatory Appendix C (see para. 6.10.2).

184

185

###### Table 22 Dimensions of Class 2500 Flanges	ð20Þ

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  | 8 | 9 | 10 |  |  | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Length Through Hub |  |  |  |  |  | Bore |  |  | Corner |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bore |  |
|  |  |  |  | Hub Diam. |  |  |  |  |  | Minimum |  |  |  |  | Radius of |  |
|  | Outside |  |  | Beginning |  |  |  |  |  | Thread |  |  |  |  | Lapped | Minimum |
| Nom. | Diam. | Min. |  | of Chamfer |  |  |  |  |  | Length |  |  |  |  | Flange | Counterbore |
| Pipe | of | Thickness | Diam. | Welding |  |  |  | Welding |  | Threaded | Min. |  | Welding |  | and | Threaded |
| Size, | Flange, | of Flange, | of Hub, | Neck, | Threaded, |  | Lapped, | Neck, |  | Flange, | Lapped, |  | Neck, |  | Pipe, | Flange, |
| NPS | O | tf | X | Ah (2) | Y |  | Y | Y |  | T (3) | B |  | B |  | r | Q |

ASME  B16.5-2020

1∕2	133	30.2	43	21.3	40	40	73	28	22.9	(4)	3	23.6

3∕4	140	31.8	51	26.7	43	43	79	32	28.2	(4)	3	29.0

| 1 | 159 | 35.1 | 57 | 33.5 | 48 | 48 | 89 | 35 | 35.1 | (4) | 3 | 35.8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11∕4 | 184 | 38.1 | 73 | 42.2 | 52 | 52 | 95 | 38 | 43.7 | (4) | 5 | 44.4 |
| 11∕2 | 203 | 44.4 | 79 | 48.3 | 60 | 60 | 111 | 44 | 50.0 | (4) | 6 | 50.5 |
| 2 | 235 | 50.8 | 95 | 60.5 | 70 | 70 | 127 | 51 | 62.5 | (4) | 8 | 63.5 |
| 21∕2 | 267 | 57.2 | 114 | 73.2 | 79 | 79 | 143 | 57 | 75.4 | (4) | 8 | 76.2 |
| 3 | 305 | 66.5 | 133 | 88.9 | … | 92 | 168 | … | 91.4 | (4) | 10 | … |
| 4 | 356 | 76.2 | 165 | 114.3 | … | 108 | 190 | … | 116.8 | (4) | 11 | … |
| 5 | 419 | 91.9 | 203 | 141.2 | … | 130 | 229 | … | 144.5 | (4) | 11 | … |
| 6 | 483 | 108.0 | 235 | 168.4 | … | 152 | 273 | … | 171.4 | (4) | 13 | … |

###### Table 22 Dimensions of Class 2500 Flanges (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13

Length Through Hub

Bore	Corner

Bore

|  |  |  |  | Hub Diam. |  |  |  | Minimum |  |  | Radius of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outside |  |  | Beginning |  |  |  | Thread |  |  | Lapped | Minimum |
| Nom. | Diam. | Min. |  | of Chamfer |  |  |  | Length |  |  | Flange | Counterbore |
| Pipe | of | Thickness | Diam. | Welding |  |  | Welding | Threaded | Min. | Welding | and | Threaded |
| Size, | Flange, | of Flange, | of Hub, | Neck, | Threaded, | Lapped, | Neck, | Flange, | Lapped, | Neck, | Pipe, | Flange, |
| NPS | O | tf | X | Ah (2) | Y | Y | Y | T (3) | B | B | r | Q |
| 8 | 552 | 127.0 | 305 | 219.2 | … | 178 | 318 | … | 222.2 | (4) | 13 | … |
| 10 | 673 | 165.1 | 375 | 273.0 | … | 229 | 419 | … | 277.4 | (4) | 13 | … |
| 12 | 762 | 184.2 | 441 | 323.8 | … | 254 | 464 | … | 328.2 | (4) | 13 | … |

ASME  B16.5-2020

186

GENERAL NOTES:

(a) Dimensions are in millimeters, except for diameters of bolts and bolt holes, which are in inches.

(b)  For tolerances, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 21.

(e)  For spot facing, see para 6.6.

(f)  For reducing threaded and slip-on flanges, see Table 6.

(g)  Blind flanges may be made with or without hubs at the manufacturer’s option.

(h) For reducing welding neck flanges, see para 6.8.

NOTES:

(1) This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded and lapped flanges. This dimension is defined as the diameter at the intersection between the hub taper and back face of the flange.

(2) For welding end bevel, see para. 6.7.

(3)  For thread of threaded flanges, see para. 6.9.

(4)  To be specified by the purchaser.

ASME  B16.5-2020

187

###### Table 22C Dimensions of Class 2500 Flanges

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  | 8 | 9 | 10 |  | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Length Through Hub |  |  |  |  |  | Bore |  | Corner Bore |  |
|  |  |  |  | Hub Diameter |  |  |  |  |  | Minimum |  |  |  | Radius of | Minimum |
|  | Outside | Minimum |  | Beginning of |  |  |  |  |  | Thread Length |  |  |  | Lapped | Counterbore |
| Diameter of |  | Thickness of | Diameter | Chamfer |  |  |  | Welding |  | Threaded | Minimum |  | Welding | Flange and | Threaded |
| ominal Flange, Flange, of Hub, Welding Neck, Threaded, Lapped, Neck Flange, Lapped, Neck, Pipe, Flange, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ipe Size O tf X Ah (2) Y Y Y T (3) B B r Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1∕2 5.25 |  | 1.19 | 1.69 | 0.84 | 1.56 |  | 1.56 | 2.88 |  | 1.12 | 0.90 |  | (4) | 0.12 | 0.93 |
| 3∕4 5.50 |  | 1.25 | 2.00 | 1.05 | 1.69 |  | 1.69 | 3.12 |  | 1.25 | 1.11 |  | (4) | 0.12 | 1.14 |
| 1 6.25 |  | 1.38 | 2.25 | 1.32 | 1.88 |  | 1.88 | 3.50 |  | 1.38 | 1.38 |  | (4) | 0.12 | 1.41 |
| 11∕4 7.25 |  | 1.50 | 2.88 | 1.66 | 2.06 |  | 2.06 | 3.75 |  | 1.50 | 1.72 |  | (4) | 0.19 | 1.75 |
| 11∕2 8.00 |  | 1.75 | 3.12 | 1.90 | 2.38 |  | 2.38 | 4.38 |  | 1.75 | 1.97 |  | (4) | 0.25 | 1.99 |
| 2 9.25 |  | 2.00 | 3.75 | 2.38 | 2.75 |  | 2.75 | 5.00 |  | 2.00 | 2.46 |  | (4) | 0.31 | 2.50 |
| 21∕2 10.50 |  | 2.25 | 4.50 | 2.88 | 3.12 |  | 3.12 | 5.62 |  | 2.25 | 2.97 |  | (4) | 0.31 | 3.00 |
| 3 12.00 |  | 2.62 | 5.25 | 3.50 | … |  | 3.62 | 6.62 |  | … | 3.60 |  | (4) | 0.38 | … |
| 4 14.00 |  | 3.00 | 6.50 | 4.50 | … |  | 4.25 | 7.50 |  | … | 4.60 |  | (4) | 0.44 | … |
| 5 16.50 |  | 3.62 | 8.00 | 5.56 | … |  | 5.12 | 9.00 |  | … | 5.69 |  | (4) | 0.44 | … |
| 6 19.00 |  | 4.25 | 9.25 | 6.63 | … |  | 6.00 | 10.75 |  | … | 6.75 |  | (4) | 0.50 | … |
| 8 21.75 |  | 5.00 | 12.00 | 8.63 | … |  | 7.00 | 12.50 |  | … | 8.75 |  | (4) | 0.50 | … |
| 10 26.50 |  | 6.50 | 14.75 | 10.75 | … |  | 9.00 | 16.50 |  | … | 10.92 |  | (4) | 0.50 | … |
| 12 30.00 |  | 7.25 | 17.38 | 12.75 | … |  | 10.00 | 18.25 |  | … | 12.92 |  | (4) | 0.50 | … |

N P

ASME  B16.5-2020

188

###### Table 22C Dimensions of Class 2500 Flanges (Cont’d)

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For tolerances, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 21C.

(e)  For spot facing, see para. 6.6.

(f)  For reducing threaded and slip-on flanges, see Table 6C.

(g)  Blind flanges may be made with or without hubs at the manufacturer’s option.

(h) For reducing welding neck flanges, see para. 6.8.

NOTES:

(1) This dimension is for the large end of the hub, which may be straight or tapered. Taper shall not exceed 7 deg on threaded and lapped flanges.

(2) For welding end bevel, see para. 6.7.

(3) For threads in threaded flanges, see para. 6.9.

(4)  To be specified by the purchaser.

ASME  B16.5-2020

## MANDATORY APPENDIX I

THREADING OF PIPE FOR AMERICAN NATIONAL STANDARD THREADED FLANGES

The length of external taper threads in ASME B1.20.1 is sufficient to provide a satisfactory joint when assembled with corresponding internal taper threads in couplings and fittings. In this Standard, the length of internal thread in Classes 150, 300, and 400 flanges also conforms to ASME B1.20.1.

In Class 600 and higher rated flanges, the length through the hub may exceed the length for internal threads in ASME B1.20.1. Where this occurs, the extended length of internal threads follows the taper of the standard taper threads. Therefore, the diameters of the extra threads are smaller than those shown in ASME B1.20.1. When threaded flanges conforming to this Standard are assembled with threaded-end pipe, it is intended that the end of the pipe be reasonably close to the mating surface of the flange. To meet this intent, the following requirements

are imposed on the mating pipe and external thread:

(a) Pipe to be threaded into flanges of Class 600 or higher rating shall be Schedule 80 or heavier in wall thick- ness.

(b)  The length of external effective thread on the pipe

end shall be greater than specified in ASME B1.20.1. When tested with the standard ring gage, the pipe end shall project beyond the gage by the distance specified in Table I-1, subject to a tolerance of one thread pitch as provided in ASME B1.20.1.

(c) The extra threads shall continue the taper specified

in ASME B1.20.1, so that the pitch diameter of the thread at the pipe end is less than specified therein.

(d)  It is recommended that power equipment be used

to assemble threaded joints having longer than standard taper threads, in order to bring the pipe end close to the flange face.

189

ASME  B16.5-2020

Classes 150, 300,

###### Table I-1 Projection of Threaded Pipe End Through a Ring Gage

400	Class 600	Class 900	Class 1500	Class 2500

Number of

Number of

Projection	Number

of

Projection	Number

of

Projection	Number of

Projection

NPS

Turns

Turns

mm	in.

Turns

mm	in.

Turns

mm	in.

Turns

mm	in.

1∕2	…	(1)	…	…	…	…	…	31∕2	6.4	0.25	7	12.7	0.50

3∕4	…	(1)	…	…	…	…	…	5	9.5	0.38	7	12.7	0.50

1	…	(1)	…	…	…	…	…	5	11.1	0.44	71∕2	16.5	0.65

11∕4	…	(1)	…	…	…	…	…	5	11.1	0.44	71∕2	16.5	0.65

11∕2	…	(1)	…	…	…	…	…	5	11.1	0.44	71∕2	16.5	0.65

2	…	(1)	…	…	…	…	11.1	5	11.1	0.44	71∕2	16.5	0.65

21∕2	…	…	…	…	…	…	…	5	15.9	0.62	8	25.4	1.00

3	…	1	3.2	0.12	3	9.5	0.38	…	…	…	…	…	…

31∕2	…	1	3.2	0.12	…	…	…	…	…	…	…	…	…

4	(1)	11∕2	4.8	0.19	31∕2	11.1	0.44	…	…	…	…	…	…

5	(1)	11∕2	4.8	0.19	31∕2	11.1	0.44	…	…	…	…	…	…

6	(1)	11∕2	4.8	0.19	31∕2	11.1	0.44	…	…	…	…	…	…

8	(1)	2	6.4	0.25	4	12.7	0.50	…	…	…	…	…	…

10	(1)	3	9.5	0.38	5	15.9	0.62	…	…	…	…	…	…

12	(1)	3	9.5	0.38	5	15.9	0.62	…	…	…	…	…	…

14	(1)	3	9.5	0.38	6	19.0	0.75	…	…	…	…	…	…

16	(1)	3	9.5	0.38	6	19.0	0.75	…	…	…	…	…	…

18	(1)	3	9.5	0.38	6	19.0	0.75	…	…	…	…	…	…

20	(1)	3	9.5	0.38	6	19.0	0.75	…	…	…	…	…	…

24	(1)	3	9.5	0.38	6	19.0	0.75	…	…	…	…	…	…

NOTE: (1) Use taper pipe thread per ASME B1.20.1 for these sizes.

190

ASME  B16.5-2020

## MANDATORY APPENDIX II

REFERENCES	ð20Þ

The following is a list of standards and specifications referenced in this Standard. Products covered by each ASTM specification are listed for convenience. For ASME Codes and Standards referenced hereunder, up to and including the latest published edition in effect at the time this edition of this Standard is specified, may be used. (See specifications for exact titles and detailed contents.) Materials manufactured to other editions of the referenced ASTM specifications may be used to manufacture flanges and flanged fittings meeting the requirements of this Standard as long as the flange/fitting manufacturer verifies that the material meets the requirements of the referenced edition of the ASTM specification.

ASME B1.1, Unified Inch Screw Threads (UN and UNR Thread  Form)

ASME B1.20.1, Pipe Threads, General Purpose (Inch) ASME B16.20, Metallic Gaskets for Pipe Flanges: Ring-

Joint, Spiral-Wound, and Jacketed

ASME B16.21, Nonmetallic Flat Gaskets for Pipe Flanges ASME B16.25, Buttwelding Ends

ASME B16.34, Valves — Flanged, Threaded, and Welding End

ASME B18.2.1, Square and Hex Bolts and Screws (Inch

Series)

ASME B18.2.2, Square and Hex Nuts (Inch Series)

ASME B18.31.2, Continuous Thread Stud, Double-End Stud, and Flange Bolting Stud (Stud Bolt) (Inch Series) ASME B36.l0M, Welded and Seamless Wrought Steel Pipe ASME B46.1, Surface Texture (Surface Roughness, Wavi-

ness, and Lay)

ASME PCC-1, Guidelines for Pressure Boundary Bolted Flange Joint Assembly

Section I, Power Boilers

Section II, Materials

Section III, Rules for Construction of Nuclear Facility Components

Section VIII, Div. 1 and 2, Pressure Vessels

Section IX, Welding, Brazing, and Fusing Qualifications Publisher: The American Society of Mechanical Engineers

(ASME), Two Park Avenue, New York, NY 10016-5990 (www.asme.org)

ASTM A105/A105M-18, Carbon Steel Forgings for Piping Applications

ASTM A106/A106M-19a, Seamless Carbon Steel Pipe for High-Temperature  Service

ASTM A182/A182M-20, Forged or Rolled Alloy and Stain- less Steel Pipe Flanges, Forged Fittings, and Valves and Parts for High Temperature Service

ASTM A193/A193M-19, Alloy-Steel and Stainless Steel Bolting for High-Temperature or High-Pressure Service and Other Special Purpose Applications

ASTM A194/A194M-18, Carbon Steel, Alloy Steel, and Stainless Steel Nuts for Bolts for High-Pressure and High-Temperature Service, or Both

ASTM A203/A203M-17, Pressure Vessel Plates, Alloy

Steel, Nickel

ASTM A204/A204M-17, Pressure Vessel Plates, Alloy Steel, Molybdenum

ASTM A216/A216M-18, Steel Castings, Carbon, Suitable

for Fusion Welding, for High-Temperature Service ASTM A217/A217M-14, Steel Castings, Martensitic Stain-

less and Alloy, for Pressure Containing Parts, Suitable for High-Temperature Service

ASTM A240/A240M-19, Chromium and Chromium-Nickel

Stainless Steel Plate, Sheet, and Strip for Pressure Vessels and for General Applications

ASTM A307-14e1, Carbon Steel Bolts, Studs, and Threaded Rod 60,000 psi Tensile Strength

ASTM A320/A320M-18, Alloy Steel and Stainless Steel

Bolting for Low-Temperature Service

ASTM A350/A350M-18, Carbon and Low-Alloy Steel Forgings, Requiring Notch Toughness Testing for Piping Components

ASTM A351/A351M-18e1, Castings, Austenitic, for Pres-

sure-Containing  Parts

ASTM A352/A352M-18a, Steel Castings, Ferritic and Martensitic, for Pressure-Containing Parts,  Suitable for  Low-Temperature  Service

ASTM A354-17e2, Quenched and Tempered Alloy Steel Bolts, Studs, and Other Externally Threaded Fasteners ASTM A387/A387M-17a, Pressure Vessel Plates, Alloy

Steel,  Chromium-Molybdenum

ASTM A449-14, Hex Cap Screws, Bolts and Studs, Steel, Heat Treated, 120/105/90 ksi Minimum Tensile Strength, General Use

ASTM A453/A453M-17, High-Temperature Bolting, with Expansion Coefficients Comparable to Austenitic Stain- less Steels

191

ASME  B16.5-2020

ASTM A515/A515M-17, Pressure Vessel Plates, Carbon Steel, for Intermediate- and Higher-Temperature Service

ASTM A516/A516M-17, Pressure Vessel Plates, Carbon Steel, For Moderate- and Lower-Temperature Service ASTM A537/A537M-20, Pressure Vessel Plates, Heat-

Treated, Carbon-Manganese-Silicon Steel

ASTM A540/540M-15, Alloy-Steel Bolting Materials for Special  Applications

ASTM  A995/A995M-19,  Castings,  Austenitic-Ferritic (Duplex) Stainless Steel, for Pressure-Containing Parts ASTM B127-19, Nickel-Copper Alloy Plate, Sheet, and Strip

ASTM B160-05 (2019), Nickel Rod and Bar

ASTM B162-99 (2019) , Nickel Plate, Sheet, and Strip ASTM B163-19 , Seamless Nickel and Nickel Alloy

Condenser and Heat-Exchanger Tubes

ASTM B164-03 (2019), Nickel-Copper Alloy Rod, Bar, and Wire

ASTM B166-19, Nickel-Chromium-Aluminum Alloy,

Nickel-Chromium-Iron Alloys, Nickel-Chromium- Cobalt-Molybdenum Alloy, Nickel-Iron-Chromium- Tungsten Alloy, and Nickel-Chromium-Molybdenum- Copper Alloy Rod, Bar, and Wire

ASTM B168-19, Nickel-Chromium-Aluminum Alloys, Nickel-Chromium-Iron Alloys, Nickel-Chromium- Cobalt-Molybdenum Alloy, Nickel-Iron-Chromium- Tungsten Alloy, and Nickel-Chromium-Molybdenum- Copper Alloy Plate, Sheet, and Strip

ASTM B333-03 (2018), Nickel-Molybdenum Alloy Plate, Sheet, and Strip

ASTM B335-18, Nickel-Molybdenum Alloy Rod

ASTM B407-08a (2019), Nickel-Iron-Chromium Alloy Seamless Pipe and Tube

ASTM B408-06 (2016), Nickel-Iron-Chromium Alloy Rod

and Bar

ASTM B409-06 (2016), Nickel-Iron-Chromium Alloy, Plate, Sheet, and Strip

ASTM B424-19e1, Nickel-Iron-Chromium-Molybdenum- Copper Alloys Plate, Sheet, and Strip

ASTM B425-19 , Nickel-Iron-Chromium-Molybdenum- Copper Alloys Rod and Bar

ASTM B434-06 (2016), Nickel-Molybdenum-Chromium- Iron Alloys (UNS N10003, UNS N10242) Plate, Sheet, and Strip

ASTM B435-06 (2016), UNS N06002, UNS N06230, UNS

N12160, and UNS R30556 Plate, Sheet, and Strip ASTM B443-19 , Nickel-Chromium-Molybdenum-Colum-

bium Alloy and Nickel-Chromium-Molybdenum-Silicon Alloy Plate, Sheet, and Strip

ASTM B446-19, Nickel-Chromium-Molybdenum-Colum- bium Alloy (UNS N06625), Nickel-Chromium-Molyb- denum-Silicon Alloy (UNS N06219), and Nickel- Chromium-Molybdenum-Tungsten Alloy (UNS N06650) Rod and Bar

ASTM B462-18, Forged or Rolled Nickel Alloy Pipe Flanges, Forged Fittings, and Valves and Parts for Corro- sive High-Temperature Service

ASTM B463-10 (2016), UNS N08020 Alloy Plate, Sheet,

and Strip

ASTM B473-07 (2018), UNS N08020, UNS N08024, and

UNS N08026 Nickel Alloy Bar and Wire

ASTM B511/B511M-16 Nickel-Iron-Chromium-Silicon Alloy Bars and Shapes

ASTM B536-19, Nickel-Iron-Chromium-Silicon Alloy Plate, Sheet, and Strip

ASTM B564-19, Nickel Alloy Forgings

ASTM B572-06 (2016), UNS N06002, UNS N06230, UNS N12160, and UNS R30556 Rod

ASTM B573-06 (2016), Nickel-Molybdenum-Chromium- Iron Alloy (UNS N10003, N10242) Rod

ASTM B574-18, Low-Carbon Nickel-Chromium-Molyb- denum, Low-Carbon Nickel-Molybdenum-Chromium, Low-Carbon Nickel-Molybdenum-Chromium- Tantalum, Low-Carbon Nickel-Chromium-Molyb- denum-Copper, and Low-Carbon Nickel-Chromium- Molybdenum-Tungsten Alloy Rod

ASTM B575-17, Low-Carbon Nickel-Chromium-Molyb- denum, Low-Carbon Nickel-Chromium-Molybdenum- Copper, Low-Carbon Nickel-Chromium-Molybdenum- Tantalum, Low-Carbon Nickel-Chromium-Molyb- denum-Tungsten, and Low-Carbon Nickel-Molyb- denum-Chromium Alloy Plate, Sheet, and Strip

ASTM B581-17, Nickel-Chromium-Iron-Molybdenum- Copper Alloy Rod

ASTM B582-07 (2018), Nickel-Chromium-Iron-Molyb- denum-Copper Alloy Plate, Sheet, and Strip

ASTM B599-20, Nickel-Iron-Chromium-Molybdenum- Niobium Stabilized Alloy (UNS N08700) Plate, Sheet, and Strip

ASTM B620-03 (2018), Nickel-Iron-Chromium-Molyb- denum Alloy (UNS N08320) Plate, Sheet, and Strip ASTM B621-02 (2016), Nickel-Iron-Chromium-Molyb-

denum Alloy (UNS N08320) Rod

ASTM B625-17, UNS N08925, UNS N08031, UNS N08034, UNS N08932, UNS N08926, UNS N08354, UNS N08830,

and UNS R20033 Plate, Sheet, and Strip

ASTM B649-17, Ni-Fe-Cr-Mo-Cu-N Low-Carbon Alloys (UNS N08925, UNS N08031, UNS N08034, UNS N08354, and UNS N08926), and Cr-Ni-Fe-N Low-

Carbon Alloy (UNS R20033) Bar and Wire, and Ni- Cr-Fe-Mo-N Alloy (UNS N08936) Wire

ASTM B672-02 (2018), Nickel-Iron-Chromium-Molyb- denum-Columbium Stabilized Alloy (UNS N08700) Bar and Wire

ASTM B688-18 (R2004), Chromium-Nickel-Molybdenum- Iron (UNS N08367) Plate, Sheet, and Strip

ASTM E29-13 (2019), Using Significant Digits in Test Data to Determine Conformance with Specifications

192

ASME  B16.5-2020

Publisher: American Society for Testing and Materials (ASTM International), 100 Barr Harbor Drive, P.O.Box C700, West Conshohocken, PA 19428-2959 (www.astm.org)

ISO 7005-1:1980, Metallic flanges — Part 1: Steel flanges ISO 9000-1:1994, Quality management and quality assur- ance standards — Part 1: Guidelines for selection and

use

ISO 9000-2:1997, Quality management and quality assur- ance standard — Part 2: Generic guidelines for the application of ISO 9001, ISO 9002m, and ISO 9003

ISO 9000-3:1997, Quality management and quality assur- ance standards — Part 3: Guidelines for the application of ISO 9001 to the development, supply, and mainte- nance of software

ISO 9001:2000, Quality management systems — Require- ments

ISO 9002:1994, Quality system — Model for quality assur-

ance in production and servicing

ISO 9003:1994, Quality systems — Model for quality assurance in final inspection and test

Publisher:1  International Organization for Standardiza-

tion (ISO), Central Secretariat, Chemin de Blandonnet 8, Case Postale 401, 1214 Vernier, Geneva, Switzerland (www.iso.org)

MSS SP-6-2001, Finishes for Contact Faces of Pipe Flanges and Connecting-End Flanges of Valves and Fittings

MSS SP-9-2001, Spot Facing for Bronze, Iron, and Steel Flanges

MSS SP-25-1998, Standard Marking System for Valves, Fittings, Flanges, and Unions

MSS SP-44-2006, Steel Pipeline Flanges

MSS SP-45-2003, Bypass and Drain Connections

MSS SP-55-2006, Quality Standard for Steel Casting for Valves, Flanges, and Fittings

MSS SP-61-2003, Pressure Testing of Steel Valves Publisher: Manufacturers Standardization Society of the

Valve and Fittings Industry, Inc. (MSS), 127 Park Street, NE, Vienna, VA 22180 (www.msshq.org)

1 ISO documents are available from the American National Standards Institute (ANSI), 25 West 43rd Street, New York, NY 10036. Publications appearing above, which have been approved as American National Standards, may also be obtained from ANSI.

193

ASME  B16.5-2020

## NONMANDATORY  APPENDIX  A

METHOD USED FOR ESTABLISHING PRESSURE–TEMPERATURE RATINGS

##### GENERAL   CONSIDERATIONS

ÅÄ	Pcd	ÉÑ

##### A-1.1 Introduction

Pressure–temperature ratings for this Standard have

where

t	1.5Å	 	 Å 2S

F

Å

Ñ

=

Ñ

Pc Ñ

(2)

been determined by the procedures described in this Appendix. These procedures are counterpart to those identified as Standard Class in ASME B16.34. The primary considerations in establishing ratings are compo- nent dimensions and material properties to sustain pres- sure and other loads.1 Other considerations affecting or limiting ratings include

    - stresses in flanges resulting from bolt-up necessary to maintain a gasket seal

    - distortion of flanges and flanged fittings due to loads transmitted through attached piping

    - limitations applying primarily to flanged compo- nents (e.g., valves) but also imposed on flanges in order to preserve compatible ratings

##### A-1.2 Bolt Cross-Sectional Area

Total flange bolting cross-sectional area requirements are based on the following relationship:

A

PcAg	(1)

d  = inside diameter of the fitting

Pc  = pressure rating class designation or number (e.g., for Class 150, Pc = 150; for Class 300, Pc = 300)

SF   = stress base constant equal to 7000

t  =  calculated thickness

The resultant units for t will be the same as those used to express d. Equation (2) results in a fitting wall thickness 50% greater than that for a simple cylinder designed for a stress of 48.28 MPa (7,000 psi) when subjected to an internal pressure equal to the pressure rating class des- ignation Pc. The actual values in the dimension tables referred to in para. 6.1 are approximately 2.5 mm (0.1 in.) larger than those given by the equation.

##### A-1.4  Material Properties

The pressure–temperature rating method uses allow- able stress, ultimate tensile strength, and yield strength values from Section II, Part D of the ASME Boiler and Pres-

sure Vessel Code. For materials listed herein that have

where

b	7000

ratings either at temperature values that are above those shown in a reference Code Section or that are not listed in any of the reference Code Sections, the allow-

A b   = total effective bolt tensile stress area

Ag   = an area whose circumference is defined by a

diameter equal to the raised face dimension R

in Table 4

Pc   = pressure rating class designation or number (e.g., for Class 150, Pc = 150; for Class 300, Pc

= 300)

##### A-1.3 Flanged Fitting Wall Thickness

Wall thickness requirements for flanged fittings are specified in para. 6.1. The minimum wall thickness values tm  are shown in the tables designated in para.

6.1. These values are all greater than those determined

by eq. (2).

able stress, ultimate tensile strength, and yield strength data have been provided directly by the ASME Boiler and Pressure Vessel Subcommittee on Materials.

##### A-1.5 Material Groups

Materials are grouped in Table 1.1-1 based on identical or closely matched allowable stress and yield strength values. When these values are not identical for each mate- rial listed, the lowest value has been used. Note that mate- rial groups in this Standard are not numbered consecutively. The unlisted numbers, numbers that are not part of this Standard, may be found in the materials tables of ASME B16.34.

1 This method is appropriate for materials listed in Table 1.1-1. It may not be appropriate for other materials.

194

ASME  B16.5-2020

##### PRESSURE–TEMPERATURE RATING METHOD

A-2.1 Rating Equation Class 300 and Higher

Pressure–temperature ratings for Class 300 and higher components, of materials listed to those in Table 1.1-1, were established by the equation

C1S1	(3)

stress values do not appear and it is noted in the allowable stress table that the allowable stress values exceed two- thirds of the yield strength at temperature, then the allow- able stress values used shall be determined as two-thirds of the tabulated yield strength at temperature.

- Yield strength shall be as listed in Section II, Part D of

the ASME Boiler and Pressure Vessel Code, for either Section III or Section VIII, Division 2.

where

pt =

8750

Pr	pc

- Allowable stress values listed in Section II, Part D of the ASME Boiler and Pressure Vessel Code, for Section III,

Class 2 or Class 3 values may only be used for a material

C1   =  10 when S1  is expressed in MPa units and the

resultant pt will be in bar units (C1 = 1 when S1 is expressed in psi units and the resultant pt  will be in psi units)

pc   = ceiling pressure, bar (psi), at temperature T as

specified in section A-3

Pr = pressure rating class index. For all designations Class 300 and above, Pr is equal to the class des- ignation (e.g., for Class 300, Pr = 300). (For Class 150, see para. A-2.4 of this Nonmandatory Appen- dix.)

pt = rated working pressure, bar (psi), for the speci- fied material at temperature T

S1 = selected stress, MPa (psi) for the specified mate- rial at temperature T. The value of S1 shall be established as described in paras. A-2.2, A-2.3, and A-2.4.

##### A-2.2 Ratings for Group No. 1 Materials

The selected stress for Group No. 1 materials in

Table 1.1-1 is determined as follows:

- At temperatures below the creep range, S1 shall be equal to or less than

  - 60% of the specified minimum yield strength at 38°C (100°F)

  - 60% of the yield strength at temperature T

  - 1.25 times 25% of the ultimate tensile strength value at temperature T, as listed in Section II, Part D of the ASME Boiler and Pressure Vessel Code for either Section I

not listed for either Section I or Section VIII, Division 1.

##### A-2.3 Method for Groups 2 and 3 Materials

Pressure–temperature ratings for Class 300 and higher, of materials corresponding to those in Materials Groups 2 and 3 of Table 1.1-1, are established by the method of paras. A-2.1 and A-2.2, except that in paras. A-2.2(a)(1) and A-2.2(a)(2), the 60% factor shall be changed to 70%. For Group 2 materials, the creep range is considered to be at temperatures in excess of 510°C (950°F) unless the material properties indicate lower temperatures should be used. For Group 3 materials, the creep range onset temperature shall be determined on an individual basis.

##### A-2.4  Method for Class 150 — All Materials

Pressure–temperature ratings for Class 150 rating des- ignation are established by the method given for the related materials in paras. A-2.1, A-2.2, and A-2.3 subject to the following exceptions:

- The value of Pr, the pressure class rating index in

eq. (3) for Class 150, shall be 115.

- The value for S1, the selected stress MPa (psi), for the specific material at temperature T shall be in accor- dance with the requirements of either para. A-2.1 or A-2.2, as applicable.

- The value of pt, the rated working pressure bar (psi), for Class 150, shall not exceed values at temperature T as given by eq. (4).

or Section VIII, Division 1

- At temperatures in the creep range, the value of S1 shall be the allowable stress at temperature T, as listed in Section II, Part D of the ASME Boiler and Pressure Vessel

where

C2  = 21.41

pt	C2

C3T

(4)

Code, for either Section I or Section VIII, Division 1, but not exceeding 60% of the listed yield strength at temperature.

- In no case shall the selected stress value increase

with increasing temperature.

- The creep range is considered to be at temperatures in excess of 370°C (700°F) for Group 1 materials.

- When the allowable stresses listed for the reference ASME Boiler and Pressure Vessel Code Section show a higher and lower value for allowable stress and the higher value is noted to the effect that these stress values exceed two-thirds of the yield strength at tempera- ture, then the lower value shall be used. If lower allowable

C3  = 0.03724 with T expressed in °C, the resultant pt

will be in bar units (C2 = 320 and C3 = 0.3, with T

expressed in °F, the resultant pt  will be in psi

units)

T  = material temperature, °C (°F)

The value of T in eq. (4) shall not exceed 538°C (1,000°F). For values of T less than 38°C (100°F), use T equal to 38°C (100°F) in eq. (4).

195

ASME  B16.5-2020

###### Table A-1 Rating Ceiling Pressure —  pc, bar

Class

| Temperature, °C | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −29 to 38 | 20.0 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 50 | 19.5 | 51.7 | 68.9 | 103.4 | 155.1 | 258.6 | 430.9 |
| 100 | 17.7 | 51.5 | 68.7 | 103.0 | 154.6 | 257.6 | 429.4 |
| 150 | 15.8 | 50.3 | 66.8 | 100.3 | 150.6 | 250.8 | 418.2 |
| 200 | 13.8 | 48.6 | 64.8 | 97.2 | 145.8 | 243.4 | 405.4 |
| 250 | 12.1 | 46.3 | 61.7 | 92.7 | 139.0 | 231.8 | 386.2 |
| 300 | 10.2 | 42.9 | 57.0 | 85.7 | 128.6 | 214.4 | 357.1 |
| 325 | 9.3 | 41.4 | 55.0 | 82.6 | 124.0 | 206.6 | 344.3 |
| 350 | 8.4 | 40.3 | 53.6 | 80.4 | 120.7 | 201.1 | 335.3 |
| 375 | 7.4 | 38.9 | 51.6 | 77.6 | 116.5 | 194.1 | 323.2 |
| 400 | 6.5 | 36.5 | 48.9 | 73.3 | 109.8 | 183.1 | 304.9 |
| 425 | 5.5 | 35.2 | 46.5 | 70.0 | 105.1 | 175.1 | 291.6 |
| 450 | 4.6 | 33.7 | 45.1 | 67.7 | 101.4 | 169.0 | 281.8 |
| 475 | 3.7 | 31.7 | 42.3 | 63.4 | 95.1 | 158.2 | 263.9 |
| 500 | 2.8 | 28.2 | 37.6 | 56.5 | 84.7 | 140.9 | 235.0 |
| 525 | 1.9 | 25.8 | 34.4 | 51.6 | 77.4 | 129.0 | 214.9 |
| 538 | 1.4 | 25.2 | 33.4 | 50.0 | 75.2 | 125.5 | 208.9 |
| 550 | (1) | 25.0 | 33.3 | 49.8 | 74.8 | 124.9 | 208.0 |
| 575 | (1) | 24.0 | 31.9 | 47.9 | 71.8 | 119.7 | 199.5 |
| 600 | (1) | 21.6 | 28.6 | 42.9 | 64.2 | 107.0 | 178.5 |
| 625 | (1) | 18.3 | 24.3 | 36.6 | 54.9 | 91.2 | 152.0 |
| 650 | (1) | 14.1 | 18.9 | 28.1 | 42.5 | 70.7 | 117.7 |
| 675 | (1) | 12.4 | 16.9 | 25.2 | 37.6 | 62.7 | 104.5 |
| 700 | (1) | 10.1 | 13.4 | 20.0 | 29.8 | 49.7 | 83.0 |
| 725 | (1) | 7.9 | 10.5 | 15.4 | 23.2 | 38.6 | 64.4 |
| 750 | (1) | 5.9 | 7.9 | 11.7 | 17.6 | 29.6 | 49.1 |
| 775 | (1) | 4.6 | 6.2 | 9.0 | 13.7 | 22.8 | 38.0 |
| 800 | (1) | 3.5 | 4.8 | 7.0 | 10.5 | 17.4 | 29.2 |
| 816 | (1) | 2.8 | 3.8 | 5.9 | 8.6 | 14.1 | 23.8 |

NOTE:  (1)  Ratings for flanges and flanged fittings terminate at 538°C (1,000°F).

##### MAXIMUM  RATINGS

The rules for establishing pressure–temperature ratings include consideration of ceiling pressures pc that effectively set limits on the selected stress. The ceiling pressure–temperature values set an upper

bound for high-strength materials and are imposed to limit deflection. Ceiling pressure values are listed in Tables A-1 and A-2. Ratings in excess of these ceiling values are not permitted under this Standard.

196

ASME  B16.5-2020

Temperature,

###### Table A-2 Rating Ceiling Pressure —  pc, psi

Class

| °F | 150 | 300 | 400 | 600 | 900 | 1500 | 2500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| −20 to 100 | 290 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 200 | 260 | 750 | 1,000 | 1,500 | 2,250 | 3,750 | 6,250 |
| 300 | 230 | 730 | 970 | 1,455 | 2,185 | 3,640 | 6,070 |
| 400 | 200 | 705 | 940 | 1,410 | 2,115 | 3,530 | 5,880 |
| 500 | 170 | 665 | 885 | 1,330 | 1,995 | 3,325 | 5,540 |
| 600 | 140 | 605 | 805 | 1,210 | 1,815 | 3,025 | 5,040 |
| 650 | 125 | 590 | 785 | 1,175 | 1,765 | 2,940 | 4,905 |
| 700 | 110 | 570 | 755 | 1,135 | 1,705 | 2,840 | 4,730 |
| 750 | 95 | 530 | 710 | 1,065 | 1,595 | 2,660 | 4,430 |
| 800 | 80 | 510 | 675 | 1,015 | 1,525 | 2,540 | 4,230 |
| 850 | 65 | 485 | 650 | 975 | 1,460 | 2,435 | 4,060 |
| 900 | 50 | 450 | 600 | 900 | 1,350 | 2,245 | 3,745 |
| 950 | 35 | 385 | 515 | 775 | 1,160 | 1,930 | 3,220 |
| 1,000 | 20 | 365 | 485 | 725 | 1,090 | 1,820 | 3,030 |
| 1,050 | (1) | 360 | 480 | 720 | 1,080 | 1,800 | 3,000 |
| 1,100 | (1) | 325 | 430 | 645 | 965 | 1,610 | 2,685 |
| 1,150 | (1) | 275 | 365 | 550 | 825 | 1,370 | 2,285 |
| 1,200 | (1) | 205 | 275 | 410 | 620 | 1,030 | 1,715 |
| 1,250 | (1) | 180 | 245 | 365 | 545 | 910 | 1,515 |
| 1,300 | (1) | 140 | 185 | 275 | 410 | 685 | 1,145 |
| 1,350 | (1) | 105 | 140 | 205 | 310 | 515 | 860 |
| 1,400 | (1) | 75 | 100 | 150 | 225 | 380 | 630 |
| 1,450 | (1) | 60 | 80 | 115 | 175 | 290 | 485 |
| 1,500 | (1) | 40 | 55 | 85 | 125 | 205 | 345 |

NOTE:  (1)  Ratings of flanges and flanged fittings terminate at 1,000°F (538°C).

197

ASME  B16.5-2020

## NONMANDATORY  APPENDIX  B LIMITING DIMENSIONS OF GASKETS OTHER THAN RING JOINT GASKETS

##### GASKET MATERIALS AND CONSTRUCTION

Classification of gasket materials and types is shown in Table B-1. Other gaskets that result in no increase in bolt loads or flange moment over those resulting from the gaskets included in the respective groups may be used (see para. 5.4).

##### GASKET DIMENSIONS

Reference to a dimensional standard for gaskets (e.g., ASME B16.21) is recommended. In any event, selected dimensions should be based on the type of gasket and

its characteristics. These characteristics include density, flexibility, compatibility with the fluid being contained, and gasket compression needed to maintain sealing. Consideration should be given to the need for a “pocket” at the gasket inside diameter (between the flange facings) or of intrusion of the gasket into the flange bore. Consideration should also be given to the effects that the contained fluid may have upon the gasket, including damage that may result from partial disintegration of the gasket material.

198

ASME  B16.5-2020

###### Table B-1 Gasket Groups and Typical Materials

| Gasket Group Number | Gasket Material Sketches |
| --- | --- |
| Ia | Self-energizing types: O-rings, metallic, elastomer, other gasket types considered as self-sealing |
|  | Elastomer without fabric |
|  | Compressed sheet suitable for the operating conditions |
|  | Fluoropolymer, elastomer with cotton fabric insertion |
|  | Elastomer with or without wire reinforcement |
|  | Vegetable fiber |
| Ib | Spiral-wound metal, with nonmetallic filler |
|  | Corrugated aluminum, copper or copper alloy, or corrugated aluminum, copper or copper alloy double jacketed with nonmetallic filler |
|  | Corrugated aluminum, copper, or brass |
| IIa and IIb IIIa and IIIb | Corrugated metal or corrugated metal double jacketed with nonmetallic filler |
|  | Corrugated metal |
|  | Flat metal jacketed with nonmetallic filler |
|  | Grooved metal |
|  | Solid flat soft aluminum Solid flat metal |
|  | Ring joint |

199

ASME  B16.5-2020

## NONMANDATORY  APPENDIX C METHOD FOR CALCULATING BOLT LENGTHS1

The following equations were used in establishing dimension L as listed in various dimensional tables:

LCSB = A + n

LCMB = B + n

For lapped joints, calculate stud bolt and machine bolt length as follows:

    - For ring joint groove facing

LCSB = A + (pipe thickness for each lap) + n

LCMB = B + (pipe thickness for each lap) + n

    - For other than ring joint facing

LCSB = A	F + (Table C-1 thickness) + n

LCMB = B	F + (Table C-1 thickness) + n

where

A = 2(tf + t + d) + G + F − a (i.e., stud bolt length exclusive of negative length tolerance, n)

a = zero, except where the small female face is on the end of pipe, a = 5 mm (0.19 in.)

B = 2(tf + t) + d + G + F + p − a (i.e., machine bolt length exclusive of negative tolerance, n)

d = heavy nut thickness (equals nominal bolt diameter, see ASME B18.2.2)

F = total height of facings or depth of ring joint groove for both flanges (see Table C-2)

G = 3.0 mm (0.12 in.) gasket thickness for raised face, male and female tongue-and-groove flanges; also approximate distance between ring joint flanges listed in Table 5 (Table 5C)

LCMB = calculated machine bolt length as measured from underside of head to end of point

LCSB = calculated stud bolt length (effective thread length, excluding end points)

LSMB = specified machine bolt length (from underside of head to end, including end point), which is LCMB rounded off to the nearest 5 mm (0.25 in.) increment (see Figure C-1)

LSSB = specified stud bolt length (effective thread length, excluding end points), which is LCSB rounded off to the nearest 5 mm (0.25 in.) increment (see Figure C-2)

n  = negative tolerance on bolt length (see

Table C-3)

p  = allowance for height of point of machine bolt

(1.5 times thread pitch)

t = plus tolerance for flange thickness (see

para. 7.4)

tf   = minimum flange thickness (see applicable dimensional tables)

###### Figure C-1 Specified Machine Bolt Length

Figure C-2 Specified Stud Bolt Length

1 The equations used in this Nonmandatory Appendix are for calculated bolt lengths established to ensure full thread engagement of heavy hexagon nuts when worst case tolerances occur on all relevant dimen- sions of the flanged joint. The use of shorter bolt lengths is acceptable provided that full thread engagement is obtained at assembly (see para. 6.10.2).

200

ASME  B16.5-2020

###### Table C-1 Thickness for Lapped Joints

Lap Combination	Classes 150 Through 2500 Flanges

For lapped to 2 mm (0.06 in.) male face on flange	One lap and 2 mm (0.06 in.) For lapped to lapped	Both laps

For lapped to 7 mm (0.25 in.) male face on flange	One lap and 7 mm (0.25 in.)

For lapped to female face on flange	One lap not less than 7 mm (0.25 in.)

For male in lap to female in lap	2 × pipe wall with lap for male not less than 7 mm (0.25 in.)

###### Table C-2 F Values

|  |  |  | Male and Female |  |
| --- | --- | --- | --- | --- |
|  | 2 mm Raised | 7 mm Raised | or |  |
| Class | 0.06 in. | 0.25 in. | Tongue and Groove | Ring Joint |
| 150 and 300 400 to 2500 | 4 mm (0.12) 4 mm (0.12) | 14 mm (0.50) 14 mm (0.50) | 7 mm (0.25) 7 mm (0.25) | 2 × groove depth 2 × groove depth |

Total Height of Facings or Depth of Ring Joint Groove for Both Flanges, F, mm (in.) Type of Flange Facing (1)

NOTE:

- See Figure 7 and Tables 4 and 5 (Tables 4C and 5C).

Stud Bolt

###### Table C-3 n Values

Dimensions	Negative Tolerance on Bolt Lengths, n, mm (in.)	Length, mm (in.)

A	1.5 (0.06)	≤305 (≤12)

or

[A + (pipe thickness for each lap)]	3.0 (0.12)	>305 (>12), ≤460 (≤18) or

| B |  |  |
| --- | --- | --- |
| or |  |  |
| [B + (pipe thickness for each lap)] | For n values, use negative length | … |
| or | tolerances per ASME B18.2.1 |  |
| [B − F + (Table C-1 thickness)] |  |  |

[A − F + (Table C-1 thickness)]	7.0 (0.25)	>460 (>18) Machine Bolt

201

ASME  B16.5-2020

## NONMANDATORY  APPENDIX  D QUALITY SYSTEM PROGRAM

The products manufactured in accordance with this Standard shall be produced under a quality system program following the principles of an appropriate stan- dard from the ISO 9000 series.1 A determination of the need for registration and/or certification of the product manufacturer’s quality system program by an independent organization shall be the responsibility of the manufacturer. The detailed documentation demon-

strating program compliance shall be available to the purchaser at the manufacture’s facility. A written summary description of the program utilized by the product manufacturer shall be available to the purchaser upon request. The product manufacturer is defined as the entity whose name or trademark appears on the product in accordance with the marking or identification require- ments of this Standard.

1 The series is also available from the American National Standards Institute (ANSI) and the American Society for Quality (ASQ) as American National Standards that are identified by a prefix “Q,” replacing the prefix “ISO.” Each standard of the series is listed under References in Mandatory Appendix II.

202

ASME  B16.5-2020

## NONMANDATORY APPENDIX E  DIMENSIONS OF CLASSES 400, 600, 900, 1500, AND 2500

FLANGED FITTINGS IN U.S. CUSTOMARY UNITS

This  Nonmandatory  Appendix  contains  Tables  E-1

through E-5.

203

ASME  B16.5-2020

###### Table E-1 Dimensions of Class 400 Flanged Fittings

| Minimum Inside Outside Minimum Wall Diamete- Diameter Thickness Thickness r Nominal of of of of Pipe Flange, Flange, Fitting, Fitting, Size O tf tm d | Flange Edge | Raised Face 0.25 in. (1) |
| --- | --- | --- |
|  | Center- to- Short Flange Center- Edge, to- Elbow, Center- Long Flange Flange Tee, to- Center- Edge, Edge- Cross, Flange to- Lateral, to- and Edge, Flange and Flange True 45-deg Edge, True Edge, “Y,” Elbow, Lateral, “Y,” Reducer, A C E F G | Center- to- Short Contact Center- Surface to- of Center- Long Contact Raised to- Center- Surface Face Contact to- of Elbow, Surface Contact Raised Tee, of Surface Face, Cross, Raised of Lateral, and Face, Raised and True 45-deg Face, True “Y,” Elbow, Lateral, “Y,” AA CC EE FF |

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| 1∕2 3∕4 1 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
|  | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 11∕4 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 11∕2 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 2 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 21∕2 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 3 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 31∕2 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 4 | 10.00 | 1.38 | 0.38 | 4.00 | 7.75 | 5.25 | 15.75 | 4.25 | 7.75 | 8.00 | 5.50 | 16.00 | 4.50 |
| 5 | 11.00 | 1.50 | 0.44 | 5.00 | 8.75 | 5.75 | 16.50 | 4.75 | 8.75 | 9.00 | 6.00 | 16.75 | 5.00 |
| 6 | 12.50 | 1.62 | 0.44 | 6.00 | 9.50 | 6.00 | 18.50 | 5.00 | 9.50 | 9.75 | 6.25 | 18.75 | 5.25 |
| 8 | 15.00 | 1.88 | 0.56 | 8.00 | 11.50 | 6.50 | 22.00 | 5.50 | 11.50 | 11.75 | 6.75 | 22.25 | 5.75 |
| 10 | 17.50 | 2.12 | 0.69 | 10.00 | 13.00 | 7.50 | 25.50 | 6.00 | 13.00 | 13.25 | 7.75 | 25.75 | 6.25 |
| 12 | 20.50 | 2.25 | 0.75 | 12.00 | 14.75 | 8.50 | 29.50 | 6.25 | 14.75 | 15.00 | 8.75 | 29.75 | 6.50 |

204

ASME  B16.5-2020

###### Table E-1 Dimensions of Class 400 Flanged Fittings

| Raised Face 0.25 in. (1) | Ring Joint (1) |  |  |  |  | Diameter of Round Base or Center- Width Thickness to- of of Base, Square Base, Thickness R (4)– Base, T of Ribs, (6) S (4) (4), (7) U (4) | Base Drilling (8) |  | Nominal Pipe Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Surface- to- Contact Surface of Raised Face Reducer, GG (2) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (3) | Center- to-End, 45-deg Elbow, KK (3) | Long Center- to- End, Lateral, LL (3) | Short Center- to- End, Lateral and True “Y,” MM (3) | End-to-End Reducer, NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

15	16	17	18	19	20	21	22	23	24	25	26	1

| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 1∕2 3∕4 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |  |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |  |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 11∕4 |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 11∕2 |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 2 |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 21∕2 |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 3 |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |  |
| 8.25 | 8.06 | 5.56 | 16.06 | 4.56 | (2), (3) | 6.00 | 6.50 | 0.88 | 0.62 | 5.00 | 0.75 | 4 |
| 9.25 | 9.06 | 6.06 | 16.81 | 5.06 | (2), (3) | 6.75 | 7.50 | 1.00 | 0.75 | 5.88 | 0.88 | 5 |
| 10.00 | 9.81 | 6.31 | 18.81 | 5.31 | (2), (3) | 7.50 | 7.50 | 1.00 | 0.75 | 5.88 | 0.88 | 6 |
| 12.00 | 11.81 | 6.81 | 22.31 | 5.81 | (2), (3) | 9.00 | 10.00 | 1.25 | 0.88 | 7.88 | 0.88 | 8 |
| 13.50 | 13.31 | 7.81 | 25.81 | 6.31 | (2), (3) | 10.50 | 10.00 | 1.25 | 0.88 | 7.88 | 0.88 | 10 |
| 15.25 | 15.06 | 8.81 | 29.81 | 6.56 | (2), (3) | 12.00 | 12.50 | 1.44 | 1.00 | 10.62 | 0.88 | 12 |

205

| 14 | 23.00 | 2.38 | 0.81 | 13.12 | 16.00 | 9.00 | 32.50 | 6.75 | 16.00 | 16.25 | 9.25 | 32.75 | 7.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 25.50 | 2.50 | 0.88 | 15.00 | 17.50 | 10.00 | 36.00 | 7.75 | 18.00 | 17.75 | 10.25 | 36.25 | 8.00 |
| 18 | 28.00 | 2.62 | 0.94 | 17.00 | 19.00 | 10.50 | 39.00 | 8.25 | 19.00 | 19.25 | 10.75 | 39.25 | 8.50 |
| 20 | 30.50 | 2.75 | 1.06 | 18.88 | 20.50 | 11.00 | 42.50 | 8.75 | 20.50 | 20.75 | 11.25 | 42.75 | 9.00 |
| 24 | 36.00 | 3.00 | 1.19 | 22.62 | 24.00 | 12.50 | 50.00 | 10.25 | 24.00 | 24.25 | 12.75 | 50.25 | 10.50 |

ASME  B16.5-2020

###### Table E-1 Dimensions of Class 400 Flanged Fittings  (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| Minimum Inside Outside Minimum Wall Diamete- Diameter Thickness Thickness r Nominal of of of of Pipe Flange, Flange, Fitting, Fitting, Size O tf tm d | Flange Edge | Raised Face 0.25 in. (1) |
| --- | --- | --- |
|  | Center- to- Short Flange Center- Edge, to- Elbow, Center- Long Flange Flange Tee, to- Center- Edge, Edge- Cross, Flange to- Lateral, to- and Edge, Flange and Flange True 45-deg Edge, True Edge, “Y,” Elbow, Lateral, “Y,” Reducer, A C E F G | Center- to- Short Contact Center- Surface to- of Center- Long Contact Raised to- Center- Surface Face Contact to- of Elbow, Surface Contact Raised Tee, of Surface Face, Cross, Raised of Lateral, and Face, Raised and True 45-deg Face, True “Y,” Elbow, Lateral, “Y,” AA CC EE FF |

206

| 16.50 | 16.31 | 9.31 | 32.81 | 7.06 | (2), (3) | 13.50 | 12.50 | 1.44 | 1.00 | 10.62 | 0.88 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18.50 | 17.81 | 10.31 | 36.31 | 8.06 | (2), (3) | 14.75 | 12.50 | 1.44 | 1.12 | 10.62 | 0.88 | 16 |
| 19.50 | 19.31 | 10.81 | 39.31 | 8.56 | (2), (3) | 16.25 | 15.00 | 1.62 | 1.12 | 13.00 | 1.00 | 18 |
| 21.00 | 20.88 | 11.38 | 42.88 | 9.12 | (2), (3) | 17.88 | 15.00 | 1.62 | 1.25 | 13.00 | 1.00 | 20 |
| 24.50 | 24.44 | 12.94 | 50.44 | 10.69 | (2), (3) | 20.75 | 17.50 | 1.88 | 1.25 | 15.25 | 1.12 | 24 |

ASME  B16.5-2020

###### Table E-1 Dimensions of Class 400 Flanged Fittings  (Cont’d)

15	16	17	18	19	20	21	22	23	24	25	26	1

| Raised Face 0.25 in. (1) | Ring Joint (1) |  |  |  |  | Diameter of Round Base or Center- Width Thickness to- of of Base, Square Base, Thickness R (4)– Base, T of Ribs, (6) S (4) (4), (7) U (4) | Base Drilling (8) |  | Nominal Pipe Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Surface- to- Contact Surface of Raised Face Reducer, GG (2) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (3) | Center- to-End, 45-deg Elbow, KK (3) | Long Center- to- End, Lateral, LL (3) | Short Center- to- End, Lateral and True “Y,” MM (3) | End-to-End Reducer, NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

207

ASME  B16.5-2020

###### Table E-1 Dimensions of Class 400 Flanged Fittings (Cont’d)

GENERAL NOTES:

  - Dimensions are in inches.

  - For tolerances, see section 7.

  - For facings, see para. 6.4.

  - For flange bolt holes, see para. 6.5 and Table 13C.

  - For spot facing, see para. 6.6.

  - For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4.

  - For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

  - For reinforcement of certain fittings, see para. 6.1.

  - For drains, see para. 6.12.

NOTES:

    - For center-to-contact surface and center-to-end dimensions of reducing fittings, see para. 6.2.3.

    - These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For the center-to-end dimensions of reducing fittings or end-to- end dimensions of reducers, use center-to-flange edge or flange edge-to-flange edge dimensions for largest opening, and add the proper height to provide for ring joint groove applying to each flange. See Table 5C for ring joint facing dimensions.

    - For contact surface-to-surface and end-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

    - The base dimensions apply to all straight and reducing sizes.

    - For reducing fittings, the size and center-to-face dimensions of the base are determined by the size of the largest opening of fitting. In the case of reducing base elbows, orders shall specify whether the base shall be opposite the larger or smaller opening.

    - Bases shall be plain faced unless otherwise specified, and the center-to-base face dimension R shall be the finished dimension.

    - Bases may be cast integral or attached as weldments at the option of the manufacturer.

    - The bases of these fittings are intended for support in compression and are not to be used for anchors or supports in tension or shear.

    - Use Class 600 dimensions in this size.

208

ASME  B16.5-2020

TABLE STARTS ON NEXT PAGE

209

ASME  B16.5-2020

###### Table E-2 Dimensions of Class 600 Flanged Fittings

| Minimum Wall Inside Outside Minimum Thickness Diamete- Nominal Diameter Thickness of r of Pipe of Flange, of Flange, Fitting, Fitting, Size O tm tm d | Flange Edge | Raised Face 0.25 in. (1) |
| --- | --- | --- |
|  | Center- to- Short Flange Center- Edge, to- Elbow, Center- Long Flange Flange Tee, to- Center- Edge, Edge- Cross, Flange to- Lateral, to- and Edge, Flange and Flange True 45-deg Edge, True Edge, “Y,” Elbow, Lateral, “Y,” Reducer, A C E F G | Center- to- Short Contact Center- Surface to- of Center- Long Contact Raised to- Center- Surface Face, Contact to- of Elbow, Surface Contact Raised Tee, of Surface Face, Cross, Raised of Lateral, and Face, Raised and True 45-deg Face, True “Y,” Elbow, Lateral, “Y,” AA CC EE FF |

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| 1∕2 3∕4 | 3.75 | 0.56 | 0.16 | 0.50 | 3.00 | 1.75 | 5.50 | 1.50 | 4.50 | 3.25 | 2.00 | 5.75 | 1.75 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4.62 | 0.62 | 0.16 | 0.75 | 3.50 | 2.25 | 6.50 | 1.75 | 4.50 | 3.75 | 2.50 | 6.75 | 2.00 |
| 1 | 4.88 | 0.69 | 0.19 | 1.00 | 4.00 | 2.25 | 7.00 | 2.00 | 4.50 | 4.25 | 2.50 | 7.25 | 2.25 |
| 11∕4 | 5.25 | 0.81 | 0.19 | 1.25 | 4.25 | 2.50 | 7.75 | 2.25 | 4.50 | 4.50 | 2.75 | 8.00 | 2.50 |
| 11∕2 | 6.12 | 0.88 | 0.22 | 1.50 | 4.50 | 2.75 | 8.75 | 2.50 | 4.50 | 4.75 | 3.00 | 9.00 | 2.75 |
| 2 | 6.50 | 1.00 | 0.25 | 2.00 | 5.50 | 4.00 | 10.00 | 3.25 | 5.50 | 5.75 | 4.25 | 10.25 | 3.50 |
| 21∕2 | 7.50 | 1.12 | 0.28 | 2.50 | 6.25 | 4.25 | 11.25 | 3.25 | 6.25 | 6.50 | 4.50 | 11.50 | 3.50 |
| 3 | 8.25 | 1.25 | 0.31 | 3.00 | 6.75 | 4.75 | 12.50 | 3.75 | 6.75 | 7.00 | 5.00 | 12.75 | 4.00 |
| 31∕2 | 9.00 | 1.38 | 0.34 | 3.50 | 7.25 | 5.25 | 13.75 | 4.25 | 7.25 | 7.50 | 5.50 | 14.00 | 4.50 |
| 4 | 10.75 | 1.50 | 0.38 | 4.00 | 8.25 | 5.75 | 16.25 | 4.25 | 8.25 | 8.50 | 6.00 | 16.50 | 4.50 |
| 5 | 13.00 | 1.75 | 0.44 | 5.00 | 9.75 | 6.75 | 19.25 | 5.75 | 9.75 | 10.00 | 7.00 | 19.50 | 6.00 |
| 6 | 14.00 | 1.88 | 0.50 | 6.00 | 10.75 | 7.25 | 20.75 | 6.25 | 10.75 | 11.00 | 7.50 | 21.00 | 6.50 |
| 8 | 16.50 | 2.19 | 0.62 | 7.88 | 12.75 | 8.25 | 24.25 | 6.75 | 12.75 | 13.00 | 8.50 | 24.50 | 7.00 |
| 10 | 20.00 | 2.50 | 0.75 | 9.75 | 15.25 | 9.25 | 29.25 | 7.75 | 15.25 | 15.50 | 9.50 | 29.50 | 8.00 |
| 12 | 22.00 | 2.62 | 0.91 | 11.75 | 16.25 | 9.75 | 31.25 | 8.25 | 16.25 | 16.50 | 10.00 | 31.50 | 8.50 |
| 14 | 23.75 | 2.75 | 0.97 | 12.88 | 17.25 | 10.50 | 34.00 | 8.75 | 17.25 | 17.50 | 10.75 | 34.25 | 9.00 |

210

ASME  B16.5-2020

###### Table E-2 Dimensions of Class 600 Flanged Fittings

| Raised Face 0.25 in. (1) | Ring Joint (1) |  |  |  |  | Diameter of Round Center- Base or Thickness to- Width of of Base, Square Base, Thickness R (4)– Base, T of Ribs, (6) S (4) (4), (7) U (4) | Base Drilling (8) |  | Nominal Pipe Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Surface- to- Contact Surface of Raised Face Reducer, GG (2) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (3) | Center- to-End, 45-deg Elbow, KK (3) | Long Center- to-End Lateral, LL (3) | Short Center- to-End, Lateral, and True “Y,” MM (3) | End-to-End Reducer, NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

15	16	17	18	19	20	21	22	23	24	25	26	1

| 5.00 | 3.22 | 1.97 | 5.72 | 1.72 | (2), (3) | … | … | … | … | … | … | 1∕2 3∕4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.00 | 3.75 | 2.50 | 6.75 | 2.00 | (2), (3) | … | … | … | … | … | … |  |
| 5.00 | 4.25 | 2.50 | 7.25 | 2.25 | (2), (3) | … | … | … | … | … | … | 1 |
| 5.00 | 4.50 | 2.75 | 8.00 | 2.50 | (2), (3) | … | … | … | … | … | … | 11∕4 |
| 5.00 | 4.75 | 3.00 | 9.00 | 2.75 | (2), (3) | … | … | … | … | … | … | 11∕2 |
| 6.00 | 5.81 | 4.31 | 10.31 | 3.56 | (2), (3) | 4.75 | 6.12 | 0.81 | 0.62 | 4.50 | 0.88 | 2 |
| 6.75 | 6.56 | 4.56 | 11.56 | 3.56 | (2), (3) | 5.25 | 6.12 | 0.81 | 0.62 | 4.50 | 0.88 | 21∕2 |
| 7.25 | 7.06 | 5.06 | 12.81 | 4.06 | (2), (3) | 5.75 | 6.50 | 0.88 | 0.75 | 5.00 | 0.75 | 3 |
| 7.75 | 7.56 | 5.56 | 14.06 | 4.56 | (2), (3) | 6.50 | 6.50 | 0.88 | 0.75 | 5.00 | 0.75 | 31∕2 |
| 8.75 | 8.56 | 6.06 | 16.56 | 4.56 | (2), (3) | 7.00 | 7.50 | 1.00 | 0.75 | 5.88 | 0.88 | 4 |
| 10.25 | 10.06 | 7.06 | 19.56 | 6.06 | (2), (3) | 8.25 | 10.00 | 1.25 | 0.75 | 7.88 | 0.88 | 5 |
| 11.25 | 11.06 | 7.56 | 21.06 | 6.56 | (2), (3) | 9.00 | 10.00 | 1.25 | 0.75 | 7.88 | 0.88 | 6 |
| 13.25 | 13.06 | 8.56 | 24.56 | 7.06 | (2), (3) | 11.00 | 12.50 | 1.44 | 1.00 | 10.62 | 0.88 | 8 |
| 15.75 | 15.56 | 9.56 | 29.56 | 8.06 | (2), (3) | 12.50 | 12.50 | 1.44 | 1.00 | 10.62 | 0.88 | 10 |
| 16.75 | 16.56 | 10.06 | 31.56 | 8.56 | (2), (3) | 13.25 | 15.00 | 1.62 | 1.12 | 13.00 | 1.00 | 12 |
| 17.75 | 17.56 | 10.81 | 34.31 | 9.06 | (2), (3) | 14.75 | 15.00 | 1.62 | 1.12 | 13.00 | 1.00 | 14 |

211

| 16 | 27.00 | 3.00 | 1.09 | 14.75 | 19.25 | 11.50 | 38.25 | 9.75 | 19.25 | 19.50 | 11.75 | 38.50 | 10.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | 29.25 | 3.25 | 1.22 | 16.50 | 21.25 | 12.00 | 41.75 | 10.25 | 21.25 | 21.50 | 12.25 | 42.00 | 10.50 |
| 20 | 32.00 | 3.50 | 1.34 | 18.25 | 23.25 | 12.75 | 45.25 | 10.75 | 23.25 | 23.50 | 13.00 | 45.50 | 11.00 |
| 24 | 37.00 | 4.00 | 1.59 | 22.00 | 27.25 | 14.50 | 52.75 | 12.75 | 27.25 | 27.50 | 14.75 | 53.00 | 13.00 |

ASME  B16.5-2020

###### Table E-2 Dimensions of Class 600 Flanged Fittings  (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| Minimum Wall Inside Outside Minimum Thickness Diamete- Nominal Diameter Thickness of r of Pipe of Flange, of Flange, Fitting, Fitting, Size O tm tm d | Flange Edge | Raised Face 0.25 in. (1) |
| --- | --- | --- |
|  | Center- to- Short Flange Center- Edge, to- Elbow, Center- Long Flange Flange Tee, to- Center- Edge, Edge- Cross, Flange to- Lateral, to- and Edge, Flange and Flange True 45-deg Edge, True Edge, “Y,” Elbow, Lateral, “Y,” Reducer, A C E F G | Center- to- Short Contact Center- Surface to- of Center- Long Contact Raised to- Center- Surface Face, Contact to- of Elbow, Surface Contact Raised Tee, of Surface Face, Cross, Raised of Lateral, and Face, Raised and True 45-deg Face, True “Y,” Elbow, Lateral, “Y,” AA CC EE FF |

212

| 19.75 | 19.56 | 11.81 | 38.56 | 10.06 | (2), (3) | 16.00 | 15.00 | 1.62 | 1.25 | 13.00 | 1.00 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21.75 | 21.56 | 12.31 | 42.06 | 10.56 | (2), (3) | … | … | … | … | … | … | 18 |
| 23.75 | 23.62 | 13.12 | 45.62 | 11.12 | (2), (3) | … | … | … | … | … | … | 20 |
| 27.75 | 27.69 | 14.94 | 53.19 | 13.19 | (2), (3) | … | … | … | … | … | … | 24 |

ASME  B16.5-2020

###### Table E-2 Dimensions of Class 600 Flanged Fittings  (Cont’d)

15	16	17	18	19	20	21	22	23	24	25	26	1

| Raised Face 0.25 in. (1) | Ring Joint (1) |  |  |  |  | Diameter of Round Center- Base or Thickness to- Width of of Base, Square Base, Thickness R (4)– Base, T of Ribs, (6) S (4) (4), (7) U (4) | Base Drilling (8) |  | Nominal Pipe Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Surface- to- Contact Surface of Raised Face Reducer, GG (2) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (3) | Center- to-End, 45-deg Elbow, KK (3) | Long Center- to-End Lateral, LL (3) | Short Center- to-End, Lateral, and True “Y,” MM (3) | End-to-End Reducer, NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

213

ASME  B16.5-2020

###### Table E-2 Dimensions of Class 600 Flanged Fittings  (Cont’d)

GENERAL NOTES:

      - Dimensions are in inches.

      - For tolerances, see section 7.

      - For facings, see para. 6.4.

      - For flange bolt holes, see para. 6.5 and Table 15C.

      - For spot facing, see para. 6.6.

      - For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4.

      - For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

      - For reinforcement of certain fittings, see para. 6.1.

      - For drains, see para. 6.12.

NOTES:

        - For center-to-contact surface and center-to-end dimensions of reducing fittings, see para. 6.2.3.

        - For contact surface-to-contact surface and end-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

        - These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For the center-to-end dimensions of reducing fittings or end-to- end dimensions of reducers, use center-to-flange edge or flange edge-to-flange edge dimensions for largest opening, and add the proper height to provide for ring joint groove applying to each flange. See Table 5C for ring joint facing dimensions.

        - The base dimensions apply to all straight and reducing sizes.

        - For reducing fittings, the size and center-to-face dimensions of the base are determined by the size of the largest opening of fitting. In the case of reducing base elbows, orders shall specify whether the base shall be opposite the larger or smaller opening.

        - Bases shall be plain faced unless otherwise specified, and the center-to-base face dimension R shall be the finished dimension.

        - Bases may be cast integral or attached as weldments at the option of the manufacturer.

        - The bases of these fittings are intended for support in compression and are not to be used for anchors or supports in tension or shear.

214

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

215

ASME  B16.5-2020

###### Table E-3 Dimensions of Class 900 Flanged Fittings

| Minimum Inside Outside Minimum Wall Diamete- Nominal Diameter Thickness Thickness r of Pipe of Flange, of Flange, of Fitting, Fitting, Size O tf tm d | Flange Edge | Raised Face 0.25 in. (1) |
| --- | --- | --- |
|  | Center- to- Short Flange Center- Edge, to- Elbow, Center- Long Flange Tee, to- Center- Edge, Flange Cross, Flange to- Lateral, Edge-to- and Edge, Flange and Flange True 45-deg Edge, True Edge, “Y,” Elbow, Lateral, “Y,” Reducer, A C E F G | Center- to- Short Contact Center- Surface to- of Center- Long Contact Raised to- Center- Surface Face Contact to- of Elbow, Surface Contact Raised Tee, of Surface Face, Cross, Raised of Lateral, and Face, Raised and True 45-deg Face, True “Y,” Elbow, Lateral, “Y,” AA CC EE FF |

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| 1∕2 3∕4 1 | 4.75 | 0.88 | 0.16 | 0.50 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5.12 | 1.00 | 0.19 | 0.69 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
|  | 5.88 | 1.12 | 0.22 | 0.88 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 11∕4 | 6.25 | 1.12 | 0.25 | 1.12 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 11∕2 | 7.00 | 1.25 | 0.28 | 1.38 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 2 | 8.50 | 1.50 | 0.31 | 1.88 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 21∕2 | 9.62 | 1.62 | 0.34 | 2.25 | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |
| 3 | 9.50 | 1.50 | 0.41 | 2.88 | 7.25 | 5.25 | 14.25 | 4.25 | 7.25 | 7.50 | 5.50 | 14.50 | 4.50 |
| 4 | 11.50 | 1.75 | 0.50 | 3.88 | 8.75 | 6.25 | 17.25 | 5.25 | 8.75 | 9.00 | 6.50 | 17.50 | 5.50 |
| 5 | 13.75 | 2.00 | 0.59 | 4.75 | 10.75 | 7.25 | 20.75 | 6.25 | 10.75 | 11.00 | 7.50 | 21.00 | 6.50 |
| 6 | 15.00 | 2.19 | 0.72 | 5.75 | 11.75 | 7.75 | 22.25 | 6.25 | 11.75 | 12.00 | 8.00 | 22.50 | 6.50 |
| 8 | 18.50 | 2.50 | 0.88 | 7.50 | 14.25 | 8.75 | 27.25 | 7.25 | 14.25 | 14.50 | 9.00 | 27.50 | 7.50 |
| 10 | 21.50 | 2.75 | 1.06 | 9.38 | 16.25 | 9.75 | 31.25 | 8.25 | 16.25 | 16.50 | 10.00 | 31.50 | 8.50 |
| 12 | 24.00 | 3.12 | 1.25 | 11.12 | 18.75 | 10.75 | 34.25 | 8.75 | 17.25 | 19.00 | 11.00 | 34.50 | 9.00 |
| 14 | 25.25 | 3.38 | 1.38 | 12.25 | 20.00 | 11.25 | 36.25 | 9.25 | 18.50 | 20.25 | 11.50 | 36.50 | 9.50 |

216

ASME  B16.5-2020

###### Table E-3 Dimensions of Class 900 Flanged Fittings

| Raised Face 0.25 in. (1) | Ring Joint (1) |  |  |  |  | Diameter of Round Center- Base or Thickness to- Width of of Base, Square Base, Thickness S Base, T of Ribs, (4), (5) S (6) (6), (7) U (6) | Base Drilling (8) |  | Nominal Pipe Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Surface- to- Contact Surface of Raised Face Reducer, GG (2) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (3) | Center- to-End, 45-deg Elbow, KK (3) | Long Center- to-End, Lateral, LL (3) | Short Center- to-End, Lateral, and True “Y,” MM (3) | End-to-End Reducer, NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

15	16	17	18	19	20	21	22	23	24	25	26	1

| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 1∕2 3∕4 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |  |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) |  |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 11∕4 |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 11∕2 |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 2 |
| (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | (9) | 21∕2 |
| 7.75 | 7.56 | 5.56 | 14.56 | 4.56 | (2), (3) | 5.75 | 6.50 | 0.88 | 0.75 | 5.00 | 0.75 | 3 |
| 9.25 | 9.06 | 6.56 | 17.56 | 5.56 | (2), (3) | 7.00 | 7.50 | 1.00 | 0.75 | 5.88 | 0.88 | 4 |
| 11.25 | 11.06 | 7.56 | 21.06 | 6.56 | (2), (3) | 8.25 | 10.00 | 1.25 | 0.75 | 7.88 | 0.88 | 5 |
| 12.25 | 12.06 | 8.06 | 22.56 | 6.56 | (2), (3) | 9.00 | 10.00 | 1.25 | 0.75 | 7.88 | 0.88 | 6 |
| 14.75 | 14.56 | 9.06 | 27.56 | 7.56 | (2), (3) | 11.00 | 12.50 | 1.44 | 1.00 | 10.62 | 0.88 | 8 |
| 16.75 | 16.56 | 10.06 | 31.56 | 8.56 | (2), (3) | 12.50 | 12.50 | 1.44 | 1.00 | 10.62 | 0.88 | 10 |
| 17.75 | 19.06 | 11.06 | 34.56 | 9.06 | (2), (3) | 13.25 | 15.00 | 1.62 | 1.12 | 13.00 | 1.00 | 12 |
| 19.00 | 20.44 | 11.69 | 36.69 | 9.69 | (2), (3) | 14.75 | 15.00 | 1.62 | 1.12 | 13.00 | 1.00 | 14 |

217

ASME  B16.5-2020

###### Table E-3 Dimensions of Class 900 Flanged Fittings (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| Minimum Inside Outside Minimum Wall Diamete- Nominal Diameter Thickness Thickness r of Pipe of Flange, of Flange, of Fitting, Fitting, Size O tf tm d | Flange Edge | Raised Face 0.25 in. (1) |
| --- | --- | --- |
|  | Center- to- Short Flange Center- Edge, to- Elbow, Center- Long Flange Tee, to- Center- Edge, Flange Cross, Flange to- Lateral, Edge-to- and Edge, Flange and Flange True 45-deg Edge, True Edge, “Y,” Elbow, Lateral, “Y,” Reducer, A C E F G | Center- to- Short Contact Center- Surface to- of Center- Long Contact Raised to- Center- Surface Face Contact to- of Elbow, Surface Contact Raised Tee, of Surface Face, Cross, Raised of Lateral, and Face, Raised and True 45-deg Face, True “Y,” Elbow, Lateral, “Y,” AA CC EE FF |

| 16 | 27.75 | 3.50 | 1.56 | 14.00 | 22.00 | 12.25 | 40.50 | 10.25 | 20.50 | 22.25 | 12.50 | 40.75 | 10.25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | 31.00 | 4.00 | 1.75 | 15.75 | 23.75 | 13.00 | 45.25 | 11.75 | 24.00 | 24.00 | 13.25 | 45.50 | 12.00 |
| 20 | 33.75 | 4.25 | 1.91 | 17.50 | 25.75 | 14.25 | 50.00 | 12.75 | 26.00 | 26.00 | 14.50 | 50.25 | 13.00 |
| 24 | 41.00 | 5.50 | 2.28 | 21.00 | 30.25 | 17.75 | 59.75 | 15.25 | 30.00 | 30.50 | 18.00 | 60.00 | 15.50 |

218

ASME  B16.5-2020

###### Table E-3 Dimensions of Class 900 Flanged Fittings (Cont’d)

15	16	17	18	19	20	21	22	23	24	25	26	1

| Raised Face 0.25 in. (1) | Ring Joint (1) |  |  |  |  | Diameter of Round Center- Base or Thickness to- Width of of Base, Square Base, Thickness S Base, T of Ribs, (4), (5) S (6) (6), (7) U (6) | Base Drilling (8) |  | Nominal Pipe Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Surface- to- Contact Surface of Raised Face Reducer, GG (2) | Center- to-End Elbow, Tee, Cross, and True “Y,” HH (3) | Center- to-End, 45-deg Elbow, KK (3) | Long Center- to-End, Lateral, LL (3) | Short Center- to-End, Lateral, and True “Y,” MM (3) | End-to-End Reducer, NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

| 21.00 | 22.44 | 12.69 | 40.94 | 10.69 | (2), (3) | 16.00 | 15.00 | 1.62 | 1.25 | 13.00 | 1.00 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24.50 | 24.25 | 13.50 | 45.75 | 12.25 | (2), (3) | … | … | … | … | … | … | 18 |
| 26.50 | 26.25 | 14.75 | 50.50 | 13.25 | (2), (3) | … | … | … | … | … | … | 20 |
| 30.50 | 30.88 | 18.38 | 60.38 | 15.88 | (2), (3) | … | … | … | … | … | … | 24 |

219

ASME  B16.5-2020

###### Table E-3 Dimensions of Class 900 Flanged Fittings (Cont’d)

GENERAL NOTES:

          - Dimensions are in inches.

          - For tolerances, see section 7.

          - For facings, see para. 6.4.

          - For flange bolt holes, see para. 6.5 and Table 17C.

          - For spot facing, see para. 6.6.

          - For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4

          - For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

          - For reinforcement of certain fittings, see para. 6.1.

          - For drains, see para. 6.12.

NOTES:

            - For center-to-contact surface and center-to-end dimensions of reducing fittings, see para. 6.2.3.

            - For contact surface-to-contact surface and center-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

            - These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For the center-to-end dimensions of reducing fittings or end-to- end dimensions of reducers, use center-to-flange edge or flange edge-to-flange edge dimensions for largest opening, and add the proper height to provide for ring joint groove applying to each flange. See Table 5C for ring joint facing dimensions.

            - For reducing fittings, the size and center-to-face dimensions of the base are determined by the size of the largest opening fitting. In the case of reducing base elbows, orders shall specify whether the base shall be opposite the larger or smaller opening.

            - Bases shall be plain faced unless otherwise specified, and the center-to-base face dimension R shall be the finished dimension.

            - The base dimensions apply to all straight and reducing sizes.

            - Bases may be cast integral or attached as weldments at the option of the manufacturer.

            - The bases of these fittings are intended for support in compression and are not to be used for anchors or supports in tension or shear.

            - Use Class 1500 dimensions in this size.

220

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

221

ASME  B16.5-2020

###### Table E-4 Dimensions of Class 1500 Flanged Fittings

| Inside Outside Minimum Diamete- Diameter Minimum Wall r Nominal of Thickness Thickness of Pipe Flange, of Flange, of Fitting, Fitting, Size O tf tm d | Flange Edge | Raised Face 0.25 in. (1) |
| --- | --- | --- |
|  | Center- to- Short Flange Center- Edge, to- Elbow, Center- Long Flange Tee, to- Center- Edge, Flange Cross, Flange to- Lateral, Edge and Edge, Flange and to Flange True 45-deg Edge, True Edge “Y,” Elbow, Lateral, “Y,” Reducer, A C E F G | Center- to- Short Contact Center- Surface to- of Center- Long Contact Raised to- Center- Surface Face, Contact to- of Elbow, Surface Contact Raised Tee, of Surface Face, Cross, Raised of Lateral, and Face, Raised and True 45-deg Face, True “Y,” Elbow, Lateral, “Y,” AA CC EE FF |

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| 1∕2 3∕4 | 4.75 | 0.88 | 0.19 | 0.50 | 4.00 | 2.75 | … | … | … | 4.25 | 3.00 | … | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5.12 | 1.00 | 0.23 | 0.69 | 4.25 | 3.00 | … | … | … | 4.50 | 3.25 | … | … |
| 1 | 5.88 | 1.12 | 0.26 | 0.88 | 4.75 | 3.25 | 8.75 | 2.25 | 4.50 | 5.00 | 3.50 | 9.00 | 2.50 |
| 11∕4 | 6.25 | 1.12 | 0.31 | 1.12 | 5.25 | 3.75 | 9.75 | 2.75 | 5.25 | 5.50 | 4.00 | 10.00 | 3.00 |
| 11∕2 | 7.00 | 1.25 | 0.38 | 1.38 | 5.75 | 4.00 | 10.75 | 3.25 | 5.75 | 6.00 | 4.25 | 11.00 | 3.50 |
| 2 | 8.50 | 1.50 | 0.44 | 1.88 | 7.00 | 4.50 | 13.00 | 3.75 | 6.75 | 7.25 | 4.75 | 13.25 | 4.00 |
| 21∕2 | 9.62 | 1.62 | 0.50 | 2.25 | 8.00 | 5.00 | 15.00 | 4.25 | 7.75 | 8.25 | 5.25 | 15.25 | 4.50 |
| 3 | 10.50 | 1.88 | 0.62 | 2.75 | 9.00 | 5.50 | 17.00 | 4.75 | 8.75 | 9.25 | 5.75 | 17.25 | 5.00 |
| 4 | 12.25 | 2.12 | 0.75 | 3.62 | 10.50 | 7.00 | 19.00 | 5.75 | 10.25 | 10.75 | 7.25 | 19.25 | 6.00 |
| 5 | 14.75 | 2.88 | 0.91 | 4.38 | 13.00 | 8.50 | 23.00 | 7.25 | 13.25 | 13.25 | 8.75 | 23.25 | 7.50 |
| 6 | 15.50 | 3.25 | 1.09 | 5.38 | 13.62 | 9.12 | 24.62 | 7.88 | 14.00 | 13.88 | 9.38 | 24.88 | 8.12 |
| 8 | 19.00 | 3.62 | 1.41 | 7.00 | 16.12 | 10.62 | 29.62 | 8.88 | 16.50 | 16.38 | 10.88 | 29.88 | 9.12 |
| 10 | 23.00 | 4.25 | 1.72 | 8.75 | 19.25 | 11.75 | 35.75 | 10.00 | 19.75 | 19.50 | 12.00 | 36.00 | 10.25 |
| 12 | 26.50 | 4.88 | 2.00 | 10.38 | 22.00 | 13.00 | 40.50 | 11.75 | 22.50 | 22.25 | 13.25 | 40.75 | 12.00 |
| 14 | 29.50 | 5.25 | 2.19 | 11.38 | 24.50 | 14.00 | 43.75 | 12.25 | 25.25 | 24.75 | 14.25 | 44.00 | 12.50 |
| 16 | 32.50 | 5.75 | 2.50 | 13.00 | 27.00 | 16.00 | 48.00 | 14.50 | 27.75 | 27.25 | 16.25 | 48.25 | 14.75 |

222

ASME  B16.5-2020

###### Table E-4 Dimensions of Class 1500 Flanged Fittings

| Raised Face 0.25 in. (1) | Ring Joint (1) |  |  |  |  | Diameter of Round Base Center- or Width Thickness to- of of Base, Square Base, Thickness R (4)– Base, T of Ribs, (6) S (4) (4), (7) U (4) | Base Drilling (8) |  | Nominal Pipe Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Surface- to- Contact Surface of Raised Face Reducer, GG (2) | Center- to- End Elbow, Tee, Cross, and True “Y,” HH (3) | Center- to- End, 45-deg Elbow, KK (3) | Long Center- to-End, Lateral, LL (3) | Short Center- to-End, Lateral, and True “Y,” MM (3) | End-to-End Reducer, NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

15	16	17	18	19	20	21	22	23	24	25	26	1

| … | 4.25 | 3.00 | … | … | (2), (3) | … | … | … | … | … | … | 1∕2 3∕4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | 4.50 | 3.25 | … | … | (2), (3) | … | … | … | … | … | … |  |
| 5.00 | 5.00 | 3.50 | 9.00 | 2.50 | (2), (3) | … | … | … | … | … | … | 1 |
| 5.75 | 5.50 | 4.00 | 10.00 | 3.00 | (2), (3) | … | … | … | … | … | … | 11∕4 |
| 6.25 | 6.00 | 4.25 | 11.00 | 3.50 | (2), (3) | … | … | … | … | … | … | 11∕2 |
| 7.25 | 7.31 | 4.81 | 13.31 | 4.06 | (2), (3) | 5.50 | 6.50 | 0.88 | 0.75 | 5.00 | 0.75 | 2 |
| 8.25 | 8.31 | 5.31 | 15.31 | 4.56 | (2), (3) | 6.00 | 6.50 | 0.88 | 0.75 | 5.00 | 0.75 | 21∕2 |
| 9.25 | 9.31 | 5.81 | 17.31 | 5.06 | (2), (3) | 6.50 | 7.50 | 1.00 | 0.75 | 5.88 | 0.88 | 3 |
| 10.75 | 10.81 | 7.31 | 19.31 | 6.06 | (2), (3) | 7.75 | 10.00 | 1.25 | 0.75 | 7.88 | 0.88 | 4 |
| 13.75 | 13.31 | 8.81 | 23.31 | 7.56 | (2), (3) | 9.00 | 10.00 | 1.25 | 0.75 | 7.88 | 0.88 | 5 |
| 14.50 | 14.00 | 9.50 | 25.00 | 8.25 | (2), (3) | 9.75 | 12.50 | 1.44 | 1.00 | 10.62 | 0.88 | 6 |
| 17.00 | 16.56 | 11.06 | 30.06 | 9.31 | (2), (3) | 11.50 | 12.50 | 1.44 | 1.00 | 10.62 | 0.88 | 8 |
| 20.25 | 19.69 | 12.19 | 36.19 | 10.44 | (2), (3) | 13.75 | 15.00 | 1.62 | 1.12 | 13.00 | 1.00 | 10 |
| 23.00 | 22.56 | 13.56 | 41.06 | 12.31 | (2), (3) | 15.50 | 15.00 | 1.62 | 1.12 | 13.00 | 1.00 | 12 |
| 25.75 | 25.12 | 14.62 | 44.38 | 12.88 | (2), (3) | 17.25 | 17.50 | 1.88 | 1.25 | 15.25 | 1.12 | 14 |
| 28.25 | 27.69 | 16.69 | 48.69 | 15.19 | (2), (3) | 18.75 | 17.50 | 1.88 | 1.25 | 15.25 | 1.12 | 16 |

223

| 18 | 36.00 | 6.38 | 2.81 | 14.62 | 30.00 | 17.50 | 53.00 | 16.25 | 31.00 | 30.25 | 17.75 | 53.25 | 16.50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 38.75 | 7.00 | 3.12 | 16.38 | 32.50 | 18.50 | 57.50 | 17.50 | 33.50 | 32.75 | 18.75 | 57.75 | 17.75 |
| 24 | 46.00 | 8.00 | 3.72 | 19.62 | 38.00 | 20.50 | 67.00 | 20.25 | 39.25 | 38.75 | 20.75 | 67.25 | 20.50 |

ASME  B16.5-2020

###### Table E-4 Dimensions of Class 1500 Flanged Fittings (Cont’d)

1	2	3	4	5	6	7	8	9	10	11	12	13	14

| Inside Outside Minimum Diamete- Diameter Minimum Wall r Nominal of Thickness Thickness of Pipe Flange, of Flange, of Fitting, Fitting, Size O tf tm d | Flange Edge | Raised Face 0.25 in. (1) |
| --- | --- | --- |
|  | Center- to- Short Flange Center- Edge, to- Elbow, Center- Long Flange Tee, to- Center- Edge, Flange Cross, Flange to- Lateral, Edge and Edge, Flange and to Flange True 45-deg Edge, True Edge “Y,” Elbow, Lateral, “Y,” Reducer, A C E F G | Center- to- Short Contact Center- Surface to- of Center- Long Contact Raised to- Center- Surface Face, Contact to- of Elbow, Surface Contact Raised Tee, of Surface Face, Cross, Raised of Lateral, and Face, Raised and True 45-deg Face, True “Y,” Elbow, Lateral, “Y,” AA CC EE FF |

224

| 31.50 | 30.69 | 18.19 | 53.69 | 16.94 | (2), (3) | … | … | … | … | … | … | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34.00 | 33.19 | 19.19 | 58.19 | 18.19 | (2), (3) | … | … | … | … | … | … | 20 |
| 39.75 | 38.81 | 21.31 | 67.81 | 21.06 | (2), (3) | … | … | … | … | … | … | 24 |

ASME  B16.5-2020

###### Table E-4 Dimensions of Class 1500 Flanged Fittings (Cont’d)

15	16	17	18	19	20	21	22	23	24	25	26	1

| Raised Face 0.25 in. (1) | Ring Joint (1) |  |  |  |  | Diameter of Round Base Center- or Width Thickness to- of of Base, Square Base, Thickness R (4)– Base, T of Ribs, (6) S (4) (4), (7) U (4) | Base Drilling (8) |  | Nominal Pipe Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contact Surface- to- Contact Surface of Raised Face Reducer, GG (2) | Center- to- End Elbow, Tee, Cross, and True “Y,” HH (3) | Center- to- End, 45-deg Elbow, KK (3) | Long Center- to-End, Lateral, LL (3) | Short Center- to-End, Lateral, and True “Y,” MM (3) | End-to-End Reducer, NN |  | Bolt Circle or Bolt Spacing, W | Diameter of Drilled Holes |  |

225

ASME  B16.5-2020

###### Table E-4 Dimensions of Class 1500 Flanged Fittings  (Cont’d)

GENERAL NOTES:

              - Dimensions are in inches.

              - For tolerances, see section 7.

              - For facings, see para. 6.4.

              - For flange bolt holes, see para. 6.5 and Table 19C.

              - For spot facing, see para. 6.6.

              - For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4.

              - For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

              - For reinforcement of certain fittings, see para. 6.1.

              - For drains, see para. 6.12.

NOTES:

                - For center-to-contact surface and center-to-end dimensions of reducing fittings, see para. 6.2.3.

                - For contact surface-to-contact surface and end-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

                - These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For the center-to-end dimensions of reducing fittings or end-to- end dimensions of reducers, use center-to-flange edge or flange edge-to-flange edge dimensions for largest opening, and add the proper height to provide for ring joint groove applying to each flange. See Table 5C for ring joint facing dimensions.

                - The base dimensions apply to all straight and reducing sizes.

                - For reducing fittings, the size and center-to-face dimensions of the base are determined by the size of the largest opening fitting. In the case of reducing base elbows, orders shall specify whether the base shall be opposite the larger or smaller opening.

                - Bases shall be plain faced unless otherwise specified, and the center-to-base dimension R shall be the finished dimension.

                - Bases may be cast integral or attached as weldments at the option of the manufacturer.

                - The bases of these fittings are intended for support in compression and are not to be used for anchors or supports in tension or shear.

226

ASME  B16.5-2020

#### TABLE STARTS ON NEXT PAGE

227

ASME  B16.5-2020

###### Table E-5 Dimensions of Class 2500 Flanged Fittings

1	2	3	4	5	6	7	8	9	10

Flange Edge

Minimum

Inside

Center-to- Flange Edge,

Center-to-

Long Center-

Short Center- to-Flange Edge,

Flange Edge-to-

Outside

Minimum

Wall

Diameter

Elbow, Tee,

Flange Edge,  to-Flange

Lateral,

Flange

Nominal Pipe

Diameter of

Thickness of Flange,

Thickness of Fitting,

of Fitting,

Cross, and True “Y,”

45-deg Elbow,

Edge, Lateral,

and True “Y,”

Edge, Reducer,

Size	Flange, O	tf	tm	d	A	CE	E	F	G

1∕2	5.25	1.19	0.25	0.44	4.94	…	…	…	…

3∕4	5.50	1.25	0.28	0.56	5.12	…	…	…	…

| 1 | 6.25 | 1.38 | 0.34 | 0.75 | 5.81 | 3.75 | … | … | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11∕4 | 7.25 | 1.50 | 0.44 | 1.00 | 6.62 | 4.00 | … | … | … |
| 11∕2 | 8.00 | 1.75 | 0.50 | 1.12 | 7.31 | 4.50 | … | … | … |
| 2 | 9.25 | 2.00 | 0.62 | 1.50 | 8.62 | 5.50 | 15.00 | 5.00 | 9.00 |
| 21∕2 | 10.50 | 2.25 | 0.75 | 1.88 | 9.75 | 6.00 | 17.00 | 5.50 | 10.00 |
| 3 | 12.00 | 2.62 | 0.88 | 2.25 | 11.12 | 7.00 | 19.50 | 6.50 | 11.25 |
| 4 | 14.00 | 3.00 | 1.09 | 2.88 | 13.00 | 8.25 | 22.75 | 7.50 | 13.00 |
| 5 | 16.50 | 3.62 | 1.34 | 3.62 | 15.38 | 9.75 | 27.00 | 9.00 | 15.25 |
| 6 | 19.00 | 4.25 | 1.59 | 4.38 | 17.75 | 11.25 | 31.00 | 10.25 | 17.50 |
| 8 | 21.75 | 5.00 | 2.06 | 5.75 | 19.88 | 12.50 | 35.00 | 11.50 | 20.00 |
| 10 | 26.50 | 6.50 | 2.59 | 7.25 | 24.75 | 15.75 | 43.00 | 14.50 | 25.00 |
| 12 | 30.00 | 7.25 | 3.03 | 8.62 | 27.75 | 17.50 | 49.00 | 16.00 | 28.50 |

228

ASME  B16.5-2020

###### Table E-5 Dimensions of Class 2500 Flanged Fittings

11	12	13	14	15	16	17	18	19	20	1

Raised Face 0.25 in. (1)	Ring Joint (1)

Short Center-

Center-to- Contact Surface of Raised Face Elbow, Tee,

Cross, and True “Y,” AA

Center- to- Contact Surface of Raised Face

45-deg Elbow, CC

Long Center- to- Contact Surface of Raised Face Lateral, EE

to- Contact Surface of Raised Face Lateral and

True “Y,”

FF

Contact Surface- to- Contact Surface of Raised Face

Reducer,

GG (2)

Center-to- End, Elbow, Tee, Cross,

and True “Y,”

HH (3)

Center-to- End, 45-

deg Elbow, KK (3)

Long Center- to-End Lateral, LL (3)

Short Center- to-End Lateral and

True “Y,”

MM (3)

End-to- End Reducer, NN

(2), (3)

Nominal Pipe Size

5.19	…	…	…	…	5.19	…	…	…	…	1∕2

5.37	…	…	…	…	5.19	…	…	…	…	3∕4

| 6.06 | 4.00 | … | … | … | 6.06 | 4.00 | … | … | … | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.87 | 4.25 | … | … | … | 6.94 | 4.31 | … | … | … | 11∕4 |
| 7.56 | 4.75 | … | … | … | 7.62 | 4.81 | … | … | … | 11∕2 |
| 8.87 | 5.75 | 15.25 | 5.25 | 9.50 | 8.94 | 5.81 | 15.31 | 5.31 | … | 2 |
| 10.00 | 6.25 | 17.25 | 5.75 | 10.50 | 10.12 | 6.38 | 17.38 | 5.88 | … | 21∕2 |
| 11.37 | 7.25 | 19.75 | 6.75 | 11.75 | 11.50 | 7.38 | 19.88 | 6.88 | … | 3 |
| 13.25 | 8.50 | 23.00 | 7.75 | 13.50 | 13.44 | 8.69 | 23.19 | 7.94 | … | 4 |
| 15.62 | 10.00 | 27.25 | 9.25 | 15.75 | 15.88 | 10.25 | 27.50 | 9.50 | … | 5 |
| 18.00 | 11.50 | 31.25 | 10.50 | 18.00 | 18.25 | 11.75 | 31.50 | 10.75 | … | 6 |
| 20.12 | 12.75 | 35.25 | 11.75 | 20.50 | 20.44 | 13.06 | 35.56 | 12.06 | … | 8 |
| 25.00 | 16.00 | 43.25 | 14.75 | 25.50 | 25.44 | 16.44 | 43.69 | 15.19 | … | 10 |
| 28.00 | 17.75 | 49.25 | 16.25 | 29.00 | 28.44 | 18.19 | 49.69 | 16.62 | … | 12 |

229

ASME  B16.5-2020

###### Table E-5 Dimensions of Class 2500 Flanged Fittings (Cont’d)

GENERAL NOTES:

(a)  Dimensions are in inches.

(b)  For tolerances, see section 7.

(c)  For facings, see para. 6.4.

(d)  For flange bolt holes, see para. 6.5 and Table 21C.

(e)  For spot facing, see para. 6.6.

(f)  For intersecting centerlines, center-to-contact surface, and center-to-end dimensions of side outlet fittings, see para. 6.2.4.

(g)  For center-to-contact surface and center-to-end dimensions of special degree elbows, see para. 6.2.5.

(h) For reinforcement of certain fittings, see para. 6.1.

(i)   For drains, see para. 6.12.

NOTES:

(1) For center-to-contact surface and center-to-end dimensions of reducing fittings, see para. 6.2.3.

(2)  For contact surface-to-contact surface and end-to-end dimensions of reducers and eccentric reducers, see para. 6.2.3.

(3) These dimensions apply to straight sizes only (see paras. 6.2.3 and 6.4.2.2). For the center-to-end dimensions of reducing fittings or end-to- end dimensions of reducers, use center-to-flange edge or flange edge-to-flange edge dimensions for largest opening, and add the proper height to provide for ring joint groove applying to each flange. See Table 5C for ring joint facing dimensions.

230

ASME  B16.5-2020

## NONMANDATORY APPENDIX F

METHOD USED TO ESTABLISH SI UNIT VALUES	ð20Þ

##### INTRODUCTION

The 2003 edition of ASME B16.5 contained dimensions expressed in millimeters and pressure–temperature ratings expressed in bar-Celsius, with U.S. Customary values enclosed in parentheses or listed separately from the SI values. This Appendix describes how the SI unit values were established at that time, then subse- quently modified for the 2020 edition.

##### SI VALUES FOR THE 2003 EDITION

The B16 Subcommittee C had two primary goals during the development of the SI values shown in ASME B16.5- 2003.

    - The  dimensions  expressed  in  millimeters should

reflect the needed precision as closely as possible.

    - Flanges manufactured using existing forging dies and machinery settings based on the inch dimensions should be able to meet the requirements for the millimeter dimensions.

##### F-2.1 Conversion From Fractional Inches

Before 1977, ASME B16.5 dimensions were expressed mostly in fractional inches instead of decimal inches. For the 1977 edition, the fractional-inch values were converted to decimal-inch values. For example, 1∕8 in. was shown as 0.12 in. or 0.125 in., depending on the intended precision of the dimension.

Also, in the 1977 edition, millimeter dimensions were converted from the original fractional-inch dimensions rather than the decimal-inch dimensions. For example, 1∕16 in. (0.0625 in.) was converted to 1.6 mm and 0.06 in. wherever the intended precision was 0.01 in. However, if one converts 0.06 in. to millimeters, one gets 1.5 mm, not 1.6 mm. Thus, to a user converting decimal inches to millimeters, some of the conversions in the 2003 tables would appear to be incorrect.

In the 1977 edition, the dimension 1∕16 in. was some-

times converted to the nearest 0.1 mm, sometimes to

the nearest 0.5 mm, and sometimes to the nearest 1 mm. The conversion depended on the intended precision of the measurement. So, the millimeter equivalent for 0.06 in. was sometimes shown as 1.6 mm, sometimes as 1.5 mm, and sometimes as 2 mm.

##### F-2.2 Toleranced Dimensions

Dimensions that ensure adequate fit-up and contribute to the integrity of the pressurized flange joint require tolerances. In the 2003 edition, these toleranced dimen- sions were converted such that the millimeter dimensions were essentially the same as the fractional-inch dimen- sions, and the tolerances were selected such that the permitted deviations from the tabulated dimensions were nearly identical to those permitted by the inch dimensions. Examples include the following:

- Bolt circle diameter was converted to the nearest

0.1 mm. This level of precision is needed to minimize prob- lems with fit-up to other flanges, even though the toler- ance on the dimension is 1.5 mm. Converting with less precision may cause additional problems with centering metal gaskets.

- Length through hub was converted to the nearest

1 mm. This dimension needs to be consistent to maintain overall dimensions for fabricated spools. Maintaining this dimension to the nearest whole millimeter provides the needed precision.

##### F-2.3 Untoleranced Dimensions

Dimensions that do not contribute significantly to proper fit-up or to the integrity of the pressurized flanged joint do not require tolerances. The following are examples of untoleranced dimensions and the philo- sophies used to convert them to the millimeter dimen- sions:

- A 1∕16-in. raised face was converted to 2 mm instead

of 1.6 mm. Raised faces measuring other than 2 mm meet

the requirements of the Standard. Conversion to the nearest millimeter reflects the intended precision of the dimension.

- Outside diameter of flanges was converted to the nearest 5 mm. For example, for NPS 3∕4 Class 600 flange outside diameter, 45∕8 in. was converted to the nearest 5 mm (115 mm) instead of the nearest whole millimeter (117 mm) or tenth millimeter (117.5 mm). Outside diameters measuring other than 115 mm meet the re-

quirements of the Standard. Conversion to the nearest 5 mm reflects the intended precision of the dimension.

- Bolt hole diameters were kept in fractional inches.

Inch-dimension bolt holes were retained for flanges manufactured to millimeter dimensions. Inch bolts

231

ASME  B16.5-2020

should be used with these flanges. Extensive dimensional compatibility studies were conducted to explore the possi- bility of using millimeter- as well as inch-dimensioned bolting with ASME B16.5 flanges. The studies revealed that providing dimensions that allowed for the use of milli- meter as well as inch-dimensioned bolts, especially when combined with metal gaskets, was impossible. This conclusion was supported by experience with some flanges manufactured to the 1980 edition of ISO 7005- 1: Metallic flanges — Part 1: Steel flanges.

##### F-2.4 In Summary

The B16 Subcommittee C did not intentionally change any of the requirements for dimensions in the 2003 edition of ASME B16.5. The dimensions expressed in milli- meters reflected the needed precision as closely as possible.

Flanges that were manufactured using existing forging dies and machinery settings based on the inch dimensions were able to meet the 2003 requirements for the milli- meter dimensions. The acceptable dimension ranges for toleranced dimensions were not precisely the same in the two units of measure, but there was a significant amount of overlap. Still, it was possible for a flange to meet the requirements in one system of units and not in the other.

##### SI VALUES FOR THE 2020 EDITION

While the differences in inch and millimeter dimensions between the two versions of the same flange did not result in any known functional problem, the differences did confuse manufacturers and users alike. For example, since the precision of conversion of the flange outside diameter was to the nearest 5 mm, there sometimes was a difference of 2 mm to 3 mm between mating- flange outside diameters. This introduced a practical problem for users who welded attachments to mating flanges, and it also created an appearance problem. Engi- neers were confused when comparing inch and millimeter dimensions in the tables when the dimensions didn't match. They didn't know which dimension to use.

To address these problems, ASME B16.5-2020 features millimeter dimensions that have been converted from the decimal-inch dimensions rather than the fractional-inch dimensions and the conversions are more precise. The following are examples of changes in the 2020 edition:

    - Outside diameters of flanges have been converted

to nearest 1 mm instead of the nearest 5 mm.

    - Raised face heights have been converted from 0.06 in. to 1.5 mm and from 0.25 in. to 6.4 mm instead of from

0.06 in. to 2 mm and from 0.25 in. to 7 mm.

    - Inch tolerances have been revised to more closely match  the  millimeter  tolerances.

232

# ASME B16.5-2020